3GPP TSG-SA5 Meeting #123 	S5-191288

# Montreal,Canada, 21-25 January 2019

# Source:	ITU-T

Title:	Draft new Recommendation E.RQST – KPI targets for mobile networks

# Document for:	Information, Discussion

Agenda Item:	5.3

|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG12-TD605R1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 12 |  |  |  |
|  |  |  | Original: English |  |  |  |
| Question(s): |  | 12/12 |  | Geneva, 27 November - 6 December 2018 |  |  |
| TD |  |  |  |  |  |  |
| Source: |  | Editors E.RQST |  |  |  |  |
| Title: |  | Draft new Recommendation E.RQST – KPI targets for mobile networks |  |  |  |  |
| Purpose: |  | Discussion |  |  |  |  |
| Contact: |  | Nicholas Jatta PURA Gambia |  |  |  | Tel: +220 767 7777 Email: nic@pura.gm |
| Contact: |  | Nii Ayitey Komey NCA Ghana |  |  | Tel: +233 266000882 Email: nii.komey@nca.org.gh |  |

This TD contains the draft text for Recommendation E.RQST “KPI targets for mobile networks”.

The document considers the comments received during the Rapporteur Group meeting of Q12/12 held in Istanbul, Turkey on 6-7 September 2018.

R1 reflects the Q12/12 discussions on the base text (TD605), and on Contributions 267 and 271R1 in Geneva, 27 November – 6 December 2018.

Draft new Recommendation E.RQST – KPI targets for mobile networks

Summary

WTSA Resolution 95 (Hammamet, 2016) instructs SG 12 to provide references relating to minimal satisfactory key performance and key quality indicators for evaluating the quality of services, as well as to assist developing and least developed countries in elaborating and implementing actions to improve service quality and keep users informed.

Aiming to achieve these clear directives from WTSA-16, this draft recommendation presents a framework that stakeholders and ICT regulators in particular can use as benchmarking when monitoring and enforcing network and services performance on mobile networks.

Keywords

KPI, mobile network, GSM, UMTS, LTE

# 1	Scope

This recommendation presents a framework that stakeholders and ICT regulators in particular can use as reference for proposed targets when monitoring and enforcing network and services performance on mobile networks.

This draft recommendation is focusing on proposing thresholds that could be used to enforce compliance to KPIs.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	 ITU-T E.804 Series, Quality of Telecommunication services: concepts, models, objectives and dependability planning- Terms and definitions related to the quality of Telecommunication services. QoS aspects for popular services in mobile networks, 2014

[2]	ITU-T Quality of Service Regulations Manual, 2017

[3]	ITU-T E.600 Series, Overall Network Operation, Telephone Service, Service Operation and Human Factors. Traffic engineering – Definitions, 2007

[4]	ETSI TS 132 410, V15, Digital Cellular Telecommunications Systems (GSM); UMTS; Telecommunications Management; Key Performance Indicators (KPI) for UMTS and GSM, 2018

[5]        ETSI TS 132 450, V11, Universal Mobile Telecommunications System (UMTS); LTE; Telecommunications Management; Key Performance Indicators (KPIs) for Evolved Universal Terrestrial Radio Access (E-UTRAN): Definitions, 2012

[6]	ETSI EG 202 057 part 3

[7]	ETSI TS 102 250 part 2, 4 & 7

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

GSM	Global Standards for Mobile

UMTS	Universal Mobile Terrestrial Systems

UE	User Equipment

FTP	File Transfer Protocol

PDP	Packet Data Protocol

KPI	Key Performance Indicator

EPS	Evolved Packet System

QoS	Quality of Service

CS	Circuit Switch

PS	Packet Switch

ICMP	Internet Control Message Protocol

# 5	Conventions

“None”

# 6	Introduction

ITU-T Rec. E.800 defines quality of service as; Totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service. ETSI, in a technical report1 (ETSI-TR102157) also defined QoS from a network performance perspective as the ability to segment traffic or differentiate between traffic types in order for the network to treat certain traffic differently from others.

The various definitions have different perspectives, the ITU definition focusing on the service as the entity under consideration whilst ETSI from the network perspective. This tends to reflect views on the telecommunications systems, network, nodes and services/applications from network and user perspectives.

Ideally, QoS is to be addressed from the end-user perspective; however, there are several factors, be it objective or subjective that influences the perception. In any QoS framework the QoS offered is crucial in assessing quality of service delivery. As indicated in ITU-T Rec. G.1000, QoS offered by

the service provider is a statement of the level of quality expected to be offered to the customer by the service provider. The level of quality is expressed by values or thresholds assigned to QoS

parameters which serve as benchmarks by which the quality of service delivery may be judged.

It is in this regard that this draft recommendation is being made to be used as baseline for KPI thresholds for mobile services on GSM/UMTS/LTE networks. These proposed thresholds will become relevant matrix for benchmarking service providers and comparing their achieved QoS vis a vis offered QoS.

This document collates various QoS regulations /regimes from across the world and derives a target that will be applicable for the KPIs. This is simply a one stop document that puts together KPIs and their desired targets.

# 7	KPIs for GSM, UMTS and LTE networks.

The following sections have list of proposed KPIs targets for mobile voice and data services. It is worth noting that targets indicated in this document are proposals and should be set by the regulator in consultation with stakeholders (ie Service Providers) prior to its implementation.  According to the ITU’s Quality of Service Regulations Manual, Key Performance Indicators (KPIs) are based on network counters and are essential for operation and maintenance, and for business models which can help in other actions such as report and auditing. KPIs can be formulated with implied or specific targets.

Figure 1 below shows a model for quality of service parameters in mobile networks, which have four layers. The first layer, network availability, defines QoS from the viewpoint of the service provider rather than the service user. The second layer, network access, is the basic requirement for all other QoS aspects and parameters from the service user point of view. The third layer contains the other three QoS aspects, service access, service integrity, and service retainability. The different services are located in the fourth layer. Their outcomes are the QoS parameters

All listed KPIs following figure 1 below are related to the first three layers and their measurements/computation can be obtained from one or more raw measurements taken from specific counters or field measurements.

Fig.1 QoS aspects for mobile networks (ITU-T Rec. E804)

Setting of the targets in this document was guided by industry best practice values, operators’/systems capabilities and consumer’s latent requirement for network improvement

It must be emphasised that targets set in this document are subject change, depending on the jurisdiction’s specific circumstances. Regulators and operators need to choose those that match their main aims Parties might even wish to set targets in areas that are not listed in this document in order to address their national or local problems. Targets should be tailored to parties (which include consumers) needs, Service Providers capacities, existing regulation and average industry performance.

8 KPI Selection and Recommended Targets

Editor’s note: content to be provided in subsequent revisions

8.1 Target Setting

Editor’s note: content to be provided in subsequent revisions

## 8.2	GSM KPIs

| Category | Key Performance Indicators (KPIs) | Definition | Measurement Approach | Aggregation Level | Proposed Thresholds |
| --- | --- | --- | --- | --- | --- |
| Accessibility | Stand-Alone Dedicated Control Channel (SDCCH) Assignment Success Rate | Refer to 5.6 of 3GPP TR 32.814 V7 |  |  | ≥98% |
|  | Stand-Alone Dedicated Control Channel (SDCCH) Drop Rate | Indicates the probability of an SDCCH abnormally released after the UE has seized it and signalling ongoing |  |  | ≤1% |
|  | Stand-Alone Dedicated Control Channel (SDCCH) Congestion | Refer to 5.7 of 3GPP TR 32.814 V7 |  |  | ≤1% |
|  | Traffic Channel (TCH) Assignment Success Rate | The rate that the TCHs assigned are successfully seized by the UE to make calls. |  |  | ≥98% |
|  | Traffic Channel (TCH) Congestion | Refer to 5.8 of 3GPP TR 32.814 V7 |  |  | ≤1% |
|  | Call Setup Success Rate | Refer to 5.5 of 3GPP TR 32.814 V7 |  |  | ≥98% |
| Retainability | Traffic Channel (TCH) Drop Rate | Refer to 5.3 of 3GPP TR 32.814 V7 |  |  | ≤1% |
|  | Handover Success Rate | Refer to 5.4 of 3GPP TR 32.814 V7 |  |  | ≥95% |
| Accessibility | Call Setup Time | Is the period of time elapsing from the sending of a complete destination address (ie target telephone number) to setting up of a call to the receiving terminal |  |  | ≤10sec |
|  | PDP Context Activation Success Rate | Refer to 7.5 of ETSI TS 132 410 V15 |  |  | ≥98% |
| Service Integrity | Minimum Data Speed | This is the minimum TCH encoding rate for data (Half-Rate channel, 4.8kbps: GSM Specification 06.20 |  |  | Must meet the minimum speed specified in the 3GPP standards for GPRS and EDGE |

## 8.3	UMTS KPIs

| Category | Key Performance Indicators (KPIs) | Definition | Measurement Approach | Aggregation Level | Proposed Thresholds |
| --- | --- | --- | --- | --- | --- |
| Accessibility | Circuit Switch (CS) Radio Resource Control (RRC) Setup Success Rate | Refer to 7.2 of ETSI TS 132 410 V15 |  |  | ≥98% |
|  | Radio Access Bearer (RAB) Establishment Success Rate for Circuit Switch (CS) | Refer to 7.1.1 of ETSI TS 132 410 V15 |  |  | ≥98% |
|  | Radio Access Bearer (RAB) Establishment Success Rate for Packet Switch (PS) | Refer to 10.3 of ETSI TS 132 410 V15 |  |  | ≥98% |
| Retainability | Radio Access Bearer (RAB) Drop Rate | The ratio of number of RAB abnormal releases to the successful RAB establishments |  |  | ### ≤1% |
|  | Soft Handover Success Rate | Describes the ratio of number of successful radio link additions to the total number of radio link addition attempts |  |  | ≥98% |
|  | Circuit Switch (CS) Inter RAT Handover Success Rate | The ratio of number of successful inter RAT handover to the total number of the attempted inter RAT handover from UMTS to GSM for CS (voice) domain |  |  | ≥96% |
|  | Packet Switch (PS) Inter RAT Handover Success Rate | The ratio of number of successful inter RAT handover to the total number of the attempted inter RAT handover from UMTS to GSM for PS domain |  |  | ≥96% |
| Integrity | Downlink (DL) Throughput | Payload data volume per elapsed time on air interface |  |  | Must meet the minimum speed specified in the 3GPP / International Mobile Telecommunications standards |

## 8.4 LTE KPIs

| Category | Key Performance Indicators (KPIs) | Definition | Measurement Approach | Aggregation Level | Proposed Thresholds |
| --- | --- | --- | --- | --- | --- |
| Retainability | Radio Resource Control (RRC) Setup Success Rate | The ratio of all successful RRC establishments to RRC establishment attempts |  |  | ≥98% |
|  | (Evolved Packet System) EPS Bearer Activation Success Rate | Describes the ratio of successfully established EPS bearer to EPS bearer activation attempts. |  |  | ≥98% |
|  | E-UTRAN Radio Access Bearer (E-RAB) Establishment Success Rate | Probability of successful E-RAB establishments compared with total number of attempts for the different parts of the E-RAB establishment. |  |  | ≥98% |
|  | E-UTRAN Radio Access Bearer (E-RAB) Drop Rate | The ratio of abnormally released E-RAB with data in any of the buffers to the total established E-RABs |  |  | ≤1% |
| Integrity | Latency | Time from reception of IP packet to transmission of first packet over the Uu |  |  | <100ms |
| Integrity | Downlink (DL) Throughput | Payload data volume per elapsed time on Uu interface |  |  | Must meet the minimum speed specified in the 3GPP / International Mobile Telecommunications standards |

## Editor’s note:

## The tables in section 8 above will include a third new column with name ‘link to QoS as perceived by end users’

## Editor’s Note:

The targets were set by adapting values determined through earlier measurements by operators and operators in other countries or proposed international standards. The targets relate to the quality levels customers want, which might change as quality improves and vary for different market segments in different geographic areas. This document does not pretend to say what regulators must (always) do. However, it does suggest the targets set should be useful to customers and realistic for operators if they are to be effective

APPENDIX

Quality of Service Key Performance Indicators (KPIs) for various Jurisdictions

Regulations and QoS standards from five National Regulatory Authorities; namely National Communications Authority (NCA) –Ghana, Communications Authority (CA) – Kenya, National Communications Commission (NCC)- Nigeria, Independent Communications Authority for South Africa (ICASA) and Telecom Regulatory Authority of India were considered in setting the targets for KPIs

KPIs and target values in Kenya

Source: https://www.itu.int/dms_pub/itu-d/opb/pref/D-PREF-BB.QOS_REG01-2017-PDF-E.pdf

KPIs and target values in India

Source: https://www.itu.int/dms_pub/itu-d/opb/pref/D-PREF-BB.QOS_REG01-2017-PDF-E.pdf

KPI and target values in Ghana

Source: https://www.itu.int/dms_pub/itu-d/opb/pref/D-PREF-BB.QOS_REG01-2017-PDF-E.pdf

KPIs and target values in Nigeria

Source: https://www.ncc.gov.ng/docman-main/legal-regulatory/regulations/731-legal-regulations-quality-of-service-regulation-2013/file

KPI and target values in South Africa

Source: https://www.icasa.

_______________________