3GPP TSG-SA5 Meeting #124 	S5-192277

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192043

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0702 | rev | 1 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support status of VoLTE service delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | VBCLTE |  |  |  |  |  | Date: |  |  | 2019-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PCNs cannot collect accounting information to determine the status of VoLTE service delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce two new reasons of Cause for Record Closing field in the CDR to indicate the status of VoLTE service delivery. - VoLTE bearer normal release. - VoLTE bearer abnormal release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Volume based charging for VoLTE is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... TS32.299 CR 0820 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.2.2.5	Cause for Record Closing

This field contains a reason for the release of the CDR. In case of Rf interface is used, it is derived from Change-Condition AVP at PS-information AVP level defined in TS 32.299 [40], when received. The following is included:

-	normal release: IP-CAN bearer release or detach, TDF session release, IP-CAN session release for IP-Edge; It corresponds to "Normal Release" in Change-Condition AVP.

-	data volume limit; It corresponds to "Volume Limit" in Change-Condition AVP.

-	time (duration) limit; It corresponds to "Time Limit" in Change-Condition AVP.

-	maximum number of changes in charging conditions; It corresponds to "Max Number of Changes in Charging conditions " in Change-Condition AVP.

-	For SGSN: intra SGSN intersystem change (change of radio interface from GSM to UMTS or vice versa);

-	For P-GW, TDF and S-GW: Radio Access Technology (RAT) change; It corresponds to "RAT Change" in Change-Condition AVP.

-	abnormal termination (IP-CAN bearer or MM context or TDF session, or IP-CAN session); It corresponds to "Abnormal Release" in Change-Condition AVP.

-	For SGSN: SGSN change;

-	For S-GW, ePDG, TWAG: S-GW change; It corresponds to "S-GW Change" in Change-Condition AVP and is used for inter serving node change of all types.

-	Timezone change; It corresponds to "UE TimeZone Change" in Change-Condition AVP.

-	SGSN or S-GW PLMN change; It corresponds to "Serving Node PLMN Change" in Change-Condition  AVP.

-	For P-GW: APN-AMBR change: It corresponds to "Qos Change" in Change-Condition AVP.

-	For S-GW, P-GW: MO exception data counter receipt: It corresponds to " MO exception data counter receipt" in Change-Condition AVP.

-	unauthorized network originating a location service request;

-	unauthorized client requesting a location service;

-	position method failure at a location service execution;

-	unknown or unreachable LCS client at a location service request;

-	management intervention (request due to O&M reasons); It corresponds to "Management Intervention" in Change-Condition AVP;

-	VoLTE bearer normal release: It corresponds to "VoLTE Bearer Normal Release" in Change-Condition AVP.

-	VoLTE bearer abnormal release: It corresponds to "VoLTE Bearer Abnormal Release" in Change-Condition AVP.

A more detailed reason may be found in the diagnostics field or Enhanced Diagnostics field.

End of changes