3GPP TSG-SA5 Meeting #124 	S5-192460d1

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192338

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 002 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the DN to URI mapping rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanhai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The rules for mapping a DN to a URI and vice versa are umbigious and incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The rules for mapping a DN to a URI and vice versa are completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DNs cannot be mappded to URIs and vice versa. This mapping is essential for the network management to work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.1, 4.2.1a (new), 4.2.2, 4.2.3, 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CRUD	Create, Retrieve, Update, Delete

DC	Domain Component

DN	Distinguished Name

DNS	Domain Name Service

FQDN	Fully Qualified Doman Name

HTTP	Hypertext Transfer Protocol

JSON	JavaScript Object Notation

LDN	Local Distinguished Name

MnS	Management Service

REST	REpresentational State Transfer

RPC	Remote Procedure Call

TCP	Transmission Control Protocol

URI	Uniform Resource Identifier

End of 1st modified section

2nd modified section

### 4.2.1	Managed object naming

The Distinguished Name (DN) is used in 3GPP to uniquely identify a managed object instance within a specific name space. The DN is a comma (",") separated list  of Relative Distinguished Names (RDNs). Each managed object instance has an associated RDN. The sequence of RDNs is governed by name containment relationships in the UML class diagram describing the modelled network. The RDN consists of a naming attribute name separated by an equal sign ("=") from the naming attribute value. The naming attribute name is equal to the class name of the MOI

In addition to the RDNs associated to a managed object instance the DN may have as leftmost RDN whose naming attribute name is "DC" (Domain Component) and whose value is a domain name. A DN with DC is globally unique.

The DN concept is described in detail in TS 32.300 [3]

The following example DN has a DC.

DN = "DC=operatorA.com,subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

### 4.2.1a	Global and local namespaces

A DN in the global name space is globally unique and starts with the RDN of the global root, which is the DC. A DN in a local name space starts with the RDN of the local root and is unique only within this name space. A DN in a local namespace is also referred to as Local Distinguished Name (LDN). The DN of the local root relative to the global root is called DN prefix. The concatenation of DN prefix and LDN is equal to the globally unique DN of a managed object instance.

For example, the (local) root of an object diagram representing a manged element only is the object representing the managed element itself.

LDN = "managedElement=a,eNBFunction=1,cell=1"

DN-prefix = "DC=operatorA.com,subNetwork=south"

The (local) root of an object diagram representing a subnetwork is the object representing the subnetwork.

LDN = "subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

DN-prefix = "DC=operatorA.com"

### 4.2.2	Resource identification

HTTP uses a subset of the generic Uniform Resource Identifier (URI) scheme (RFC 3986 [4]) defined in RFC 7230 [5] for target resource identification.

http-URI = "http:" "//" authority path-abempty [ "?" query ] [ "#" fragment ]

The path component is an absolute path (one that starts with a single slash character) or empty.

The origin server is identified by the authority component, which includes a host identifier and an optional path TCP port. The hierarchical path component and optional query component serve as an identifier for a potential target resource within that origin server’s name space. The optional fragment component allows for indirect identification of a secondary resource.

The host identifier is either an IP address or an indirect identifier such as a FQDN to be resolved with DNS.

### 4.2.3	Mapping of DNs to URIs

URIs are globally unique. For this reason only a globally unique DN with DC is mappable into a URI. The mapping rules are as follow:

The DN prefix is mapped semantically to the authority component of the URI. The syntax of the DN prefix is modified to match the syntax of the authority component.

The LDN is mapped semantically to the path component of the URI. The syntax of the LDN is modified to match the syntax of the path component.

When mapping a LDN the equal sign "=" shall be used as delineator between the naming attribute name and naming attribute value when constructing a RDN.

URI-RDN = {namingAttributeName} "=" {namingAttributeValue}

The URI-LDN is the concatenation of URI-RDNs separated by a slash "/".

URI-LDN = *( "/" RDN )

For example, the LDN

LDN = "subNetwork=south,managedElement=a,eNBFunction=1,cell=1"

maps to

URI-LDN = "/subNetwork=south/managedElement=a/eNBFunction=1/cell=1"

and the LDN

LDN = "managedElement=a,eNBFunction=1,cell=1"

to

URI-LDN = "/managedElement=a/eNBFunction=1/cell=1"

When constructing the authority part from the DN prefix, it shall be reformatted according to the name conventions applying to FQDNs. For example, the DN prefix

DN-prefix = "DC=operatorA.com"

maps to

URI-DN-prefix = "operatorA.com"

and the DN prefix

DN-prefix = "DC=operatorA.com,subNetwork=south"

to

URI-DN-prefix = "south.subNetwork.operatorA.com"

The complete URIs for the examples are

http://operatorA.com/subNetwork=south/managedElement=a/eNBFunction=1/cell=1

http://south.subNetwork.operatorA.com/managedElement=a/eNBFunction=1/cell=1

The constructed URI-DN-prefix is a FQDN that can be registered into a name resolution service such as DNS. The sole presence of a constructed FQDN does not mean it can be resolved to an IP address and there is a server listening at that address.

Note that one DN can be mapped to one or more URIs.

The leftmost part of the path component may include one or more path segments ("label")

http://operatorA.com/{label}/subNetwork=south/.../cell=1

allowing to structure the resource hierarchy, for example

http://operatorA.com/3GPPmanagemen/ProvMnS/v1500/subNetwork=south/.../cell=1

The character set allowed in DNs is much bigger than the character set allowed in the path component and authority component of a URI. Care needs to be taken when selecting the naming attribute names und values that the mapping from a DN to a URI does not become impossible as a consequence of not mappable characters.

End of 2nd modified section

3rd modified section

## 4.4	URI structure

URIs identifying resources representing managed object instances shall follow a common structure given by

http://{URI-DN-PREFIX}/{root}/{MnSName}/{MnSVersion}/{URI-LDN}

where:

{URI-DN-PREFIX}		indicates the authority part of the URI constructed from the DN prefix.

{root}				indicates an optional root for structuring the resource hierarchy.

{MnSName}				indicates the MnS name.

{MnSVersion}				indicates the version of the MnS.

{URI-LDN}				indicates the resource path constructed from the LDN.

End of 3rd modified section