3GPP TS 28.541 V16.0.0 (2019-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Management and orchestration;

5G Network Resource Model (NRM);

Stage 2 and stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Management, resource,model,orchestration,5G

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	15

Introduction	15

1	Scope	16

2	References	16

3	Definitions and abbreviations	18

3.1	Definitions	18

3.2	Abbreviations	18

4	Information model definitions for NR NRM	19

4.1	Imported and associated information	19

4.1.1	Imported information entities and local labels	19

4.1.2	Associated information entities and local labels	19

4.2	Class diagram	19

4.2.1	Class diagram for gNB and en-gNB	19

4.2.1.1	Relationships	19

4.2.1.2	Inheritance	21

4.3	Class definitions	22

4.3.1	GNBDUFunction	22

4.3.1.1	Definition	22

4.3.1.2	Attributes	23

4.3.1.3	Attribute constraints	23

4.3.1.4	Notifications	23

4.3.2	GNBCUCPFunction	23

4.3.2.1	Definition	23

4.3.2.2	Attributes	23

4.3.2.3	Attribute constraints	23

4.3.2.4	Notifications	24

4.3.3	GNBCUUPFunction	24

4.3.3.1	Definition	24

4.3.3.2	Attributes	24

4.3.3.3	Attribute constraints	24

4.3.3.4	Notifications	24

4.3.4	NRCellCU	24

4.3.4.1	Definition	24

4.3.4.2	Attributes	25

4.3.4.3	Attribute constraints	25

4.3.4.4	Notifications	25

4.3.5	NRCellDU	25

4.3.5.1	Definition	25

4.3.5.2	Attributes	26

4.3.5.3	Attribute constraints	26

4.3.5.4	Notifications	26

4.3.6	NRSectorCarrier	26

4.3.6.1	Definition	26

4.3.6.2	Attributes	27

4.3.6.3	Attribute constraints	27

4.3.6.4	Notifications	27

4.3.7	BWP	27

4.3.7.1	Definition	27

4.3.7.2	Attributes	27

4.3.7.3	Attribute constraints	27

4.3.7.4	Notifications	27

4.3.8	EP_E1	28

4.3.8.1	Definition	28

4.3.8.2	Attributes	28

4.3.8.3	Attribute constraints	28

4.3.8.4	Notifications	28

4.3.9	EP_XnU	28

4.3.9.1	Definition	28

4.3.9.2	Attributes	28

4.3.9.3	Attribute constraints	28

4.3.9.4	Notifications	28

4.3.10	EP_NgC	28

4.3.10.1	Definition	28

4.3.10.2	Attributes	29

4.3.10.3	Attribute constraints	29

4.3.10.4	Notifications	29

4.3.11	EP_NgU	29

4.3.11.1	Definition	29

4.3.11.2	Attributes	29

4.3.11.3	Attribute constraints	29

4.3.11.4	Notifications	29

4.3.12	EP_F1C	29

4.3.12.1	Definition	29

4.3.12.2	Attributes	30

4.3.12.3	Attribute constraints	30

4.3.12.4	Notifications	30

4.3.13	EP_F1U	30

4.3.13.1	Definition	30

4.3.13.2	Attributes	30

4.3.13.3	Attribute constraints	30

4.3.13.4	Notifications	30

4.3.14	EP_S1U	30

4.3.14.1	Definition	30

4.3.14.2	Attributes	30

4.3.14.3	Attribute constraints	30

4.3.14.4	Notifications	31

4.3.15	EP_X2C	31

4.3.15.1	Definition	31

4.3.15.2	Attributes	31

4.3.15.3	Attribute constraints	31

4.3.15.4	Notifications	31

4.3.16	EP_X2U	31

4.3.16.1	Definition	31

4.3.16.2	Attributes	31

4.3.16.3	Attribute constraints	31

4.3.16.4	Notifications	31

4.3.17	EP_XnC	31

4.3.17.1	Definition	31

4.3.17.2	Attributes	32

4.3.17.3	Attribute constraints	32

4.3.17.4	Notifications	32

4.3.18	ExternalGNBCUCPFunction	32

4.3.18.1	Definition	32

4.3.18.2	Attributes	32

4.3.18.3	Attribute constraints	32

4.3.18.4	Notifications	32

4.3.19	ExternalGNBCUUPFunction	32

4.3.19.1	Definition	32

4.3.19.2	Attributes	32

4.3.19.3	Attribute constraints	32

4.3.19.4	Notifications	32

4.3.20	ExternalGNBDUFunction	33

4.3.20.1	Definition	33

4.3.20.2	Attributes	33

4.3.20.3	Attribute constraints	33

4.3.20.4	Notifications	33

4.3.21	ExternalUPFFunction	33

4.3.21.1	Definition	33

4.3.21.2	Attributes	33

4.3.21.3	Attribute constraints	33

4.3.21.4	Notifications	33

4.3.22	ExternalAMFFunction	33

4.3.22.1	Definition	33

4.3.22.2	Attributes	33

4.3.22.3	Attribute constraints	33

4.3.22.4	Notifications	34

4.3.23	ExternalMFs <<ProxyClass>>	34

4.3.23.1	Definition	34

4.3.23.2	Attributes	34

4.3.23.3	Attribute constraints	34

4.3.23.4	Notifications	34

4.3.24	ENBFunction <<ProxyClass>>	34

4.3.24.1	Definition	34

4.3.24.2	Attributes	34

4.3.24.3	Attribute constraints	34

4.3.24.4	Notifications	34

4.3.25	GNBCUCPFunction <<ProxyClass>>	34

4.3.25.1	Definition	34

4.3.25.2	Attributes	34

4.3.25.3	Attribute constraints	35

4.3.25.4	Notifications	35

4.3.26	GNBCUUPFunction <<ProxyClass>>	35

4.3.26.1	Definition	35

4.3.26.2	Attributes	35

4.3.26.3	Attribute constraints	35

4.3.26.4	Notifications	35

4.3.27	GNBDUFunction <<ProxyClass>>	35

4.3.27.1	Definition	35

4.3.27.2	Attributes	35

4.3.27.3	Attribute constraints	35

4.3.27.4	Notifications	35

4.3.28	ServingGWFFunction <<ProxyClass>>	35

4.3.28.1	Definition	35

4.3.28.2	Attributes	35

4.3.28.3	Attribute constraints	36

4.3.28.4	Notifications	36

4.3.29	UPFFunction <<ProxyClass>>	36

4.3.29.1	Definition	36

4.3.29.2	Attributes	36

4.3.29.3	Attribute constraints	36

4.3.29.4	Notifications	36

4.3.30	AMFFunction <<ProxyClass>>	36

4.3.30.1	Definition	36

4.3.30.2	Attributes	36

4.3.30.3	Attribute constraints	36

4.3.30.4	Notifications	36

4.3.31	Void	36

4.3.32	NRCellRelation	36

4.3.32.1	Definition	36

4.3.32.2	Attributes	37

4.3.32.3	Attribute constraints	37

4.3.32.4	Notifications	37

4.3.33	NRFreqRelation	37

4.3.33.1	Definition	37

4.3.33.2	Attributes	37

4.3.33.3	Attribute constraints	37

4.3.33.4	Notifications	37

4.3.34	EutranFreqRelation	37

4.3.34.1	Definition	37

4.3.34.2	Attributes	37

4.3.34.3	Attribute constraints	37

4.3.34.4	Notifications	37

4.3.35	ExternalNRCellCU	38

4.3.35.1	Definition	38

4.3.35.2	Attributes	38

4.3.35.3	Attribute constraints	38

4.3.35.4	Notifications	38

4.3.36	RRMPolicyRatio2 <<dataType>>	38

4.3.36.1	Definition	38

4.3.36.2	Attributes	38

4.3.37	S-NSSAI <<dataType>>	39

4.3.37.1	Definition	39

4.3.37.2	Attributes	39

4.3.37.3	Attribute constraints	39

4.3.37.4	Notifications	39

4.4	Attribute definitions	39

4.4.1	Attribute properties	39

4.5	Common notifications	46

5	Information Model definitions for 5GC NRM	47

5.1 	Imported information entities and local labels	47

5.2 	Class diagram	47

5.2.1	Class diagram of 5GC NFs	47

5.2.1.1	Relationships	47

5.2.1.2	Inheritance	54

5.2.2	Class diagram of AMF Region/AMF Set	55

5.2.2.1	Relationships	55

5.2.2.2	Inheritance	56

5.3	Class definitions	56

5.3.1	AMFFunction	56

5.3.1.1	Definition	56

5.3.1.2	Attributes	57

5.3.1.3	Attribute constraints	57

5.3.1.4	Notifications	57

5.3.2	SMFFunction	57

5.3.2.1	Definition	57

5.3.2.2	Attributes	57

5.3.2.3	Attribute constraints	57

5.3.2.4	Notifications	57

5.3.3	UPFFunction	57

5.3.3.1	Definition	57

5.3.3.2	Attributes	58

5.3.3.3	Attribute constraints	58

5.3.3.4	Notifications	58

5.3.4	N3IWFFunction	58

5.3.4.1	Definition	58

5.3.4.2	Attributes	58

5.3.4.3	Attribute constraints	58

5.3.4.4	Notifications	58

5.3.5	PCFFunction	58

5.3.5.1	Definition	58

5.3.5.2	Attributes	58

5.3.5.3	Attribute constraints	58

5.3.5.4	Notifications	58

5.3.6	AUSFFunction	59

5.3.6.1	Definition	59

5.3.6.2	Attributes	59

5.3.6.3	Attribute constraints	59

5.3.6.4	Notifications	59

5.3.7	UDMFunction	59

5.3.7.1	Definition	59

5.3.7.2	Attributes	59

5.3.5.3	Attribute constraints	59

5.3.5.4	Notifications	59

5.3.8	UDRFunction	59

5.3.8.1	Definition	59

5.3.8.2	Attributes	60

5.3.8.3	Attribute constraints	60

5.3.8.4	Notifications	60

5.3.9	UDSFFunction	60

5.3.9.1	Definition	60

5.3.9.2	Attributes	60

5.3.9.3	Attribute constraints	60

5.3.9.4	Notifications	60

5.3.10	NRFFunction	60

5.3.10.1	Definition	60

5.3.10.2	Attributes	61

5.3.10.3	Attribute constraints	61

5.3.10.4	Notifications	61

5.3.11	NSSFFunction	61

5.3.11.1	Definition	61

5.3.11.2	Attributes	61

5.3.11.3	Attribute constraints	61

5.3.11.4	Notifications	61

5.3.12	AFFunction	61

5.3.12.1	Definition	61

5.3.13	DNFunction	61

5.3.13.1	Definition	61

5.3.14	SMSFFunction	62

5.3.14.1	Definition	62

5.3.14.2	Attributes	62

5.3.14.3	Attribute constraints	62

5.3.14.4	Notifications	62

5.3.15	LMFFunction	62

5.3.15.1	Definition	62

5.3.15.2	Attributes	62

5.3.15.3	Attribute constraints	62

5.3.15.4	Notifications	62

5.3.16	NGEIRFunction	62

5.3.16.1	Definition	62

5.3.16.2	Attributes	63

5.3.16.3	Attribute constraints	63

5.3.16.4	Notifications	63

5.3.17	SEPPFunction	63

5.3.17.1	Definition	63

5.3.17.2	Attributes	63

5.3.17.3	Attribute constraints	63

5.3.17.4	Notifications	63

5.3.18	NWDAFFunction	63

5.3.18.1	Definition	63

5.3.18.2	Attributes	63

5.3.18.3	Attribute constraints	63

5.3.18.4	Notifications	63

5.3.19	EP_N2	64

5.3.19.1	Definition	64

5.3.19.2	Attributes	64

5.3.19.3	Attribute constraints	64

5.3.19.4	Notifications	64

5.3.20	EP_N3	64

5.3.20.1	Definition	64

5.3.20.2	Attributes	64

5.3.20.3	Attribute constraints	64

5.3.20.4	Notifications	64

5.3.21	EP_N4	64

5.3.21.1	Definition	64

5.3.21.2	Attributes	64

5.3.21.3	Attribute constraints	64

5.3.21.4	Notifications	65

5.3.22	EP_N5	65

5.3.22.1	Definition	65

5.3.22.2	Attributes	65

5.3.22.3	Attribute constraints	65

5.3.22.4	Notifications	65

5.3.23	EP_N6	65

5.3.23.1	Definition	65

5.3.23.2	Attributes	65

5.3.23.3	Attribute constraints	65

5.3.23.4	Notifications	65

5.3.24	EP_N7	65

5.3.24.1	Definition	65

5.3.24.2	Attributes	66

5.3.24.3	Attribute constraints	66

5.3.24.4	Notifications	66

5.3.25	EP_N8	66

5.3.25.1	Definition	66

5.3.25.2	Attributes	66

5.3.25.3	Attribute constraints	66

5.3.25.4	Notifications	66

5.3.26	EP_N9	66

5.3.26.1	Definition	66

5.3.26.2	Attributes	66

5.3.26.3	Attribute constraints	66

5.3.26.4	Notifications	66

5.3.27	EP_N10	67

5.3.27.1	Definition	67

5.3.27.2	Attributes	67

5.3.27.3	Attribute constraints	67

5.3.27.4	Notifications	67

5.3.28	EP_N11	67

5.3.28.1	Definition	67

5.3.28.2	Attributes	67

5.3.28.3	Attribute constraints	67

5.3.28.4	Notifications	67

5.3.29	EP_N12	67

5.3.29.1	Definition	67

5.3.29.2	Attributes	67

5.3.29.3	Attribute constraints	67

5.3.29.4	Notifications	68

5.3.30	EP_N13	68

5.3.30.1	Definition	68

5.3.30.2	Attributes	68

5.3.30.3	Attribute constraints	68

5.3.30.4	Notifications	68

5.3.31	EP_N14	68

5.3.31.1	Definition	68

5.3.31.2	Attributes	68

5.3.31.3	Attribute constraints	68

5.3.31.4	Notifications	68

5.3.32	EP_N15	68

5.3.32.1	Definition	68

5.3.32.2	Attributes	69

5.3.32.3	Attribute constraints	69

5.3.32.4	Notifications	69

5.3.33	EP_N16	69

5.3.33.1	Definition	69

5.3.33.2	Attributes	69

5.3.33.3	Attribute constraints	69

5.3.33.4	Notifications	69

5.3.34	EP_N17	69

5.3.34.1	Definition	69

5.3.34.2	Attributes	69

5.3.34.3	Attribute constraints	69

5.3.34.4	Notifications	69

5.3.35	EP_N20	70

5.3.35.1	Definition	70

5.3.35.2	Attributes	70

5.3.35.3	Attribute constraints	70

5.3.35.4	Notifications	70

5.3.36	EP_N21	70

5.3.36.1	Definition	70

5.3.36.2	Attributes	70

5.3.36.3	Attribute constraints	70

5.3.36.4	Notifications	70

5.3.37	EP_N22	70

5.3.37.1	Definition	70

5.3.37.2	Attributes	70

5.3.37.3	Attribute constraints	70

5.3.37.4	Notifications	71

5.3.38	EP_N26	71

5.3.38.1	Definition	71

5.3.38.2	Attributes	71

5.3.38.3	Attribute constraints	71

5.3.38.4	Notifications	71

5.3.39	EP_SBI_X	71

5.3.39.1	Definition	71

5.3.39.2	Attributes	71

5.3.39.3	Attribute constraints	71

5.3.39.4	Notifications	71

5.3.40	EP_SBI_IPX	71

5.3.40.1	Definition	71

5.3.40.2	Attributes	72

5.3.40.3	Attribute constraints	72

5.3.40.4	Notifications	72

5.3.41	EP_S5C	72

5.3.41.1	Definition	72

5.3.41.2	Attributes	72

5.3.41.3	Attribute constraints	72

5.3.41.4	Notifications	72

5.3.42	EP_S5U	72

5.3.42.1	Definition	72

5.3.42.2	Attributes	72

5.3.42.3	Attribute constraints	72

5.3.42.4	Notifications	72

5.3.43	EP_Rx	73

5.3.43.1	Definition	73

5.3.43.2	Attributes	73

5.3.43.3	Attribute constraints	73

5.3.43.4	Notifications	73

5.3.44	EP_MAP_SMSC	73

5.3.44.1	Definition	73

5.3.44.2	Attributes	73

5.3.44.3	Attribute constraints	73

5.3.44.4	Notifications	73

5.3.45	EP_NLS	73

5.3.45.1	Definition	73

5.3.45.2	Attributes	73

5.3.45.3	Attribute constraints	73

5.3.45.4	Notifications	74

5.3.46	EP_NLG	74

5.3.46.1	Definition	74

5.3.46.2	Attributes	74

5.3.46.3	Attribute constraints	74

5.3.46.4	Notifications	74

5.3.47	EP_N27	74

5.3.47.1	Definition	74

5.3.47.2	Attributes	74

5.3.47.3	Attribute constraints	74

5.3.47.4	Notifications	74

5.3.48	EP_N31	74

5.3.48.1	Definition	74

5.3.48.2	Attributes	75

5.3.48.3	Attribute constraints	75

5.3.48.4	Notifications	75

5.3.49	ExternalNRFFunction	75

5.3.49.1	Definition	75

5.3.49.2	Attributes	75

5.3.49.3	Attribute constraints	75

5.3.49.4	Notifications	75

5.3.50	ExternalNSSFFunction	75

5.3.50.1	Definition	75

5.3.50.2	Attributes	75

5.3.50.3	Attribute constraints	75

5.3.50.4	Notifications	75

5.3.51	AMFSet	76

5.3.51.1	Definition	76

5.3.51.2	Attributes	76

5.3.51.3	Attribute constraints	76

5.3.51.4	Notifications	76

5.3.52	AMFRegion	76

5.3.52.1	Definition	76

5.3.52.2	Attributes	76

5.3.52.3	Attribute constraints	76

5.3.52.4	Notifications	76

5.3.53	ExternalAMFFunction	77

5.3.53.1	Definition	77

5.3.53.2	Attributes	77

5.3.53.3	Attribute constraints	77

5.3.53.4	Notifications	77

5.4	Attribute definitions	77

5.4.1	Attribute properties	77

5.5	Common notifications	79

6	Information Model definitions for network slice NRM	79

6.1	Imported information entities and local labels	79

6.2	Class diagram	80

6.2.1	Relationships	80

6.2.2	Inheritance	80

6.3	Class definitions	81

6.3.1	NetworkSlice	81

6.3.1.1	Definition	81

6.3.1.2	Attributes	81

6.3.1.3	Attribute constraints	81

6.3.1.4	Notifications	81

6.3.2	NetworkSliceSubnet	81

6.3.2.1	Definition	81

6.3.2.2	Attributes	81

6.3.2.3	Attribute constraints	81

6.3.2.4	Notifications	81

6.3.3	ServiceProfile <<dataType>>	82

6.3.3.1	Definition	82

6.3.3.2	Attributes	82

6.3.3.3	Attribute constraints	82

6.3.3.4	Notifications	82

6.3.4	SliceProfile <<dataType>>	82

6.3.4.1	Definition	82

6.3.4.2	Attributes	82

6.3.4.3	Attribute constraints	82

6.3.4.4	Notifications	83

6.4	Attribute definition	83

6.4.1	Attribute properties	83

6.5	Common notifications	86

7	Solution Set (SS)	86

Annex A (normative):	Cell state handling	88

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	88

A.2	Combined state diagram for gNB cell	88

Annex B (normative):	 NSI and NSSI state handling	92

B.1 	NSI state handling	92

B.2	State handling of NSSI	93

Annex C (normative):	XML definitions for NR NRM	96

C.1	General	96

C.2	Architectural features	96

C.3	Mapping	96

C.3.1	General mapping	96

C.3.2	Information Object Class (IOC) mapping	96

C.4	Solution Set definitions	96

C.4.1	XML definition structure	96

C.4.2	Graphical representation	96

C.4.3	XML schema "nRNrm.xsd"	97

Annex D (normative):	JSON definitions for NR NRM	105

D.1	General	105

D.2	Architectural features	105

D.3	Mapping	105

D.3.1	IOC mapping	105

D.3.2	Attributes mapping	105

D.4	Solution Set (SS) definitions	106

D.4.1	JSON definition structure	106

D.4.2	Graphical representation	106

D.4.3	JSON schema "nrNrm.json"	106

Annex E (normative):	YANG definitions for NR NRM	115

E.1	General	115

E.2	Architectural features	115

E.3	Mapping	115

E.3.1	IOC mapping	115

E.3.2	Attributes mapping	116

E.4	Solution Set (SS) definitions	116

E.4.1	YANG definition structure	116

E.4.2	Graphical representation	116

E.4.3	YANG schema	120

E.4.3.1	General type definition "nrm-types-3gpp.yang"	120

E.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "Subnetwork.yang", “vsDataContainer.yang”, “MeContext.yang”, “ManagementNode.yang”	127

E.4.3.3	NGRAN, composition of all gNB ManagedFunctions in "ngran.yang"	136

E.4.3.4	NGRAN submodule, "ngran-gNBCUCPFunction.yang"	137

E.4.3.5	NGRAN submodule, "ngran-gNBCUUPFunction.yang"	138

E.4.3.6	NGRAN submodule, "ngran-gNBDUFunction.yang"	139

E.4.3.7	NGRAN submodule, "ngran-nRCellCU.yang"	139

E.4.3.8	NGRAN submodule, "ngran- nRCellDU.yang"	140

E.4.3.9	NGRAN submodule, "ngran-nRSectorCarrier.yang"	142

E.4.3.10	NGRAN submodule, "ngran-bWP.yang"	143

Annex F (normative):	XML definitions for 5GC NRM	144

F.1	General	144

F.2	Architectural features	144

F.3	Mapping	144

F.3.1	General mapping	144

F.3.2	Information Object Class (IOC) mapping	144

F.4	Solution Set definitions	144

F.4.1	XML definition structure	144

F.4.2	Graphical representation	144

F.4.3	XML schema "ngcNrm.xsd"	145

Annex G (normative):	JSON definitions for 5GC NRM	164

G.1	General	164

G.2	Architectural features	164

G.3	Mapping	164

G.3.1	IOC mapping	164

G.3.2	Attributes mapping	165

G.4	Solution Set definitions	166

G.4.1	JSON definition structure	166

G.4.2	Graphical representation	166

G.4.3	JSON schema "ngcNrm.json"	166

Annex H (normative):	YANG definitions for 5GC	185

H.1	General	185

H.2	Architectural features	185

H.3	Mapping	185

H.3.1	IOC mapping	185

H.3.2	Attributes mapping	186

H.4	Solution Set definitions	186

H.4.1	YANG definition structure	186

H.4.2	Graphical representation	186

H.4.3	YANG schema	201

H.4.3.1	General type definition "nrm-types-3gpp.yang"	201

H.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork"	201

H.4.3.3	NGC module, composition of all 5GC ManagedFunctions in "ngc.yang"	201

H.4.3.4	NGC submodule, "ngc-AMFFunction.yang", “ngc-AMFRegion.yang”, “ngc-AMFSet.yang”	202

H.4.3.5	NGC submodule, "ngc-AUSFFunction.yang"	205

H.4.3.6	NGC submodule, "ngc-LMFFunction.yang"	206

H.4.3.7	NGC submodule, "ngc-N3IWFFunction.yang"	206

H.4.3.8	NGC submodule, "ngc-NGEIRFunction.yang"	207

H.4.3.9	NGC submodule, "ngc-NRFFunction.yang"	207

H.4.3.10	NGC submodule, "ngc-NSSFFunction.yang"	213

H.4.3.11	NGC submodule, "ngc-NWDAFunction.yang"	214

H.4.3.12	NGC submodule, "ngc-PCFFunction.yang"	215

H.4.3.13	NGC submodule, "ngc-SEPPFunction.yang"	216

H.4.3.14	NGC submodule, "ngc-SMFFunction.yang"	216

H.4.3.15	NGC submodule, "ngc-SMSFFunction.yang"	217

H.4.3.16	NGC submodule, "ngc-UDMFunction.yang"	218

H.4.3.17	NGC submodule, "ngc-UDRFunction.yang"	219

H.4.3.18	NGC submodule, "ngc-UDSFFunction.yang"	219

H.4.3.19	NGC submodule, "ngc-UPFFunction.yang"	220

Annex I (normative):	XML definitions for network slice	222

I.1	General	222

I.2	Architectural features	222

I.3	Mapping	222

I.3.1	General mapping	222

I.3.2	Information Object Class (IOC) mapping	222

I.4	Solution Set (SS) definitions	222

I.4.1	XML definition structure	222

I.4.2	Graphical representation	222

I.4.3	XML schema "sliceNrm.xsd"	223

Annex J (normative):	JSON definitions for network slice	226

J.1	General	226

J.2	Architectural features	226

J.3	Mapping	226

J.3.1	IOC mapping	226

J.3.2	Attributes mapping	226

J.4	Solution Set (SS) definitions	227

J.4.1	JSON definition structure	227

J.4.2	Graphical representation	227

J.4.3	JSON schema "sliceNrm.json"	227

Annex K (normative):	YANG definitions for Network Slice NRM	232

K.1	General	232

K.2	Architectural features	232

K.3	Mapping	232

K.3.1	IOC mapping	232

K.3.2	Attributes mapping	232

K.4	Solution Set definitions	232

K.4.1	YANG definition structure	232

K.4.2	Graphical representation	232

K.4.3	YANG schema	234

K.4.3.1	General type definition "nrm-types-3gpp.yang"	234

K.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork"	234

K.4.3.3	NetworkSlice Module, "NetworkSlice.yang"	234

K.4.3.4	ServiceProfile Submodule, "ServiceProfile.yang"	235

K.4.3.5	NetworkSliceSubnet Module, "NetworkSliceSubnet.yang"	236

K.4.3.6	SliceProfile Submodule, "SliceProfile.yang"	237

K.4.3.7	perfReq Module, "perfReq.yang"	238

Annex L (informative):	 Change history	239

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in 3GPP TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], 3GPP TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 28.540 [10].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.501 [2], 3GPP TS 38.401 [4], 3GPP TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] , 3GPP TS 23.501 [2], 3GPP TS 38.401 [4] and 3GPP TS 28.540 [10].

BWP	Bandwidth part

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function

# 4	Information model definitions for NR NRM

## 4.1	Imported and associated information

### 4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [30], IOC, ManagedElement |  |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.625 [17], attribute, availabilityStatus | availabilityStatus |
| TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.658 [19], IOC, EUtranCellFDD | EUTranCellFDD |
| TS 28.658 [19], IOC, EUtranCellTDD | EUTranCellTDD |
| 3GPP TS 28.622 [30], IOC, SubNetwork |  |

### 4.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for the representation of gNB and en-gNB.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

Figure 4.2.1.1-5: View on frequencies and external entities

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

4.3	Class definitions

### 4.3.1	GNBDUFunction

#### 4.3.1.1	Definition

This IOC represents the logical function DU of gNB or en-gNB defined in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

#### 4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | M | T | T | F | T |
| gNBDUName | O | T | T | F | T |
| gNBId | M | T | F | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | T | T |

#### 4.3.1.3	Attribute constraints

None.

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	GNBCUCPFunction

#### 4.3.2.1	Definition

This IOC represents the logical function CU-CP of gNB and en-gNB defined in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C <<IOC>>EP_F1U. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |

#### 4.3.2.3	Attribute constraints

None.

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	GNBCUUPFunction

#### 4.3.3.1	Definition

This IOC represents the logical function CU-UP of gNB or en-gNB defined in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.3.3	Attribute constraints

None.

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	NRCellCU

#### 4.3.4.1	Definition

This IOC represents the information required by CU that is responsible for the management of inter-cell mobility and neighbour relations via ANR.

4.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| rRMPolicyType | CM | T | T | F | T |
| rRMPolicyNSSIId | CM | T | T | F | T |
| rRMPolicyRatio | CM | T | T | F | T |
| rRMPolicyRatio2 | CM | T | T | F | T |
| rRMPolicy | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

Note 1:	 Void.

Note 2: 	Void.

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |
| rRMPolicyRatio2 Support Qualifier | Condition: Network slicing feature is supported and rRMPolicyType is 2. |
| rRMPolicyType Support Qualifier | Condition: RRM policy for network slicing feature is supported. |
| rRMPolicyNSSIId Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 1. |
| rRMPolicyRatio Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 1. |
| rRMPolicy Support Qualifier | Condition: RRM policy for network slicing feature is supported and rRMPolicyType is 0. |

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	NRCellDU

#### 4.3.5.1	Definition

This IOC represents the information of a cell known by DU. The procedure for cell setup is initiated from the DU, thus requiring a notion of a cell. The DU owns the resources realizing the cell, thus requiring a notion of a cell owning the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in subclause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See subclauses 5.3 and 5.4.2 of TS 38.104 for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

#### 4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| operationalState | M | T | F | F | T (see Note 2) |
| administrativeState | M | T | T | F | T (see Note 2) |
| availabilityStatus | M | T | F | F | T (see Note 2) |
| cellState | M | T | F | F | T (see Note 2) |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| ssbFrequency | M | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | M | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrier | M | T | T | F | T |
| bWP | M | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: The attribute value change is conveyed by the notifyStateChange notification. |  |  |  |  |  |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |
| arfcnUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| arfcnSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| bSChannelBwUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| nrTAC Support Qualifier | Condition: 5G Standalone solution is supported. |

#### 4.3.5.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point included in the cell. These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bSChannelBwDL or bSChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes have to be present and their values need to be set.

#### 4.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bSChannelBwDL | CM | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunction | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bSChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bSChannelBwDL. |
| bSChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bSChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	BWP

#### 4.3.7.1	Definition

This IOC represents a bandwidth part (BWP) defined in 3GPP TS 38.211 [32], subclause 4.4.5. A bandwidth part is related to downlink, uplink or supplementary uplink resource grids, and is defined by its subcarrier spacing (SCS), cyclic prefix and location and size related to the common resource grid for the applicable SCS.

A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics.

#### 4.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bwpContext | M | T | T | F | T |
| isInitialBwp | M | T | T | F | T |
| subCarrierSpacing | M | T | T | F | T |
| cyclicPrefix | M | T | T | F | T |
| startRB | M | T | T | F | T |
| numberOfRBs | M | T | T | F | T |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.8	EP_E1

#### 4.3.8.1	Definition

This IOC represents the local end point of the logical link, supporting E1 interface between gNB-CU-CP and gNB-CU-UP. The E1 interface is defined in 3GPP TS 38.401 [4].

#### 4.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.8.3	Attribute constraints

None.

#### 4.3.8.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.9	EP_XnU

#### 4.3.9.1	Definition

This IOC represents the one end-point of a logical link supporting the Xn user plane (Xn-U) interface. The Xn-U interface provides non-guaranteed delivery of user plane PDUs between two NG-RAN nodes. The user plane PDUs are carried on GTP-U/UDP/IP/Data link layer/Physical layer stack. See subclause 7.2 of 3GPP TS 38.420 [6].

#### 4.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.10	EP_NgC

#### 4.3.10.1	Definition

This IOC represents the local end point of the control plane interface (NG-C) between the gNB and NG-Core entity. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol).

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.10.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.11	EP_NgU

#### 4.3.11.1	Definition

This IOC represents the local end point of the NG user plane (NG-U) interface between the gNB and the UPGW. The interface provides non-guaranteed delivery of user plane PDUs between the gNB and the UPGW. GTP-U is baseline for this interface.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.12	EP_F1C

#### 4.3.12.1	Definition

This IOC represents the local end point of the control plane interface (F1-C) between the DU and CU or CU-CP. The transport network layer is based on IP transport with the SCTP on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol). See subclause 7.1 of 3GPP TS 38.470 [7].

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.12.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.13	EP_F1U

#### 4.3.13.1	Definition

This IOC represents the local end point of the user plane interface (F1-U) between the DU and CU or CU-UP. The transport network layer is based on IP transport, with the UDP and GTP-U on top of IP.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.14	EP_S1U

#### 4.3.14.1	Definition

This IOC represents the local end point of the logical link, supporting S1-U interface towards a S-GW node. The S1-U interface is defined in 3GPP TS 36.410 [14].

#### 4.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.15	EP_X2C

#### 4.3.15.1	Definition

This IOC represents the local end point of the logical link, supporting X2-C application protocols used in EN-DC, to a neighbour eNB or en-gNB node, which is defined in 3GPP TS 36.423 [15]. EN-DC is defined in 3GPP TS 37.340 [9].

#### 4.3.15.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.16	EP_X2U

#### 4.3.16.1	Definition

This IOC represents the local end-point of a logical link supporting the X2 user plane (X2-U) interface used in EN-DC, which is defined in 3GPP TS 36.425 [16].

#### 4.3.16.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.17	EP_XnC

#### 4.3.17.1	Definition

This IOC represents the local gNB node end point of the logical link, supporting Xn Application protocols, to a neighbour gNB node. The Xn Application PDUs are carried over SCTP/IP/Data link layer/Physical layer stack. See subclause 7 of 3GPP TS 38.420 [6].

#### 4.3.17.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.17.3	Attribute constraints

None

#### 4.3.17.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.18	ExternalGNBCUCPFunction

#### 4.3.18.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUCPFunction managed by another management function. For more information about GNBCUCPFunction, see subclause 4.3.2.

#### 4.3.18.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.18.3	Attribute constraints

None.

#### 4.3.18.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.19	ExternalGNBCUUPFunction

#### 4.3.19.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUUPFunction managed by another management function. For more information about GNBCUUPFunction, see subclause 4.3.3.

#### 4.3.19.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.20	ExternalGNBDUFunction

#### 4.3.20.1	Definition

This IOC represents the properties, known by the management function, of a GNBDUFunction managed by another management function. For more information about GNBDUFunction, see subclause 4.3.1.

#### 4.3.20.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | T | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.21	ExternalUPFFunction

#### 4.3.21.1	Definition

This IOC represents the properties, known by the management function, of a UPFFunction managed by another management function. For more information about UPFFunction, see subclause 5.3.3.

#### 4.3.21.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.22	ExternalAMFFunction

#### 4.3.22.1	Definition

This IOC represents the properties, known by the management function, of an AMFFunction managed by another management function. For more information about AMFFunction, see subclause 5.3.

#### 4.3.22.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.22.3	Attribute constraints

None

#### 4.3.22.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.23	ExternalMFs <<ProxyClass>>

#### 4.3.23.1	Definition

This represents the following IOCs:

<<IOC>>ExternalAMFFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ExternalGNBDUFunction, <<IOC>>ExternalServingGWFunction, <<IOC>>ExternalUPFFunction.

#### 4.3.23.2	Attributes

See respective IOCs.

#### 4.3.23.3	Attribute constraints

See respective IOCs.

#### 4.3.23.4	Notifications

See respective IOCs.

### 4.3.24	ENBFunction <<ProxyClass>>

#### 4.3.24.1	Definition

This IOC represents an <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.2	Attributes

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.3	Attribute constraints

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBPFunction.

#### 4.3.24.4	Notifications

See respective IOCs.

### 4.3.25	GNBCUCPFunction <<ProxyClass>>

#### 4.3.25.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.3	Attribute constraints

See respective IOCs.

#### 4.3.25.4	Notifications

See respective IOCs.

### 4.3.26	GNBCUUPFunction <<ProxyClass>>

#### 4.3.26.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.4	Notifications

See respective IOCs.

### 4.3.27	GNBDUFunction <<ProxyClass>>

#### 4.3.27.1	Definition

This IOC represents an <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.2	Attributes

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.3	Attribute constraints

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.4	Notifications

See respective IOCs.

### 4.3.28	ServingGWFFunction <<ProxyClass>>

#### 4.3.28.1	Definition

This IOC represents an <<IOC>>ServingGWFFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.2	Attributes

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.3	Attribute constraints

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.4	Notifications

See respective IOCs.

### 4.3.29	UPFFunction <<ProxyClass>>

#### 4.3.29.1	Definition

This IOC represents an <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.2	Attributes

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.3	Attribute constraints

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.4	Notifications

See respective IOCs.

### 4.3.30	AMFFunction <<ProxyClass>>

#### 4.3.30.1	Definition

This IOC represents an <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.2	Attributes

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.3	Attribute constraints

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.4	Notifications

See respective IOCs.

#### 4.3.31	Void

#### 4.3.32	NRCellRelation

#### 4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

Neighbour cell relations are unidirectional.

#### 4.3.32.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | T | T | F |
| nRTCI | O | T | T | F | T |
| adjacentCell | M | T | T | F | T |

#### 4.3.32.3	Attribute constraints

#### 4.3.32.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.33	NRFreqRelation

#### 4.3.33.1	Definition

This IOC represents a neighbour frequency relation for one or more NRCellRelations.

#### 4.3.33.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.33.3	Attribute constraints

#### 4.3.33.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.34	EutranFreqRelation

#### 4.3.34.1	Definition

This IOC represents a neighbour frequency relation for one or more EutranCellRelations.

#### 4.3.34.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.34.3	Attribute constraints

#### 4.3.34.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.35	ExternalNRCellCU

#### 4.3.35.1	Definition

This abstract IOC represents the properties of an NRCellCU controlled by another Management Service Provider. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by Management Service Provider. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 4.3.35.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| arfcnDL | M | T | T | F | T |

#### 4.3.35.3	Attribute constraints

None.

#### 4.3.35.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.36	RRMPolicyRatio2 <<dataType>>

#### 4.3.36.1	Definition

This data type represents the properties of RRMPolicyRatio2. The RRM policy setting the ratios for the split of the Radio resources between the supported S-NSSAILists, in average over time. RRMPolicyRatio2List is a list and each item of the list has seven attributes. The list of the seven attributes is data type RRMPolicyRatio2. Four of them specify the percentage of radio resources to be allocated to the corresponding sNSSAIList. The attribute rRMPolicyMaxRatio defines the maximum resource limitation for the sNSSAIList. The attribute rRMPolicyMarginMaxRatio defines the safety margin that allows for maximum float limit use with other sNSSAIs borrowing free resources while keeping safety margin to ensure that resources are available for quota specific sNSSAIList. The attribute rRMPolicyMinRatio defines the minimum resource limitation for the sNSSAIList. The attribute rRMPolicyMarginMinRatio defines the safety margin that allows for minimum float limit use with other sNSSAIs borrowing free resources while keeping safety margin to ensure that resources are available for quota specific sNSSAIList.

#### 4.3.36.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | M | T | F | T | F |
| sNSSAIList | M | T | T | F | T |
| quotaType | M | T | T | F | T |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMarginMaxRatio | O | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyMarginMinRatio | O | T | T | F | T |

### 4.3.37	S-NSSAI <<dataType>>

#### 4.3.37.1	Definition

This data type represents an S-NSSAI. An NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of an SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).

#### 4.3.37.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sST | M | T | T | F | T |
| sD | O | T | T | F | T |

#### 4.3.37.3	Attribute constraints

None

#### 4.3.37.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: "LOCKED", "SHUTTING DOWN", "UNLOCKED". The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: "ENABLED", "DISABLED". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." "allowedValues: "IDLE", "INACTIVE", "ACTIVE". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. allowedValues:TBD | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], subclause 4.2. allowedValues: "NORMAL", "EXTENDED". | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id. (See IEEE 802.1Q [39]), | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB. See 'gNB-DU ID' in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellLocalId | It identifies a NR cell of a gNB. It, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [3]), The NCI can be constructed by encoding the gNB Identifier using gNBId (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction) and cellLocalId where the gNB Identifier field is of length specified by gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI subclause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. | type: Bitstring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUCPFunction.pLMNId | It specifies the PLMN identifier to be used as part of the global RAN node identity. allowedValues: Not applicable. | Type: <<dataType>>PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBDUFunction.pLMNId | It specifies the PLMN identifier to be used as part of the NR Cell Global Identity (NCGI). allowedValues: Not applicable. | type: <<dataType>>PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUUPFunction.pLMNIdList | This is a list of PLMN identifiers. It defines from which set of PLMNs an UE must have as its serving PLMN to be allowed to use the GNB-CU-UP. allowedValues: Not applicable. | type: <<dataType>>PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: TrueN/A defaultValue: None isNullable: False |
| NRCellDU.pLMNIdList | It defines which PLMNs that can be served by the NR cell. The first entry of the list is the PLMN used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: <<dataType>>PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: TrueN/A defaultValue: None isNullable: False |
| ExternalNRCellCU.pLMNIdList | It defines which PLMNs that are assumed to be served by the NR Cell in another gNB-CU-CP. This list is either updated by the managed element itself (e.g. due to ANR, signalling over Xn etc) or by consumer over the standard interface. allowedValues: Not applicable. | Type: <<datatype>>PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: TrueN/A defaultValue: None isNullable: False |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting. The S-NSSAI is defined in 3GPP TS 23.003 [13]. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This attribute specifies the Slice/Service type (SST) of the network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sD | This attribute specifies the Slice Differentiator (SD), which is optional information that complements the slice/service type(s) to differentiate amongst multiple Network Slices. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyType | Type of the RRM policy. The value 0 denotes use of the rRMPolicy. The value 1 denotes use of the rRMPolicyNSSIId, rRMPolicyRatio The value 2 denotes use of the rRMPolicyRatio2. allowedValues: 0 : 65535. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicyNSSIId | The list of S-NSSAIs for which a rRMPolicyRatio value is specified allowedValues: Not applicable. | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyRatio | The RRM policy setting the ratio for the split of the Radio resources between the supported S-NSSAI lists A S-NSSAI list is defined in rRMPolicyNSSIId. rRMPolicyRatio is the list of target percentage values assigned to the corresponding rRMPolicyNSSIId values. Every value specifies the percentage of PRBs to be allocated to the corresponding S-NSSAIs, in average over time. The sum of the values shall be less or equal 100. allowedValues: 0 : 100 See NOTE | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyRatio2List | The attribute specifies a list of RRMPolicyRatio2 which is defined as a datatype. The attribute is used to set the ratios for the split of the Radio resources between the sNSSAILists for radio resources (e.g. RRC connected users, PDCP resource, etc.) in average time (see NOTE 2 and NOTE 3). The sum of the values included in the item of rRMPolicyRatio2 shall be less or equal 100 (see NOTE 4). allowedValues: Not applicable. | type: RRMPolicyRatio2 multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| groupId | The attribute identifies one sNSSAIList group inside NRCellCU. The rRMPolicyRatio2 is configured for each group. The value of the groupId is unique inside one NRCellCU instance | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| quotaType | The attribute indicates the type of the quota which allows to allocate resources as strictly usable for defined slice(s) ( “strict quota”) or allows that resources to be used by other slice(s) when defined slice(s) do not need them ( “float quota”). allowedValues: "STRICT", "FLOAT". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicyMaxRatio | The RRM policy setting the maximum percentage of radio resources to be allocated to the corresponding S-NSSAIList. This quota can be strict or float quota. Strict quota means resources are not allowed for other sNSSAIs even when they are not used by the defined sNSSAIList. Float quota resources can be used by other sNSSAIs when the defined sNSSAIList do not need them. Value 0 indicates that there is no maximum limit. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMarginMaxRatio | Maximum quota margin ratio is applicable when maximum quota policy ratio is of type “float quota”. It defines the resource quota within maximum quota to reserve buffers for new resource requirements for the specified S-NSSAIList. With the margin ratio, unused resources of the maximum resource quota can be allocated to other S-NSSAIs when the free resources are more than resource amount indicated by the margin. The margin resource quota can only be used for the specific S-NSSAIList. Value 0 indicates that no margin is used. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMinRatio | The RRM policy setting the minimum percentage of radio resources to be allocated to the corresponding S-NSSAIList. This quota can be strict or float quota. Strict quota means resources are not allowed for other sNSSAIs even when they are not used by the defined sNSSAIList. Float quota resources can be used by other sNSSAIs when the defined sNSSAIList do not need them. Value 0 indicates that there is no minimum limit. allowedValues: 0 : 100 NOTE: The averaging time interval is implementation dependent. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMarginMinRatio | Minimum quota margin ratio is applicable when minimum quota policy ratio is of type “float quota”. It defines the resource quota within minimum quota to reserve buffers for new resource requirements for the specified S-NSSAIList. With the margin ratio, unused resources of the minimum resource quota can be allocated to other S-NSSAIs when the free resources are more than resource amount indicated by the margin. The margin resource quota can only be used for the specific S-NSSAIList. Value 0 indicates that no margin is used. allowedValues: 0 : 100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicy | It represents RRM policy which includes guidance for split of radio resources between the S-NSSAIs that the cell supports in case when the rRMPolicyType is absent or equal to 0. The RRM policy is implementation dependent. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| subcarrierSpacing | Subcarrier spacing configuration for a BWP. See subclause 5 in TS 38.104 [12]. AllowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: "DL", "UL", "DL and UL" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: "DL", "UL", "SUL" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: "INITIAL", "OTHER" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTCI | This is the Target NR Cell Identifier. It consists of NR Cell Identifier (NCI) and Physical Cell Identifier of the target NR cell (nRPCI). The NRRelation.nRTCI identifies the target cell from the perspective of the NRCell, the name-containing instance of the subject NRCellCU instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| adjacentCell | This attribute contains the DN of an AdjacentNRCell (NRCellCU or ExternalNRCellCU) or AdjacentEutranCell (EutranCellFDD or EutranCellTDD or ExternalEutranCellFDD or ExternalEutranCellTDD) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ssbFrequency | Indicates cell defining SSB frequency domain position Frequency of the cell defining SSB transmission. The frequency provided in this attribute identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The frequency must be positioned on the NR global frequency raster, as defined in TS 38.101 [42] subclause 5.4.2. and within bSChannelBwDL. allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbPeriodicity | Indicates cell defined SSB periodicity in number of subframes (ms). The SSB periodicity in msec is used for the rate matching purpose. allowedValues: 5, 10, 20, 40, 80, 160. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbSubCarrierSpacing | Subcarrier spacing of SSB. See subclause 7.4.3.1 of TS 38.211 [32]. allowedValues: Only the values 15kHz or 30 kHz (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbOffset | Indicates cell defining SSB time domain position. Defined as the offset of the measurement window, in number of subframes (ms), in which to receive SS/PBCH blocks, where allowed values depend on the ssbPeriodicity. allowedValues: ssbPeriodicity5 ms 0..4, ssbPeriodicity10 ms 0..9, ssbPeriodicity20 ms 0..19, ssbPeriodicity40 ms 0..39, ssbPeriodicity80 ms 0..79, ssbPeriodicity160 ms 0..159. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbDuration | Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (ms) (see 38.213 [41], subclause 4.1. allowedValues: 1, 2, 3, 4, 5. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: The averaging time interval is implementation dependent. NOTE 2: The radio resource can be signaling resources (e.g. RRC connected users) or user plane resources (e.g. PDCP). The detail resource and how to map the ratio to exact number of resources is implementation dependant. NOTE 3: The averaging time interval is implementation dependent. NOTE 4: How to calculate the sum of the ratio is implementation dependent. |  |  |

## 4.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 5		Information Model definitions for 5GC NRM

## 5.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], attribute, pLmnIdList | pLMNIDList |

## 5.2 	Class diagram

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

The Figure 5.2.1.1-2 shows the transport view of AMF NRM.

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-9 shows the transport view of UDR NRM.

Figure 5.2.1.1-9: Transport view of UDR NRM

The Figure 5.2.1.1-10 shows the transport view of UDSF NRM.

Figure 5.2.1.1-10: Transport view of UDSF NRM

The Figure 5.2.1.1-11 shows the transport view of NWDAF NRM.

Figure 5.2.1.1-11: Transport view of NWDAF NRM

The Figure 5.2.1.1-12 shows the transport view of NRF NRM.

«InformationObjectClass»

NRFFunction

«InformationObjectClass»

EP_SBI_X

«Name»

*

«ProxyClass»

FarEnd_SBI_X1

1

1

«InformationObjectClass»

EP_N27

«Name»

*

«InformationObjectClass»

ExternalNRFFunction

1

1

It represents any other 5GC node

Figure 5.2.1.1-12: Transport view of NRF NRM

The Figure 5.2.1.1-13 shows the transport view of NSSF NRM.

«InformationObjectClass»

NSSFunction

«Name»

*

«Name»

*

It represents

NRFFunction or NEFFunction or UDRFunction or UDSFFunction or NWDAFFunction

«InformationObjectClass»

EP_SBI_X

«ProxyClass»

FarEnd_SBI_X2

1

1

«InformationObjectClass»

EP_N22

«InformationObjectClass»

AMFFunction

1

1

«InformationObjectClass»

ExternalNSSFFunction

«InformationObjectClass»

EP_N31

«Name»

*

1

1

Figure 5.2.1.1-13: Transport view of NSSF NRM

The Figure 5.2.1.1-14 shows the transport view of SMSF NRM.

Figure 5.2.1.1-14: Transport view of SMSF NRM

The Figure 5.2.1.1-15 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-15: Transport view of LMF NRM

The Figure 5.2.1.1-16 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-16: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-17 shows the transport view of SEPP NRM.

Figure 5.2.1.1-17: Transport view of SEPP NRM

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

### 5.2.2	Class diagram of AMF Region/AMF Set

#### 5.2.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of AMF Region/AMF Set definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.2.1-1 shows the AMF Region/AMF Set NRM containment/naming relationship.

Figure 5.2.2.1-1: AMF Region/AMF Set NRM

#### 5.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.2.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the AMF Region/AMF Set NRM.

Figure 5.2.2.2-1: AMF Region/AMF Set Inheritance

## 5.3	Class definitions

### 5.3.1	AMFFunction

#### 5.3.1.1	Definition

This IOC represents the AMF functionality in 5GC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| weightFactor | M | T | T | F | T |
| sNSSAIList | CM | T | F | F | T |

#### 5.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.1.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.2	SMFFunction

#### 5.3.2.1	Definition

This IOC represents the SMF function in 5GC. For more information about the SMF, see 3GPP TS 23.501 [2].

#### 5.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.2.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.3	UPFFunction

#### 5.3.3.1	Definition

This IOC represents the UPF function in 5GC. For more information about the UPF, see 3GPP TS 23.501 [2].

#### 5.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList CM Support Qualifier | The condition is "network slicing feature is supported". |

#### 5.3.3.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.4	N3IWFFunction

#### 5.3.4.1	Definition

This IOC represents the N3IWF function which is used to enable non-3GPP access networks connected to the 5GC. For more information about the N3IWF, see 3GPP TS 23.501 [2].

#### 5.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |

#### 5.3.4.3	Attribute constraints

None.

#### 5.3.4.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.5	PCFFunction

#### 5.3.5.1	Definition

This IOC represents the PCF function in 5GC. For more information about the PCF, see 3GPP TS 23.501 [2].

#### 5.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.6	AUSFFunction

#### 5.3.6.1	Definition

This IOC represents the AUSF function in 5GC. For more information about the AUSF, see 3GPP TS 23.501 [2].

#### 5.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.6.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.7	UDMFunction

#### 5.3.7.1	Definition

This IOC represents the UDM function in 5GC. For more information about the UDM, see 3GPP TS 23.501 [2].

#### 5.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.8	UDRFunction

#### 5.3.8.1	Definition

This IOC represents the UDR function in 5GC. For more information about the UDR, see 3GPP TS 23.501 [2].

#### 5.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.8.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.9	UDSFFunction

#### 5.3.9.1	Definition

This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in 3GPP TS 23.501 [2]. For more information about the UDSF, see 3GPP TS 23.501 [2].

#### 5.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.9.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.10	NRFFunction

#### 5.3.10.1	Definition

This IOC represents the NRF function in 5GC. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.10.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nFProfileList | CM | T | T | F | T |
| nSIIdList | O | T | T | F | T |

#### 5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |
| nfProfileList Support Qualifier | Condition: NF profile is registered and deregistered by management system. |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.10.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.11	NSSFFunction

#### 5.3.11.1	Definition

This IOC represents the NSSF function in 5GC. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | M | T | T | F | T |
| nSIIdList | O | T | T | F | T |

#### 5.3.11.3	Attribute constraints

None.

#### 5.3.11.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.12	AFFunction

#### 5.3.12.1	Definition

This IOC is defined for only purpose to describe the IOCs representing its interaction interface with 5GC (i.e. EP_Rx and EP_N5). It has no any attributes defined.

### 5.3.13	DNFunction

#### 5.3.13.1	Definition

This IOC is defined for only purpose to describe the IOCs representing Data Network (DN) interaction interface with 5GC (i.e. EP_N6). It has no any attributes defined.

### 5.3.14	SMSFFunction

#### 5.3.14.1	Definition

This IOC represents the SMSF function defined in 3GPP TS 23.501 [2].

#### 5.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |

#### 5.3.14.3	Attribute constraints

None.

#### 5.3.14.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.15	LMFFunction

#### 5.3.15.1	Definition

This IOC represents the LMF function defined in 3GPP TS 23.501 [2].

#### 5.3.15.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |

#### 5.3.15.3	Attribute constraints

None.

#### 5.3.15.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.16	NGEIRFunction

#### 5.3.16.1	Definition

This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see 3GPP TS 23.501 [2].

#### 5.3.16.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.16.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.17	SEPPFunction

#### 5.3.17.1	Definition

This IOC represents the SEPP function which support message filtering and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501 [2].

#### 5.3.17.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |

#### 5.3.17.3	Attribute constraints

None.

#### 5.3.17.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.18	NWDAFFunction

#### 5.3.18.1	Definition

This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see 3GPP TS 23.501 [2].

#### 5.3.18.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sBIServiceList | M | T | F | F | T |
| sNSSAIList | CM | T | T | F | T |

#### 5.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.18.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.19	EP_N2

#### 5.3.19.1	Definition

This IOC represents the N2 interface between (R)AN and AMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.19.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.19.3	Attribute constraints

None.

#### 5.3.19.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.20	EP_N3

#### 5.3.20.1	Definition

This IOC represents the N3 interface between (R)AN and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.20.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.20.3	Attribute constraints

None.

#### 5.3.20.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.21	EP_N4

#### 5.3.21.1	Definition

This IOC represents the N4 interface between SMF and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.21.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.21.3	Attribute constraints

None.

#### 5.3.21.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.22	EP_N5

#### 5.3.22.1	Definition

This IOC represents the N5 interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.22.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.22.3	Attribute constraints

None.

#### 5.3.22.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.23	EP_N6

#### 5.3.23.1	Definition

This IOC represents the N6 interface between UPF and DN, which is defined in 3GPP TS 23.501 [2].

#### 5.3.23.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.23.3	Attribute constraints

None.

#### 5.3.23.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.24	EP_N7

#### 5.3.24.1	Definition

This IOC represents the N7 interface between SMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.24.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.24.3	Attribute constraints

None.

#### 5.3.24.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.25	EP_N8

#### 5.3.25.1	Definition

This IOC represents the N8 interface between AMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.25.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.25.3	Attribute constraints

None.

#### 5.3.25.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.26	EP_N9

#### 5.3.26.1	Definition

This IOC represents the N7 interface between two UPFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.26.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.26.3	Attribute constraints

None.

#### 5.3.26.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.27	EP_N10

#### 5.3.27.1	Definition

This IOC represents the N10 interface between SMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.27.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.27.3	Attribute constraints

None.

#### 5.3.27.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.28	EP_N11

#### 5.3.28.1	Definition

This IOC represents the N11 interface between AMF and SMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.28.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.28.3	Attribute constraints

None.

#### 5.3.28.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.29	EP_N12

#### 5.3.29.1	Definition

This IOC represents the N12 interface between AMF and AUSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.29.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.29.3	Attribute constraints

None.

#### 5.3.29.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.30	EP_N13

#### 5.3.30.1	Definition

This IOC represents the N13 interface between AUSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.30.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.30.3	Attribute constraints

None.

#### 5.3.30.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.31	EP_N14

#### 5.3.31.1	Definition

This IOC represents the N14 interface between two AMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.31.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.31.3	Attribute constraints

None.

#### 5.3.31.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.32	EP_N15

#### 5.3.32.1	Definition

This IOC represents the N15 interface between AMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.32.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.32.3	Attribute constraints

None.

#### 5.3.32.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.33	EP_N16

#### 5.3.33.1	Definition

This IOC represents the N16 interface between two SMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.33.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.33.3	Attribute constraints

None.

#### 5.3.33.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.34	EP_N17

#### 5.3.34.1	Definition

This IOC represents the N17 interface between AMF and 5G-EIR, which is defined in 3GPP TS 23.501 [2].

#### 5.3.34.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.34.3	Attribute constraints

None.

#### 5.3.34.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.35	EP_N20

#### 5.3.35.1	Definition

This IOC represents the N20 interface between AMF and SMSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.35.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.35.3	Attribute constraints

None.

#### 5.3.35.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.36	EP_N21

#### 5.3.36.1	Definition

This IOC represents the N21 interface between SMSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.36.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.36.3	Attribute constraints

None.

#### 5.3.36.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.37	EP_N22

#### 5.3.37.1	Definition

This IOC represents the N22 interface between AMF and NSSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.37.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.37.3	Attribute constraints

None.

#### 5.3.37.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.38	EP_N26

#### 5.3.38.1	Definition

This IOC represents the N26 interface between AMF and MME, which is defined in 3GPP TS 23.501 [2].

#### 5.3.38.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.38.3	Attribute constraints

None.

#### 5.3.38.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.39	EP_SBI_X

#### 5.3.39.1	Definition

This IOC represents the service-based interfaces from any 5GC NF towards UDSF, UDR, NEF, NRF and NWDAF, as these NFs can be interacted with any other 5GC NF, see detail in 3GPP TS 23.501 [2].

#### 5.3.39.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| sBIserviceList | O | T | T | F | T |

#### 5.3.39.3	Attribute constraints

None.

#### 5.3.39.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.40	EP_SBI_IPX

#### 5.3.40.1	Definition

This IOC represents the inter-connection interface from any 5GC control plane NF to SEPP, see detail in 3GPP TS 23.501 [2].

#### 5.3.40.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| sBIserviceList | O | T | T | F | T |

#### 5.3.40.3	Attribute constraints

None.

#### 5.3.40.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.41	EP_S5C

#### 5.3.41.1	Definition

This IOC represents the S5-C interface between SGW and SMF/PGW-C, which is defined in 3GPP TS 23.501 [2].

#### 5.3.41.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.41.3	Attribute constraints

None.

#### 5.3.41.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.42	EP_S5U

#### 5.3.42.1	Definition

This IOC represents the S5-U interface between SGW and UPF/PGW-U, which is defined in 3GPP TS 23.501 [2].

#### 5.3.42.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.42.3	Attribute constraints

None.

#### 5.3.42.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.43	EP_Rx

#### 5.3.43.1	Definition

This IOC represents the Rx interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.43.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.43.3	Attribute constraints

None.

#### 5.3.43.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.44	EP_MAP_SMSC

#### 5.3.44.1	Definition

This IOC represents the MAP interface between SMSF and MSC-IWMSC/GMSC, which is defined in 3GPP TS 23.040 [22].

#### 5.3.44.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.44.3	Attribute constraints

None.

#### 5.3.44.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.45	EP_NLS

#### 5.3.45.1	Definition

This IOC represents the NLs interface between AMF and LMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.45.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.45.3	Attribute constraints

None.

#### 5.3.45.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.46	EP_NLG

#### 5.3.46.1	Definition

This IOC represents the NLg interface between AMF and GMLC, which is defined in 3GPP TS 23.501 [2].

#### 5.3.46.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.46.3	Attribute constraints

None.

#### 5.3.46.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.47	EP_N27

#### 5.3.47.1	Definition

This IOC represents an end point of N27 interface between vNRF and hNRF, which is defined in 3GPP TS 29.510 [10].

#### 5.3.47.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.47.3	Attribute constraints

None.

#### 5.3.47.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.48	EP_N31

#### 5.3.48.1	Definition

This IOC represents an end point of N31 interface between vNSSF and hNSSF, which is defined in 3GPP TS 29.531 [11].

#### 5.3.48.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.48.3	Attribute constraints

None.

#### 5.3.48.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.49	ExternalNRFFunction

#### 5.3.49.1	Definition

This IOC represents external NRF function controlled by another management domain. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.49.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.49.3	Attribute constraints

None.

#### 5.3.49.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.50	ExternalNSSFFunction

#### 5.3.50.1	Definition

This IOC represents external NSSF function controlled by another management domain. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.50.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.50.3	Attribute constraints

None.

#### 5.3.50.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.51	AMFSet

#### 5.3.51.1	Definition

This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice. For more information about the AMF Set, see 3GPP TS 23.501 [2].

#### 5.3.51.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFRegion | M | T | T | F | T |
| aMFSetMemberList | M | T | T | F | T |

#### 5.3.51.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.51.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.52	AMFRegion

#### 5.3.52.1	Definition

This IOC represents the AMF Region which consists one or multiple AMF Sets. For more information about the AMF Region, see 3GPP TS 23.501 [2].

#### 5.3.52.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFRegionId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSet | M | T | T | F | T |

#### 5.3.52.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.52.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.53	ExternalAMFFunction

#### 5.3.53.1	Definition

This IOC represents an external AMF functionality used in EN-DC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.53.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.53.3	Attribute constraints

None.

#### 5.3.53.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  | Documentation and Allowed Values |  | Properties |
| --- | --- | --- | --- | --- |
| aMFIdentifier |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetId |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetMemberList |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aMFRegionId |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| nfProfileList |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nSIIdList |  | It is a set of NSI Id. The NSI ID is defined in subclause 6.1.6.2.8 of 3GPP TS 29.531 [24]. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList |  | See subclause 4.4.1. |  |  |
| sBIFQDN |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sBIServiceList |  | It is used to indicate the all supported NF services registered on service-based interface. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nRTACList |  | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| weightFactor |  | The weight factor is typically set according to the capacity of local node (AMF) relative to other nodes in the same type. And it is used to achieve load balancing among a set of same type of network functions. (Ref. 3GPP TS 23.501 [2]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

## 5.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 6	Information Model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], attribute, pLmnIdList | pLMNIDList |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

### 6.2.2	Inheritance

Figure 6.2.2-1: Network slice inheritance relationship

## 6.3	Class definitions

### 6.3.1	NetworkSlice

#### 6.3.1.1	Definition

This IOC represents the properties of a network slice instance in a 5G network. For more information about the network slice instance, see 3GPP TS 28.531 [26].

#### 6.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| serviceProfileList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nSSIId | M | T | F | F | T |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.2	NetworkSliceSubnet

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet instance in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.531 [26].

#### 6.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mFIdList | M | T | F | F | T |
| constituentNSSIIdList | M | T | F | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |

#### 6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo Support Qualifier | Condition: It shall be supported if the NSS instance is realized in the virtualized environment. Otherwise this attribute shall be absent. |

#### 6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This  data type represents the properties of network slice related requirement that should be supported by the network slice instance in 5G network.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.4	SliceProfile <<dataType>>

#### 6.3.4.1	Definition

This  data type represents the properties of network slice subnet related requirement that should be supported by the network slice subnet instance in a 5G network.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| availability | This parameter specifies the availability requirement for an network slice instance, expressed as a percentage. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| nSSIId | It is thea DN of the MOI for the NSSI associated with the network slice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| constituentNSSIIdList | It is a list of DN of MOI(s) for the constituent NSSI associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| mFIdList | It is a list of DN of the MOI(s) for the NF instances associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| operationalState | It indicates the operational state of the network slice instance or the network slice subnet instance. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice instance or the network slice subnet instance. It describes the permission to use or prohibition against using the imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| perfReq | This parameter specifies the requirements to the NSI in terms of the scenarios defined in the TS 22.261 [28], such as Experienced data rate, Area traffic capacity (density) information of UE density. It is a structure containing the following elements: - list of perfRequirements Depending on the sST value, the list of perfRequirements will be - list of eMBBPerfReq or - list of uRLLCPerfReq or - list of mIoTPerfReq NOTE: the list of mIoTPerfReq is not addressed in the present document. allowedValues: - list of eMBBPerfReq is a list of entries where an entry identifies the performance requirements to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. An entry has the following attributes: expDataRateDL (Integer), expDataRateUL (Integer), areaTrafficCapDL (Integer), areaTrafficCapUL (Integer), userDensity (Integer), activityFactor (Integer), uESpeed (Integer), coverage (String) (see Table 7.1-1 of TS 22.261 [28]). - list of uRLLCPerfReq is a list of entries where an entry identifies the performance requirements to the NSI in terms of the scenarios defined in the Table 7.2.2-1 of TS 22.261 [28]. An entry has the following attributes: e2eLatency (Integer), jitter (Integer), survivalTime (Integer), cSAvailability (Float), reliability (Float), expDataRate (Integer), payloadSize (String), trafficDensity (Integer), connDensity (Integer), serviceAreaDimension (String) (see Table 7.2-1 of TS 22.261 [28]). NOTE: Limitation on attribute values in instances of ServiceProfile is not addressed in the present document. | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the new NSI to be created or the existing NSI to be re-used. sNSSAList is defined in subclause 4.4.1 |  |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of TrackingAreas where the NSI can be selected. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice instance. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice instance. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice instance may be shared with another network slice instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet instance may be shared with another network slice subnet instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice instance | type: ServiceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet instance | type: SliceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type for a ServiceProfile.. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

## 6.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

# 7	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for NR and NG-RAN:

-	XML based 3GPP NR and NG-RAN NRM Solution Set (Annex C).

-	JSON based 3GPP NR and NG-RAN NRM Solution Set (Annex D).

-	YANG based 3GPP NR and NG-RAN NRM Solution Set (Annex E).

The present document defines the following NRM Solution Set definitions for 5GC:

-	XML based 3GPP 5GC NRM Solution Set (Annex F).

-	JSON based 3GPP 5GC NRM Solution Set (Annex G).

-	YANG based 3GPP 5GC NRM Solution Set (Annex H).

The present document defines the following NRM Solution Set definitions for network slice and network slice subnet:

-	XML based 3GPP Network Slice NRM Solution Set (Annex I).

-	JSON based 3GPP Network Slice NRM Solution Set (Annex J).

-	YANG based 3GPP Network Slice NRM Solution Set (Annex K).

Annex A (normative):
Cell state handling

# A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"

The administrative state indicates the permission to use or prohibition against using the cell, imposed through the OAM services. The administrative state has three values: "LOCKED", "SHUTTING DOWN" or "UNLOCKED"

The meanings of these values are defined in ITU-T Recommendation X.731 [18].

The relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" is defined in subclause 8.5 of TS 38.401 [4]. See below an extract from subclause 8.5 of TS 38.401 [4] on the F1 startup and cell activation.

If the operationalState is "ENABLED" (i.e. the resource is physically installed and working) and if the administrativeState is "UNLOCKED", the step "0: Pre-operational state" will exit and the step "1: F1 Setup Request" will be executed."

8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

# A.2	Combined state diagram for gNB cell

This is the Combined state diagram for gNB cell.

Figure A.2-1: Combined gNB cell state diagram

The gNB-DU maintains cell states. The following table is the gNB cell state transition table.

In 3-split and 2-split deployment scenarios, the interactions between gNB-CU and gNB-DU are standardized. The interactions specified under the column "The state transition events and actions" of "The gNB Cell state transition table" below shall be present for the state transition.

In the non-split deployment scenarios, the interactions between gNB-CU and gNB-DU are not standardized. The interactions between gNB-CU and gNB-DU specified under the column "The state transition events and actions" of "The gNB Cell state transition table" can be replaced by other means that is not standardized.

Table A.2-1: The gNB Cell state transition table

| Transition number | The state transition event and actions |
| --- | --- |
| 1 | Event: Receive request to unlock. Action: None. |
| 2 | Event: Receive request to lock. Action: None. |
| 2a | Event: Receive request to lock Action: Send to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. |
| 3 | Event: When the required cell resource is physically installed and working. Action: none. |
| 4 | Event: When the required cell resource is not physically installed or is not working. Action: Send to gNB-CU the "gNB-DU Configuration update message" with cell to delete. |
| 4a | Event: When the required cell resource is physically uninstalled or is not working. Action: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 5 | Event: Receive from gNB-CU the "F1 Setup Response message" (identifying the cell to be activated). The cell is activated successfully. Actions: Do nothing or send gNB-CU the "gNB-DU Configuration Update message" with Cell stated as active' ----- or ----- Event: Receive from gNB-CU the "gNB-CU Configuration Update message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message"). Actions: The cell is activated successfully. Send to gNB-CU the "gNB-CU Configuration Update Response" to confirm the cell is in active state. ----- or ----- Event: Receive from gNB-CU the "gNB-DU Configuration Update Acknowledge message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message") and the cell is activated successfully Actions: Do nothing. |
| 6 | Event: Receive from gNB-CU the "gNB-CU Configuration Update message" and responds with gNB-CU Configuration Update Acknowledge messages. Actions: Respond with gNB-CU Configuration Update Acknowledge messages. ----- or ----- Event: Event: DU experiences an internal failure and decided to place the cell into inactive state. Actions: Send to gNB-CU the "gNB-DU Cell status Update message" |
| 7 | Event: Send to gNB-CU the "F1 Setup request" (identifying the cell that is configured and ready to be activated). Actions: none. ----- or ----- Send to gNB-CU the "gNB-DU Configuration Update message" with the served cell to add. Actions: none. |
| 8 | Event: Sends to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. Receive response from gNB-CU the "gNB-DU Configuration Update Acknowledge message". Actions: None. |
| 9 | Event: Receive request to shut down. Actions: None. |
| 10 | Event: Last user quit. Actions: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 11 | Event: When a cell is created and is configured. Actions: None |
| 12 | Event: When a cell is deleted. Action: None. |

Annex B (normative): 
NSI and NSSI state handling

# B.1 	NSI state handling

An NSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time, the management system needs to know the state of an NSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.1: Combined NSI state diagram

In an NSI deployment scenario, the interactions between communication service management function, network slice management function and network slice subnet management function are standardized. The interactions specified under the column "The state transition events and actions" of "NSI state transition table" below shall be present for the state transition.

Figure B.2: NSI state diagram with state transition triggers

Table B.1: The NSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSMF responds positively to the "Create NSI request" message, the NSI is created and the state is set to Locked |
| 1 | NSMF responds positively to the "Activate NSI request" message (identifying the NSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSI stops using the NSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSI and its constituents are installed and working NSMF receives positive response to the "Allocate NSSI" message (applicable to the NSI to be enabled). |
| 4 | When the NSI or its constituents are not installed or not working NSMF receives positive response to the "Deallocate NSSI" message (applicable to the NSI to be disabled) |
| 5 | NSMF responds positively to the "Deallocate NSI request" message, the NSI is deleted and the state is set to NULL |

# B.2	State handling of NSSI

An NSSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time the management system needs to know the state of an NSSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.2.1: Combined NSSI state diagram

In an NSSI deployment scenario, the interactions between CSMF, NSMF and NSSMF are standardized. The interactions specified under the column "The state transition events and actions" of "NSSI state transition table" below shall be present for the state transition.

Figure B.2.2: NSSI state diagram with state transition triggers

Table B.2.1: The NSSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSSMF responds positively to the "Create NSSI request" message, the NSSI is created and the state is set to Locked |
| 1 | NSSMF responds positively to the "Activate NSSI request" message (identifying the NSSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSSI stops using the NSSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSSI constituents are installed and working NSSMF receives positive response to the "Create NSSI constituent" message (applicable to the NSSI to be enabled). |
| 4 | When the NSSI constituents are not installed or not working NSSMF receive positive response to the "Delete NSSI constituent" message (applicable to the NSSI to be disabled) |
| 5 | NSSMF responds positively to the "Delete NSSI request" message, the NSSI is deleted and the state is set to NULL. |

Annex C (normative):
XML definitions for NR NRM

# C.1	General

This annex contains the XML definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM Information Model definitions specified in clause 4.

# C.2	Architectural features

The overall architectural feature of NR NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations

# C.3	Mapping

## C.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## C.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# C.4	Solution Set definitions

## C.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

The present document defines the NRM-specific XML schema nrNrm.xsd for the NR NRM Information Model defined in clause 4.

XML schema nrNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## C.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## C.4.3	XML schema "nRNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 NR Network Resource Model

XML schema definition

nrNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<simpleType name="GnbId">

<restriction base="unsignedLong">

<maxInclusive value="4294967295"/>

</restriction>

</simpleType>

<simpleType name="GnbIdLength">

<restriction base="integer">

<minLength value="22"/>

<maxLength value="32"/>

</restriction>

</simpleType>

<simpleType name="Nci">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

</restriction>

</simpleType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<simpleType name="NrTac">

<restriction base="unsignedLong">

<maxInclusive value="16777215"/>

<!—5G TAC is 3-octets length -->

</restriction>

</simpleType>

<simpleType name="GnbDuId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbName">

<restriction base="string">

<minLength value="1"/>

<maxLength value="150"/>

</restriction>

</simpleType>

<simpleType name="CyclicPrefix">

<restriction base="integer">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="60"/>

<enumeration value="120"/>

</restriction>

</simpleType>

<simpleType name="QuotaType">

<restriction base="string">

<enumeration value="STRICT"/>

<enumeration value="FLOAT"/>

</restriction>

</simpleType>

<complexType name="IpEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<simpleType name="CellState">

<restriction base="string">

<enumeration value="IDLE"/>

<enumeration value="INACTIVE"/>

<enumeration value="ACTIVE"/>

</restriction>

</simpleType>

<simpleType name="BwpContext">

<restriction base="string">

<enumeration value="DL"/>

<enumeration value="UL"/>

<enumeration value="SUL"/>

</restriction>

</simpleType>

<simpleType name="IsInitialBwp">

<restriction base="string">

<enumeration value="INITIAL"/>

<enumeration value="OTHER"/>

</restriction>

</simpleType>

<complexType name="RRMPolicyRation2">

<sequence>

<element name="groupId" type="integer"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="quotaType" type="QuotaType"/>

<element name="rRMPolicyMaxRation" type="integer"/>

<element name="rRMPolicyMarginMaxRation" type="integer"/>

<element name="rRMPolicyMinRation" type="integer"/>

<element name="rRMPolicyMarginMinRation" type="integer"/>

</sequence>

</complexType>

<element name="GNBDUFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbDUId" type="nn:GnbDuId"/>

<element name="gnbDuName" type="nn:GnbName" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellDU"/>

<element ref="nn:BWP"/>

<element ref="nn:NRSectorCarrier"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_F1U"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbCuName" type=" nn:GnbName" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellCU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_E1"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_NgC"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUUPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="gNBId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:EP_E1"/>

<element ref="nn:EP_F1U"/>

<element ref="nn:EP_XnU"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_S1U"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="rRMPolicyType" type="integer" minOccurs="0"/>

<element name="rRMPolicyNSSIId" type="xn:dn" minOccurs="0"/>

<element name="rRMPolicyRatio" type="integer" minOccurs="0"/>

<element name="rRMPolicy" type="string" minOccurs="0"/>

<element name="rRMPolicyRatio2" type="RRMPolicyRation2" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellDU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="nCGI" type="nn:Ncgi" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType"

minOccurs="0"/>

<element name="availabilityStatus" type="sm:availabilityStatusType"

minOccurs="0"/>

<element name="cellState" type="nn:CellState"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="nRpci" type="nn:Pci" />

<element name="nRTac" type="nn:NrTac" />

<element name="arfcnDL" type="integer"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="arfcnSUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwSUL" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="nRSectorCarrier" type="xn:dn" minOccurs="0"/>

<element name="bWP" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRSectorCarrier">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="txDirection" type="nn:TxDirection"/>

<element name="configuredMaxTxPower" type="integer"/>

<element name="arfcnDL" type="integer" minOccurs="0"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer" minOccurs="0"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element name="sectorEquipmentFunction" type="xn:dn" minOccurs="0"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="BWP">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="bwpContext" type="nn:BwpContext"/>

<element name="isInitialBwp" type="nn:IsInitialBwp"/>

<element name="subCarrierSpacing" type="integer"/>

<element name="cyclicPrefix" type="nn:CyclicPrefix"/>

<element name="startRB" type="integer"/>

<element name="numberOfRBs" type="integer"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_E1">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex D (normative):
JSON definitions for NR NRM

# D.1	General

This annex contains the JSON definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# D.2	Architectural features

# D.3	Mapping

## D.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| GNBDUFunction | gnbDuFunction | object |
| GNBCUCPFunction | gnbCuCpFunction | object |
| GNBCUUPFunction | gnbCuUpFunction | object |
| NRCellCU | nrCellCu | object |
| NRCellDU | nrCellDu | object |
| NRSectorCarrier | nrSectorCarrier | object |
| BWP | bwp | object |
| EP_E1 | ep_E1 | object |
| EP_XnC | ep_XnC | object |
| EP_XnU | ep_XnU | object |
| EP_NgC | ep_NgC | object |
| EP_NgU | ep_NgU | object |
| EP_F1C | ep_F1C | object |
| EP_F1U | ep_F1U | object |
| EP_S1U | ep_S1U | object |
| EP_X2C | ep_X2C | object |
| EP_X2U | ep_X2U | object |

## D.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| cellState | cellState | CellState |
| txDirection | txDirection | TxDirection |
| bwpContext | BwpContext | bwpContext |
| isInitialBwp | isInitialBwp | IsInitialBwp |
| configuredMaxTxPower | configuredMaxTxPower | ConfiguredMaxTxPower |
| fDDCenterFrequencyUL | fddCenterFrequencyUL | FddCenterFrequencyUL |
| fDDCenterFrequencyDL | fddCenterFrequencyDL | FddCenterFrequencyDL |
| fDDBandwidthUL | fddBandwidthUL | FddBandwidthUL |
| fDDBandwidthDL | fddBandwidthDL | FddBandwidthDL |
| localAddress | localAddress | IpEndPoint |
| remoteAddress | remoteAddress | IpEndPoint |
| gNBId | gnbId | GnbId |
| gNBDUId | gnbDuId | GnbDUId |
| gNBCUName | gnbCuName | gNBName |
| gNBDUName | gnbDuName | gNBName |
| nCI | nCi | NCi |
| nRPCI | nrPci | NrPCI |
| nRTac | nrTac | NrTac |
| sNSSAIList | snssaiList | SnssaiList |
| pointAArfcnULFDD | pointAArfcnULFDD | PointAArfcnULFDD |
| pointAArfcnDLFDD | pointAArfcnDLFDD | PointAArfcnDLFDD |
| pointAArfcnTDD | pointAArfcnTDD | PointAArfcnTDD |
| rRMPolicy | rrmPolicy | RrmPolicy |
| RRMPolicyRation2 | rrmPolicyRation2 | RrmPolicyRation2 |
| plmnId | plmnId | PlmnId |
| plmnIdList | plmnIdList | PlmnIdList |

# D.4	Solution Set (SS) definitions

## D.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## D.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## D.4.3	JSON schema "nrNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#,"

"id": "http://3gpp.org/28541/nrNrm.json",

"description": "JSON based solution set definitions for NR NRM",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"gnbDuFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbDuId": {

"$ref": "#/definitions/GnbDuId"

},

"gnbDuName": {

"$ref": "#/definitions/GnbName"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

}

},

"required": [

"userLabel",

"gnbDuId",

"gnbId",

" gnbIdLength"

]

},

"gnbCuCpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"gnbCuName": {

"$ref": "#/definitions/GnbName"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

" gnbIdLength",

"plmnIdList"

]

},

"gnbCuUpFunction": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"peeParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/PeeParametersList"

},

"gnbId": {

"$ref": "#/definitions/GnbId"

},

"gnbIdLength": {

"$ref": "#/definitions/GnbIdLength"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"gnbId",

"gnbIdLength",

"plmnIdList"

]

},

"nrCellCu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"rrmPolicyType": {

"type": "integer"

},

"rrmPolicyNSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"rrmPolicyRatio": {

"type": "integer"

},

"rrmPolicy": {

"$ref": "#/definitions/RrmPolicy"

},

"rrmPolicyRatio2": {

"$ref": "#/definitions/RrmPolicyRatio2"

}

},

"required": [

"userLabel",

"nCI",

"plmnIdList"

]

},

"nrCellDu": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AdministrativeState"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/OperationalState"

},

"availabilityStatus": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AvailabilityStatus"

},

"cellState": {

"$ref": "#/definitions/CellState"

},

"nCi": {

"$ref": "#/definitions/NCi"

},

"plmnIdList": {

"$ref": "#/definitions/PlmnIdList"

},

"snssaiList": {

"$ref": "#/definitions/SnssaiList"

},

"nrPci": {

"$ref": "#/definitions/NrPci"

},

"nrTac": {

"$ref": "#/definitions/NrTac"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"arfcnSUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

"bSChannelBwSUL": {

"type": "integer"

}

},

"required": [

"userLabel",

"ngi",

"plmnIdList",

"nrPci",

"nrTac",

"arfcnDL",

"bSChannelBwDL"

]

},

"nrSectorCarrier": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"txDirection": {

"$ref": "#/definitions/TxDirection"

},

"configuredMaxTxPower": {

"type": "integer"

},

"arfcnDL": {

"type": "integer"

},

"arfcnUL": {

"type": "integer"

},

"bSChannelBwDL": {

"type": "integer"

},

"bSChannelBwUL": {

"type": "integer"

},

},

"required": [

"userLabel",

"configuredMaxTxPower",

"txDirection"

]

},

"bwp": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"bwpContext": {

"$ref": "#/definitions/BwpContext"

},

"isInitialBwp": {

"$ref": "#/definitions/IsInitialBwp"

},

"subCarrierSpacing": {

"type": "integer"

},

"cyclicPrefix": {

"$ref": "#/definitions/CyclicPrefix"

},

"startRB": {

"type": "integer"

},

"numberOfRBs": {

"type": "integer"

},

},

"required": [

"userLabel",

"bwpContext",

"isInitialBwp",

"subCarrierSpacing",

"cyclicPrefix",

"startRB",

"numberOfRBs"

]

},

"eP_E1": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_XnU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgC": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_NgU": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_F1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_S1U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2C": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"eP_X2U": {

"type": "object",

"properties": {

"userLabel": {

"type": "string"

},

"farEndEntity": {

"type": "string"

},

"localAddress": {

"$ref": "#/definitions/IpEndPoint"

},

"remoteAddress": {

"$ref": "#/definitions/IpEndPoint"

}

},

},

"GnbId": {

"type": "string",

"description": "gNBId identifies a gNB within a PLMN."

},

"GnbIdLength": {

"type": "integer",

"description":  "This indicates the number of bits for encoding the gNB ID.",

"minimum": 22,

"maximum": 32

},

"GnbName": {

"type": "string",

"maxLength": 150

},

"GnbDuId": {

"type": "number",

"description": "GnbDuId represents the NB-DU ID defined in TS 38.473.",

"minimum": 0,

"maximum": 68719476735

},

"NCi": {

"type": "object",

"description": "NCgi uniquely identifies a NR cell  within a PLMN.",

"properties": {

"plmnId": {

"$ref": "#/definitions/PlmnId"

},

"nCI": {

"$ref": "#/definitions/NrCellId"

}

}

},

"SnssaiList": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"RrmPolicy": {

"type": "string",

"description": "RrmPolicy represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.",

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"type": "string"

},

"ipv6Address": {

"type": "string"

},

"ipv6Prefix": {

"type": "string"

},

"transport": {

"type": "string"

},

"vlanId": {

"type": "integer"

}

}

},

"NrPci": {

"type": "integer",

"maximum": 503

},

"NrTac": {

"type": "integer",

"maximum": 16777215

},

"NrCellId": {

"type": "integer",

"maximum": 68719476735

},

"Sst": {

"type": "integer",

"maximum": 255

},

"Snssai": {

"type": "object",

"properties": {

"sst": {

"$ref": "#/definitions/Sst"

},

"sd": {

"type": "string"

}

}

},

"CellState": {

"enum": ["IDLE", "INACTIVE", "ACTIVE"]

},

"CyclicPrefix": {

"enum": ["15", "30", "60", "120"]

},

"TxDirection": {

"enum": ["DL", "UL", "DL and UL"]

},

"BwpContext": {

"enum": ["DL", "UL", "SUL"]

},

"IsInitialBwp": {

"enum": ["INITIAL", "OTHER", "SUL"]

},

"QuotaType": {

"enum": ["STRICT", "FLOAT"]

},

"RrmPolicyRatio2": {

"type": "object",

"properties": {

"groupId": {

"type": "integer"

},

"sNSSAIList": {

"$ref": "#/definitions/SnssaiList"

},

"quotaType": {

"$ref ": "#/definitions/QuotaType"

},

"rRMPolicyMaxRation": {

"type": "integer"

},

"rRMPolicyMarginMaxRation": {

"type": "integer"

},

"rRMPolicyMinRation": {

"type": "integer"

},

"rRMPolicyMarginMinRation": {

"type": "integer"

}

}

},

"Mcc": {

"type": "string",

"pattern": "[02-79][0-9][0-9]"

},

"Mnc": {

"type": "string",

"pattern": "[0-9]{3}|[0-9]{2}"

},

"PlmnId": {

"type": "object",

"properties": {

"mcc": {

"$ref": "#/definitions/Mcc"

},

"mnc": {

"$ref": "#/definitions/Mnc"

}

}

},

"PlmnIdList": {

"type": "array",

"item": {

"$ref": "#/definitions/PlmnId"

}

}

}

}

Annex E (normative):
YANG definitions for NR NRM

# E.1	General

This annex contains the YANG definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# E.2	Architectural features

# E.3	Mapping

## E.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| ManagedElement | ManagedElement | Container in module |
| GNBDUFunction | GNBDUFunction | Container in submodule |
| GNBCUCPFunction | GNBCUCPFunction | Container in submodule |
| GNBCUUPFunction | GNBCUUPFunction | Container in submodule |
| NRCellCU | NRCellCU | Container in submodule |
| NRCellDU | NRCellDU | Container in submodule |
| NRSectorCarrier | NRSectorCarrier | Container in submodule |
| BWP | BWP | Container in submodule |
| EP_E1 | EP_E1 | Container in module |
| EP_XnC | EP_XnC | Container in module |
| EP_XnU | EP_XnU | Container in module |
| EP_NgC | EP_NgC | Container in module |
| EP_NgU | EP_NgU | Container in module |
| EP_F1C | EP_F1C | Container in module |
| EP_F1U | EP_F1U | Container in module |
| EP_S1U | EP_S1U | Container in module |
| EP_X2C | EP_X2C | Container in module |
| EP_X2U | EP_X2U | Container in module |
| NOTE 1: Modularize (include module and submodules) the data model design to be easier to maintain and extend. NOTE 2: Facilitate the reusability. Technically use grouping statement of YANG to define common node. NOTE 3: The YANG definitions' Name is identical to object's name in information model. NOTE 4: Define a ngran module which included/imported other modules/submodules to constitute a completed NG-RAN system. The module augments the ManagedElement IoC to add required ManagedFunctions NOTE 5: Reflect the class hierarchy in Inheritance view. Define basic IoCs such as Top, ManagedFunction, EP_RP, etc., and inherit xxxFunction, EP_xxx, etc., from the parent IoCs. NOTE 6: To simplify, only define common IoC in 28.622 [30] as base class, and integrate attributes of common IoCs in 28.620 into the common classes. |  |  |

## E.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| Simple Type Attributes | Same to the Name in Stage 2 | leaf |
| Complex Type Attribute | Same to the Name in Stage 2 | Reusable Container |
| Simple Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | leaf-list |
| Complex Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | list |
| Note: Basically, use Leaf to define Attributes, and use typedef/grouping to define specific/complex type if required. |  |  |

# E.4	Solution Set (SS) definitions

## E.4.1	YANG definition structure

TBD

## E.4.2	Graphical representation

module: ngran

+--rw ManagedElement

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro vendorName                string

+--rw userDefinedState          string

+--ro swVersion                 string

+--ro dnPrefix                  string

+--rw userLabel                 string

+--ro locationName              string

+--ro managedBy*                nrm-type:t_dn

+--ro managedElementTypeList*   string

+--rw GNBDUFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBDUId              uint32

|  +--rw gNBDUName           string

|  +--ro gNBId                uint32

|  +--ro gNBIdLength          uint32

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1U

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellDU* [k_nCI]

|  |  +--rw k_nCI                          string

|  |  +--ro objectClass                     string

|  |  +--ro objectInstance                  nrm-type:t_dn

|  |  +--ro id                              nrm-type:t_dn

|  |  +--rw userLabel                      string

|  |  +--rw vnfParametersList

|  |  |  +--rw vnfInstanceId    string

|  |  |  +--rw vnfdId          string

|  |  |  +--rw flavourId       string

|  |  |  +--rw autoScalable    boolean

|  |  +--rw peeParametersList

|  |  |  +--rw siteIdentification    string

|  |  |  +--rw siteLatitude         decimal64

|  |  |  +--rw siteLongitude        decimal64

|  |  |  +--rw siteDescription       string

|  |  |  +--rw equipmentType         enumeration

|  |  |  +--rw environmentType       enumeration

|  |  |  +--rw powerInterface        enumeration

|  |  +--rw nCI		t_NCI

|  |  +--rw pLMNId* [MCC]

|  |  |  +--rw MCC    t_mcc

|  |  |  +--rw MNC   t_mnc

|  |  +--rw s-NSSAI*                        nrm-type:t_s-NSSAI

|  |  +--ro operationalState               nrm-type:t_operationalState

|  |  +--rw administrativeState            nrm-type:t_administrativeState

|  |  +--ro availabilityStatus             nrm-type:t_availabilityStatus

|  |  +--ro cellState                       nrm-type:t_cellState

|  |  +--rw nRPCI                           nrm-type:t_nRPCI

|  |  +--rw nRTAC                           nrm-type:t_tAC

|  |  +--rw bSChannelBw                     uint16

|  |  +--rw fDDCarrierDL                    uint16

|  |  +--rw fDDCarrierUL                    uint16

|  |  +--rw tDDCarrier                      uint16

|  |  +--rw subcarrierSpacingDL             nrm-type:t_subcarrierSpacing

|  |  +--rw subcarrierSpacingUL             nrm-type:t_subcarrierSpacing

|  |  +--rw cyclicPrefixDL                  nrm-type:t_cyclicPrefix

|  |  +--rw cyclicPrefixUL                  nrm-type:t_cyclicPrefix

|  |  +--rw fDDBwpCommonCarrierFreqDL      uint16

|  |  +--rw fDDBwpCommonCarrierFreqUL      uint16

|  |  +--rw fDDBwpCommonChannelBwDL        uint16

|  |  +--rw fDDBwpCommonChannelBwUL        uint16

|  |  +--rw tDDBwpCommonCarrierFreq        uint16

|  |  +--rw tDDBwpCommonChannelBw          uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqDL   uint16

|  |  +--rw fDDBwpDedicatedCarrierFreqUL   uint16

|  |  +--rw fDDBwpDedicatedChannelBwDL     uint16

|  |  +--rw fDDBwpDedicatedChannelBwUL     uint16

|  |  +--rw tDDBwpDedicatedCarrierFreq     uint16

|  |  +--rw tDDBwpDedicatedChannelBw       uint16

|  |  +--rw NRSectorCarrier*                nrm-type:t_dn

|  +--rw NRSectorCarrier* [sectorEquipmentFunction]

|     +--ro configuredMaxTxPower       uint16

|     +--ro pointAArfcnDLFDD          uint32

|     +--ro pointAArfcnULFDD          uint32

|     +--ro pointAArfcnTDD            uint32

|     +--ro fDDBwpTxStartDL           uint16

|     +--ro fDDBwpTxStartUL           uint16

|     +--ro fDDBwpTxBwDL              uint16

|     +--ro fDDBwpTxBwUL              uint16

|     +--ro tDDBwpTxStart             uint16

|     +--ro tDDBwpTxBw                uint16

|     +--rw sectorEquipmentFunction    nrm-type:t_dn

+--rw GNBCUCPFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw gNBCUName           string

|  +--ro gNBId                uint32

|  +--ro gNBIdLength          uint32

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw (gNB-type)

|  |  +--:(gNB)

|  |  |  +--rw EP-NgC

|  |  |  |  +--ro objectClass       string

|  |  |  |  +--ro objectInstance    nrm-type:t_dn

|  |  |  |  +--rw id                nrm-type:t_dn

|  |  |  |  +--rw userLabel        string

|  |  |  |  +--rw farEndEntity     nrm-type:t_dn

|  |  |  |  +--rw localAddress

|  |  |  |  |  +--rw ip_address   inet:ip-address

|  |  |  |  |  +--rw vlan_id      uint16

|  |  |  |  +--rw remoteAddress    inet:ip-address

|  |  |  +--rw EP-XnC

|  |  |     +--ro objectClass       string

|  |  |     +--ro objectInstance    nrm-type:t_dn

|  |  |     +--rw id                nrm-type:t_dn

|  |  |     +--rw userLabel        string

|  |  |     +--rw farEndEntity     nrm-type:t_dn

|  |  |     +--rw localAddress

|  |  |     |  +--rw ip_address   inet:ip-address

|  |  |     |  +--rw vlan_id      uint16

|  |  |     +--rw remoteAddress    inet:ip-address

|  |  +--:(en-gNB)

|  |     +--rw EP-X2C

|  |        +--ro objectClass       string

|  |        +--ro objectInstance    nrm-type:t_dn

|  |        +--rw id                nrm-type:t_dn

|  |        +--rw userLabel        string

|  |        +--rw farEndEntity     nrm-type:t_dn

|  |        +--rw localAddress

|  |        |  +--rw ip_address   inet:ip-address

|  |        |  +--rw vlan_id      uint16

|  |        +--rw remoteAddress    inet:ip-address

|  +--rw EP_F1C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_E1

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw NRCellCU* [k_nCI]

|     +--rw k_nCI               string

|     +--ro objectClass          string

|     +--ro objectInstance       nrm-type:t_dn

|     +--ro id                   nrm-type:t_dn

|     +--rw userLabel           string

|     +--rw vnfParametersList

|     |  +--rw vnfInstanceId    string

|     |  +--rw vnfdId          string

|     |  +--rw flavourId       string

|     |  +--rw autoScalable    boolean

|     +--rw peeParametersList

|     |  +--rw siteIdentification    string

|     |  +--rw siteLatitude         decimal64

|     |  +--rw siteLongitude        decimal64

|     |  +--rw siteDescription       string

|     |  +--rw equipmentType         enumeration

|     |  +--rw environmentType       enumeration

|     |  +--rw powerInterface        enumeration

|     +--rw nCGI		t_NCI

|     +--rw pLMNId* [MCC]

|     |  +--rw MCC    t_mcc

|     |  +--rw MNC   t_mnc

|     +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|     +--rw rRMPolicy           string

+--rw GNBCUUPFunction

+--ro objectClass          string

+--ro objectInstance       nrm-type:t_dn

+--ro id                   nrm-type:t_dn

+--rw userLabel           string

+--rw vnfParametersList

|  +--rw vnfInstanceId    string

|  +--rw vnfdId          string

|  +--rw flavourId       string

|  +--rw autoScalable    boolean

+--rw peeParametersList

|  +--rw siteIdentification    string

|  +--rw siteLatitude         decimal64

|  +--rw siteLongitude        decimal64

|  +--rw siteDescription       string

|  +--rw equipmentType         enumeration

|  +--rw environmentType       enumeration

|  +--rw powerInterface        enumeration

+--ro gNBId                uint32

|  +--ro gNBIdLength                uint32

+--rw pLMNId* [MCC MNC]

|  +--rw MCC    t_mcc

|  +--rw MNC    t_mnc

+--rw EP_F1U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_E1

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw (gNB-type)

+--:(gNB)

|  +--rw EP_NgU

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_XnU

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--:(en-gNB)

+--rw EP_X2U

|  +--ro objectClass       string

|  +--ro objectInstance    nrm-type:t_dn

|  +--rw id                nrm-type:t_dn

|  +--rw userLabel        string

|  +--rw farEndEntity     nrm-type:t_dn

|  +--rw localAddress

|  |  +--rw ip_address   inet:ip-address

|  |  +--rw vlan_id      uint16

|  +--rw remoteAddress    inet:ip-address

+--rw EP_S1U

+--ro objectClass       string

+--ro objectInstance    nrm-type:t_dn

+--rw id                nrm-type:t_dn

+--rw userLabel        string

+--rw farEndEntity     nrm-type:t_dn

+--rw localAddress

|  +--rw ip_address   inet:ip-address

|  +--rw vlan_id      uint16

+--rw remoteAddress    inet:ip-address

## E.4.3	YANG schema

### E.4.3.1	General type definition "nrm-types-3gpp.yang"

module nrm-types-3gpp {

namespace "urn:3gpp:tsg:sa5:nrm:types";

prefix "nrm-type";

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

import ietf-yang-types { prefix yang; revision-date "2010-09-24"; }

revision 2018-07-31 {

description

"Initial revision.";

}

typedef t_dn {

type string;

description

"The dn type represents a Distinguish Name of the object.";

}

typedef t_mcc {

description "The mobile country code consists of three decimal digits, The first digit of the mobile country code identifies the geographic region (the digits 1 and 8 are not used):";

type string {

pattern '[02-79][0-9][0-9]';

}

}

typedef t_mnc {

description "the mobile network code consists of two or three decimal digits (for example: MNC of 001 is not the same as MNC of 01)";

type string {

pattern '[0-9][0-9][0-9]|[0-9][0-9]';

}

}

grouping pLMNId {

leaf MCC {

type t_mcc;

}

leaf MNC {

type t_mnc;

}

}

typedef t_NCI {

description "The NCI shall be of fixed length of 36 bits and shall be coded using full hexadecimal representation. The exact coding of the NCI is the responsibility of each PLMN operator";

reference "23.003";

type union {

type string {

length 36;

pattern '[01]*';

}

type string {

length 9;

pattern '[a-fA-F0-9]*';

}

}

}

typedef t_operationalState {

type enumeration {

enum "Enabled";

enum "Disabled";

}

}

typedef t_administrativeState {

type enumeration {

enum "Locked";

enum "Shutdown";

enum "Unlocked";

}

}

typedef t_availabilityStatus {

type enumeration {

enum "IN TEST";

enum "FAILED";

enum "POWER OFF";

enum "OFF LINE";

enum "OFF DUTY";

enum "DEPENDENCY";

enum "DEGRADED";

enum "NOT INSTALLED";

enum "LOG FULL";

}

}

typedef t_cellState {

type enumeration {

enum "Idle";

enum "Inactive";

enum "Active";

}

}

typedef t_s-NSSAI {

type union {

type uint8;

type uint32;

}

}

typedef t_SST {

type uint8;

}

typedef t_nRPCI {

type uint32;

}

typedef t_tAC {

type union {

type string {

length 4;

pattern '[a-fA-F0-9]*';

}

type string {

length 6;

pattern '[a-fA-F0-9]*';

}

}

}

typedef t_subcarrierSpacing {

type enumeration {

enum 15;

enum 30;

enum 60;

enum 120;

}

}

typedef t_cyclicPrefix {

type enumeration {

enum "Normal";

enum "Extended";

}

}

typedef t_aMF-Region-id {

type string {

length 8;

pattern '[01]*';

}

}

typedef t_aMF-Set-id {

type string {

length 10;

pattern '[01]*';

}

}

typedef t_aMF-Pointer {

type string {

length 6;

pattern '[01]*';

}

}

grouping aMFI {

leaf AMF-Region-id {

type t_aMF-Region-id;

}

leaf AMF-Set-id {

type t_aMF-Set-id;

}

leaf AMF-Pointer {

type t_aMF-Pointer;

}

}

typedef t_TransportProtocol {

type enumeration {

enum "TCP";

enum "STCP";

enum "UDP";

}

}

// type definition especially for core NF

typedef NFType {

type enumeration {

enum "NRF";

enum "UDM";

enum "AMF";

enum "SMF";

enum "AUSF";

enum "NEF";

enum "PCF";

enum "SMSF";

enum "NSSF";

enum "UDR";

enum "LMF";

enum "GMLC";

enum "5G_EIR";

enum "SEPP";

enum "UPF";

enum "N3IWF";

enum "AF";

enum "UDSF";

enum "BSF";

enum "CHF";

}

}

typedef NFStatus {

type enumeration {

enum "REGISTERED";

enum "SUSPENDED";

}

}

typedef NotificationType {

type enumeration {

enum "N1_MESSAGES";

enum "N2_INFORMATION";

enum "LOCATION_NOTIFICATION";

}

}

typedef NotificationEventType {

type enumeration {

enum "NF_REGISTERED";

enum "NF_DEREGISTERED";

enum "NF_PROFILE_CHANGED";

}

}

typedef DataSetId {

type enumeration {

enum "SUBSCRIPTION";

enum "POLICY";

enum "EXPOSURE";

enum "APPLICATION";

}

}

typedef UPInterfaceType {

type enumeration {

enum "N3";

enum "N6";

enum "N9";

}

}

typedef N1MessageClass {

type enumeration {

enum "5GMM";

enum "SM";

enum "LPP";

enum "SMS";

}

}

typedef N2InformationClass {

type enumeration {

enum "SM";

enum "NRPPa";

enum "PWS";

enum "PWS-BCAL";

enum "PWS-RF";

}

}

typedef t_Load {

description "Latest known load information of the NF ranged from 0 to 100 in percentage ";

type uint8 {

range 0..100;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping SupiRange {

leaf start {

type string;

}

leaf end {

type string;

}

leaf pattern {

type string;

}

}

grouping IdentityRange {

leaf start {

type string;

}

leaf end {

type string;

}

leaf pattern {

type string;

}

}

grouping Guami {

container plmnId { uses pLMNId; }

container amfId { uses aMFI; }

}

grouping Tai {

container plmnId { uses pLMNId; }

leaf tac { type t_tAC; }

}

typedef t_Dnn {

type string;

}

grouping DnnUpfInfoItem {

leaf dnn {

type t_Dnn;

}

}

grouping SnssaiUpfInfoItem {

leaf sNssai {

type t_s-NSSAI;

}

list dnnUpfInfo {

key dnn;

uses DnnUpfInfoItem;

}

}

grouping NFServiceVersion {

description "";

leaf apiVersionInUri {

description "";

type string;

}

leaf apiFullVersion {

type string;

description "";

}

leaf expiry {

description "";

type yang:date-and-time;

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type t_TransportProtocol;

}

leaf port {

type uint16;

}

}

grouping defaultNotificationSubscription {

leaf notificationType {

type NotificationType;

}

leaf callbackUri {

type inet:uri;

}

leaf n1MessageClass {

type N1MessageClass;

}

leaf n2InformationClass {

type N2InformationClass;

}

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

type UPInterfaceType;

}

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

leaf endpointFqdn {

type inet:domain-name;

}

}

leaf networkInstance {

type string;

}

}

grouping Ipv4AddressRange {

leaf start {

type inet:ipv4-address;

}

leaf end {

type inet:ipv4-address;

}

}

grouping Ipv6PrefixRange {

leaf start {

type inet:ipv6-prefix;

}

leaf end {

type inet:ipv6-prefix;

}

}

typedef t_NsiId {

type string;

}

typedef t_uEMobilityLevel {

type enumeration {

enum "stationary";

enum "nomadic";

enum "restricted mobility";

enum "fully mobility";

}

}

typedef t_resourceSharingLevel {

type enumeration {

enum "shared";

enum "not-shared";

}

}

typedef TxDirection {

type enumeration {

enum "DL";

enum "UL";

enum "DL and UL";

}

}

typedef BwpContext {

type enumeration {

enum "DL";

enum "UL";

enum "SUL";

}

}

typedef IsInitialBwp {

type enumeration {

enum "INITIAL";

enum "OTHER";

}

}

typedef t_quotaType {

type enumeration {

enum "strict";

enum "float";

}

}

grouping RRMPolicyRatio2{

leaf groupId {

type uint16;

}

leaf-list sNSSAI {

type t_s-NSSAI;

}

leaf quotaType {

type t_quotaType;

}

leaf rRMPolicyMaxRation {

type uint8;

}

leaf rRMPolicyMarginMaxRation {

type uint8;

}

leaf rRMPolicyMinRation {

type uint8;

}

leaf rRMPolicyMarginMinRation {

type uint8;

}

}

}

### E.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "Subnetwork.yang", “vsDataContainer.yang”, “MeContext.yang”, “ManagementNode.yang”

module Top {

namespace "urn:3gpp:tsg:sa5:nrm:Top";

prefix "top";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

organization "3gpp SA5";

description "root class to be inherited/reused by other child class including ME, MF, EP, etc.";

reference "28.622";

revision 2018-07-31 {

description "Initial revision";

}

grouping Top {

leaf objectClass {

mandatory "true";

config "false";

description "";

type string;

}

leaf objectInstance {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

description "";

}

}

module SubNetwork {

namespace "urn:3gpp:tsg:sa5:nrm:SubNetwork";

prefix "subnet";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

description "define basic SubNetwork which will be augmented by other IOCs";

revision 2018-07-31 {

description "Initial revision";

}

grouping SubNetwork {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "derived from domain_";

type string;

}

leaf userLabel {

description "derived from domain_";

mandatory "true";

config "true";

type string;

}

leaf userDefinedNetworkType {

description "derived from domain_";

mandatory "true";

config "true";

type string;

}

leaf-list setOfMcc {

min-elements "0";

description "";

config "false";

type nrm-type:t_mcc;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

container SubNetwork {

uses SubNetwork;

}

}

module ManagedElement {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedElement";

prefix "me";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

description "define basic ManagedElement which will be augmented by other IOCs";

revision 2018-07-31 {

description "Initial revision";

}

grouping ManagedElement {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf vendorName {

mandatory "true";

config "false";

description "";

type string;

}

leaf userDefinedState {

mandatory "true";

config "true";

description "";

type string;

}

leaf swVersion {

mandatory "true";

config "false";

description "";

type string;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "";

type string;

}

leaf userLabel {

description "";

mandatory "true";

config "true";

type string;

}

leaf locationName {

mandatory "true";

config "false";

type string;

}

leaf-list managedBy {

description "";

config "false";

type nrm-type:t_dn;

}

leaf-list managedElementTypeList {

min-elements "1";

config "false";

description "";

type string;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module ManagedFunction {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedFunction";

prefix "mf";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

description "common/basic MF class to be inherited/reused by other child MF classes ";

revision 2018-07-31 {

description "Initial revision";

}

grouping  ManagedFunction {

description "";

uses top:Top;

leaf id {

description "";

mandatory "true";

config "false";

type nrm-type:t_dn;

}

leaf userLabel {

mandatory "false";

config "true";

type string;

}

container vnfParametersList {

description "It shall be supported if the MF instance is realized by one or more VNF instance(s). Otherwise this attribute shall be absent ";

leaf vnfInstanceId {

mandatory "true";

type string;

}

leaf vnfdId  {

type string;

}

leaf flavourId  {

description "Identifier of the VNF Deployment Flavour applied to this VNF instance";

reference "clause 9.4.3 of ETSI GS NFV-IFA 008 ";

type string;

}

leaf autoScalable {

description "Indicator of whether the auto-scaling of this VNF instance is enabled or disabled. The type is Boolean";

type boolean;

}

}

container peeParametersList {

description "It shall be supported if the control and monitoring of PEE parameters is supported by the ManagedFunction or sub-class instance.";

leaf siteIdentification {

mandatory "true";

description "The identification of the site where the ManagedFunction resides.";

type string;

}

leaf siteLatitude  {

description "The latitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global referenceframe (WGS 84). Positive values correspond to the northern hemisphere. This attribute is optional in case of BTSFunction and RNCFunction  instance(s). allowedValues: -90.0000 to +90.0000 ";

when "contains(../../objectClass,'BTSFunction') or contains(../../objectClass,'RNCFunction')";

type decimal64 {

fraction-digits 4;

range "-90.0000 .. 90.0000";

}

}

leaf siteLongitude {

description "The longitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -180.0000to +180.0000 ";

when "contains(../../objectClass,'BTSFunction') or contains(../../objectClass,'RNCFunction')";

type decimal64 {

fraction-digits 4;

range "-180.0000 .. 180.0000";

}

}

leaf siteDescription {

mandatory "true";

description "An operator defined description of the site where the ManagedFunction instance resides.";

type string;

}

leaf equipmentType {

mandatory "true";

description "The type of equipment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. ";

type enumeration {

enum RRU;

enum BBU;

enum Wide-area-BS-cabinet;

enum Medium-range-BS;

}

}

leaf environmentType {

mandatory "true";

description "The type of environment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. ";

type enumeration {

enum Indoor;

enum Outdoor;

}

}

leaf powerInterface {

mandatory "true";

description "The type of power. allowedValues: see clause 4.4.1 of ETSI ES202 336-12 [18]. ";

type enumeration {

enum AC;

enum DC;

}

}

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module EP_RP {

namespace "urn:3gpp:tsg:sa5:nrm:EP_RP";

prefix "ep-rp";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

description "common/basic MF class to be inherited/reused by other child MF classes ";

revision 2018-07-31 {

description "Initial revision";

}

grouping local-address {

leaf ip_address {

type inet:ip-address;

}

leaf vlan_id {

type uint16;

}

}

grouping EP_RP {

description "";

uses top:Top;

leaf id {

description "";

mandatory "true";

type nrm-type:t_dn;

}

leaf userLabel {

description "";

mandatory "false";

type string;

}

leaf farEndEntity {

mandatory "false";

type nrm-type:t_dn;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

grouping EP_XX {

description "as all EPs defined in NGRAN and 5GC have same attributes, define a common EP for all endpoints for reusing";

uses EP_RP;

container localAddress  {

presence "true";

description "";

uses local-address;

}

leaf remoteAddress  {

description "";

mandatory "false";

type inet:ip-address;

}

}

grouping EP_E1 {

description "";

uses EP_XX;

}

grouping EP_F1C {

description "";

uses EP_XX;

}

grouping EP_F1U {

description "";

uses EP_XX;

}

grouping EP_XnC {

description "";

uses EP_XX;

}

grouping EP_XnU {

description "";

uses EP_XX;

}

grouping EP_NgC {

description "";

uses EP_XX;

}

grouping EP_NgU {

description "";

uses EP_XX;

}

grouping EP_X2C {

description "";

uses EP_XX;

}

grouping EP_X2U {

description "";

uses EP_XX;

}

grouping EP_S1U {

description "";

uses EP_XX;

}

grouping EP_N2 {

description "";

uses EP_XX;

}

grouping EP_N3 {

description "";

uses EP_XX;

}

grouping EP_N4 {

description "";

uses EP_XX;

}

grouping EP_N5 {

description "";

uses EP_XX;

}

grouping EP_N6 {

description "";

uses EP_XX;

}

grouping EP_N7 {

description "";

uses EP_XX;

}

grouping EP_N8 {

description "";

uses EP_XX;

}

grouping EP_N9 {

description "";

uses EP_XX;

}

grouping EP_N10 {

description "";

uses EP_XX;

}

grouping EP_N11 {

description "";

uses EP_XX;

}

grouping EP_N12 {

description "";

uses EP_XX;

}

grouping EP_N13 {

description "";

uses EP_XX;

}

grouping EP_N14 {

description "";

uses EP_XX;

}

grouping EP_N15 {

description "";

uses EP_XX;

}

grouping EP_N16 {

description "";

uses EP_XX;

}

grouping EP_N17 {

description "";

uses EP_XX;

}

grouping EP_N20 {

description "";

uses EP_XX;

}

grouping EP_N21 {

description "";

uses EP_XX;

}

grouping EP_N22 {

description "";

uses EP_XX;

}

grouping EP_N26 {

description "";

uses EP_XX;

}

grouping EP_S5C {

description "";

uses EP_XX;

}

grouping EP_S5U {

description "";

uses EP_XX;

}

grouping EP_Rx {

description "";

uses EP_XX;

}

grouping EP_MAP_SMSC {

description "";

uses EP_XX;

}

grouping EP_NLS {

description "";

uses EP_XX;

}

grouping EP_NLG {

description "";

uses EP_XX;

}

grouping EP_N27 {

description "";

uses EP_XX;

}

grouping EP_N31 {

description "";

uses EP_XX;

}

grouping EP_N32 {

description "";

uses EP_XX;

}

grouping EP_SBI_X {

description "It's used for the services consumed by the contol plane function. Far entity and remoteAdress are requied for this endpoint";

uses EP_XX;

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

grouping EP_SBI_IPX {

description "It's used for the SEPP services consumed by the contol plane function. Far entity and remoteAdress are requied for this endpoint";

uses EP_XX;

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

grouping sAP {

leaf dn {

type inet:domain-name;

}

leaf port {

type uint16;

}

}

grouping EP_NBI_X {

description "The service access point of the control plane function. It's used for the services (including SEPP service) provided by the contol plane function. Far entity and remoteAdress are uncessary for this endpoint ";

uses EP_RP;

container localSAP  {

presence "true";

description "";

uses sAP;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

}

}

module ManagementNode {

namespace "urn:3gpp:tsg:sa5:nrm:ManagedElement";

prefix "managementNode";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

import VsDataContainer {prefix vsData; revision-date "2018-12-18"; }

organization "3gpp SA5";

revision 2018-12-18 {

description "15.1.0";

}

container ManagementNode {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf userLabel {

description "";

mandatory "true";

config "true";

type string;

}

leaf-list managedElements {

description "";

config "false";

type nrm-type:t_dn;

}

leaf vendorName {

mandatory "true";

config "false";

description "";

type string;

}

leaf locationName {

mandatory "true";

config "false";

type string;

}

leaf userDefinedState {

mandatory "true";

config "true";

description "";

type string;

}

leaf swVersion {

mandatory "true";

config "false";

description "";

type string;

}

list vsDataContainer {

min-elements "0";

uses vsData:VsDataContainer;

}

}

}

module MeContext {

namespace "urn:3gpp:tsg:sa5:nrm:MeContext";

prefix "meContext";

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import Top { prefix top; revision-date "2018-07-31"; }

organization "3gpp SA5";

revision 2018-12-18 {

description "Initial Version";

}

container MeContext {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf dnPrefix {

mandatory "true";

config "false";

description "derived from domain_";

type string;

}

}

}

module VsDataContainer {

namespace "urn:3gpp:tsg:sa5:nrm::VsDataContainer";

prefix "vsData";

import Top { prefix top; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

revision 2018-12-18 {

description "Initial revision";

}

grouping VsDataContainer {

uses top:Top;

leaf id {

mandatory "true";

config "false";

description "derived from top_";

type nrm-type:t_dn;

}

leaf vsDataType {

mandatory "true";

config "true";

description "";

type string;

}

leaf vsDataFormatVersion {

mandatory "true";

config "false";

description "";

type string;

}

anydata vsData;

list vsDataContainer {

min-elements "0";

uses VsDataContainer;

}

}

}

### E.4.3.3	NGRAN, composition of all gNB ManagedFunctions in "ngran.yang"

module ngran {

namespace "urn:3gpp:tsg:sa5:nrm:ngran";

prefix "ngran";

import ManagedElement { prefix me; revision-date "2018-07-31"; }

include ngran-nRCellCU;

include ngran-nRCellDU;

include ngran-nRSectorCarrier;

include ngran-gNBDUFunction;

include ngran-gNBCUCPFunction;

include ngran-gNBCUUPFunction;

organization "3gpp SA5";

description "Main YANG module for the NRM NG-RAN Defined gNB as ManagedElement

(subclass of ME) for overaching all other functions supported

ngran Define constitued MFs and EPs as container in submodule";

revision 2018-07-27 {

description "15.0.2";

}

container ManagedElement {

description "list constituted MF and EP below, let's discuss if umbrella ManagedElement needed for NR

if it's not required, we can just remove the me and keep the container below";

uses me:ManagedElement;

list GNBDUFunction { uses GNBDUFunction; }

list GNBCUCPFunction { uses GNBCUCPFunction; }

list GNBCUUPFunction { uses GNBCUUPFunction; }

}

}

### E.4.3.4	NGRAN submodule, "ngran-gNBCUCPFunction.yang"

submodule ngran-gNBCUCPFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include ngran-nRCellCU;

description "GNBCUCPFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBCUCPFunction {

uses mf:ManagedFunction;

leaf gNBCUName {

description "PrintableString(SIZE(1..150,))";

mandatory "false";

config "true";

type string {

length "1..150";

}

}

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range "22..32";

}

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

choice gNB-type {

case gNB {

list EP-NgC { uses ep-rp:EP_NgC; }

list EP-XnC { uses ep-rp:EP_XnC; }

}

case en-gNB {

container EP-X2C { uses ep-rp:EP_X2C; }

}

}

list EP_F1C { uses ep-rp:EP_F1C; }

list EP_E1 { uses ep-rp:EP_E1; }

list NRCellCU {

uses NRCellCU;

}

}

}

### E.4.3.5	NGRAN submodule, "ngran-gNBCUUPFunction.yang"

submodule ngran-gNBCUUPFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "GNBCUUPFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBCUUPFunction {

uses mf:ManagedFunction;

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range "22..32";

}

}

list pLMNId {

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_F1U { uses ep-rp:EP_F1U; }

container EP_E1 { uses ep-rp:EP_E1; }

choice gNB-type {

case gNB {

list EP_NgU { uses ep-rp:EP_NgU; }

list EP_XnU { uses ep-rp:EP_XnU; }

}

case en-gNB {

list EP_X2U { uses ep-rp:EP_X2U; }

list EP_S1U { uses ep-rp:EP_S1U; }

}

}

}

}

### E.4.3.6	NGRAN submodule, "ngran-gNBDUFunction.yang"

submodule ngran-gNBDUFunction {

belongs-to ngran { prefix ngran; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

include ngran-nRCellDU;

include ngran-nRSectorCarrier;

description "GNBDUFunction derived from basic ManagedFunction, for 3 split deployment mode only";

revision 2018-08-02 {

description "15.0.2";

}

grouping GNBDUFunction {

uses mf:ManagedFunction;

leaf gNBDUId {

description "It uniquely identifies the DU at least within a gNB. See gNB-DU ID in subclause 9.3.1.9 of 3GPP TS 38.473 ";

mandatory "true";

config "true";

type uint32;

}

leaf gNBDUName {

description "PrintableString(SIZE(1..150,))";

mandatory "false";

config "true";

type string {

length "1..150";

}

}

leaf gNBId {

description "It identifies a gNB within a PLMN. See gNB Identifier (gNB ID) of subclause 8.2 of 3GPP TS 38.300 [3]). See Global gNB ID in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. ";

mandatory "true";

config "false";

type uint32;

}

leaf gNBIdLength {

description "It indicates the number of bits for encoding the gNB ID.";

mandatory "true";

config "false";

type uint32 {

range " 22..32";

}

}

container EP_F1C { uses ep-rp:EP_F1C; }

list EP_F1U { uses ep-rp:EP_F1U; }

list NRCellDU {

uses NRCellDU;

}

list NRSectorCarrier {

uses NRSectorCarrier;

}

list BWP {

uses BWP;

}

}

}

### E.4.3.7	NGRAN submodule, "ngran-nRCellCU.yang"

submodule ngran-nRCellCU {

belongs-to ngran { prefix ngran; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRCellCU derived from basic ManagedFunction";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRCellCU {

uses mf:ManagedFunction;

leaf nCI {

config "true";

description "It uniquely identifies a NR cell within a PLMN.";

reference "See NR Cell identifier (NCI) subclause 8.2 of 3GPP

TS 38.300 [3].";

type nrm-type:t_NCI;

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key MCC;

uses nrm-type:pLMNId;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPPTS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf rRMPolicyType {

config "true";

type uint16;

}

leaf rRMPolicyNSSIId {

config "true";

type nrm-type:t_dn;

}

leaf rRMPolicyRatio {

config "true";

type uint8 {	range 0..100; }

}

leaf rRMPolicy {

description "It represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent.";

config "true";

type string;

}

container rRMPolicyRatio2 {

presence "true";

config "true";

uses nrm-type:RRMPolicyRatio2;

}

}

}

### E.4.3.8	NGRAN submodule, "ngran- nRCellDU.yang"

submodule ngran-nRCellDU {

belongs-to ngran { prefix ngran; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "NRCellDU derived from basic ManagedFunction";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRCellDU {

uses mf:ManagedFunction;

leaf nCI {

config "true";

description "It uniquely identifies a NR cell within a PLMN.";

reference "See NR Cell identifier (NCI) subclause 8.2 of 3GPP TS 38.300 [3].";

type nrm-type:t_NCI;

}

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key MCC;

uses nrm-type:pLMNId;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

type nrm-type:t_s-NSSAI;

}

leaf operationalState  {

config "false";

description "CM, It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled).";

type nrm-type:t_operationalState;

}

leaf administrativeState  {

description "CM, It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services.";

config "true";

type nrm-type:t_administrativeState;

}

leaf availabilityStatus  {

config "false";

description "CM";

type nrm-type:t_availabilityStatus;

}

leaf cellState  {

description "It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active)";

mandatory "true";

config "false";

type nrm-type:t_cellState;

}

leaf nRPCI {

description "";

mandatory "true";

config "true";

type nrm-type:t_nRPCI;

}

leaf nRTAC {

mandatory "true";

config "true";

type nrm-type:t_tAC;

}

leaf arfcnDL {

mandatory "true";

config "true";

type uint32;

}

leaf arfcnUL {

config "true";

type uint32;

}

leaf arfcnSUL {

config "true";

type uint32;

}

leaf bSChannelBwDL {

mandatory "true";

config "true";

type uint32;

}

leaf bSChannelBwUL {

config "true";

type uint32;

}

leaf bSChannelBwSUL {

config "true";

type uint32;

}

//refer to nRSectorCarrier

leaf-list NRSectorCarrier {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

//refer to bwp

leaf-list bWP {

min-elements "0";

description "";

config "true";

type nrm-type:t_dn;

}

}

}

### E.4.3.9	NGRAN submodule, "ngran-nRSectorCarrier.yang"

submodule ngran-nRSectorCarrier {

belongs-to ngran { prefix ngran; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

description "NRSectorCarrier";

revision 2018-08-02 {

description "Initial revision";

}

grouping NRSectorCarrier {

description "";

uses mf:ManagedFunction;

leaf txDirection {

mandatory "true";

config "true";

type nrm-type:TxDirection;

}

leaf configuredMaxTxPower {

mandatory "true";

config "false";

description "";

type uint16;

}

leaf arfcnDL {

config "true";

type uint32;

}

leaf arfcnUL {

config "true";

type uint32;

}

leaf bSChannelBwDL {

config "true";

type uint32;

}

leaf bSChannelBwUL {

config "true";

type uint32;

}

leaf sectorEquipmentFunction {

mandatory "true";

config "true";

description "fqBand for NR should be defined";

reference "23.622";

type nrm-type:t_dn;

}

}

}

### E.4.3.10	NGRAN submodule, "ngran-bWP.yang"

submodule ngran-bWP {

belongs-to ngran { prefix ngran; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

description "BWP";

revision 2018-09-28 {

description "Initial revision";

}

grouping BWP {

description "";

uses mf:ManagedFunction;

leaf bwpContext {

mandatory "true";

config "true";

type nrm-type:BwpContext;

}

leaf isInitialBwp {

mandatory "true";

config "false";

description "";

type nrm-type:IsInitialBwp;

}

leaf subCarrierSpacing {

mandatory "true";

config "true";

type uint32;

}

leaf cyclicPrefix {

mandatory "true";

config "true";

type nrm-type:t_cyclicPrefix;

}

leaf startRB {

mandatory "true";

config "true";

type uint32;

}

leaf numberOfRBs {

mandatory "true";

config "true";

type uint32;

}

}

}

Annex F (normative):
XML definitions for 5GC NRM

# F.1	General

This annex contains the XML definitions for the 5GC NRM, in accordance with 5G NRM Information Model definitions specified in clause 4.

# F.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# F.3	Mapping

## F.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## F.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# F.4	Solution Set definitions

## F.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

The present document defines the NRM-specific XML schema ngcNrm.xsd for the 5GC NRM Information Model defined in clause 4.

XML schema ngcNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## F.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## F.4.3	XML schema "ngcNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 5GC Network Resource Model

XML schema definition

ngcNrm.xsd

-->

<schema

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"     xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<!--NGC NRM IM class associated XML elements -->

<complexType name="aMFIdentifier">

<sequence>

<element name="amfRegionId" type="ngc:AmfRegionId"/>

<element name="amfSetId" type="ngc:AmfSetId"/>

<element name="amfPointer" type="ngc:AmfPointer"/>

</sequence>

</complexType>

<simpleType name="AmfRegionId">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- The AMF Region ID is 8-bitslength, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfSetId">

<restriction base="integer">

<maxInclusive value="1023"/>

<!-- The AMF Region ID is 10-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfPointer">

<restriction base="integer">

<maxInclusive value="63"/>

<!-- The AMF Pointer is 6-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<complexType name="NrTACList">

<sequence>

<element name="tac" type="nn:NrTac" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="NFProfileList">

<sequence>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NfProfile">

<sequence>

<element name="nfInstanceId" type="string"/>

<!-- nfInstanceId is uuid of NF instance -->

<element name="nfType" type="ngc:NfType"/>

<element name="nfStatus" type="ngc:NfStatus"/>

<element name="plmn" type="en:PLMNId"/>

<element name="nfType" type="ngc:NfType"/>

<element name="sNssais" type="ngc: SnssaiList"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="fqdn" type="string"/>

<element name="ipv4Addresses" type="string"/>

<element name="ipv6Addresses" type="string"/>

<element name="ipv6Prefixes" type="string"/>

<element name="capacity" type="string"/>

<element name="udrInfo" type="ngc:UdrInfo"/>

<element name="amfInfo" type="ngc:AmfInfo"/>

<element name="smfInfo" type="ngc:SmfInfo"/>

<element name="upfInfo" type="ngc:UpfInfo"/>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<complexType name="NfServices">

<sequence>

<element name="serviceInstanceId" type="string"/>

<element name="serviceName" type="string"/>

<element name="version" type="string"/>

<element name="schema" type="string"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="ipEndPoints" type="ngc:IpEndpoints"/>

<element name="apiPrefix" type="string"/>

<element name="defaultNotificationSubscriptions" type="ngc:DefaultNotificationSubscriptions"/>

<element name="allowedPlmns" type="en:PLMNIdList"/>

<element name="allowedNfTypes" type="ngc:NFTypeList"/>

<element name="allowedNssais" type="ngc:Nssai"/>

<element name="capacity" type="string"/>

<element name="supportedFeatures" type="string"/>

</sequence>

</complexType>

<simpleType name="NfType">

<restriction base="string">

<!-- NF name is defined in TS 23.501 -->

<enumeration value="NRF"/>

<enumeration value="UDM"/>

<enumeration value="AMF"/>

<enumeration value="SMF"/>

<enumeration value="AUSF"/>

<enumeration value="NEF"/>

<enumeration value="PCF"/>

<enumeration value="SMSF"/>

<enumeration value="NSSF"/>

<enumeration value="UDR"/>

<enumeration value="LMF"/>

<enumeration value="GMLC"/>

<enumeration value="5GEIR"/>

<enumeration value="SEPP"/>

<enumeration value="UPF"/>

<enumeration value="N3IWF"/>

<enumeration value="AF"/>

<enumeration value="UDSF"/>

<enumeration value="DN"/>

</restriction>

</simpleType>

<complexType name="NFTypeList">

<sequence>

<element name="NFType" type="ngc:NfType"/>

</sequence>

</complexType>

<complexType name="IpEndpoints">

<sequence>

<element name="ipEndPoint" type="nn:IpEndPoint"/>

</sequence>

</complexType>

<complexType name="UdrInfo">

<sequence>

<element name="supiRange" type="ngc:SupiRange"/>

</sequence>

</complexType>

<complexType name="SupiRange">

<sequence>

<element name="start" type="string"/>

<element name="end" type="string"/>

<element name="pattern" type="string"/>

</sequence>

</complexType>

<complexType name="AmfInfo">

<sequence>

<element name="amfSetId" type="ngc:AmfSetId"/>

</sequence>

</complexType>

<complexType name="SmfInfo">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="UpfInfo">

<sequence>

<element name="snssaiUpfInfo" type="ngc:SnssaiUpfInfo"/>

</sequence>

</complexType>

<complexType name="SnssaiUpfInfo">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

<element name="dnnUpfInfoList" type="ngc:DnnUpfInfoList"/>

</sequence>

</complexType>

<complexType name="DnnUpfInfoList">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="DefaultNotificationSubscription">

<sequence>

<element name="notificationType" type="ngc:NotificationType"/>

<element name="callbackUri" type="string"/>

<element name="n1MessageClass" type="string"/>

<element name="n2InformationClass" type="string"/>

</sequence>

</complexType>

<simpleType name="NotificationType">

<restriction base="string">

<enumeration value="N1_MESSAGES"/>

<enumeration value="N2_INFORMATION"/>

<enumeration value="LOCATION_NOTIFICATION"/>

</restriction>

</simpleType>

<simpleType name="TransportProtocol">

<restriction base="string">

<enumeration value="TCP"/>

</restriction>

</simpleType>

<simpleType name="NfStatus">

<restriction base="string">

<enumeration value="REGISTERED"/>

<enumeration value="SUSPENDED"/>

</restriction>

</simpleType>

<complexType name="NfRegistrationData">

<sequence>

<element name="heartBeatTimer" type="integer"/>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NSILdList">

<sequence>

<element name="nSIId" type="string"/>

<!-- NSI Id is defined in TS 29.531 -->

</sequence>

</complexType>

<complexType name="SnssaiList">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

</sequence>

</complexType>

<complexType name="SNssai">

<sequence>

<element name="sst" type="ngc:Sst" minOccurs="0"/>

<element name="sd" type="ngc:Sd"/>

</sequence>

</complexType>

<simpleType name="Sst">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- SST is 1-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<simpleType name="Sd">

<restriction base="integer">

<maxInclusive value="65535"/>

<!-- SST is 2-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<complexType name="SBIServiceList">

<sequence>

<element name="nfServices" type="ngc:NfServices"/>

</sequence>

</complexType>

<simpleType name="WeightFactor">

<restriction base="integer">

</restriction>

</simpleType>

<element name="AMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="aMFIdentifier" type="ngc:aMFIdentifier"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="weightFactor" type="ngc:WeightFactor"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="aMFSet" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N14"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N17"/>

<element ref="ngc:EP_N22"/>

<element ref="ngc:EP_N26"/>

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_NLS"/>

<element ref="ngc:EP_NLG"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_S5C"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UPFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N3"/>

<element ref="ngc:EP_N9"/>

<element ref="ngc:EP_S5U"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="ngc:EP_N6"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="N3IWFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N3"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="PCFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

<element name="sBIFqdn" type="string" />

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_N5"/>

<element ref="ngc:EP_Rx"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AUSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDMFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N13"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList" minOccurs="0"/>

<element name="nFProfileList" type="ngc:NFProfileList" minOccurs="0"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NSSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="nSIIdList" type="ngc:NSIIdList"/>

<element name="snssaiList" type="ngc: SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_N31"/>

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMSFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_N21"/>

<element ref="ngc:EP_MAP_SMSC"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="LMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_NLS"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NGEIRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N17"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SEPPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N32"/>

<element ref="ngc:EP_SBI_IPX"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NWDAFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="sBISerivceList" type="ngc:SBIServiceList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_SBI_X"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N2">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N3">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N4">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N5">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N6">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N7">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N8">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N9">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N10">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N11">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N12">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N13">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N14">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N15">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N16">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N17">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N20">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N21">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N22">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N26">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N27">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N31">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N32">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_X">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_SBI_IPX">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Rx">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_MAP_SMSC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLS">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLG">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:IpEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:IpEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex G (normative):
JSON definitions for 5GC NRM

# G.1	General

This annex contains the JSON definitions for the 5GC NRM, in accordance with 5GC NRM information model definitions specified in clause 5.

# G.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 5, this annex specifies features that are specific to the Schema definitions.

# G.3	Mapping

## G.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| AMFFunction | amfFunction | object |
| SMFFunction | smfFunction | object |
| UPFFunction | upfFunction | object |
| N3IWFunction | n3wifFunction | object |
| PCFFunction | pcrfFunction | object |
| AUSFFunction | ausfunction | object |
| UDMFunction | udmFunction | object |
| UDRFunction | udrFunction | object |
| UDSFFunction | udsfFunction | object |
| NRFFunction | nrfFunction | object |
| NSSFFunction | nssfFunction | object |
| SMSFFunction | smsfFunction | object |
| LMFFunction | lmfFunction | object |
| NGEIRFunction | ngeirFunction | object |
| SEPPFunction | seppFunction | object |
| NWDAFFunction | nwdafFunction | object |
| EP_N2 | eP_N2 | object |
| EP_N3 | eP_N3 | object |
| EP_N4 | eP_N4 | object |
| EP_N5 | eP_N5 | object |
| EP_N6 | eP_N6 | object |
| EP_N7 | eP_N7 | object |
| EP_N8 | eP_N8 | object |
| EP_N9 | eP_N9 | object |
| EP_N10 | eP_N10 | object |
| EP_N11 | eP_N11 | object |
| EP_N12 | eP_N12 | object |
| EP_N13 | eP_N13 | object |
| EP_N14 | eP_N14 | object |
| EP_N15 | eP_N15 | object |
| EP_N16 | eP_N16 | object |
| EP_N17 | eP_N17 | object |
| EP_N20 | eP_N20 | object |
| EP_N21 | eP_N21 | object |
| EP_N22 | eP_N22 | object |
| EP_N26 | eP_N26 | object |
| EP_SBI_X | eP_SBI_X | object |
| EP_SBI_IPX | eP_SBI_IPX | object |
| EP_S5C | eP_S5C | object |
| EP_S5U | eP_S5U | object |
| EP_Rx | eP_Rx | object |
| EP_MAP_SMSC | eP_MAP_SMSC | object |
| EP_NLC | eP_NLC | object |
| EP_NLG | eP_NLG | object |
| EP_N27 | eP_N27 | object |
| EP_N31 | eP_N31 | object |
| ExternalNRFFunction | externalNrfFunction | object |
| ExternalNSSFFunction | externalNssfFunction | object |
| AMFSet | amfSet | object |
| AMFRegion | amfRegion | object |
| ExternalAMFFunction | externalAmfFunction | object |

## G.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| aMFIdentifier | amfIdentifier | AmfIdentifier |
| aMFSetId | amfSetId | AmfSetId |
| aMFRegionId | amfRegionId | AmfRegionId |
| localAddress | localAddress | nrNrm:IPEndPoint |
| remoteAddress | remoteAddress | nrNrm:IPEndPoint |
| nfProfileList | nfProfileList | NfProfileList |
| nSIIdList | nSIIdList | NSIIdList |
| sNSSAIList | snssaiList | nrNrm:SnssaiList |
| pLMNIdList | plmnIdList | nrNrm:PlmnIdList |
| sBIFQDN | sBIFQDN | SBIFqdn |
| sBIServiceList | sBIServiceList | SBIServiceList |
| nRTACList | tACList | TACList |
| weightFactor | weightFactor | WeightFactor |

# G.4	Solution Set definitions

## G.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## G.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## G.4.3	JSON schema "ngcNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#",

"id": "http://3gpp.org/28541/ngcNrm.json",

"description": "JSON based solution set definitions for 5GC",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"definitions": {

"amfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/definitions/AmfIdentifier"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"weightFactor": {

"$ref": "#/definitions/WeightFactor"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"amfSet": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

}

},

"required": [

"userLabel",

"plmnIdList",

"amfIdentifier",

"sBIFqdn",

"sBIServiceList",

"weightFactor"

]

},

"smfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList",

"sBIFqdn",

"sBIServiceList"

]

},

"upfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"nRTACList"

]

},

"n3iwfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"pcfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"ausfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udmFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udrFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"udsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"nrfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"nssfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/definitions/NSIIdList"

},

"nFProfileList": {

"$ref": "#/definitions/NFProfileList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn"

]

},

"smsfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"lmfFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList"

]

},

"ngeirFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"seppFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

}

},

"required": [

"userLabel",

"plmnIdList",

]

},

"nwdafFunction": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"vnfParametersList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/VnfParametersList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBISerivceList": {

"$ref": "#/definitions/SBIServiceList"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"userLabel",

"plmnIdList",

"sBIFqdn",

"sBIServiceList"

]

},

"eP_N2": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N3": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N4": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N5": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N6": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N7": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N8": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N9": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N10": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N11": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N12": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N13": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N14": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N15": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N16": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N17": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N20": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N21": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N22": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn "

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_N26": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_X": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_SBI_IPX": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5C": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_S5U": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_Rx": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_MAP_SMSC": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLS": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"eP_NLG": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"farEndEntity": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"localAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

},

"remoteAddress": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/IPEndPoint"

}

}

},

"amfSet": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfSetId"

]

},

"amfRegion": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"nRTACList": {

"$ref": "#/definitions/TACList"

},

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

}

},

"required": [

"plmnIdList",

"nRTACList",

"amfRegionId"

]

},

"AmfIdentifier": {

"type": "object",

"description": "AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer",

"properties": {

"amfRegionId": {

"$ref": "#/definitions/AmfRegionId"

},

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

},

"amfPointer": {

"$ref": "#/definitions/AmfPointer"

}

}

},

"AmfRegionId": {

"type": "integer",

"description": "AmfRegionId is defined in TS 23.003",

"maximum": 255

},

"AmfSetId": {

"type": "string",

"description": "AmfSetId is defined in TS 23.003",

"maximum": 1023

},

"AmfPointer": {

"type": "integer",

"description": "AmfPointer is defined in TS 23.003",

"maximum": 63

},

"nFProfileList": {

"type": "array",

"description": "List of NF profile",

"items": {

"$ref": "#/definitions/NFProfile"

}

},

"NFProfile": {

"type": "object",

"description": "NF profile stored in NRF, defined in TS 29.510",

"properties": {

"nFInstanceId": {

"type": "string",

"description": "uuid of NF instance"

},

"nFType": {

"$ref": "#/definitions/NFType"

},

"nFStatus": {

"$ref": "#/definitions/NFStatus"

},

"plmn": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"sNssais": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/Snssai"

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipv4Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv4Addr"

}

},

"ipv6Addresses": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Addr"

}

},

"ipv6Prefixes": {

"type": "array",

"item": {

"$ref": "#/definitions/Ipv6Prefix"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"udrInfo": {

"$ref": "#/definitions/UdrInfo"

},

"amfInfo": {

"$ref": "#/definitions/amfInfo"

},

"smfInfo": {

"$ref": "#/definitions/SmfInfo"

},

"upfInfo": {

"$ref": "#/definitions/UpfInfo"

},

"nfServices": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

}

}

},

"NFService": {

"type": "object",

"description": "NF Service is defined in TS 29.510",

"properties": {

"serviceInstanceId": {

"type": "string",

},

"serviceName": {

"type": "string",

},

"version": {

"type": "string",

},

"schema": {

"type": "string",

},

"fqdn": {

"$ref": "#/definitions/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/definitions/Fqdn"

},

"ipEndPoints": {

"type": "array",

"item": {

"$ref": "#/definitions/IpEndPoint"

}

},

"apiPrfix": {

"type": "string",

},

"defaultNotificationSubscriptions": {

"type": "array",

"item": {

"$ref": "#/definitions/DefaultNotificationSubscriptions"

}

},

"allowedPlmns": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnId"

},

"allowedNfTypes": {

"type": "array",

"item": {

"$ref": "#/definitions/NFType"

}

},

"allowedNssais": {

"type": "array",

"item": {

"$ref": "#/definitions/Snssai"

}

},

"capacity": {

"$ref": "#/definitions/Capacity"

},

"supportedFeatures": {

"$ref": "#/definitions/SupportedFeatures"

}

}

},

"NFType": {

"type": "string",

"description": " NF name defined in TS 23.501",

"enum": [

"NRF",

"UDM",

"AMF",

"SMF",

"AUSF",

"NEF",

"PCF",

"SMSF",

"NSSF",

"UDR",

"LMF",

"GMLC",

"5G_EIR",

"SEPP",

"UPF",

"N3IWF",

"AF",

"UDSF",

"DN"

]

},

"Fqdn": {

"type": "string"

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "#/definitions/Ipv4Addr"

},

"ipv6Address": {

"$ref": "#/definitions/Ipv6Addr"

},

"ipv6Prefix": {

"$ref": "#/definitions/Ipv6Prefix"

},

"transport": {

"$ref": "#/definitions/TransportProtocol"

},

"port": {

"type": "integer"

}

}

},

"UdrInfo": {

"type": "object",

"properties": {

"supiRanges": {

"type": "array",

"items": {

"$ref": "#/definitions/SupiRange"

}

}

}

},

"SupiRange": {

"type": "object",

"properties": {

"start": {

"type": "string"

},

"end": {

"type": "string"

},

"pattern": {

"type": "string"

}

}

},

"AmfInfo": {

"type": "object",

"properties": {

"amfSetId": {

"$ref": "#/definitions/AmfSetId"

}

}

},

"SmfInfo": {

"type": "object",

"properties": {

"dnnList": {

"type": "array",

"items": {

"$ref": "#/definitions/Dnn"

}

}

}

},

"UpfInfo": {

"type": "object",

"properties": {

"sNssaiUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/SnssaiUpfInfoItem"

}

}

}

},

"SnssaiUpfInfoItem": {

"type": "object",

"properties": {

"sNssai": {

"$ref": "#/definitions/Snssai"

},

"dnnUpfInfoList": {

"type": "array",

"items": {

"$ref": "#/definitions/DnnUpfInfoListInfo"

}

}

}

},

"DnnUpfInfoItem": {

"type": "object",

"properties": {

"dnn": {

"$ref": "#/definitions/Dnn"

}

}

},

"DefaultNotificationSubscription": {

"type": "object",

"properties": {

"notificationType": {

"$ref": "#/definitions/NotificationType"

},

"callbackUri": {

"$ref": "#/definitions/Uri"

},

"n1MessageClass": {

"$ref": "#/definitions/N1MessageClass"

},

"n2InformationClass": {

"$ref": "#/definitions/n2InformationClass"

}

}

},

"NotificationType": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"N1_MESSAGES",

"N2_INFORMATION",

"LOCATION_NOTIFICATION"

]

},

"TranportProtocol": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"TCP"

]

},

"NFStatus": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"REGISTERED",

"SUSPENDED"

]

},

"NFRegistrationData": {

"type": "object",

"properties": {

"heartBeatTimer": {

"type": "integer"

},

"nfProfile": {

"$ref": "#/definitions/NFProfile"

}

}

},

"NSIIdList": {

"type": "array",

"items": {

"nSIId": {

"$ref": "#/definitions/NSIId"

}

}

},

"NSIId": {

"type": "string",

"description": "NSI Id is defined in TS 29.531"

},

"NrCellId": {

"type": "string",

},

"SBIFqdn": {

"$ref": "#/definitions/Fqdn"

},

"SBIServiceList": {

"type": "array",

"item": {

"$ref": "#/definitions/NFService"

}

},

"TACList": {

"type": "array",

"item": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/NrTac"

}

},

"weightFactor": {

"type": "integer"

}

}

}

Annex H (normative):
YANG definitions for 5GC

# H.1	General

This annex contains the YANG definitions for the 5GC NRM, in accordance with 5GC information model definitions specified in clause 4.

# H.2	Architectural features

# H.3	Mapping

## H.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| SubNetwork | SubNetwork | Container in module |
| ManagedElement | ManagedElement | Container in module |
| AMFFunction | AMFFunction | Container in submodule |
| SMFFunction | SMFFunction | Container in submodule |
| UPFFunction | UPFFunction | Container in submodule |
| N3IWFFunction | N3IWFFunction | Container in submodule |
| PCFFunction | PCFFunction | Container in submodule |
| AUSFFunction | AUSFFunction | Container in submodule |
| UDMFunction | UDMFunction | Container in submodule |
| UDRFunction | UDRFunction | Container in submodule |
| UDSFFunction | UDSFFunction | Container in submodule |
| NRFFunction | NRFFunction | Container in submodule |
| NSSFFunction | NSSFFunction | Container in submodule |
| SMSFFunction | SMSFFunction | Container in submodule |
| LMFFunction | LMFFunction | Container in submodule |
| NGEIRFunction | NGEIRFunction | Container in submodule |
| SEPPFunction | SEPPFunction | Container in submodule |
| NWDAFFunction | NWDAFFunction | Container in submodule |
| EP_*** | EP_*** | Reusable container in module |

## H.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| Simple Type Attributes | Same to the Name in Stage 2 | leaf |
| Complex Type Attribute | Same to the Name in Stage 2 | Reusable Container |
| Simple Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | leaf-list |
| Complex Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | list |

# H.4	Solution Set definitions

## H.4.1	YANG definition structure

## H.4.2	Graphical representation

module: ngc

+--rw SubNetwork

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--rw ManagedElement

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro vendorName                string

+--rw userDefinedState          string

+--ro swVersion                 string

+--ro dnPrefix                  string

+--rw userLabel                 string

+--ro locationName              string

+--ro managedBy*                nrm-type:t_dn

+--ro managedElementTypeList*   string

+--rw AMFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw aMFIdentifier

|  |  +--rw AMF-Region-id   t_aMF-Region-id

|  |  +--rw AMF-Set-id      t_aMF-Set-id

|  |  +--rw AMF-Pointer     t_aMF-Pointer

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw weightFactor         uint8

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N2

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N8

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N11

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N12

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N14

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N15

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N17

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N20

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N22

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N26

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_NLS

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_NLG

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw SMFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--ro tAC*                 nrm-type:t_tAC

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N4

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N7

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N10

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N11

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N16

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_S5C

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw UPFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--ro tAC*                 nrm-type:t_tAC

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N3

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N4

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N9

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N6

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_S5U

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--rw N3IWFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw EP_N2

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N3

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--rw PCFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N5

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N7

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N15

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N16

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_Rx

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw AUSFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N12

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N13

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw UDMFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N8

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N10

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N13

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw UDRFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw UDSFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw NRFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw nFProfile* [nfInstanceId]

|  |  +--rw nfInstanceId     string

|  |  +--rw nfType           nrm-type:NFType

|  |  +--rw nfStatus         nrm-type:NFStatus

|  |  +--rw plmn

|  |  |  +--rw MCC   t_mcc

|  |  |  +--rw MNC   t_mnc

|  |  +--rw sNssais*         nrm-type:t_s-NSSAI

|  |  +--rw nsi*             string

|  |  +--rw fqdn            inet:domain-name

|  |  +--rw interPlmnFqdn   inet:domain-name

|  |  +--rw ipv4Address*     inet:ipv4-address

|  |  +--rw ipv6Address*     inet:ipv6-address

|  |  +--rw ipv6Prefix*      inet:ipv6-prefix

|  |  +--rw capacity        uint8

|  |  +--rw load            nrm-type:t_Load

|  |  +--rw locality        string

|  |  +--rw priority        uint8

|  |  +--rw udrInfo

|  |  |  +--rw groupId                         string

|  |  |  +--rw supiRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw gpsiRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw externalGroupIdentifiersRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw supportedDataSet*                nrm-type:DataSetId

|  |  +--rw udmInfo

|  |  |  +--rw groupId                         string

|  |  |  +--rw supiRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw gpsiRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw externalGroupIdentifiersRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw routingIndicator                string

|  |  +--rw ausfInfo

|  |  |  +--rw groupId            string

|  |  |  +--rw supiRange* [start end pattern]

|  |  |  |  +--rw start      string

|  |  |  |  +--rw end        string

|  |  |  |  +--rw pattern    string

|  |  |  +--rw routingIndicator   string

|  |  +--rw amfInfo

|  |  |  +--rw amfRegionId            string

|  |  |  +--rw amfSetId               string

|  |  |  +--rw guami* [k_id]

|  |  |  |  +--rw k_id      string

|  |  |  |  +--rw plmnId

|  |  |  |  |  +--rw MCC   t_mcc

|  |  |  |  |  +--rw MNC   t_mnc

|  |  |  |  +--rw amfId

|  |  |  |     +--rw AMF-Region-id   t_aMF-Region-id

|  |  |  |     +--rw AMF-Set-id      t_aMF-Set-id

|  |  |  |     +--rw AMF-Pointer     t_aMF-Pointer

|  |  |  +--rw tai* [k_id]

|  |  |  |  +--rw k_id      string

|  |  |  |  +--rw plmnId

|  |  |  |  |  +--rw MCC   t_mcc

|  |  |  |  |  +--rw MNC   t_mnc

|  |  |  |  +--rw tac      t_tAC

|  |  |  +--rw backupInfoAmfFailure* [k_id]

|  |  |  |  +--rw k_id      string

|  |  |  |  +--rw plmnId

|  |  |  |  |  +--rw MCC   t_mcc

|  |  |  |  |  +--rw MNC   t_mnc

|  |  |  |  +--rw amfId

|  |  |  |     +--rw AMF-Region-id   t_aMF-Region-id

|  |  |  |     +--rw AMF-Set-id      t_aMF-Set-id

|  |  |  |     +--rw AMF-Pointer     t_aMF-Pointer

|  |  |  +--rw backupInfoAmfRemoval* [k_id]

|  |  |     +--rw k_id      string

|  |  |     +--rw plmnId

|  |  |     |  +--rw MCC   t_mcc

|  |  |     |  +--rw MNC   t_mnc

|  |  |     +--rw amfId

|  |  |        +--rw AMF-Region-id   t_aMF-Region-id

|  |  |        +--rw AMF-Set-id      t_aMF-Set-id

|  |  |        +--rw AMF-Pointer     t_aMF-Pointer

|  |  +--rw smfInfo

|  |  |  +--rw dnn*       nrm-type:t_Dnn

|  |  |  +--rw tai* [k_id]

|  |  |  |  +--rw k_id      string

|  |  |  |  +--rw plmnId

|  |  |  |  |  +--rw MCC   t_mcc

|  |  |  |  |  +--rw MNC   t_mnc

|  |  |  |  +--rw tac      t_tAC

|  |  |  +--rw pgwFqdn   inet:domain-name

|  |  +--rw upfInfo

|  |  |  +--rw sNssaiUpfInfo* [sNssai]

|  |  |  |  +--rw sNssai        t_s-NSSAI

|  |  |  |  +--rw dnnUpfInfo* [dnn]

|  |  |  |     +--rw dnn    t_Dnn

|  |  |  +--rw smfServingArea     string

|  |  |  +--rw interfaceUpfInfo* [k_id]

|  |  |     +--rw k_id               string

|  |  |     +--rw interfaceType     UPInterfaceType

|  |  |     +--rw (address)

|  |  |     |  +--:(ipv4Address)

|  |  |     |  |  +--rw ipv4Address       inet:ipv4-address

|  |  |     |  +--:(ipv6Address)

|  |  |     |  |  +--rw ipv6Address       inet:ipv6-address

|  |  |     |  +--:(ipv6Prefix)

|  |  |     |  |  +--rw ipv6Prefix        inet:ipv6-prefix

|  |  |     |  +--:(endpointFqdn)

|  |  |     |     +--rw endpointFqdn      inet:domain-name

|  |  |     +--rw networkInstance   string

|  |  +--rw pcfInfo

|  |  |  +--rw dnn*   nrm-type:t_Dnn

|  |  +--rw bsfInfo

|  |  |  +--rw ipv4AddressRange* [start end]

|  |  |  |  +--rw start    inet:ipv4-address

|  |  |  |  +--rw end      inet:ipv4-address

|  |  |  +--rw ipv6PrefixRange* [start end]

|  |  |     +--rw start    inet:ipv6-prefix

|  |  |     +--rw end      inet:ipv6-prefix

|  |  +--rw nfService* [serviceInstanceID]

|  |     +--rw serviceInstanceID                  string

|  |     +--rw serviceName                        string

|  |     +--rw version

|  |     |  +--rw apiVersionInUri   string

|  |     |  +--rw apiFullVersion    string

|  |     |  +--rw expiry            yang:date-and-time

|  |     +--rw schema                             string

|  |     +--rw fqdn                              inet:domain-name

|  |     +--rw interPlmnFqdn                     inet:domain-name

|  |     +--rw ipEndPoint* [k_id]

|  |     |  +--rw k_id           string

|  |     |  +--rw (address)

|  |     |  |  +--:(ipv4Address)

|  |     |  |  |  +--rw ipv4Address   inet:ipv4-address

|  |     |  |  +--:(ipv6Address)

|  |     |  |  |  +--rw ipv6Address   inet:ipv6-address

|  |     |  |  +--:(ipv6Prefix)

|  |     |  |     +--rw ipv6Prefix    inet:ipv6-prefix

|  |     |  +--rw transport     t_TransportProtocol

|  |     |  +--rw port          uint16

|  |     +--rw apiPrefix                         string

|  |     +--rw defaultNotificationSubscription* [notificationType]

|  |     |  +--rw notificationType      NotificationType

|  |     |  +--rw callbackUri          inet:uri

|  |     |  +--rw n1MessageClass       N1MessageClass

|  |     |  +--rw n2InformationClass   N2InformationClass

|  |     +--rw allowedPlmn* [MCC MNC]

|  |     |  +--rw MCC    t_mcc

|  |     |  +--rw MNC    t_mnc

|  |     +--rw allowedNfType*                     nrm-type:NFType

|  |     +--rw allowedDomain*                     string

|  |     +--rw allowedNssais*                     nrm-type:t_s-NSSAI

|  |     +--rw capacity                          uint8

|  |     +--rw load                              nrm-type:t_Load

|  |     +--rw supportedFeatures                 nrm-type:SupportedFeatures

|  +--ro nSIId*               nrm-type:t_NsiId

|  +--rw EP_N27

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw NSSFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--ro nSIId*               nrm-type:t_NsiId

|  +--rw EP_N22

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N31

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw SMSFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw EP_N20

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_N21

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_MAP_SMSC

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--rw LMFFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw EP_NLS

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

+--rw NGEIRFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw s-NSSAI*             nrm-type:t_s-NSSAI

|  +--rw EP_N17

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_X* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw SEPPFunction

|  +--ro objectClass          string

|  +--ro objectInstance       nrm-type:t_dn

|  +--ro id                   nrm-type:t_dn

|  +--rw userLabel           string

|  +--rw vnfParametersList

|  |  +--rw vnfInstanceId    string

|  |  +--rw vnfdId          string

|  |  +--rw flavourId       string

|  |  +--rw autoScalable    boolean

|  +--rw peeParametersList

|  |  +--rw siteIdentification    string

|  |  +--rw siteLatitude         decimal64

|  |  +--rw siteLongitude        decimal64

|  |  +--rw siteDescription       string

|  |  +--rw equipmentType         enumeration

|  |  +--rw environmentType       enumeration

|  |  +--rw powerInterface        enumeration

|  +--rw pLMNId* [MCC MNC]

|  |  +--rw MCC    t_mcc

|  |  +--rw MNC    t_mnc

|  +--rw sBIFQDN              inet:domain-name

|  +--ro sBIService*          string

|  +--rw EP_N32

|  |  +--ro objectClass       string

|  |  +--ro objectInstance    nrm-type:t_dn

|  |  +--rw id                nrm-type:t_dn

|  |  +--rw userLabel        string

|  |  +--rw farEndEntity     nrm-type:t_dn

|  |  +--rw localAddress

|  |  |  +--rw ip_address   inet:ip-address

|  |  |  +--rw vlan_id      uint16

|  |  +--rw remoteAddress    inet:ip-address

|  +--rw EP_SBI_IPX* [id]

|     +--ro objectClass       string

|     +--ro objectInstance    nrm-type:t_dn

|     +--rw id                nrm-type:t_dn

|     +--rw userLabel        string

|     +--rw farEndEntity     nrm-type:t_dn

|     +--rw localAddress

|     |  +--rw ip_address   inet:ip-address

|     |  +--rw vlan_id      uint16

|     +--rw remoteAddress    inet:ip-address

|     +--ro sBIService*       string

+--rw NWDAFunction

+--ro objectClass          string

+--ro objectInstance       nrm-type:t_dn

+--ro id                   nrm-type:t_dn

+--rw userLabel           string

+--rw vnfParametersList

|  +--rw vnfInstanceId    string

|  +--rw vnfdId          string

|  +--rw flavourId       string

|  +--rw autoScalable    boolean

+--rw peeParametersList

|  +--rw siteIdentification    string

|  +--rw siteLatitude         decimal64

|  +--rw siteLongitude        decimal64

|  +--rw siteDescription       string

|  +--rw equipmentType         enumeration

|  +--rw environmentType       enumeration

|  +--rw powerInterface        enumeration

+--rw pLMNId* [MCC MNC]

|  +--rw MCC    t_mcc

|  +--rw MNC    t_mnc

+--rw sBIFQDN              inet:domain-name

+--ro sBIService*          string

+--rw s-NSSAI*             nrm-type:t_s-NSSAI

+--rw EP_SBI_X* [id]

+--ro objectClass       string

+--ro objectInstance    nrm-type:t_dn

+--rw id                nrm-type:t_dn

+--rw userLabel        string

+--rw farEndEntity     nrm-type:t_dn

+--rw localAddress

|  +--rw ip_address   inet:ip-address

|  +--rw vlan_id      uint16

+--rw remoteAddress    inet:ip-address

+--ro sBIService*       string

## H.4.3	YANG schema

### H.4.3.1	General type definition "nrm-types-3gpp.yang"

Refer to Annex E.4.3.1.

### H.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork"

Refer to Annex G.

### H.4.3.3	NGC module, composition of all 5GC ManagedFunctions in "ngc.yang"

module ngc {

namespace "urn:3gpp:tsg:sa5:nrm:ngc";

prefix "ngc";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import ManagedElement { prefix me; revision-date "2018-07-31"; }

include ngc-AMFFunction;

include ngc-SMFFunction;

include ngc-UPFFunction;

include ngc-N3IWFFunction;

include ngc-PCFFunction;

include ngc-AUSFFunction;

include ngc-UDMFunction;

include ngc-UDRFunction;

include ngc-UDSFFunction;

include ngc-NRFFunction;

include ngc-NSSFFunction;

include ngc-SMSFFunction;

include ngc-LMFFunction;

include ngc-NGEIRFunction;

include ngc-SEPPFunction;

include ngc-NWDAFunction;

include NFProfile;

revision 2018-08-07 {

description "Initial revision";

}

container SubNetwork {

uses subnet:SubNetwork;

list ManagedElement {

uses me:ManagedElement;

list AMFFunction { uses AMFFunction; }

list SMFFunction { uses SMFFunction; }

list UPFFunction { uses UPFFunction; }

list N3IWFFunction { uses N3IWFFunction; }

list PCFFunction { uses PCFFunction; }

list AUSFFunction { uses AUSFFunction; }

list UDMFunction { uses UDMFunction; }

list UDRFunction { uses UDRFunction; }

list UDSFFunction { uses UDSFFunction; }

list NRFFunction { uses NRFFunction; }

list NSSFFunction { uses NSSFFunction; }

list SMSFFunction { uses SMSFFunction; }

list LMFFunction { uses LMFFunction; }

list NGEIRFunction { uses NGEIRFunction; }

list SEPPFunction { uses SEPPFunction; }

list NWDAFunction { uses NWDAFunction; }

}

list AMFSet { uses AMFSet; }

list AMFRegion { uses AMFRegion; }

}

}

### H.4.3.4	NGC submodule, "ngc-AMFFunction.yang", “ngc-AMFRegion.yang”, “ngc-AMFSet.yang”

submodule ngc-AMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "AMFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping AMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

container aMFIdentifier {

presence "true";

config "true";

description "";

uses nrm-type:aMFI;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description "";

min-elements "1";

config "false";

type string;

}

leaf weightFactor {

mandatory "true";

config "true";

description "";

type uint8;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N2 { uses ep-rp:EP_N2; }

list EP_N8 { uses ep-rp:EP_N8; }

list EP_N11 { uses ep-rp:EP_N11; }

list EP_N12 { uses ep-rp:EP_N12; }

list EP_N14 { uses ep-rp:EP_N14; }

list EP_N15 { uses ep-rp:EP_N15; }

list EP_N17 { uses ep-rp:EP_N17; }

list EP_N20 { uses ep-rp:EP_N20; }

list EP_N22 { uses ep-rp:EP_N22; }

list EP_N26 { uses ep-rp:EP_N26; }

list EP_NLS { uses ep-rp:EP_NLS; }

list EP_NLG { uses ep-rp:EP_NLS; }

list EP_SBI_X {

uses ep-rp:EP_SBI_X;

}

}

}

submodule ngc-AMFRegion {

belongs-to ngc { prefix ngc; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-12-18 {

description "initial verison";

}

grouping AMFRegion {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf aMFRegionId {

config "true";

description "";

type nrm-type:t_aMF-Region-id;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list aMFSet {

description "AMF Set list in the aMFRegion";

min-elements "0";

config "true";

type nrm-type:t_dn;

}

}

}

submodule ngc-AMFSet {

belongs-to ngc { prefix ngc; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-12-18 {

description "initial verison";

}

grouping AMFSet {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf aMFSetId {

config "true";

description "";

type nrm-type:t_aMF-Set-id;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list aMFSetMember {

description "AMF funciton list in the aMFSet";

min-elements "0";

config "true";

type nrm-type:t_dn;

}

leaf aMFRegion {

description "associated aMFRegion the aMFSet belong to";

type nrm-type:t_dn;

}

}

}

H.4.3.5	NGC submodule, "ngc-AUSFFunction.yang"

submodule ngc-AUSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "AUSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping AUSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N12 { uses ep-rp:EP_N12; }

list EP_N13 { uses ep-rp:EP_N13; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.6	NGC submodule, "ngc-LMFFunction.yang"

submodule ngc-LMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "LMFFunction derived from basic ManagedFunction";

revision 2018-12-18 {

description "15.1.0";

}

grouping LMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_NLS { uses ep-rp:EP_NLS; }

}

}

H.4.3.7	NGC submodule, "ngc-N3IWFFunction.yang"

submodule ngc-N3IWFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "N3IWFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping N3IWFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_N2 { uses ep-rp:EP_N2; }

list EP_N3 { uses ep-rp:EP_N3; }

}

}

### H.4.3.8	NGC submodule, "ngc-NGEIRFunction.yang"

submodule ngc-NGEIRFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "NGEIRFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NGEIRFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N17 { uses ep-rp:EP_N17; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.9	NGC submodule, "ngc-NRFFunction.yang"

submodule ngc-NRFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

include NFProfile;

description "NRFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NRFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description "";

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list nFProfile {

description "CM only required when NF profile is registered and deregistered by management system.";

min-elements "1";

config "true";

key nfInstanceId;

uses NFProfile;

}

leaf-list nSIId {

description "Is it really needed if network slicing feature enabled Seems it's only required for NSSF";

min-elements "1";

config "false";

type nrm-type:t_NsiId;

}

list EP_N27 { uses ep-rp:EP_N26; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

submodule NFProfile {

belongs-to ngc { prefix ngc; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

revision 2018-08-08 {

description "Initial revision";

}

grouping UdrInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

list gpsiRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

list externalGroupIdentifiersRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

leaf-list supportedDataSet {

type nrm-type:DataSetId;

}

}

grouping UdmInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

list gpsiRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

list externalGroupIdentifiersRange {

key "start end pattern";

uses nrm-type:IdentityRange;

}

leaf routingIndicator {

type string;

}

}

grouping AusfInfo {

leaf groupId {

type string;

}

list supiRange {

key "start end pattern";

uses nrm-type:SupiRange;

}

leaf routingIndicator {

type string;

}

}

grouping AmfInfo {

leaf amfRegionId {

type string;

}

leaf amfSetId {

type string;

}

list guami {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

list tai {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Tai;

}

list backupInfoAmfFailure {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

list backupInfoAmfRemoval {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Guami;

}

}

grouping SmfInfo {

leaf-list dnn {

type nrm-type:t_Dnn;

}

list tai {

//FFS to use embedded identifier

key k_id;

leaf k_id {

type string;

}

uses nrm-type:Tai;

}

leaf pgwFqdn {

type inet:domain-name;

}

}

grouping UpfInfo {

list sNssaiUpfInfo {

key sNssai;

uses nrm-type:SnssaiUpfInfoItem;

}

leaf smfServingArea {

type string;

}

list interfaceUpfInfo {

//FFS to use "choiced" identifier

key k_id;

leaf k_id {

type string;

}

//key "interfaceType address";

uses nrm-type:InterfaceUpfInfoItem;

}

}

grouping PcfInfo {

leaf-list dnn {

type nrm-type:t_Dnn;

}

}

grouping BsfInfo {

list ipv4AddressRange {

key "start end";

uses nrm-type:Ipv4AddressRange;

}

list ipv6PrefixRange {

key "start end";

uses nrm-type:Ipv6PrefixRange;

}

}

grouping NFService {

leaf serviceInstanceID {

description "";

mandatory "true";

type string;

}

leaf serviceName {

mandatory "true";

type string;

}

container version { uses nrm-type:NFServiceVersion; }

leaf schema {

mandatory "true";

type string;

}

leaf fqdn {

type inet:domain-name;

}

leaf interPlmnFqdn {

type inet:domain-name;

}

list ipEndPoint {

//FFS to use "choiced" identifier

key k_id;

leaf k_id {

type string;

}

//     		key "address port";

uses nrm-type:ipEndPoint;

}

leaf apiPrefix {

type string;

}

list defaultNotificationSubscription {

key notificationType;

uses nrm-type:defaultNotificationSubscription;

}

list allowedPlmn {

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf-list allowedNfType {

type nrm-type:NFType;

}

leaf-list allowedDomain {

type string;

}

leaf-list allowedNssais {

type nrm-type:t_s-NSSAI;

}

leaf capacity {

type uint8;

}

leaf load {

type nrm-type:t_Load;

}

leaf supportedFeatures {

type nrm-type:SupportedFeatures;

}

}

grouping NFProfile {

leaf nfInstanceId {

mandatory "true";

type string;

}

leaf nfType {

mandatory "true";

type nrm-type:NFType;

}

leaf nfStatus {

mandatory "true";

type nrm-type:NFStatus;

}

container plmn {

uses nrm-type:pLMNId;

}

leaf-list sNssais {

type nrm-type:t_s-NSSAI;

}

leaf-list nsi {

type string;

}

leaf fqdn {

type inet:domain-name;

}

leaf interPlmnFqdn {

type inet:domain-name;

}

leaf-list ipv4Address {

type inet:ipv4-address;

}

leaf-list ipv6Address {

type inet:ipv6-address;

}

leaf-list ipv6Prefix {

type inet:ipv6-prefix;

}

leaf capacity {

type uint8;

}

leaf load {

type nrm-type:t_Load;

}

leaf locality {

type string;

}

leaf priority {

type uint8;

}

container udrInfo { uses UdrInfo; }

container udmInfo { uses UdmInfo; }

container ausfInfo { uses AusfInfo; }

container amfInfo { uses AmfInfo; }

container smfInfo { uses SmfInfo; }

container upfInfo { uses UpfInfo; }

container pcfInfo { uses PcfInfo; }

container bsfInfo { uses BsfInfo; }

list nfService {

min-elements "0";

description "";

key serviceInstanceID;

uses NFService;

}

}

}

### H.4.3.10	NGC submodule, "ngc-NSSFFunction.yang"

submodule ngc-NSSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "NSSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NSSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description " ";

type inet:domain-name;

}

leaf-list sBIService {

description " ";

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

leaf-list nSIId {

description "Is it really needed if network slicing feature enabled Seems it's only required for NSSF";

min-elements "1";

config "false";

type nrm-type:t_NsiId;

}

list EP_N22 { uses ep-rp:EP_N22; }

list EP_N31 { uses ep-rp:EP_N31; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.11	NGC submodule, "ngc-NWDAFunction.yang"

submodule ngc-NWDAFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "NWDAFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping NWDAFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.12	NGC submodule, "ngc-PCFFunction.yang"

submodule ngc-PCFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "PCFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping PCFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N5 { uses ep-rp:EP_N5; }

list EP_N7 { uses ep-rp:EP_N7; }

list EP_N15 { uses ep-rp:EP_N15; }

list EP_N16 { uses ep-rp:EP_N16; }

list EP_Rx { uses ep-rp:EP_Rx; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.13	NGC submodule, "ngc-SEPPFunction.yang"

submodule ngc-SEPPFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "SEPPFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping SEPPFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

container EP_N32 { uses ep-rp:EP_N32; }

list EP_SBI_IPX {

key id;

uses ep-rp:EP_SBI_IPX;

}

}

}

### H.4.3.14	NGC submodule, "ngc-SMFFunction.yang"

submodule ngc-SMFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "SMFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping SMFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N4 { uses ep-rp:EP_N4; }

list EP_N7 { uses ep-rp:EP_N7; }

list EP_N10 { uses ep-rp:EP_N10; }

list EP_N11 { uses ep-rp:EP_N11; }

list EP_N16 { uses ep-rp:EP_N16; }

list EP_S5C { uses ep-rp:EP_S5C; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.15	NGC submodule, "ngc-SMSFFunction.yang"

submodule ngc-SMSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "SMSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping SMSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

list EP_N20 { uses ep-rp:EP_N20; }

list EP_N21 { uses ep-rp:EP_N21; }

list EP_MAP_SMSC { uses ep-rp:EP_MAP_SMSC; }

}

}

### H.4.3.16	NGC submodule, "ngc-UDMFunction.yang"

submodule ngc-UDMFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "UDMFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UDMFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N8 { uses ep-rp:EP_N8; }

list EP_N10 { uses ep-rp:EP_N10; }

list EP_N13 { uses ep-rp:EP_N13; }

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.17	NGC submodule, "ngc-UDRFunction.yang"

submodule ngc-UDRFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "UDRFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UDRFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.18	NGC submodule, "ngc-UDSFFunction.yang"

submodule ngc-UDSFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import ietf-inet-types { prefix inet; revision-date "2010-09-24"; }

description "UDSFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UDSFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf sBIFQDN {

mandatory "true";

config "true";

description "";

type inet:domain-name;

}

leaf-list sBIService {

min-elements "1";

config "false";

type string;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_SBI_X {

key id;

uses ep-rp:EP_SBI_X;

}

}

}

### H.4.3.19	NGC submodule, "ngc-UPFFunction.yang"

submodule ngc-UPFFunction {

belongs-to ngc { prefix ngc; }

import EP_RP { prefix ep-rp; revision-date "2018-07-31"; }

import ManagedFunction { prefix mf; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

description "UPFFunction derived from basic ManagedFunction";

revision 2018-08-07 {

description "Initial revision";

}

grouping UPFFunction {

uses mf:ManagedFunction;

list pLMNId {

min-elements "1";

description "a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)|| Mobile Network Codes(MNC) ";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

leaf-list tAC {

min-elements "1";

config "false";

type nrm-type:t_tAC;

}

leaf-list s-NSSAI {

min-elements "0";

config "true";

description "It represents the list of S-NSSAI the managed object is capable

of supporting, an S-NSSAI is comprised of a SST (Slice/Service

type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP

TS 23.003 [13]). ";

reference "3GPP TS 23.003";

type nrm-type:t_s-NSSAI;

}

list EP_N3 { uses ep-rp:EP_N4; }

list EP_N4 { uses ep-rp:EP_N7; }

list EP_N9 { uses ep-rp:EP_N10; }

list EP_N6 { uses ep-rp:EP_N11; }

list EP_S5U { uses ep-rp:EP_S5C; }

}

}

Annex I (normative):
XML definitions for network slice

# I.1	General

This annex contains the XML definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

# I.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# I.3	Mapping

## I.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## I.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# I.4	Solution Set (SS) definitions

## I.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

This annex defines the NRM-specific XML schema sliceNrm.xsd for the network slice Information Model defined in clause 6.

XML schema sliceNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

## I.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## I.4.3	XML schema "sliceNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 network slice Network Resource Model

XML schema definition

sliceNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sl="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<simpleType name="MobilityLevel">

<restriction base="string">

<enumeration value="STATIONARY"/>

<enumeration value="NOMADIC"/>

<enumeration value="RESTRICTED MOBILITY"/>

<enumeration value="FULLY MOBILITY"/>

</restriction>

</simpleType>

<simpleType name="SharingLevel">

<restriction base="string">

<enumeration value="SHARED"/>

<enumeration value="NON-SHARED"/>

</restriction>

</simpleType>

<complexType name="PerfReq">

<!-- Refer to definitions in TS 22.261-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="perfReqEMBB" type="sl:PerfReqEmbb"/>

<element name="perfReqUrllc" type="sl:PerfReqUrllc"/>

</choice>

</sequence>

</complexType>

<complexType name="PerfReqEmbb">

<sequence>

<element name="expDataRateDL" type="integer" minOccurs="0"/>

<element name="expDataRateUL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapDL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapUL" type="integer" minOccurs="0"/>

<element name="userDensity" type="integer" minOccurs="0"/>

<element name="activityFactor" type="integer " minOccurs="0"/>

<element name="uESpeed" type="integer" minOccurs="0"/>

<element name="coverage" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PerfReqUrllc">

<sequence>

<element name="e2eKatency" type="integer" minOccurs="0"/>

<element name="jitter" type="integer" minOccurs="0"/>

<element name="survivalTime" type="integer" minOccurs="0"/>

<element name="cSAvailability" type="float" minOccurs="0"/>

<element name="reliability" type="float" minOccurs="0"/>

<element name="expDataRate" type="integer " minOccurs="0"/>

<element name="payloadSize" type="integer" minOccurs="0"/>

<element name="trafficDensity" type="string" minOccurs="0"/>

<element name="connDensity" type="float" minOccurs="0"/>

<element name="serviceDimension" type="string " minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfile">

<sequence>

<element name="serviceProfileId" type="string"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="integer" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="sst" type="ngc:Sst"/>

<element name="availability" type="float" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfileList">

<sequence>

<element name="serviceProfile" type="sl:ServiceProfile"/>

</sequence>

</complexType>

<complexType name="SliceProfile">

<sequence>

<element name="sliceProfileId" type="string"/>

<element name="sNSSAIList" type=" ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="sl: MobilityLevel" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SliceProfileList">

<sequence>

<element name="sliceProfile" type="sl:SliceProfile"/>

</sequence>

</complexType>

<complexType name="NsInfo">

<!-- Refer to definitions in subclause 8.3.3.2.2 of ETSI NFV IFA013 -->

<sequence>

<element name="nsInstanceId" type="string"/>

<element name="nsName" type="string"/>

<element name="description" type="string"/>

</sequence>

</complexType>

<element name="NetworkSlice" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="nSSIId" type="xn:dn"/>

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="serviceProfileList" type="sl:ServiceProfileList"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NetworkSliceSubnet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="mFIdList" type="xn:dnList"/>

<element name="ConstituentNSSIIdList" type="xn:dnList"/>

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="nsInfo" type="sl:NsInfo" minOccurs="0"/>

<element name="sliceProfileList" type="sl:SliceProfileList"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex J (normative):
JSON definitions for network slice

# J.1	General

This annex contains the JSON definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

# J.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

# J.3	Mapping

## J.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| NetworkSlice | networkSlice | object |
| NetworkSliceSubnet | networkSliceSubnet | object |
| ServiceProfile | serviceProfile | object |
| SliceProfile | sliceProfile | object |

## J.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| nSSIId | nSSIId | genericNrm:Dn |
| serviceProfileId | serviceProfileId | string |
| sliceProfileId | sliceProfileId | string |
| constituentNSSIIdList | constituentNSSIIdList | genericNrm:DnList |
| mFIdList | mFIdList | genericNrm:DnList |
| operationalState | operationalState | stateManagementNrm: operationalStateType |
| administrativeState | administrativeState | stateManagementNrm: administrativeStateType |
| nsInfo | nsInfo | NsInfo |
| plmnIdList | plmnIdList | nrNrm: PlmnIdList |
| perfReq | perfReq | PerfReq |
| sNSSAIList | snssaiList | nrNrm: SnssaiList |
| maxNumberofUEs | maxNumberofUEs | number |
| coverageAreaTAList | coverageAreaTAList | ngcNrm:TACList |
| latency | latency | number |
| uEMobilityLevel | uEMobilityLevel | number |
| resourceSharingLevel | resourceSharingLevel | number |
| serviceProfileList | serviceProfileList | object |
| sliceProfileList | sliceProfileList | object |

# J.4	Solution Set (SS) definitions

## J.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## J.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## J.4.3	JSON schema "sliceNrm.json"

{

"$schema": "http://json-schema.org/draft-04/schema#",

"id": "http://3gpp.org/28541/sliceNrm.json",

"description": "JSON based solution set definitions for network slice and network slice subnet",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"networkSlice": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"nSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/Dn"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/operationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/administrativeState"

},

"serviceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

}

},

"required": [

"userLabel",

"userDefinedNetworkType",

"nSSIId",

"operationalState",

"administrativeState",

"serviceProfileList"

]

},

"networkSliceSubnet": {

"type": "object",

"properties": {

"id": {

"type": "string"

},

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"mFIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/DnList"

},

"constituentNSSIIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#/definitions/DnList"

},

"operationalState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/OperationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28626/stateManagementNrm.json#/definitions/AdministrativeState"

},

"nsInfo": {

"$ref": "#/definitions/NsInfo"

},

"sliceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

}

},

"required": [

"userLabel",

"mFIdList",

"constituentNSSIIdList",

"operationalState",

"administrativeState",

"sliceProfileList"

]

},

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY",

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"PrefReq": {

"type": "object",

"properties": {

"prefReqEmbb": {

"$ref": "#/definitions/PrefReqEmbb"

},

"prefReqUrllc": {

"$ref": "#/definitions/PrefReqUrllc"

}

}

},

"PerfReqEmbb":{

"type": "object",

"properties": {

"expDataRateDL": {

"type": "number"

},

"expDataRateUL": {

"type": "number"

},

"areaTrafficCapDL": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"userDensity": {

"type": "number"

},

"activityFactor": {

"type": "number"

},

"uESpeed": {

"type": "number"

},

"coverage": {

"type": "string"

}

}

},

"PerfReqUrrlc":{

"type": "object",

"properties": {

"e2eLatency": {

"type": "number"

},

"jitter": {

"type": "number"

},

"survivalTime": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"cSAvailability": {

"type": "number"

},

"reliability": {

"type": "number"

},

"expDataRate": {

"type": "number"

},

"payloadSize": {

"type": "number"

},

"trafficDensity": {

"type": "string"

},

"connDensity": {

"type": "number"

},

"serviceDimension": {

"type": "string"

}

}

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#/definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number"

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

},

"resourceSharingLevel": {

"type": "integer"

}

"availability": {

"type": "number"

}

}

},

"ServiceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/ServiceProfile"

}

},

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/SnssaiList"

},

"plmnIdList": {

"$ref": "http://3gpp.org/28541/nrNrm.json#/definitions/PlmnIdList"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#/definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number"

},

"resourceSharingLevel": {

"type": "integer"

}

}

},

"SliceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/SliceProfile"

}

},

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

},

"description ": {

"type": "string"

}

}

}

}

}

Annex K (normative):
YANG definitions for Network Slice NRM

# K.1	General

This annex contains the YANG definitions for the Network Slice NRM, in accordance with Network Slicing information model definitions specified in clause 6.

# K.2	Architectural features

# K.3	Mapping

## K.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| NetworkSlice | NetworkSlice | Contianer in module |
| NetworkSliceSubnet | NetworkSliceSubnet | Contianer in module |
| ServiceProfile | serviceProfile | Reusable Container in Module |
| SliceProfile | SliceProfile | Reusable Container in Module |

## K.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| Simple Type Attributes | Same to the Name in Stage 2 | leaf |
| Complex Type Attribute | Same to the Name in Stage 2 | Reusable Container |
| Simple Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | leaf-list |
| Complex Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | list |

# K.4	Solution Set definitions

## K.4.1	YANG definition structure

## K.4.2	Graphical representation

module: NetworkSlice

+--rw NetworkSlice

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro nSSIId                    nrm-type:t_dn

+--ro operationalState          nrm-type:t_operationalState

+--rw administrativeState       nrm-type:t_administrativeState

+--rw serviceProfile* [serviceProfileId]

|  +--rw serviceProfileId        nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

|  +--rw sST                       nrm-type:t_SST

module: NetworkSliceSubnet

+--rw NetworkSliceSubnet

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro mFId*                     nrm-type:t_dn

+--ro constituentNSSIIdList*    nrm-type:t_dn

+--rw State                     string

+--ro nsInfo                   nrm-type:t_dn

+--rw sliceProfile* [sliceProfileId]

|  +--rw sliceProfileId          nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

+--rw sST                       nrm-type:t_SST

## K.4.3	YANG schema

### K.4.3.1	General type definition "nrm-types-3gpp.yang"

Refer to Annex E.4.3.1.

### K.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork"

Refer to Annex E.4.3.2.

### K.4.3.3	NetworkSlice Module, "NetworkSlice.yang"

module NetworkSlice {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSlice";

prefix "ns";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include ServiceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSlice {

uses subnet:SubNetwork;

leaf nSSIId {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf operationalState {

mandatory "true";

config "false";

description "";

type nrm-type:t_operationalState;

}

leaf administrativeState {

mandatory "true";

config "true";

description "";

type nrm-type:t_administrativeState;

}

list serviceProfile {

min-elements "1";

config "true";

description "";

key serviceProfileId;

uses ServiceProfile;

}

}

}

### K.4.3.4	ServiceProfile Submodule, "ServiceProfile.yang"

submodule ServiceProfile {

belongs-to NetworkSlice { prefix ns; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import perfReq { prefix perf; revision-date "2018-08-10"; }

revision 2018-08-09 {

description "Initial revision";

}

grouping ServiceProfile {

leaf serviceProfileId {

mandatory "true";

description "";

type nrm-type:t_dn;

}

leaf-list s-NSSAI  {

min-elements "1";

config "true";

description "";

type nrm-type:t_s-NSSAI;

}

list pLMNId {

min-elements "1";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

container perfReq {

config "true";

description "";

uses perf:perfReq;

}

leaf maxNumberofUEs {

mandatory "false";

config "true";

description "";

type uint64;

}

leaf-list coverageAreaTA {

min-elements "1";

config "true";

description "";

type nrm-type:t_tAC;

}

leaf latency {

mandatory "false";

config "true";

description "";

type uint16;

}

leaf uEMobilityLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_uEMobilityLevel;

}

leaf resourceSharingLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_resourceSharingLevel;

}

leaf availability {

mandatory "false";

config "true";

description "";

type decimal64;

}

}

}

### K.4.3.5	NetworkSliceSubnet Module, "NetworkSliceSubnet.yang"

module NetworkSliceSubnet {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSliceSubnet";

prefix "nss";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include SliceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSliceSubnet {

uses subnet:SubNetwork;

leaf-list mFId {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf-list constituentNSSIIdList {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf operationalState {

mandatory "true";

config "false";

description "";

type nrm-type:t_operationalState;

}

leaf administrativeState {

mandatory "true";

config "true";

description "";

type nrm-type:t_administrativeState;

}

leaf nsInfo {

mandatory "false";

config "false";

description "";

type nrm-type:t_dn;

}

list sliceProfile {

min-elements  "1";

config "true";

description "";

key sliceProfileId;

uses SliceProfile;

}

}

}

### K.4.3.6	SliceProfile Submodule, "SliceProfile.yang"

submodule SliceProfile {

belongs-to NetworkSliceSubnet { prefix nss; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import perfReq { prefix perf; revision-date "2018-08-10"; }

revision 2018-08-09 {

description "Initial revision";

}

grouping SliceProfile {

leaf sliceProfileId {

mandatory "true";

description "";

type nrm-type:t_dn;

}

leaf-list s-NSSAI  {

min-elements "1";

config "true";

description "";

type nrm-type:t_s-NSSAI;

}

list pLMNId {

min-elements "1";

config "true";

key "MCC MNC";

uses nrm-type:pLMNId;

}

container perfReq {

config "true";

description "";

uses perf:perfReq;

}

leaf maxNumberofUEs {

mandatory "false";

config "true";

description "";

type uint64;

}

leaf-list coverageAreaTA {

min-elements "1";

config "true";

description "";

type nrm-type:t_tAC;

}

leaf latency {

mandatory "false";

config "true";

description "";

type uint16;

}

leaf uEMobilityLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_uEMobilityLevel;

}

leaf resourceSharingLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_resourceSharingLevel;

}

}

}

### K.4.3.7	perfReq Module, "perfReq.yang"

module perfReq {

namespace "urn:3gpp:tsg:sa5:nrm:perfReq";

prefix "perf";

revision 2018-08-10 {

description "Initial revision";

}

grouping perfReq {

choice SST {

case eMBB {

leaf expDataRateDL { type uint16; }

leaf expDataRateU {type uint16; }

leaf areaTrafficCapDL { type uint16; }

leaf areaTrafficCapUL { type uint16; }

leaf userDensity { type uint16; }

leaf activityFactor { type uint16; }

leaf uESpeed { type uint16; }

leaf coverage { type string; }

}

case uRLLC {

leaf e2eLatency { type uint16; }

leaf jitter {type uint16; }

leaf survivalTime { type uint16; }

leaf cSAvailability {

type decimal64 {

fraction-digits 4;

range 1..99.9999;

}

}

leaf reliability {

type decimal64 {

fraction-digits 4;

range 1..99.9999;

}

}

leaf expDataRate { type uint16; }

leaf payloadSize { type string; }

leaf trafficDensity { type uint16; }

leaf connDensity { type uint16; }

leaf serviceAreaDimension { type string; }

}

}

}

}

Annex L (informative):	
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EdiHelp review | 15.0.1 |
| 2018-12 | SA#82 | SP-181046 | 0001 | 1 | F | Fix issues raised by EditHelp | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0002 | 2 | F | Update NR Stage 2 definition to align with TS 37.340 for MR-DC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0003 | 1 | F | Update NRM Stage 2 defintion to align with TS 23.501 for 5G architecture | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0005 | 1 | F | Update Stage 3 XML definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0006 | 1 | F | Update Stage 3 JSON definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0007 | 1 | F | Update Stage 3 YANG definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0008 | 1 | F | Update Stage 3 XML definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0009 | 1 | F | Update Stage 3 JSON definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0011 | 1 | F | Update stage 3 XML definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0012 | 1 | F | Update Stage 3 JSON definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0013 | 1 | F | Update stage 3 YANG definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0014 | 1 | F | Correct the term sNSSAIList and nRTAClist | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0015 | 1 | F | Update the inheritance hierarchy figure for NR NRM to include BWP IOC and NRSectorCarrier IOC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0016 | 1 | F | Change the term nCGI to nCI | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0019 | 1 | F | Align properties of cell state | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0021 | 1 | F | Add missing attribute definition and condition | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0022 | 1 | F | Add missing detail definition for attribute | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0023 | 1 | F | Adding missing attribute, and correction of reference | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0025 | - | F | Remove NSSF from the abbrevations | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0027 | - | F | Replace symbol for network slice state management | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0031 | 1 | F | Remove the ExternalENBFunction definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0033 | 1 | F | Align the management of external function and cell with TS 28.658 | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0034 | 1 | F | Update NR NRM with Cell Relation | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0038 | 3 | F | RRM Policy enhancements | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0039 | 1 | F | Fix containment issue in YANG definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0040 | - | F | Implement minor corrections | 15.1.0 |
| 2018-12 | SA#82 | SP-181042 | 0041 | - | F | Update Stage 3 NRM for RRM Policy enhancements | 15.1.0 |
| 2019-03 | SA#83 | SP-190121 | 0043 | 1 | F | Align NR attributes definition related to SSB with corresponding NG-RAN IE definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0044 | 1 | F | Correct the use of nCI and PLMN | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0045 | - | F | Remove duplicate definition for ExternalNRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0046 | 2 | F | Correct class diagram for view on external entities | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0047 | 1 | F | Correct the definition for resourceSharingLevel | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0048 | 1 | F | Correction of references | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0052 | 1 | F | Align the term mFIdList and constituentNSSIIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0053 | 1 | F | Correct the definition of nSSIId | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0054 | 1 | F | Add missing attribute constraint for class definition of NSSFFunction | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0055 | 1 | F | Correct attribute constraints for RRMpolicy related attributes in NRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0057 | - | F | Correct cardinality of End Point (EP) to target | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0058 | 0 | F | Correct Import table | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0059 | - | F | Remove ExternalNRCellCU.pLMNIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0060 | - | F | Use 'bS' (not 'bs') to prefix all BS (base station) attributes | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0061 | 1 | F | Correction of State attributes descriptions | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0062 | - | F | Update 5G JSON Solution Set to align with generic NRM | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0063 | 1 | F | Update YANG Solution Set to align with Stage 2 definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0064 | 1 | F | Update Information Service to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0065 | 1 | F | Update Solution Set to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0066 | 1 | F | Add availability in service profile of network slice resource model | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0068 | 1 | F | Add sST attribute to ServiceProfile | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0069 | 1 | F | Update to sST attribute stage 3 | 15.2.0 |
| 2019-03 | SA#83 | SP-190149 | 0073 | 2 | F | Replace CoverageAreaTAList type definition | 16.6.0 |
| 2019-03 | SA#83 | SP-190149 | 0074 | 1 | F | Name datatypes SliceProfile and ServiceProfile | 16.6.0 |
| 2019-03 | SA#83 | SP-190149 | 0075 | 1 | F | Add datatype definition for S-NSSAI | 16.6.0 |
| 2019-03 | SA#83 | SP-190149 | 0076 | 1 | F | Remove incomplete description for TAC | 16.6.0 |
| 2019-03 | SA#83 | SP-190149 | 0079 | 1 | F | Name datatype RRMPolicyRatio2 | 16.6.0 |