3GPP TSG-SA5 Meeting #124	S5-192074

# Taipei,Taïwan, 25 Feb. - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Add RESTful HTTP-based solution set of fault supervision for integration with ONAP VES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ONAP DCAE VES Collector is becoming a potential consumer of 3GPP fault supervision data report service. It is therefore required that a new RESTful HTTP-based solution set be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new RESTful HTTP-based solution set for enabling ONAP DCAE VES Collector to consume 3GPP fault supervision date report service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ONAP DCAE VES Collector will not be able to consume 3GPP fault supervision data report service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration of networks and network slicing; Provisioning; Stage 1".

[6]	3GPP TS 28.554: "Management and orchestration of networks and network slicing; 5G End to end Key Performance Indicators (KPI); performance measurements and assurance data".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS) "

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[x]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://docs.onap.org/en/latest/_downloads/2c2b5962df52a0c1f2862f3bba3d67c7/CommonEventFormat_30.1_ONAP.json, accessed 13.02.2019.

Next change

# X	RESTful HTTP-based solution set of fault supervision for integration with ONAP VES API

## X.1	Mapping of operations

NOTE: all operations from clause 9.1 apply here except operations subscribe (cf. clause 9.1.8) and unsubscribe (cf. clause 9.1.9).

## X.2	Mapping of notifications

### X.2.1	Introduction

#### X.2.1.1	General

The IS notifications are mapped to SS equivalents according to table X.2.1.1-1.

Table X.2.1.1-1: Mapping of IS notifications to SS equivalents

| IS operations | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyNewAlarm | POST | /notificationSink | M |
| notifyNewSecurityAlarm | POST | /notificationSink | M |
| notifyAckStateChanged | POST | /notificationSink | M |
| notifyClearedAlarm | POST | /notificationSink | M |
| notifyAlarmListRebuilt | POST | /notificationSink | M |
| notifyChangedAlarm | POST | /notificationSink | M |
| notifyComments | POST | /notificationSink | M |
| notifyPotentialFaultyAlarmList | POST | /notificationSink | M |
| notifyCorrelatedNotificationChanged | POST | /notificationSink | M |
| notifyChangedAlarmGeneral | POST | /notificationSink | O |

#### X.2.1.2	Notification parameter mapping principles

Fault supervision alarm notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields [x], or to

- ONAP VES API fault3GPP fields.

The ONAP VES API commonEventHeader datatype consists of fields common to all events, as in table X.2.1.2-1 [x].

Table X.2.1.2-1: ONAP VES API common event header fields

| Field | Type | Required? | Description |
| --- | --- | --- | --- |
| domain | string | Yes | Event domain enumeration: ‘fault’, ‘heartbeat’, ‘measurement’, ‘mobileFlow’, ‘notification’, ‘other’, ‘pnfRegistration’, ‘sipSignaling’, ‘stateChange’, ‘syslog’, ‘thresholdCross ingAlert’, ‘voiceQuality’ |
| eventId | string | Yes | Event key that is unique to the event source. The key must be unique within notification life cycle similar to EventID from 3GPP. It could be a sequential number, or a composite key formed from the event fields, such as domain_sequence . The eventId should not include whitespace. For fault events, eventId is the eventId of the initial alarm; if the same alarm is raised again for changed, acknowledged or cleared cases, eventId must be the same as the initial alarm (along with the same startEpochMicro sec but with a different sequence number). Note: see section 1.3 for eventId use case examples. |
| eventName | string | Yes | Unique event name (see section 1.2 for more information) |
| eventType | string | No | For example: ‘applicationNf’ , ‘guestOS’, ‘hostOS’, ‘platform’ |
| internalHeader Fields | internalHeader Fields | No | Fields (not supplied by event sources) that the VES Event Listener service can use to enrich the event if needed for efficient internal processing. This is an empty object which is intended to be defined separately by each service provider (e.g., AT&T) implementing the VES Event Listener. |
| lastEpochMicros ec | number | Yes | the latest unix time aka epoch time associated with the event from any component–as microseconds elapsed since 1 Jan 1970 not including leap seconds |
| nfcNamingCode | string | No | Network function component type: 3 characters (aligned with vfc naming standards) |
| nfNamingCode | string | No | Network function type: 4 characters (aligned with vnf and pnf naming standards) |
| nfVendorName | string | No | Network function vendor name |
| priority | string | Yes | Processing priority enumeration: ‘High’, ‘Medium’, ‘Normal’, ‘Low’ |
| reportingEntity Id | string | No | UUID identifying the entity reporting the event or detecting a problem in another vnf/vm or pnf which is experiencing the problem. (Note: the AT&T internal enrichment process shall ensure that this field is populated). The reportingEntity Id is an id for the reportingEntity Name. See ‘reportingEntit yName’ for more information. |
| reportingEntity Name | string | Yes | Name of the entity reporting the event or detecting a problem in another vnf/vm or pnf which is experiencing the problem. May be the same as the sourceName. For synthetic events generated by DCAE, it is the name of the app generating the event. |
| sequence | integer | Yes | Ordering of events communicated by an event source instance (or 0 if not needed) |
| sourceId | string | No | UUID identifying the entity experiencing the event issue, which may be detected and reported by a separate reporting entity (note: the AT&T internal enrichment process shall ensure that this field is populated). The sourceId is an id for the sourceName. See ‘sourceName’ for more information. |
| sourceName | string | Yes | Name of the entity experiencing the event issue, which may be detected and reported by a separate reporting entity. The sourceName identifies the device for which data is collected. A valid sourceName must be inventoried in A&AI. If sourceName is a xNFC or VM, then the event must be reporting data for that particular xNFC or VM. If the sourceName is a xNF, comprised of multiple xNFCs, the data must be reported/aggreg ated at the xNF leveI. Data for individual xNFC must not be included in the xNF sourceName event. |
| startEpochMicro sec | number | Yes | the earliest unix time aka epoch time associated with the event from any component–as microseconds elapsed since 1 Jan 1970 not including leap seconds. For measurements and heartbeats, where events are collected over predefined intervals, startEpochMicro sec shall be rounded to the nearest interval boundary (e.g., the epoch equivalent of 3:00PM, 3:10PM, 3:20PM, etc…). For fault events, startEpochMicro sec is the timestamp of the initial alarm; if the same alarm is raised again for changed, acknowledged or cleared cases, startEpoch Microsec must be the same as the initial alarm (along with the same eventId and an incremental sequence number). For devices with no timing source (clock), the default value will be 0 and the VES collector will replace it with Collector time stamp (when the event is received) |
| timeZoneOffset | string | No | Offset to GMT to indicate local time zone for device formatted as ‘UTC+/-hh.mm’; see https://en.wiki pedia.org/wiki/ List_of_time_zo ne_abbreviation s for UTC offset examples |
| version | string | Yes | Version of the event header as “#.#” where # is a digit; see section 1 for the correct digits to use. |
| vesEventListene rVersion | string | Yes | Version of the ves event listener api spec that this event is compliant with (as “#” or “#.#” or “#.#.#” where # is a digit; see section 1 for the correct digits to use). |

NOTE: Each following sub-clause of clause X.2 contains two mapping tables. The first table has the following columns:

- IS notification parameter name: name of the input parameter of the notification as specified in clauses 9.2.X;

- SS parameter location;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses 9.2.X;

- SS parameter name (Corresponding VES common event header field): name of the field of the VES common event header (cf. [x]);

- SQ: Support Qualifier of the VES common event header field, as specified in [x], where ‘M’ corresponds to ‘Required = Yes’ in [x] and ‘O’ corresponds to ‘Required = No’ in [x];

- SS parameter type: type of the SS parameter.

The second table has the following columns:

- IS notification parameter name: name of the input parameter of the notification as specified in clauses 9.2.X;

- SS parameter location;

- SQ: Support Qualifier for the notification input parameter, as specified in clauses 9.2.X;

- SS parameter name (Corresponding VES ‘Fault3GPP’ domain field): name of the field of the VES ‘Fault3GPP’ domain;

- SQ: Support Qualifier of the VES ‘Fault3GPP’ domain field. It has the same value as the IS notification parameter Support Qualifier;

- SS parameter type: type of the SS parameter.

### X.2.2	Notification notifyNewAlarm

The IS notification parameters are mapped to SS equivalents according to table X.2.2-1 and to table X.2.2-2.

Table X.2.2-1: Mapping of IS notification parameters to SS equivalents in ONAP VES common event header fields

| IS notification parameter name | SS parameter location | SQ | SS parameter name (Corresponding VES common event header field) | SQ | SS parameter type |
| --- | --- | --- | --- | --- | --- |
| notificationId | request body | M | eventId | M | notificationId-Type |
| notificationType | request body | M | eventName | M | alarmNotificationType-Type |
| eventTime | request body | M | lastEpochMicrosec | M | dateTime-Type |
| no corresponding IS parameter | request body | N/A | domain | M | domain-Type (value = “fault3GPP”) () |
| no corresponding IS parameter | N/A | N/A | eventType | O | N/A |
| no corresponding IS parameter | N/A | N/A | internalHeaderFields | O | N/A |
| no corresponding IS parameter | N/A | N/A | nfNamingCode | O | N/A |
| no corresponding IS parameter | N/A | N/A | nfcNamingCode | O | N/A |
| no corresponding IS parameter | N/A | N/A | nfVendorName | O | N/A |
| no corresponding IS parameter | request body | N/A | priority | M | priority-Type (value set to “High”) |
| no corresponding IS parameter | N/A | N/A | reportingEntityId | O | N/A |
| no corresponding IS parameter | request body | N/A | reportingEntityName | M | reportingEntityName-Type (value set to “”) |
| no corresponding IS parameter | request body | N/A | sequence | M | sequence-Type (value set to 0) |
| no corresponding IS parameter | N/A | N/A | sourceId | O | N/A |
| no corresponding IS parameter | request body | N/A | sourceName | M | sourceName-Type (value set to “”) |
| no corresponding IS parameter | request body | N/A | startEpochMicrosec | M | dateTime-Type (value = same value as lastEpochMicrosec) |
| no corresponding IS parameter | N/A | N/A | timeZoneOffset | O | N/A |
| no corresponding IS parameter | request body | N/A | version | M | version-Type |
| no corresponding IS parameter | request body | N/A | vesEventListenerVersion | M | vesEventListenerVersion-Type |

Table X.2.2-2: Mapping of IS notification parameters to SS equivalents in ONAP Fault3GPP domain fields

| IS notification parameter name | SS parameter location | SQ | SS parameter name (Corresponding VES ‘Fault3GPP’ domain field) | SQ | SS parameter type |
| --- | --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | M | href | M | uri-Type |
| systemDN | request body | C | systemDN | C | systemDN-Type |
| probableCause | request body | M | probableCause | M | probableCause-Type |
| perceivedSeverity | request body | M | perceivedSeverity | M | perceivedSeverity-Type |
| rootCauseIndicator | request body | O | rootCauseIndicator | O | rootCauseIndicator-Type |
| alarmType | request body | M | alarmType | M | alarmType-Type |
| specificProblem | request body | O | specificProblem | O | specificProblem-Type |
| correlatedNotifications | request body | O | correlatedNotifications | O | array(correlatedNotification-Type) |
| backedUpStatus | request body | O | backedUpStatus | O | backedUpStatus-Type |
| backUpObject | request body | O | backUpObject | O | backUpObject-Type |
| trendIndication | request body | O | trendIndication | O | trendIndication-Type |
| thresholdInfo | request body | O | thresholdInfo | O | thresholdInfo-Type |
| stateChangeDefinition | request body | O | stateChangeDefinition | O | array(attributeValueChange-Type) |
| monitoredAttributes | request body | O | monitoredAttributes | O | array(attributeNameValuePair-Type) |
| proposedRepairActions | request body | O | proposedRepairActions | O | proposedRepairActions-Type |
| additionalText | request body | O | additionalText | O | additionalText-Type |
| additionalInformation | request body | O | additionalInformation | O | array(attributeNameValuePair-Type) |
| alarmId | request body | O | alarmId | O | alarmId-Type |

### X.2.3	Notification notifyNewSecurityAlarm

Editor’s note: this is FFS.

### X.2.4	Notification notifyAckStateChanged

Editor’s note: this is FFS.

### X.2.5	Notification notifyClearedAlarm

Editor’s note: this is FFS.

### X.2.6	Notification notifyAlarmListRebuilt

Editor’s note: this is FFS.

### X.2.7	Notification notifyChangedAlarm

Editor’s note: this is FFS.

### X.2.8	Notification notifyComments

Editor’s note: this is FFS.

### X.2.9	Notification notifyPotentialFaultyAlarmList

Editor’s note: this is FFS.

### X.2.10	Notification notifyCorrelatedNotificationChanged

Editor’s note: this is FFS.

### X.2.11	Notification notifyChangedAlarmGeneral

Editor’s note: this is FFS.

## X.3	Resources

All resources defined in clause 9.3 are valid here, except:

- subscriptions

- subscription.

## X.4	Data type definitions

Editor’s note: this is FFS.

Next modification

Annex A (normative): 
OpenAPI specification

## A.x	OpenAPI specification of ‘Fault3GPP’ for integration with ONAP VES

Editor’s note: this is FFS.

End