3GPP TSG-SA5 Meeting #124	S5-192091

# Taipei, Taiwan 25 February-1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0061 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements of RRC connection resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RRC connection re-establishment is for the purpose for UE resumes the suspended connection quickly or perform an RNA update.Resume with ResumeCause ::= ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess,spare1, spare2, spare3, spare4, spare5 } The performance of RRC connection resume procedure needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements of RRC connection resume |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of RRC connection resume cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.1.1.X	RRC Connection Resume

5.1.1.X.1 	Number of RRC connection resume attempts

a)	This measurement provides the number of RRC connection resume attempts.

b)	CC

c)	On Receipt of the RRCResumeRequest message or RRCResumeRequest1 from UE.Each RRCResumeRequest is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeAtt.cause

Where cause indicates the resume cause defined in 38.331.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

5.1.1.X.2	Successful RRC connection resume

a)	This measurement provides the successful number of RRC connection resume.

b)	CC

c)	On Receipt of a RRCResumeComplete message from UE for RRC connection resume .Each successful RRC Resume is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeSucc.cause

Where cause indicates the resume cause defined in 38.331.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

5.1.1.X.3	Successful RRC connection resume with fallback

a)	This measurement provides the successful number of RRC connection resume by fallback to RRC connection establishment.

b)	CC

c)	On Receipt of a RRCSetupComplete message from UE for RRC connection resume by fallback to RRC connection establishment. Each successful RRC Resume is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeSuccByFallback.cause

Where cause indicates the resume cause defined in 38.331.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

5.1.1.X.4	RRC connection resume followed by network release

a)	This measurement provides the number of RRC connection resume followed by network release.

b)	CC

c)	On Transmission of a RRCRelease  message to UE after RRC connection resume request.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ResumeFollowedbyNetworkRelease.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

5.1.1.X.5	RRC connection resume followed by network suspend

a)	This measurement provides the number of RRC connection resume followed by network suspend.

b)	CC

c)	On Transmission of a RRCRelease with suspend configuration message to UE after RRC connection resume request.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ResumeFollowedbySuspend.

f)	NRCellCU

g)	Valid for packet switching

h)	5GS

Next change

A.X Monitor of RRC connection resume

RRC connection resume is one of important step to start delivering services by the networks to users or for RNA update.

The success or failure of a RRC connection resume directly impacts the quality level for delivering the service by the networks, and also the feeling of the end user. So the success or failure of RRC connection resume needs be monitored, this can be achieved by the calculation of RRC connection resume success rate which gives a direct view to evaluate the resume performance, and the analysis of the specific reason causing the failure to find out the problem and ascertain the solutions.

End of changes