3GPP TSG-SA5 Meeting #124 	S5-192095

# Taipei, Taiwan 25 February - 1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0064 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of Split bear PDCP throughput measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/02/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Monitoring of PDCP layer split bear fully buffered throughput in the Dual Connectivity scenario is helpful to evaluate the performance of Dual Connectivity. It is also useful to analyse the performance of flow controlling features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of Split bear PDCP throughput are respectively added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The monitoring of split bear PDCP layer throughput is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), 5.1.1.X.1 (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X  	PDCP throughput measurements

5.1.1.X.1	Average split bear DL UE fully buffered Throughput

a) This measurement provides the average split bear down link fully buffered UE throughput. This measurement is intended for UE throughput when PDCP buffer size is more the desired buffer size for DU’s. This measurement only consider  PDCP buffering excluding the volume and time of the not fully buffered based on  DDDS feedback [TS 38.425].

b) DER(N=1)

c) This measurement is obtained according to the following formula based on the “ThroughputVolumeSplit”, “ThroughputTimeSplit”.

Figure 1

Table 5.1.1.X-1

| T0 | T0 : PDCP SDU volume size to be delivered is more than desired buffer size for the data radio bearer of RLC entities (for split bear considering first & secondary) based on desired buffer size per UE per bear through F1 interface message DL DATA DELIVERY STATUS [TS 38.425] . |
| --- | --- |
| T1 | T1 : PDCP SDU volume size to be delivered is less than desired buffer size for the data radio bearer of RLC entities (for split bear considering first & secondary) based on desired buffer size per UE per bear through F1 interface message DL DATA DELIVERY STATUS after T0. |
| T2 | T2 : PDCP SDU volume size to be delivered begins more than buffer size for the data radio bearer of first RLC entities based on the DL DATA DELIVERY STATUS . |
| T3 | T3 : PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of first RLC entities based on the last second DL DATA DELIVERY STATUS before T4. |
| T4 | T4 : PDCP SDU volume size to be delivered is less than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS after T2. |
| T5 | T5 : PDCP SDU volume size to be delivered begins more than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS . |
| T6 | T6 : PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of secondary RLC entities based on the last second DL DATA DELIVERY STATUS before T4. |
| T7 | T7 : PDCP SDU volume size to be delivered is less than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS after T2. |
| Tfullybufferedleg1 | T3-T2 |
| Tfullybufferedleg2 | T6-T5 |
| ThroughputTimeSplit | T1-T0 |
| ThroughputVolumeSplit | The data PDCP SDU volume for the first RLC entity per UE per bear, in kbits successfully transmitted (acknowledged by) in DL when PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of first and secondary RLC entities. |

d) Each measurement is a real value representing the throughput in kbits per second.

e) The measurement name has the form 
DRB.ThroughputVolumeSplit, DRB.ThroughputTimeSplit,

f) GNBCUUPFunction

NRCellCU.

g) Valid for packet switched traffic

h) 5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Next modified section

A.x Use case monitoring of split bear DL PDCP fully buffered Throughput

To monitor split bear DL PDCP fully buffered throughput is essential to ensure end user satisfaction and well functioning and well configured cells. Split bear DL PDCP fully buffered throughput helps the evaluation of the RAN congestion and network expanding, it is also useful to analyse the performance of flow controlling features.

End of modifications