3GPP TSG-SA5 Meeting #124 	S5-192115

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0050 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline charging service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The offline only charging service needs to be added in TS 32.291. The proper operations and procedures when applies offline only charging service should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support offline only charging service definition, a number of change are added in TS 32.291: The support of offline only charging service through Nchf interface; The operations of offline only charging service are clarified; The procedures of operators from protocol perspective are given. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The offline charging service through Nchf interface is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.291

4.1	Service Architecture

The Converged Charging Service or offline charging service is provided by the CHF to the consumer and shown in the SBI representation model in figure 4. 1.1. The 5G Data connectivity domain charging is depicted in 3GPP TS 32.255 [30].

The ConvergedCharging Service (Nchf_ ConvergedCharging) or offline charging serviceis part of the Nchf service-based interface exhibited by the Charging Function (CHF) , with SMF as the NF Service Consume.

# Figure 4. 1.1: Reference Architecture for the Nchf_ConvergedCharging Service; SBI representation

2nd change to TS 32.291

5.1	Introduction

The following services are provided by the CHF.

# Table 5.1-1: NF Services provided by CHF

| Service Name | Description | Consumer |
| --- | --- | --- |
| Nchf_ConvergedCharging service | This service provides a converged charging for session and event based NF services, with and without quota management, as well as charging information record generation | SMF |
| Nchf_OfflineCharging service | This service provides an offline only charging for session and event based NF service. | SMF |
| Nchf_SpendingLimitControl | This service enables the PCF to retrieve policy counter status information per UE from the CHF by subscribing to spending limit reporting (i.e. notifications of policy counter status changes). | PCF |

3rd change to TS 32.291

5.x	Nchf_OfflineCharging service

5.x.1	Service Description

This service provides charging in offline only charging scenario by the CHF to the NF service consumer (i.e. SMF) as defined in subclause 6.x in 3GPP TS 32.290 [58].

It includes the following functionalities:

-	Create resource at service establishment based on the request from NF consumer;

-	During the service consumption lifecycle, update resource based on the request from NF consumer;

-	Release upon service termination; and,

-	Charging information record generation.

5.x.2	Service Operations

5.x.2.1	Introduction

The service operations defined for Nchf_ OfflineCharging are shown in table 5.x.2.1-1.

# Table 5.x.2.1-1: Nchf_ OfflineCharging Operations

| Service Operation Name | Description | Initiated by | Corresponding Offline only charging messages in 3GPP TS 32.290[58] |
| --- | --- | --- | --- |
| Nchf_ OfflineCharging_Create | Initial report of service usage. | NF consumer | Charging Data Request/Response [Initial] |
| Nchf_ OfflineCharging_Update | Intermediate report of service usage. | NF consumer | Charging Data Request/Response [Update] |
| Nchf_ OfflineCharging_Release | Add last report of service usage. | NF consumer | Charging Data Request/Response [Termination] |

5.x.2.2	Nchf_OfflineCharging_Create Operation

The Nchf_OfflineCharging_Create service operation provides means for NF (CTF) to request initial report of service usage.

The following procedures using the Nchf_OfflineCharging_Create service operation are supported:

-	Create chargingdata resource.

Figure 5.x.2.2-1: Nchf_ OfflineCharging_Create Service Operation

1.  NF (CTF) sends a Nchf_OfflineCharging_Create request to the CHF to create resource for starting charging.

2a.  At successful operation, "201 Created" response is returned. In the "201 Created" response, the CHF includes a Location header field in the body. The Location header field shall contain the URI of the created resource. The NF (CTF) shall use the URI received in the Location header in subsequent requests to the CHF for the same PDU session.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

5.x.2.3	Nchf_OfflineCharging_Update Operation

The Nchf_OfflineCharging_Update service operation provides means for NF (CTF) to update the charging data.

The following procedures using the Nchf_OfflineCharging_Update service operation are supported:

-	charging events occur, which might affect the rating of the current service.

Figure 5.x.2.3-1: Nchf_OfflineCharging_Update Service Operation

1. NF (CTF) sends a Nchf_OfflineCharging_Update request to the CHF. The {ChargingSessionRef } in the URI identifies the "Charging Data" to be updated. The requested service unit and previous used service unit is included in the request body.

2a. At successful operation, "200 OK" response is returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

5.x.2.4	Nchf_OfflineCharging_Release Operation

The Nchf_OfflineCharging_Release service operation provides means for NF (CTF) to terminate charging Session.

The following procedures using the Nchf_OfflineCharging_Release service operation are supported:

-	Terminate chargingdata resource.

Figure 5.x.2.4-1: Nchf_OfflineCharging_Release Service Operation

1. NF(CTF) sends a Nchf_OfflineCharging_Release request to the CHF. The {ChargingSessionRef } in the URI identifies the "Charging Data" to be updated and then released. The final used service unit is included in the request body.

2a. At successful operation, "204 No Content" response is returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.3.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

End of change