3GPP TSG-SA5 Meeting #124 	S5-192190

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.626 | CR | 0007 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update State management data definition Solution Set to support JSON |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Complete JSON solution set with generic IOC and attribute type definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add JSON schema in state management data definition Solution Set |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The JSON solution is imcompleted which can’t be used by the developmer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4, New clause Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 28.623: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions”.

[2]	3GPP TS 28.625: "Telecommunication management; State Management Data Definition Integration Reference Point (IRP): Information Service (IS)".

[3]	ITU-T Recommendation X.721: "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[4]	ITU-T Recommendation M.3100: "Generic network information model".

[5]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[7]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[8]	Void.

[9]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures..

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[11]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[12]	ITU-T Recommendation X.721: "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[x]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS) "

End of 1st modification

Start of 2nd modification

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CM	Configuration Management

CORBA	Common Object Request Broker Architecture

DTD	Document Type Definition

EDGE	Enhanced Data for GSM Evolution

GERAN	GSM/EDGE Radio Access Network

GSM	Global System for Mobile communication

IDL	Interface Definition Language

IOC	Information Object Class

IRP	Integration Reference Point

IS	Information Service

JSON	JavaScript Object Notation

NE	Network Element

NRM	Network Resource Model

OMG	Object Management Group

SS	Solution Set

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

XML	eXtensible Markup Language

End of 2nd modification

Start of 3rd modification

# 4	Solution Set definitions

This specification defines the following 3GPP State Management Data Definition IRP Solution Set definitions:

-	3GPP State Management Data Definition IRP CORBA SS (Annex A)

-	3GPP State Management Data Definition IRP XML definitions (Annex B)

-	3GPP State Management Data Definition IRP JSON definitions (Annex X)

End of 3rd modification

Start of 4th modification

Annex X (normative): 
JSON definitions

# X.0	General

This annex specifies the JSON file format definition for the IRP whose semantics is specified in State Management Data Definition IRP: Information Service (IS) (3GPP TS  28.625  [2]).

# X.1	Architectural features

## X.1.0	Introduction

The overall architectural feature of State Management IRP is specified in 3GPP TS 28.625 [2].

This clause specifies features that are specific to the JSON Schema definitions.

## X.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [5].

# X.2	Mapping

## X.2.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| StateManagementEntity | stateManagementNRM | object |

## X.2.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| operationalState | operationalState | OperationalState |
| usageState | usageState | UsageState |
| administrativeState | administrativeState | AdministrativeState |
| alarmStatus | alarmStatus | AlarmStatus |
| proceduralStatus | proceduralStatus | ProceduralStatus |
| availabilityStatus | availabilityStatus | AvailabilityStatus |
| controlStatus | controlStatus | ControlStatus |
| standbyStatus | standbyStatus | StandbyStatus |
| unknownStatus | unknownStatus | UnknownStatus |

# X.3	Solution Set definitions

## X.3.1		JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

The definition of the JSON resource object complies with the generic rules defined in 3GPP TS 32.158 [x].

## X.3.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## X.3.3	JSON schema " stateManagementNrm.json"

{

"$schema": "http://json-schema.org/draft-05/schema#",

"id": "http://3gpp.org/28626/stateManagementNrm.json",

"description": "JSON based solution set definitions for State Management NRM",

"definitions": {

"AdministrativeState": {

"enum": ["Locked", "Unlocked", "ShuttingDown"]

},

"OperationalState": {

"enum": ["Enabled", "Disabled"]

},

"availabilityStatus": {

"enum": ["InTest", "Failed", "PowerOff", "OffLine", "OffDuty", "Dependency", "Degraded", "NotInstalled", "LogFull"]

},

"usageState": {

"enum": ["Idle", "Active", "Busy"]

},

"alarmStatus": {

"enum": ["Cleared", "Indeterminate", "Warning", "Minor", "Major", "Critical"]

},

"proceduralStatus": {

"enum": ["InitialisationRequired", "NotInitialised", "Initialising", "Reporting", "Terminating"]

},

"controlStatus": {

"enum": ["SubjectToTest", "PartOfServicesLocked", "ReservedForTest", "Suspended"]

},

"standbyStatus": {

"enum": ["HotStandby", "ColdStandby", "ProvidingService"]

},

"unknownStatus": {

"enum": ["True", "False"]

}

}

}

End of 4th modification