3GPP TSG-SA5 Meeting #124	S5-192216

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0071 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to DRB retainability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5GS, DRBs are the key and limited resources for NG-RAN to deliver services. The numbers of abnormally released DRBs and the time duration of the activity on the DRBs impacts the end-users experiences. To support the monitor of end-users experiences, the performance measurements related to DRB retainability should be measured. It is however considered not necessary to measure the UE-level DRB retainability since the UE will anyway have different requirements for different DRBs. To simply aggregate the time of the individual DRBs to UE level will not be accurate (e.g. for an UE with multiple DRBs there might be DRBs that are active at the same time, hence aggregating the DRB level measurements for session time will give a larger session time than the total UE session time). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DRB retainablity related measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.10, 5.1.10.x(new), 5.1.10.x.1(new), 5.1.10.y(new), 5.1.10.y.1(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.1.1.10	DRB related measurements

Next change

### 5.1.1.10.x	DRB release

#### 5.1.1.10.x.1	Number of released active DRBs

a)	This measurement provides the number of released DRBs that were active at the time of release. DRBs with bursty flow are seen as being active if any DRB data on a Data Radio Bearer (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are always seen as active DRBs in the context of this measurement.
The measurement is split into subcounters per mapped 5QI and per S-NSSAI.

b)	CC

c)	On

- transmission by the NG-RAN of a PDU SESSION RESOURCE RELEASE RESPONSE message for the PDU release initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE RELEASE COMMAND message with "Cause" equal to "Normal Release" or "User inactivity", "Load balancing TAU required", "Release due to CN-detected mobility", "O&M intervention", or

- transmission by the NG-RAN of a PDU SESSION RESOURCE MODIFY RESPONSE message for the PDU modification initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE MODIFY REQUEST message with the "Cause" equal to "Normal Release", or

- transmission by the NG-RAN of a UE CONTEXT RELEASE COMPLETE for the UE context release initiated by the NG-RAN with the exception of the corresponding UE CONTEXT RELEASE REQUEST message with the cause equal to "Normal Release" or "User inactivity", "Partial handover", "Successful handover", or

- transmission by the NG-RAN of a UE CONTEXT RELEASE COMPLETE message for the UE context release initiated by the AMF with the exception of the corresponding UE CONTEXT RELEASE COMMAND message with "Cause" equal to "Normal Release", "Handover Cancelled" or a successful mobility activity (e.g., cause "Successful Handover", or "NG Intra system Handover triggered"), or

- receipt by the NG-RAN of a PATH SWITCH REQUEST ACKNOWLEDGE or PATH SWITCH REQUEST FAILED message by which some or all DRBs in the corresponding PATH SWITCH REQUEST need to be released, or

- transmission of a NG RESET ACKNOWLEDGE message to AMF; or

- receipt of a NG RESET ACKNOWLEDGE message from AMF,

if any of the UL or DL DRBs are considered active, then corresponding counter is steped.

DRBs with bursty flow are considered active if any data (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are always seen as active DRBs in the context of this measurement. Each corresponding DRB to release is added to the relevant measurement per mapped 5QI and S-NSSAI. 

How to define for a particular DRB if the DRB is of type bursty flow or continuous flow is outside the scope of this document.

d)	Each measurement is an integer value. The number of measurements is equal to the number of mapped 5QI levels plus the number of S-NSSAIs.

e)	The measurements name has the form DRB.RelActNbr.5QI, where 5QI identifies the mapped 5QI and DRB.RelActNbr.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	This measurement is to support the Retainability KPI "DRB Retainability" defined in TS 28.554 [8].

### 5.1.1.10.y 	DRB activity

#### 5.1.1.10.y.1	In-session activity time for DRB

a)	This measurement provides the aggregated active session time for DRBs in a cell. The measurement is split into subcounters per mapped 5QI and per S-NSSAI.

b)	CC

c)	Number of "in session" seconds aggregated for DRBs with a certain mapped 5QI level or for a certain S-NSSAI, where "in session" has the following definitions:

- DRBs with bursty flow is said to be "in session" if any data (UL or DL) has been transferred during the last 100 ms for that DRB.

- DRBs with continuous flow are always seen as being "in session" in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

How to decide for a particular DRB if the DRB is of type continuous flow is outside the scope of this document.

d)	Each measurement is an integer value. The number of measurements is equal to the number of mapped 5QI levels plus the number of S-NSSAIs.

e)	The measurement name has the form DRB.SessionTime.5QI, where 5QI identifies the mapped 5QI and DRB.SessionTime.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	This measurement is to support the Retainability KPI "DRB Retainability" defined in TS 28.554 [8].

Next change

# A.X	Monitor of DRB release

DRB is the key and limited resource for NG-RAN to deliver services. The release of the DRB needs to be monitored as:

-	an abnormal release of the DRB will cause the call(/session) drop, which directly impacts the QoS and slice delivered by the networks and the, and the satisfaction degree of the end user;

-	a successfully released DRB can be used to setup other requested calls(/sessions). The DRB failed to be released will still occupy the limited resource and hence it can not be used to admit other requested calls(/sessions).

From a retainability measurement aspect, DRBs do not need to be released because they are inactive, they can be kept to give fast access when new data arrives.

To define (from a DRB release measurement point of view) if a DRB is considered active or not, the DRB can be divided into two groups:

- For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms.

- For DRBs with continuous flow, the DRB is always seen as being active in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

How to decide for a particular DRB if the DRB is of type bursty flow or continuous flow is outside the scope of this document.

The specific reason causing the abnormal and failed release of the DRB is required in order to find out the problem and ascertain the solutions. And due to different priority and tolerance for different service type with different QoS level (5QI) in the networks, the monitor needs to be opened on each service type with QoS level (5QI).

Similarly, the abnormal and failed DRB releases will affect different Service Level Agreements in the networks. Therefore, each S-NSSAI needs to be monitored.

The DRB can be released by PDU Session Resource Release procedure, UE Context Release procedure, Reset procedure either initiated by NG-RAN or AMF and NG Path Switch procedure (see 3GPP TS 38.413 [11]).

Therefore, performance measurements related to DRB Release (see 3GPP TS 38.413 [11]) and UE Context Release (see 3GPP TS 38.413 [11]) procedure for each QoS level (5QI) and each S-NSSAI are necessary to support the monitor of DRB release.

End of changes