3GPP TSG-SA5 Meeting #124	S5-192219

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0010 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add KPI for DRB Retainability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5GS, DRB retainability is a key performance indicator of end-user experience that shows how often an end-user abnormally looses an DRB during the time the DRB is used. To measure DRB retainability is therefore essetial for operators, and differentiation for 5QI and S-NSSAI will increase the usefulness further. It is however considered not necessary to measure the UE-level DRB retainability since the UE will anyway have different requirements for its different DRBs. It is also complex to measure the normalizing time for UE-level retainability: to simply aggregate the time of the individual DRBs to UE level will not be accurate (e.g. for an UE with multiple DRBs there might be DRBs that are active at the same time, hence aggregating the DRB level measurements for session time will give a larger session time than the total UE session time). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add KPI of DRB Retainability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 6.X.y(new), 6.X.y.1(new), 6.X.y.2(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This KPI to be placed under 6.X.y, there 6.X is introduced in S5-191386 in SA5#123 meeting. |  |  |  |  |  |  |  |  |

First change

## 6.X	Retainability KPI

### 6.X.y	DRB Retainability

#### 6.X.y.1	Definition

- DRB Retainability

- A measurement that shows how often an end-user abnormally looses a DRB during the time the DRB is active.

- Number of DRBs that were abnormally released and that were active at the time of release, normalized with number of data session time units.

- DRB Retainability for a single mapped 5QI level (R1) and for a single S-NSSAI (R1) are defined as:

R15QI_x=DRB.RelActNbr.5QI5QI_xDRB.SessionTime.5QI5QI_x

and

R1SNSSAI_x=DRB.RelActNbr.SNSSAISNSSAI_xDRB.SessionTime.SNSSAISNSSAI_x

- DRB.RelActNbr.5QI and DRB.RelActNbr.SNSSAI 

DRB.SessionTime.5QI and DRB.SessionTime.SNSSAI

- 5GS

- Retainability

- Active release/second

- MEAN

- The definition of the service provided by 5GS is DRBs

#### 6.X.y.2	Extended definition

To define (from a DRB Retainability point of view) if a DRB is considered active or not, the DRB can be divided into two groups:

- For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms.

- For DRBs with continuous flow, the DRB (and the UE) is always seen as being active in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

Next change

A.X	Use case for DRB retainability related KPI

DRB is the key and limited resource for 5GS to deliver services. The release of the DRB needs to be monitored. DRB retainability is a key performance indicator of how often an end-user abnormally loosing a DRB during the time the DRB is used. This key performance indicator is of great important to estimate the end users’ experiences.

End of changes