3GPP TSG-SA5 Meeting #124 	S5-192248

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0710 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE IP Addresses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For IP type PDU session, UE is allocated by SMF, an "Ipv4 Address" or "Ipv6 Prefix" or both "Ipv4 Address and Ipv6 Prefix": the current description refers to Ipv6 Address instead of IPv6 Prefix. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create a new PDUAddress data type to include all types Relocate "DynamicAddressflag" from PS domain CDRs to Generic ASN.1 definitions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unexpected format on UE IP address when IPv6 prefix is allocated may cause issues on correlation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2.2, 5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291. CR#0054 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

.$GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

ISDN-AddressString,

LCSClientExternalID,

LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic,

UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17)  version18 (18)}

-- from TS 29.002 [214]

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) modules (0) protocol (3)}

-- from Rec. X.711 [304]

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

-- from Rec. X.721 [305]

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) modules (0) apdus (0) version1 (1) };

-- Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227[306) / ISO 8650 corrigendum and not "ANY"

--

--  Generic Data Types

--

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

-- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

CalledNumber			::= BCDDirectoryNumber

CallingNumber	::= BCDDirectoryNumber

CellId	::= OCTET STRING (SIZE(2))

--

-- Coded according to TS 24.008 [208]

--

ChargeIndicator			::= INTEGER

{

noCharge			(0),

charge				(1)

}

CauseForRecClosing	::= INTEGER

--

-- Cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- sWGChange value is used in both the S-GW, TWAG and ePDG for inter serving node change

--

{

normalRelease					(0),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

sGWChange						(25),

aPNAMBRChange					(26),

mOExceptionDataCounterReceipt	(27),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

CauseForTerm			::= INTEGER

--

-- Cause codes from 16 up to 31 are defined as 'CauseForRecClosing'

-- (cause for record closing).

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214].

--

{

normalRelease						(0),

partialRecord						(1),

partialRecordCallReestablishment	(2),

unsuccessfulCallAttempt				(3),

abnormalRelease						(4),

cAMELInitCallRelease				(5),

unauthorizedRequestingNetwork		(52),

unauthorizedLCSClient				(53),

positionMethodFailure				(54),

unknownOrUnreachableLCSClient		(58)

}

CivicAddressInformation		::= OCTET STRING

--

-- as defined in subclause 3.1 of IETF RFC 4776 [409] excluding the first 3 octets.

--

DataVolumeOctets		::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

CNIPMulticastDistribution		::= ENUMERATED

{

nO-IP-MULTICAST				(0),

iP-MULTICAST				(1)

}

Diagnostics						::= CHOICE

{

gsm0408Cause								[0] INTEGER,

-- See TS 24.008 [208]

gsm0902MapErrorValue						[1] INTEGER,

--

-- Note: The value to be stored here corresponds to the local values defined in the MAP-Errors

-- and MAP-DialogueInformation modules, for full details see TS 29.002 [214].

--

itu-tQ767Cause							[2] INTEGER,

-- See Q.767 [309]

networkSpecificCause						[3] ManagementExtension,

-- To be defined by network operator

manufacturerSpecificCause				[4] ManagementExtension,

-- To be defined by manufacturer

positionMethodFailureCause				[5] PositionMethodFailure-Diagnostic,

-- see TS 29.002 [214]

unauthorizedLCSClientCause				[6] UnauthorizedLCSClient-Diagnostic,

-- see TS 29.002 [214]

diameterResultCodeAndExperimentalResult	[7] INTEGER

-- See TS 29.338 [230], TS 29.337 [231], TS 29.128 [244]

}

DiameterIdentity		::= OCTET STRING

EnhancedDiagnostics					::= SEQUENCE

{

rANNASCause						[0] SEQUENCE OF RANNASCause

}

GSNAddress	::= IPAddress

IPAddress	::= CHOICE

{

iPBinaryAddress	 IPBinaryAddress,

iPTextRepresentedAddress	 IPTextRepresentedAddress

}

IPBinaryAddress	::= CHOICE

{

iPBinV4Address			[0] IPBinV4Address,

iPBinV6Address			    IPBinV6AddressWithOrWithoutPrefixLength

}

IPBinV4Address	::= OCTET STRING (SIZE(4))

IPBinV6Address	::= OCTET STRING (SIZE(16))

IPBinV6AddressWithOrWithoutPrefixLength ::= CHOICE

{

iPBinV6Address             [1] IPBinV6Address,

iPBinV6AddressWithPrefix   [4] IPBinV6AddressWithPrefixLength

}

IPBinV6AddressWithPrefixLength ::= SEQUENCE

{

iPBinV6Address                    IPBinV6Address,

pDPAddressPrefixLength            PDPAddressPrefixLength DEFAULT 64

}

IPTextRepresentedAddress	::= CHOICE

{	--

-- IP address in the familiar "dot" notation

--

iPTextV4Address		[2] IA5String (SIZE(7..15)),

iPTextV6Address		[3] IA5String (SIZE(15..45))

}

LCSCause				::= OCTET STRING (SIZE(1))

--

-- See LCS Cause Value, TS 49.031 [227]

--

LCSClientIdentity 		::= SEQUENCE

{

lcsClientExternalID	[0] LCSClientExternalID OPTIONAL,

lcsClientDialedByMS	[1] AddressString OPTIONAL,

lcsClientInternalID	[2] LCSClientInternalID OPTIONAL

}

LCSQoSInfo				::= OCTET STRING (SIZE(4))

--

-- See LCS QoS IE, TS 49.031 [227]

--

LevelOfCAMELService		::= BIT STRING

{

basic						(0),

callDurationSupervision		(1),

onlineCharging				(2)

}

LocalSequenceNumber ::= INTEGER (0..4294967295)

--

-- Sequence number of the record in this node

-- 0.. 4294967295 is equivalent to 0..2**32-1, unsigned integer in four octets

--

LocationAreaAndCell		::= SEQUENCE

{

locationAreaCode	[0] LocationAreaCode,

cellId				[1] CellId,

mCC-MNC				[2] MCC-MNC OPTIONAL

}

LocationAreaCode		::= OCTET STRING (SIZE(2))

--

-- See TS 24.008 [208]

--

ManagementExtensions	::= SET OF ManagementExtension

MBMS2G3GIndicator		::= ENUMERATED

{

twoG				(0),	-- For GERAN access only

threeG			(1),	-- For UTRAN access only

twoG-AND-threeG	(2) 	-- For both UTRAN and GERAN access

}

MBMSInformation			::= SET

{

tMGI							[1] TMGI OPTIONAL,

mBMSSessionIdentity			[2] MBMSSessionIdentity OPTIONAL,

mBMSServiceType				[3] MBMSServiceType OPTIONAL,

mBMSUserServiceType			[4] MBMSUserServiceType OPTIONAL, -- only supported in the BM-SC

mBMS2G3GIndicator			[5] MBMS2G3GIndicator OPTIONAL,

fileRepairSupported			[6] BOOLEAN OPTIONAL,			  -- only supported in the BM-SC

rAI							[7] RoutingAreaCode OPTIONAL,	  -- only supported in the BM-SC

mBMSServiceArea				[8] MBMSServiceArea OPTIONAL,

requiredMBMSBearerCaps		[9] RequiredMBMSBearerCapabilities OPTIONAL,

mBMSGWAddress				[10] GSNAddress OPTIONAL,

cNIPMulticastDistribution	[11] CNIPMulticastDistribution OPTIONAL,

mBMSDataTransferStart		[12] MBMSTime OPTIONAL,

mBMSDataTransferStop			[13] MBMSTime OPTIONAL

}

MBMSServiceArea		::= OCTET STRING

MBMSServiceType		::= ENUMERATED

{

mULTICAST		(0),

bROADCAST		(1)

}

MBMSSessionIdentity		::= OCTET STRING (SIZE (1))

--

-- This octet string is a 1:1 copy of the contents of the MBMS-Session-Identity

-- AVP specified in TS 29.061 [82]

--

MBMSTime	::= OCTET STRING (SIZE (8))

--

-- This value indicates the time in seconds relative to 00:00:00 on 1 January 1900 (calculated as

-- continuous time without leap seconds and traceable to a common time reference) where binary

-- encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in

-- the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second as

-- specified in TS 29.061 [82].

--

MBMSUserServiceType		::= ENUMERATED

{

dOWNLOAD		(0),

sTREAMING	(1)

}

MCC-MNC		::= OCTET STRING (SIZE(3))

--

-- See TS 24.008 [208]

--

MessageReference		::= OCTET STRING

MSCAddress		::= AddressString

MscNo					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSISDN					::= ISDN-AddressString

--

-- See TS 23.003 [200]

--

MSTimeZone	::= OCTET STRING (SIZE (2))

--

-- 1.Octet: Time Zone and 2. Octet: Daylight saving time, see TS 29.060 [215]

--

NodeAddress ::= CHOICE

{

iPAddress 	[0] IPAddress,

domainName	[1] GraphicString

}

PDPAddressPrefixLength		::=INTEGER (1..64)

--

-- This is an interger indicating the leght of the PDP/PDN IPv6 address prefix

-- and the default value is 64 bits.

--

PositioningData	::= OCTET STRING (SIZE(1..33))

--

-- See Positioning Data IE (octet 3..n), TS 49.031 [227]

--

RANNASCause				::=	OCTET STRING

-- This octet string is a 1:1 copy of the contents (i.e. starting with octet 5)

-- of the "RAN/NAS Cause" information element specified in TS 29.274 [223].

RecordingEntity 		::= AddressString

RecordType 	::= INTEGER

--

--	Record values 0..17 and 87,89  are CS specific. The contents are defined in TS 32.250 [10]

--

{

moCallRecord			(0),

mtCallRecord			(1),

roamingRecord		(2),

incGatewayRecord		(3),

outGatewayRecord		(4),

transitCallRecord	(5),

moSMSRecord			(6),

mtSMSRecord			(7),

moSMSIWRecord		(8),

mtSMSGWRecord		(9),

ssActionRecord		(10),

hlrIntRecord			(11),

locUpdateHLRRecord	(12),

locUpdateVLRRecord	(13),

commonEquipRecord	(14),

moTraceRecord		(15),	-- used in earlier releases

mtTraceRecord		(16),	-- used in earlier releases

termCAMELRecord	    (17),

--

--	Record values 18..22 are GPRS specific. The contents are defined in TS 32.251 [11]

--

sgsnPDPRecord		(18),

sgsnMMRecord			(20),

sgsnSMORecord		(21),	-- also MME UE originated SMS record

sgsnSMTRecord		(22),	-- also MME UE terminated SMS record

--

--  Record values 23..25 are CS-LCS specific. The contents are defined in TS 32.250 [10]

--

mtLCSRecord			(23),

moLCSRecord			(24),

niLCSRecord			(25),

--

--  Record values 26..28 are GPRS-LCS specific. The contents are defined in TS 32.251 [11]

--

sgsnMTLCSRecord		(26),

sgsnMOLCSRecord		(27),

sgsnNILCSRecord		(28),

--

--  Record values 30..62 are MMS specific. The contents are defined in TS 32.270 [30]

--

mMO1SRecord			(30),

mMO4FRqRecord		(31),

mMO4FRsRecord		(32),

mMO4DRecord			(33),

mMO1DRecord			(34),

mMO4RRecord			(35),

mMO1RRecord			(36),

mMOMDRecord			(37),

mMR4FRecord			(38),

mMR1NRqRecord		(39),

mMR1NRsRecord		(40),

mMR1RtRecord			(41),

mMR1AFRecord			(42),

mMR4DRqRecord		(43),

mMR4DRsRecord		(44),

mMR1RRRecord			(45),

mMR4RRqRecord		(46),

mMR4RRsRecord		(47),

mMRMDRecord			(48),

mMFRecord			(49),

mMBx1SRecord			(50),

mMBx1VRecord			(51),

mMBx1URecord			(52),

mMBx1DRecord			(53),

mM7SRecord			(54),

mM7DRqRecord			(55),

mM7DRsRecord			(56),

mM7CRecord			(57),

mM7RRecord			(58),

mM7DRRqRecord		(59),

mM7DRRsRecord		(60),

mM7RRqRecord			(61),

mM7RRsRecord			(62),

--

--  Record values 63..70, 82, 89..91 are IMS specific.

--  The contents are defined in TS 32.260 [20]

--

sCSCFRecord			(63),

pCSCFRecord			(64),

iCSCFRecord			(65),

mRFCRecord			(66),

mGCFRecord			(67),

bGCFRecord			(68),

aSRecord				(69),

eCSCFRecord			(70),

iBCFRecord			(82),

tRFRecord			(89),

tFRecord				(90),

aTCFRecord			(91),

--

--  Record values 71..75 are LCS specific. The contents are defined in TS 32.271 [31]

--

lCSGMORecord			(71),

lCSRGMTRecord		(72),

lCSHGMTRecord		(73),

lCSVGMTRecord		(74),

lCSGNIRecord			(75),

--

--  Record values 76..79,86 are MBMS specific.

--  The contents are defined in TS 32.251 [11] and TS 32.273 [33]

--

--  Record values 76,77 and 86 are MBMS bearer context specific

--

sgsnMBMSRecord		(76),

ggsnMBMSRecord		(77),

gwMBMSRecord		(86),

--

--  Record values 78 and 79 are MBMS service specific and defined in TS 32.273 [33]

--

sUBBMSCRecord		(78),

cONTENTBMSCRecord	(79),

--

--  Record Values 80..81 are PoC specific. The contents are defined in TS 32.272 [32]

--

pPFRecord			(80),

cPFRecord			(81),

--

--	Record values 84,85 and 92,95,96, 97 are EPC specific.

--  The contents are defined in TS 32.251 [11]

--

sGWRecord			(84),

pGWRecord			(85),

tDFRecord			(92),

iPERecord			(95),

ePDGRecord			(96),

tWAGRecord			(97),

--

--  Record Value 83 is MMTel specific. The contents are defined in TS 32.275 [35]

--

mMTelRecord			(83),

--

--  Record value  87,88 and 89 are CS specific. The contents are defined in TS 32.250 [10]

--

mSCsRVCCRecord		(87),

mMTRFRecord			(88),

iCSRegisterRecord	(99),

--

--  Record values 93 and 94 are SMS specific. The contents are defined in TS 32.274 [34]

--

sCSMORecord			(93),

sCSMTRecord			(94),

--

--  Record values 100, 101 and 102 are ProSe specific. The contents are defined in TS 32.277 [36]

--

pFDDRecord			(100),

pFEDRecord			(101),

pFDCRecord			(102),

--

--  Record values103 and 104 are Monitoring Event specific. The contents are defined in TS

--	32.278 [38]

--

mECORecord			(103),

mERERecord			(104),

--

--  Record values 105 to 106 are CP data transfer specific. The contents are defined in TS

--	32.253 [13]

--

cPDTSCERecord			(105),

cPDTSNNRecord			(106), --

--  Record values 110 to 113 are SMS specific. The contents are defined in TS

--	32.274 [34]

--

sCDVTT4Record			(110),

sCSMOT4Record			(111),

iSMSMORecord			(112),

iSMSMTRecord			(113),

--

--  Record values 120 are Exposure Function API specific. The contents are defined in TS

--	32.254 [14]

--

eASCERecord			(120),

--

--  Record values from 200 are specific to Charging Function domain

--

chargingFunctionRecord		(200)

--

}

RequiredMBMSBearerCapabilities		::= OCTET STRING (SIZE (3..14))

--

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 5) of the

-- "Quality of service Profile" information element specified in TS 29.060 [75].

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [208]

--

SCSASAddress		::= SET

--

--

--

{

sCSAddress	[1] IPAddress,

sCSRealm		[2] DiameterIdentity

}

ServiceContextID		::= UTF8String

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData		[0] GraphicString OPTIONAL, 
	serviceSpecificType		[1] INTEGER OPTIONAL

}

SMSResult					::= Diagnostics

SmsTpDestinationNumber ::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- the SMS address field the encoding of the octet string is in

-- accordance with the definition of address fields in TS 23.040 [201].

-- This encoding includes type of number and numbering plan indication

-- together with the address value range.

--

SubscriberEquipmentNumber	::= SET

--

-- If SubscriberEquipmentType is set to IMEISV and IMEI is received, the number of digits is 15.

--

{

subscriberEquipmentNumberType	[0]	SubscriberEquipmentType,

subscriberEquipmentNumberData	[1]	OCTET STRING

}

SubscriberEquipmentType	::= ENUMERATED

--

-- It should be noted that depending on the services, not all equipment types are applicable.

-- For IMS equipment types 0 and 3 are applicable.

--

{

iMEISV			(0),

mAC				(1),

eUI64			(2),

modifiedEUI64	(3)

}

SubscriptionID	::= SET

--

-- used for ExternalIdentifier with SubscriptionIdType = END-User-NAI. See TS 23.003 [200]

--

{

subscriptionIDType	[0]	SubscriptionIDType,

subscriptionIDData	[1]	UTF8String

}

SubscriptionIDType	::= ENUMERATED

{

eND-USER-E164		(0),

eND-USER-IMSI		(1),

eND-USER-SIP-URI		(2),

eND-USER-NAI			(3),

eND-USER-PRIVATE		(4)

}

SystemType	::= ENUMERATED

--

--  "unknown" is not to be used in PS domain.

--

{

unknown				(0),

iuUTRAN				(1),

gERAN				(2)

}

ThreeGPPPSDataOffStatus		::= ENUMERATED

{

active		    (0),

inactive		(1)

}

TimeStamp	::= OCTET STRING (SIZE(9))

--

-- The contents of this field are a compact form of the UTCTime format

-- containing local time plus an offset to universal time. Binary coded

-- decimal encoding is employed for the digits to reduce the storage and

-- transmission overhead

-- e.g. YYMMDDhhmmssShhmm

-- where

-- YY 	= 	Year 00 to 99		BCD encoded

-- MM 	= 	Month 01 to 12 		BCD encoded

-- DD	=	Day 01 to 31		BCD encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

-- ss	=	second 00 to 59		BCD encoded

-- S	=	Sign 0 = "+", "-"	ASCII encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

--

TMGI		::= OCTET STRING

--

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 4)

-- of the "TMGI" information element specified in TS 29.060 [75].

--

.#END

Next change

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

.$GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

CallDuration,

CallingNumber,

CauseForRecClosing,

CellId,

CivicAddressInformation,

Diagnostics,

DiameterIdentity,

DynamicAddressFlag,

EnhancedDiagnostics,

GSNAddress,

IPAddress,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocalSequenceNumber,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MBMSInformation,

MessageReference,

MSISDN,

MSTimeZone,

PositioningData,

RecordingEntity,

RecordType,

RoutingAreaCode,

SCSASAddress,

ServiceSpecificInfo,

SMSResult,

SmsTpDestinationNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DefaultGPRS-Handling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version18 (18)}

-- from TS 29.002 [214]

IMEI,

IMSI,

ISDN-AddressString,

RAIdentity

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

-- from TS 29.002 [214]

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CH-DataTypes (13) version18 (18)}

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version18 (18) }

-- from TS 29.002 [214]

LocationMethod

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version12 (12)}

-- from TS 24.080 [209]

;

--

--  GPRS RECORDS

--

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78,79 and 92, 95, 96 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnMTLCSRecord			[25] SGSNMTLCSRecord,

sgsnMOLCSRecord			[26] SGSNMOLCSRecord,

sgsnNILCSRecord			[27] SGSNNILCSRecord,

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord,

gwMBMSRecord			[86] GWMBMSRecord,

tDFRecord				[92] TDFRecord,

iPERecord				[95] IPERecord,

ePDGRecord				[96] EPDGRecord,

tWAGRecord				[97] TWAGRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

s-GWiPv6Address				[48] GSNAddress OPTIONAL,

servingNodeiPv6Address		[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

userLocationInfoTime		[52] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[53] CNOperatorSelectionEntity OPTIONAL,

presenceReportingAreaInfo	[54] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation	[55] OCTET STRING OPTIONAL,

lastMSTimeZone				[56] MSTimeZone OPTIONAL,

enhancedDiagnostics			[57] EnhancedDiagnostics OPTIONAL,

cPCIoTEPSOptimisationIndicator [59] CPCIoTEPSOptimisationIndicator OPTIONAL,

uNIPDUCPOnlyFlag			[60] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl		[61] ServingPLMNRateControl OPTIONAL,

pDPPDNTypeExtension			[62] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter		[63] MOExceptionDataCounter OPTIONAL,

listOfRANSecondaryRATUsageReports [64] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

PGWRecord 	::= SET

--

--	List of traffic volumes is only applicable when Charging per IP-CAN session is active and

--	IP-CAN bearer charging is being performed for the session.

--

--	EPC QoS Information is only applicable when Charging per IP-CAN session is active.

--

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

lowPriorityIndicator				[46] NULL OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

userLocationInfoTime				[53] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		    	[54] CNOperatorSelectionEntity OPTIONAL,

ePCQoSInformation					[55] EPCQoSInformation OPTIONAL,

presenceReportingAreaInfo			[56] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation			[57] OCTET STRING OPTIONAL,

lastMSTimeZone						[58] MSTimeZone OPTIONAL,

enhancedDiagnostics					[59] EnhancedDiagnostics OPTIONAL,

nBIFOMMode							[60] NBIFOMMode OPTIONAL,

nBIFOMSupport						[61] NBIFOMSupport OPTIONAL,

uWANUserLocationInformation			[62] UWANUserLocationInfo OPTIONAL,

sGiPtPTunnellingMethod				[64] SGiPtPTunnellingMethod OPTIONAL,

uNIPDUCPOnlyFlag					[65] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl				[66] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[67] APNRateControl OPTIONAL,

pDPPDNTypeExtension					[68] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter				[69] MOExceptionDataCounter OPTIONAL,

chargingPerIPCANSessionIndicator    [70] ChargingPerIPCANSessionIndicator OPTIONAL,

threeGPPPSDataOffStatus				[71] ThreeGPPPSDataOffStatus OPTIONAL,

sCSASAddress						[72] SCSASAddress OPTIONAL,

listOfRANSecondaryRATUsageReports	[73] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

TDFRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

tDFAddress							[53] GSNAddress,

tDFiPv6AddressUsed					[54] GSNAddress OPTIONAL,

tDFPLMNIdentifier					[55] PLMN-Id OPTIONAL,

servedFixedSubsID					[56] FixedSubsID OPTIONAL,

accessLineIdentifier				[57] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation		[59] FixedUserLocationInformation OPTIONAL

}

IPERecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

iPEdgeAddress					[4] GSNAddress,

chargingID						[5] ChargingID,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

iPCANsessionType				[8] PDPType OPTIONAL,

servedIPCANsessionAddress		[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI		   				[29] IMEI OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servedMNNAI						[36] SubscriptionID OPTIONAL,

iPEdgeOperatorIdentifier		[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

servedIPCANsessionAddressExt 	[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt			[47] DynamicAddressFlag OPTIONAL,

iPEdgeiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission   				[52] NULL OPTIONAL,

servedFixedSubsID				[55] FixedSubsID OPTIONAL,

accessLineIdentifier			[56] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation	[57] FixedUserLocationInformation OPTIONAL

}

EPDGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

ePDGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

ePDGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

uWANUserLocationInformation	[53] UWANUserLocationInfo OPTIONAL,

userLocationInfoTime		[54] TimeStamp OPTIONAL,

iMSIunauthenticatedFlag 	[55] NULL OPTIONAL

}

TWAGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

tWAGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

tWAGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

tWANUserLocationInformation	[53] TWANUserLocationInfo OPTIONAL,

iMSIunauthenticatedFlag 	[54] NULL OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL,

servingNodePLMNIdentifier	[24] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[25] CNOperatorSelectionEntity OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL,

servingNodePLMNIdentifier	[38] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[39] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMORecord	::= SET

--

--   also for MME UE originated SMS record

--

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL,

servingNodeType				[21] ServingNodeType,

servingNodeAddress			[22] GSNAddress OPTIONAL,

servingNodeiPv6Address		[23] GSNAddress OPTIONAL,

mMEName						[24] DiameterIdentity OPTIONAL,

mMERealm					[25] DiameterIdentity OPTIONAL,

userLocationInformation		[26] OCTET STRING OPTIONAL,

retransmission				[27] NULL OPTIONAL,

servingNodePLMNIdentifier	[28] PLMN-Id OPTIONAL,

userLocationInfoTime		[29] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[30] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMTRecord	::= SET

--

--   also for MME UE terminated SMS record

--

{

recordType					 [0] RecordType,

servedIMSI					 [1] IMSI,

servedIMEI					 [2] IMEI OPTIONAL,

servedMSISDN				 [3] MSISDN OPTIONAL,

msNetworkCapability			 [4] MSNetworkCapability OPTIONAL,

serviceCentre				 [5] AddressString OPTIONAL,

recordingEntity				 [6] RecordingEntity OPTIONAL,

locationArea				 [7] LocationAreaCode OPTIONAL,

routingArea					 [8] RoutingAreaCode OPTIONAL,

cellIdentifier				 [9] CellId OPTIONAL,

eventTimeStamp				 [10] TimeStamp,

smsResult					 [11] SMSResult OPTIONAL,

recordExtensions			 [12] ManagementExtensions OPTIONAL,

nodeID						 [13] NodeID OPTIONAL,

localSequenceNumber			 [14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [15] ChargingCharacteristics,

rATType						 [16] RATType OPTIONAL,

chChSelectionMode			 [17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS			 [18] CAMELInformationSMS OPTIONAL,

originatingAddress			 [19] AddressString OPTIONAL,

servingNodeType				 [20] ServingNodeType,

servingNodeAddress			 [21] GSNAddress OPTIONAL,

servingNodeiPv6Address		 [22] GSNAddress OPTIONAL,

mMEName						 [23] DiameterIdentity OPTIONAL,

mMERealm					 [24] DiameterIdentity OPTIONAL,

userLocationInformation		 [25] OCTET STRING OPTIONAL,

retransmission				 [26] NULL OPTIONAL,

servingNodePLMNIdentifier	 [27] PLMN-Id OPTIONAL,

userLocationInfoTime		 [28] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType,

lcsClientIdentity			 [3] LCSClientIdentity,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationType				 [7] LocationType,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

notificationToMSUser		 [13] NotificationToMSUser OPTIONAL,

privacyOverride				 [14] NULL OPTIONAL,

location					 [15] LocationAreaAndCell OPTIONAL,

routingArea					 [16] RoutingAreaCode OPTIONAL,

locationEstimate			 [17] Ext-GeographicalInformation OPTIONAL,

positioningData				 [18] PositioningData OPTIONAL,

lcsCause					 [19] LCSCause OPTIONAL,

diagnostics					 [20] Diagnostics OPTIONAL,

nodeID						 [21] NodeID OPTIONAL,

localSequenceNumber			 [22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [23] ChargingCharacteristics,

chChSelectionMode			 [24] ChChSelectionMode OPTIONAL,

rATType						 [25] RATType OPTIONAL,

recordExtensions			 [26] ManagementExtensions OPTIONAL,

causeForRecClosing			 [27] CauseForRecClosing,

servingNodePLMNIdentifier	 [28] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMOLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationMethod				 [7] LocationMethod,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNNILCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI OPTIONAL,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

servedIMEI					 [7] IMEI OPTIONAL,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] INTEGER OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

--

--  PS DATA TYPES

--

AccessAvailabilityChangeReason		::= INTEGER (0..4294967295)

--

-- 0 (RAN rule indication)   : This value shall be used to indicate that the availability

--                             of an access is changed due to the RAN rule indication.

-- 1 (Access usable/unusable): This value shall be used to indicate that the availability

--                             of an access is changed due to the access is unusable or usable

--                             again.

--

AccessLineIdentifier	::= SEQUENCE

--

-- "Physical Access Id" includes a port identifier and the identity of the access node where the

--  port resides. "logical Access Id" contains a Circuit-ID. Both are defined ETSI TS 283 034 [314]

--

{

physicalAccessID	[0] UTF8String OPTIONAL,

logicalAccessID		[1] OCTET STRING OPTIONAL

}

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

ADCRuleBaseName		::= IA5String

--

-- identifier for the group of charging rules

-- see ADC-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

AdditionalExceptionReports		::= ENUMERATED

{

notAllowed		(0),

allowed			(1)

}

AFChargingIdentifier	::= OCTET STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNRateControl		::= SEQUENCE

--

-- See TS 24.008 [208] for more information

--

{

aPNRateControlUplink	[0] APNRateControlParameters OPTIONAL,

aPNRateControlDownlink	[1] APNRateControlParameters OPTIONAL

}

APNRateControlParameters		::= SEQUENCE

{

additionalExceptionReports	[0] AdditionalExceptionReports OPTIONAL,

rateControlTimeUnit			[1] RateControlTimeUnit OPTIONAL,

rateControlMaxRate			[2] INTEGER OPTIONAL,

rateControlMaxMessageSize	[3] DataVolumeGPRS OPTIONAL -- aPNRateControlDownlink only

}

APNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7] FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8] FreeFormatData OPTIONAL,

fFDAppendIndicator				[9] FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2] ServiceKey OPTIONAL,

defaultSMSHandling						[3] DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7] FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

ChangeCondition	::= ENUMERATED

{

qoSChange						(0),

tariffTime						(1),

recordClosure					(2),

cGI-SAICHange					(6),	-- bearer modification. "CGI-SAI Change"

rAIChange						(7),	-- bearer modification. "RAI Change"

dT-Establishment				(8),

dT-Removal						(9),

eCGIChange						(10),	-- bearer modification. "ECGI Change"

tAIChange						(11),	-- bearer modification. "TAI Change"

userLocationChange				(12),	-- bearer modification. "User Location Change"

userCSGInformationChange		(13), 	-- bearer modification. "User CSG info Change"

presenceInPRAChange				(14),	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

removalOfAccess					(15),	-- NBIFOM "Removal of Access"

unusabilityOfAccess				(16),	-- NBIFOM "Unusability of Access"

indirectChangeCondition			(17),	-- NBIFOM "Indirect Change Condition"

userPlaneToUEChange				(18),	-- bearer modification. "Change of user plane to UE"

servingPLMNRateControlChange	(19),

-- bearer modification “Serving PLMN Rate Control Change"

threeGPPPSDataOffStatusChange	(20)   -- "Change of 3GPP PS DataO ff Status"

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR,PGW-CDR, IPE-CDR, TWAG-CDR and ePDG-CDR only

-- userLocationInformation is used only in S-CDR, SGW-CDR and PGW-CDR

-- chargingID used in PGW-CDR only when Charging per IP-CAN session is active

-- accessAvailabilityChangeReason and relatedChangeOfCharCondition applicable only in PGW-CDR

-- cPCIoTOptimisationIndicator is used in SGW-CDR only

--

{

qosRequested					[1] QoSInformation OPTIONAL,

qosNegotiated					[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink			[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink			[4] DataVolumeGPRS OPTIONAL,

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

ePCQoSInformation				[9] EPCQoSInformation OPTIONAL,

chargingID						[10] ChargingID OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

diagnostics						[13] Diagnostics OPTIONAL,

enhancedDiagnostics				[14] EnhancedDiagnostics OPTIONAL,

rATType							[15] RATType OPTIONAL,

accessAvailabilityChangeReason	[16] AccessAvailabilityChangeReason OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL,

relatedChangeOfCharCondition	[18] RelatedChangeOfCharCondition OPTIONAL,

cPCIoTEPSOptimisationIndicator	[19] CPCIoTEPSOptimisationIndicator OPTIONAL,

servingPLMNRateControl			[20] ServingPLMNRateControl OPTIONAL,

threeGPPPSDataOffStatus			[21] ThreeGPPPSDataOffStatus OPTIONAL,

listOfPresenceReportingAreaInformation	[22] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging and Application based Charging service data container

-- presenceReportingAreaStatus is used in PGW-CDR Only

--

{

ratingGroup 						[1] RatingGroupId,

chargingRuleBaseName				[2] ChargingRuleBaseName OPTIONAL,

resultCode							[3] ResultCode OPTIONAL,

localSequenceNumber					[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[5] TimeStamp OPTIONAL,

timeOfLastUsage						[6] TimeStamp OPTIONAL,

timeUsage 							[7] CallDuration OPTIONAL,

serviceConditionChange				[8] ServiceConditionChange,

qoSInformationNeg					[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 					[10] GSNAddress OPTIONAL,

datavolumeFBCUplink					[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink				[13] DataVolumeGPRS OPTIONAL,

timeOfReport						[14] TimeStamp,

failureHandlingContinue				[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier					[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation		[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation					[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation				[20] OCTET STRING OPTIONAL,

eventBasedChargingInformation		[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism					[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo					[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

sponsorIdentity						[25] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[26] OCTET STRING OPTIONAL,

aDCRuleBaseName						[27] ADCRuleBaseName OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedChangeOfServiceCondition		[33] RelatedChangeOfServiceCondition OPTIONAL,

servingPLMNRateControl				[35] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[36] APNRateControl OPTIONAL,

threeGPPPSDataOffStatus             [37] ThreeGPPPSDataOffStatus OPTIONAL,

trafficSteeringPolicyIDDownlink     [38] TrafficSteeringPolicyIDDownlink OPTIONAL,

trafficSteeringPolicyIDUplink       [39] TrafficSteeringPolicyIDUplink OPTIONAL,

tWANUserLocationInformation			[40] TWANUserLocationInfo OPTIONAL,

listOfPresenceReportingAreaInformation	[41] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP-CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

ChargingPerIPCANSessionIndicator	::= ENUMERATED

{

inactive		(0),

active               (1)

}

ChargingRuleBaseName	::= IA5String

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

roamingDefault				(4),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

visitingDefault				(5),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

fixedDefault				(6)		-- For TDF and IP-Edge

}

CNOperatorSelectionEntity	::= ENUMERATED

{

servCNSelectedbyUE		(0),

servCNSelectedbyNtw		(1)

}

CPCIoTEPSOptimisationIndicator ::= BOOLEAN

CSGAccessMode		::= ENUMERATED

{

closedMode  (0),

hybridMode  (1)

}

CSGId		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 23.003 [200]. Coded according to TS 29.060 [215] for GTP, and

-- in TS 29.274 [223] for eGTP.

--

CTEID		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier

-- of MBMS GW for user plane, defined in TS 23.246 [207].

--

DataVolumeGPRS		::= INTEGER

--

-- The volume of data transferred in octets.

--

DataVolumeMBMS ::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI							[1] INTEGER,

maxRequestedBandwithUL		[2] INTEGER OPTIONAL,

maxRequestedBandwithDL		[3] INTEGER OPTIONAL,

guaranteedBitrateUL			[4] INTEGER OPTIONAL,

guaranteedBitrateDL			[5] INTEGER OPTIONAL,

aRP							[6] INTEGER OPTIONAL,

aPNAggregateMaxBitrateUL	[7] INTEGER OPTIONAL,

aPNAggregateMaxBitrateDL	[8] INTEGER OPTIONAL,

extendedMaxRequestedBWUL	[9] INTEGER OPTIONAL,

extendedMaxRequestedBWDL	[10] INTEGER OPTIONAL,

extendedGBRUL				[11] INTEGER OPTIONAL,

extendedGBRDL				[12] INTEGER OPTIONAL,

extendedAPNAMBRUL			[13] INTEGER OPTIONAL,

extendedAPNAMBRDL			[14] INTEGER OPTIONAL

}

EventBasedChargingInformation		::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue		::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FixedSubsID	::= OCTET STRING

--

-- The fixed subscriber Id identifier is defined in Broadband Forum TR 134 [601].

--

FixedUserLocationInformation	::= SEQUENCE

--

-- See format in IEEE Std 802.11-2012 [408] for "SSID" and "BSSID".

--

{

sSID					[0] OCTET STRING OPTIONAL ,

bSSID					[1] OCTET STRING OPTIONAL,

accessLineIdentifier	[2] AccessLineIdentifier OPTIONAL

}

Flows		::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData		::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress		::= IPAddress

MOExceptionDataCounter		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

counterValue		[0] INTEGER,

counterTimestamp	[1] TimeStamp

}

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NBIFOMMode		::= ENUMERATED

{

uEINITIATED				(0),

nETWORKINITIATED		(1)

}

NBIFOMSupport 	::= ENUMERATED

{

nBIFOMNotSupported			(0),

nBIFOMSupported				(1)

}

NetworkInitiatedPDPContext		::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NodeID		::= IA5String (SIZE(1..20))

NumberOfDPEncountered  ::= INTEGER

PDPAddress		::= CHOICE

{

iPAddress				[0] IPAddress

--	eTSIAddress				[1] ETSIAddress

--								has only been used in earlier releases for X.121 format

}

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for encoding details.

--

PDPPDNTypeExtension	::= INTEGER

--

-- This integer is 1:1 copy of the PDP type value as defined in TS 29.061 [215].

--

PLMN-Id		::= OCTET STRING (SIZE (3))

--

--	This is in the same format as octets 2,3,and 4 of the Routing Area Identity (RAI) IE specified

--	in TS 29.060 [215]

--

PresenceReportingAreaElementsList ::= OCTET STRING

--

-- See Presence-Reporting-Area-Elements-List AVP defined in TS 29.212 [220]

--

PresenceReportingAreaInfo	::= SEQUENCE

{

presenceReportingAreaIdentifier	 [0] OCTET STRING,

presenceReportingAreaStatus		 [1] PresenceReportingAreaStatus OPTIONAL,

presenceReportingAreaElementsList[2] PresenceReportingAreaElementsList OPTIONAL,

presenceReportingAreaNode		 [3] PresenceReportingAreaNode OPTIONAL

}

PresenceReportingAreaNode ::= ENUMERATED

{

oCS   		 (0),

pCRF  		 (1),

oCSandPCRF 	 (2)

}

PresenceReportingAreaStatus ::= ENUMERATED

{

insideArea   (0),

outsideArea  (1),

inactive 	 (2)

}

PSFurnishChargingInformation		::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RANSecondaryRATUsageReport	::= SEQUENCE

--

{

dataVolumeUplink				[1] DataVolumeGPRS,

dataVolumeDownlink				[2] DataVolumeGPRS,

rANStartTime					[3] TimeStamp,

rANEndTime						[4] TimeStamp,

secondaryRATType				[5] SecondaryRATType OPTIONAL,

chargingID						[6] ChargingID OPTIONAL

}

RateControlTimeUnit ::= INTEGER

{	unrestricted	(0),

minute			(1),

hour			(2),

day				(3),

week			(4)

}

RatingGroupId		::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0... 4294967295)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RATType		::= INTEGER (0..255)

--

-- Ihis integer is 1:1 copy of the RAT type value as defined in TS 29.061 [215].

--

RelatedChangeOfCharCondition	::= SEQUENCE

{

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

rATType							[15] RATType OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL

}

RelatedChangeOfServiceCondition	::= SEQUENCE

{

userLocationInformation				[20] OCTET STRING OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedServiceConditionChange		[33] ServiceConditionChange OPTIONAL

}

ResultCode		::= INTEGER

--

-- charging protocol return value, range of 4 byte (0... 4294967295)

-- see Result-Code AVP as used in 32.299 [40]

--

SecondaryRATType	::= INTEGER

{

nR				(0)		-- New Radio 5G

}

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1),	-- bearer modification:

-- apply to Gn-SGSN /SGW Change

sGSNPLMNIDChange 						 (2),	-- bearer modification

tariffTimeSwitch 						 (3),	-- tariff time change

pDPContextRelease 						 (4),	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9),	-- IP flow termination.From "Service Stop" in

-- Change-Condition AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11),	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12),	-- DCCA quota reauthorization

dCCATimeExhausted 						(13),	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14),	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15),	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17),	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18),	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19),	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20),	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21),	-- bearer modification. "CGI-SAI Change"

rAIChange								(22),	-- bearer modification. "RAI Change"

dCCAServiceSpecificUnitExhausted		(23),	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From "Service Data

--	Time Limit" Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From "Service Data

--	Volume Limit" Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29),	-- bearer modification. "ECGI Change"

tAIChange								(30),	-- bearer modification. "TAI Change"

userLocationChange						(31),	-- bearer modification. "User Location Change"

userCSGInformationChange				(32),	-- bearer modification. "User CSG info Change"

presenceInPRAChange						(33), 	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

accessChangeOfSDF						(34),	-- "access change of service data flow"

indirectServiceConditionChange			(35),	-- NBIFOM: "indirect service condition change"

servingPLMNRateControlChange			(36), 	-- bearer modification. "Serving PLMNRate

-- Control Change"

aPNRateControlChange					(37)	-- bearer modification. "APN Rate ControlChange

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5),

tWAN		(6)

}

ServingPLMNRateControl		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

sPLMNDLRateControlValue	[0] INTEGER,

sPLMNULRateControlValue	[1] INTEGER

}

SGiPtPTunnellingMethod		::= ENUMERATED

{

uDPIPbased		(0),

others			(1)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange		::= BOOLEAN

--

-- present if first record after inter serving node change (SGW, ePDG, TWAG, HSGW)

--

TimeQuotaMechanism		::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] INTEGER

}

TimeQuotaType		::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

TrafficSteeringPolicyIDDownlink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-DL AVP as defined in TS 29.212[220]

TrafficSteeringPolicyIDUplink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-UL AVP as defined in TS 29.212[220]

TWANUserLocationInfo		::= SEQUENCE

{

sSID					 [0] OCTET STRING,			-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [1] OCTET STRING OPTIONAL, 	-- see format in IEEE Std 802.11-2012 [408]

civicAddressInformation	[2] CivicAddressInformation OPTIONAL,

wLANOperatorId			[3] WLANOperatorId OPTIONAL,

logicalAccessID			[4] OCTET STRING OPTIONAL

}

UNIPDUCPOnlyFlag	::= BOOLEAN

UserCSGInformation		::= SEQUENCE

{

cSGId						[0] CSGId,

cSGAccessMode				[1] CSGAccessMode,

cSGMembershipIndication		[2] NULL OPTIONAL

}

UWANUserLocationInfo		::= SEQUENCE

{

uELocalIPAddress		 [0] IPAddress,

uDPSourcePort			 [1] OCTET STRING (SIZE(2)) OPTIONAL,

sSID					 [2] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [3] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

tCPSourcePort			[4] OCTET STRING (SIZE(2)) OPTIONAL,

civicAddressInformation	[5] CivicAddressInformation OPTIONAL,

wLANOperatorId			[6] WLANOperatorId OPTIONAL,

logicalAccessID			[7] OCTET STRING OPTIONAL

}

WLANOperatorId ::= SEQUENCE

{

wLANOperatorName	[0] OCTET STRING,

wLANPLMNId			[1] PLMN-Id

}

.#END

Next change

#### 5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

CauseForRecClosing,

DataVolumeOctets,

Diagnostics,

DynamicAddressFlag,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageReference,

MSTimeZone,

RecordType,

ServiceSpecificInfo,

SubscriberEquipmentType,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

ChargingCharacteristics,

ChargingID,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

PLMN-Id,

QoSInformation,

RatingGroupId,

RATType,

ServiceIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

InvolvedParty,

Session-Id

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

OriginatorInfo,

RecipientInfo,

SMMessageType,

SMSResult,

SMSStatus

FROM SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

MessageClass,

PriorityType

FROM MMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  mmsChargingDataTypes (5) asn1Module (0) version2 (1)}

;

--

--  CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord

}

ChargingRecord 	::= SET

{

recordType						[0] RecordType,

recordingNetworkFunctionID		[1] NetworkFunctionName,

subscriberIdentifier			[2] SubscriptionID OPTIONAL,

networkFunctionInformation		[3] NetworkFunctionInformation,

triggers						[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage			[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime				[6] TimeStamp,

duration						[7] CallDuration,

recordSequenceNumber			[8] INTEGER OPTIONAL,

causeForRecClosing				[9] CauseForRecClosing,

diagnostics						[10] Diagnostics OPTIONAL,

localRecordSequenceNumber		[11] INTEGER OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation	[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation			[14] RoamingQBCInformation OPTIONAL,

sMSChargingInformation			[15] SMSChargingInformation OPTIONAL

}

--

-- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID			[0] ChargingID,

userIdentifier					[1] InvolvedParty OPTIONAL,

userEquipmentInfo				[2] SubscriberEquipmentType OPTIONAL,

userLocationInformation			[3] OCTET STRING OPTIONAL,

userRoamerInOut					[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo		[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId					[6] Session-Id,

networkSliceInstanceID			[7] NetworkSliceInstanceID OPTIONAL,

pDUType							[8] PDUSessionType OPTIONAL,

sSCMode							[9] SSCMode OPTIONAL,

sUPIPLMNIdentifier				[10] PLMN-Id OPTIONAL,

servingNetworkFunctionID		[11] SEQUENCE OF NetworkFunctionInformation OPTIONAL,

rATType							[12] RATType OPTIONAL,

dataNetworkNameIdentifier		[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress						[14] PDUAddress OPTIONAL,

qoSInformation					[15] QoSInformation OPTIONAL,

uETimeZone 						[16] MSTimeZone OPTIONAL,

pDUSessionstartTime				[17] TimeStamp OPTIONAL,

pDUSessionstopTime				[18] TimeStamp OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

chargingCharacteristics			[20] ChargingCharacteristics,

chChSelectionMode				[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus			[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 	[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL

}

--

-- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1]	NetworkFunctionName OPTIONAL,

roamingChargingProfile			[2] RoamingChargingProfile OPTIONAL,

}

--

-- SMS Charging Information

--

SMSChargingInformation	::= SET

{

sMSNodeAddress				[0] AddressString,

originatorInfo				[1] OriginatorInfo OPTIONAL,

recipientInfos				[2] SEQUENCE OF RecipientInfo OPTIONAL,

userEquipmentInfo			[3] SubscriberEquipmentType OPTIONAL,

userLocationInformation		[4] OCTET STRING OPTIONAL,

uETimeZone 					[5] MSTimeZone OPTIONAL,

rATType						[6] RATType OPTIONAL,

sMSCAddress					[7] AddressString OPTIONAL,

eventtimestamp				[8]	TimeStamp,

-- 9 to 19 is for future use

sMDataCodingScheme			[20] INTEGER OPTIONAL,

sMMessageType				[21] SMMessageType OPTIONAL,

sMReplyPathRequested		[22] SMReplyPathRequested OPTIONAL,

sMUserDataHeader			[23] OCTET STRING OPTIONAL,

sMSStatus					[24] SMSStatus OPTIONAL,

sMDischargeTime				[25] TimeStamp OPTIONAL,

sMTotalNumber 				[26] INTEGER OPTIONAL,

sMServiceType				[27] SMServiceType OPTIONAL,

sMSequenceNumber 			[28] INTEGER OPTIONAL,

sMSResult					[29] SMSResult OPTIONAL,

submissionTime				[30] TimeStamp OPTIONAL,

sMPriority					[31] PriorityType OPTIONAL,

messageReference			[32] MessageReference,

messageSize					[33] INTEGER OPTIONAL,

messageClass				[34] MessageClass OPTIONAL,

sMdeliveryReportRequested	[35] SMdeliveryReportRequested OPTIONAL

}

--

-- PDU Container Information

--

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName				[0] ChargingRuleBaseName OPTIONAL,

aFCorrelationInformation			[1] OCTET STRING OPTIONAL,

timeOfFirstUsage					[2] TimeStamp OPTIONAL,

timeOfLastUsage						[3] TimeStamp OPTIONAL,

qoSInformation						[4] QoSInformation OPTIONAL,

userLocationInformation				[5] OCTET STRING OPTIONAL,

presenceReportingAreaInfo			[6] PresenceReportingAreaInfo OPTIONAL,

rATType								[7] RATType OPTIONAL,

sponsorIdentity						[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[9] OCTET STRING OPTIONAL

}

--

-- QFI Container Information

--

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId							[0] QoSFlowId OPTIONAL,

triggers							[1] SEQUENCE OF Trigger,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

dataTotalVolume						[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[5] DataVolumeOctets OPTIONAL,

localSequenceNumber					[6]	INTEGER OPTIONAL,

timeOfFistUsage						[8] TimeStamp OPTIONAL,

timeOfLastUsage						[9] TimeStamp OPTIONAL,

qoSInformation						[10] QoSInformation OPTIONAL,

userLocationInformation				[11] OCTET STRING OPTIONAL,

uETimeZone	 						[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[13] PresenceReportingAreaInfo OPTIONAL,

rATType								[14] RATType OPTIONAL,

reportTime							[15] TimeStamp,

servingNetworkFunctionID			[16] SEQUENCE OF NetworkFunctionInformation OPTIONAL,

threeGPPPSDataOffStatus				[17] ThreeGPPPSDataOffStatus OPTIONAL

}

--

--  CHF CHARGING TYPES

--

AMFID	::= OCTET STRING (SIZE(6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainer					[1] UsedUnitContainer OPTIONAL,

uPFID								[2]	NetworkFunctionName OPTIONAL

}

NGRANSecondaryRATType	::= OCTET STRING

--

-- "NR" or "EUTRA"

--

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink				[4] DataVolumeOctets

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality				[0]	NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName,

networkFunctionAddress				[2]	IPAddress,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL

}

NetworkFunctionName	::= IA5String (SIZE(1..20))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkFunctionality	::= ENUMERATED

{

cHF			(0),

sMF			(1)

}

NetworkSliceInstanceID	::= OCTET STRING (SIZE(1..4))

-- See S-NSSAI subclause 5.15.2 of 3GPP TS 23.501 [247] for encoding.

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUAddress 	::= SEQUENCE

{

pDUIPv4Address			[0] IPAddress OPTIONAL,

pDUIPv6Prefix			[1] IPAddress OPTIONAL,

iPV4dynamicAddressFlag	[2]	DynamicAddressFlag OPTIONAL,

iPV6dynamicPrefixFlag	[3]	DynamicAddressFlag OPTIONAL

}

PDUSessionType		::= ENUMERATED

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

QoSFlowId		::= INTEGER

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoamingTrigger 		::= SEQUENCE

{

trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeOctets OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

SSCMode	::= INTEGER

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3)

}

-- See 3GPP TS 29.501 [248] for details.

SMdeliveryReportRequested ::= ENUMERATED

{

yes		(0),

no		(1)

}

SMFTrigger				::= INTEGER

{

startOfPDUSession							(1),

atartOfServiceDataFlowNoSession			(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

aMFChange									(102),

presenceReportingAreaChange				(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit			(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unusedQuotaTimerExpiry						(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601)

}

-- See TS 32.255 [15] for details.

SMServiceType 	::= INTEGER

{

-- 0 to 10 VAS4SMS Short Message, see TS TS 22.142 [x] for details

contentProcessing					(0),

forwarding							(1),

forwardingMultipleSubscriptions	(2),

filtering 							(3),

receipt								(4),

networkStorage						(5),

toMultipleDestinations				(6),

virtualPrivateNetwork				(7),

autoreply							(8),

personalSignature					(9),

deferredDelivery					(10)

-- 11 to 99	Reserved for 3GPP defined SM services

-- 100 to 199 Vendor specific SM services

}

SMReplyPathRequested	::= ENUMERATED

{

noReplyPathSet 			(0),

replyPathSet			(1)

}

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger

}

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9]	INTEGER OPTIONAL,

ratingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL

}

.#END

End of changes