3GPP TSG-SA5 Meeting #124 	S5-192279

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-192109

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0051 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add offline only charging for CHF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since offline only charging is separated from converged charging service, the selection for offline charging service is proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution is to add text for the purpose of CHF selection when do offline only charging. This concept is that SMF (CTF) retrieve charging method(s) and charging service in CHF selection part in this document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Offline charging service selection is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.255

A.1	General

A subscriber may have Charging Characteristics assigned to his subscription and/or his subscribed DNNs. Default Charging Characteristics may also be pre-provisioned on the SMF.

During UE PDU session establishment, when the SMF retrieves subscription data from the UDM, if a subscribed Charging Characteristics for the requested DNN is identified, it may override the SMF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as Behaviours (B), freely defined by Operators, as shown in TS 32.298 [51]. Each bit corresponds to a specific charging behaviour which is configured on a per operator basis, and pointed when bit is set to "1" value.

A charging behaviour is defined as an association to a specific usage designed.

One usage may consist of a set of trigger profiles associated to the PDU session in converged charging as described in the Table A.1 example:

Table A.1: Example of Charging Characteristics behaviours for SMF

|  |  |  |  |  | Converged charging |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behaviour index | Default charging method | Charging service |  | Primary and Secondary CHF addresses |  | Active | Time Limit Per PDU session | Vol Limit Per PDU session | Change Cond. Per PDU session | Tariff times | … |
| 0 | Online | Converged charging |  | URI 1 URI 2 |  | Yes | 10 min | 1Mb | 2 | 0-7, 7-12 | ... |
| 1 | Offline | Converged charging |  | URI 1 URI 2 |  | Yes | 15 min | 5Mb | 3 | 0-24 | … |
| 2 | Online | Converged charging |  | URI 1 URI 2 |  | Yes | 30 min | 2Mb | 2 | 0-7, 7-12 | ... |
| 3 | Offline | Offline only charging |  | URI 1 URI 2 |  | No | 15 min | 1MB | 1 | 0-24 | ... |
| … | … |  |  | … |  | … | … | … | … | … | … |

Associated to the behaviour, the following may also be configured:

-	the CHF addresses to be used by the SMF;

-	the "Default charging method" (online or offline) to be used as the default one for every PCC rules of the PDU Session;

-	charging service.

The "Default charging method" and CHF addresses configured in the applicable Charging Characteristics behaviour, are superseded by "Default charging method" and CHF addresses supplied by the PCF if any, during SMF interaction with the PCF at PDU session establishment, as described in TS 23.503 [202]

End