3GPP TSG-SA5 Meeting #125 	S5-193214

# Newport Beach, USA,  8 – 12 April 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0079 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to inter gNB Handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Complement the inter gNB Handover measurements so that it becomes possible to measure the complete intra RAT Handover procedure for NR and create a KPI for Handover Success Rate. Further add Inter to the names of the already defined inter gNB measurements in chapter 5.1.1.6 to clarify that those measurements are just for inter gNB handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements related to inter gNB handover execution. Added “Inter” to existing gNB Handover measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to monitor the full inter gNB handover procedure and there will be no possibility to create an overall KPI to monitor the Handover success rate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.1.1, 5.1.1.6.1.2, 5.1.1.6.1.3, 5.1.1.6.1.4, 5.1.1.6.1.5, 5.1.1.6.1.6, 5.1.1.6.1.X (new), 5.1.1.6.1.Y (new), 5.1.1.6.1.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

###### 5.1.1.6.1.1	Number of requested handover preparations

a)	This measurement provides the number of handover preparations requested by the source gNB.

b)	CC.

c)	On transmission of HANDOVER REQUIRED message (see 3GPP TS 38.413 [12]) by the NR cell CUto the AMF, or transmission of HANDOVER REQUEST message (see 3GPP TS 38.423 [13]) by the source NR cell CUto target NR cell CU, for requesting the preparation of resources at the target NR cell CU.

d)	A single integer value.

e)	MM.HoPrepInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.2	Number of successful handover preparations

a)	This measurement provides the number of successful handover preparations received by the source NR cell CU.

b)	CC

c)	On receipt of HANDOVER COMMAND message by the NR cell CUfrom the AMF (see 3GPP TS 38.413 [12]), or receipt of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.423 [13]) by the source NR cell CUfrom the target NR cell CU, for informing that the resources for the handover have been prepared at the target NR cell CU.

d)	A single integer value.

e)	MM.HoPrepInterSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.3	Number of failed handover preparations

a)	This measurement provides the number of failed handover preparations received by the source NR cell CU. This measurement is split into subcounters per failure cause.

b)	CC

c)	On receipt of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.413 [12]) by the NR cell CUfrom the AMF, or receipt of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.423 [13]) by the source NR cell CUfrom the target NR cell CU, for informing that the preparation of resources at the target NR cell CUhas failed. Each received HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoPrepInterFail.cause.

Where cause identifies the failure cause of the handover preparations.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.4	Number of requested handover resource allocations

a)	This measurement provides the number of handover resource allocation requests received by the target NR cell CU.

b)	CC

c)	On receipt of HANDOVER REQUEST message (see 3GPP TS 38.413 [12]) by the NR cell CUfrom the AMF, or receipt of HANDOVER REQUEST message (see 3GPP TS 38.423 [13]) by the target NR cell CUfrom the source NR cell CU, for requesting the preparation of resources for handover.

d)	A single integer value.

e)	MM.HoResAlloInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.5	Number of successful handover resource allocations

a)	This measurement provides the number of successful handover resource allocations at the target NR cell CUfor the handover.

b)	CC.

c)	On transmission of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.413 [12]) by the NR cell CUto the AMF, or transmission of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.423 [13]) by the target NR cell CUto the source NR cell CU, for informing that the resources for the handover have been prepared.

d)	A single integer value.

e)	MM.HoResAlloInterSucc

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.6	Number of failed handover resource allocations

a)	This measurement provides the number of failed handover resource allocations at the target NR cell CUfor the handover. This measurement is split into subcounters per failure cause.

b)	CC

c)	On transmission of HANDOVER FAILURE message (see 3GPP TS 38.413 [12]) by the NR cell CUto the AMF, or transmission of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.423 [13]) by the target NR cell CUto the source NR cell CU, for informing that the preparation of resources has failed. Each transmitted HANDOVER FAILURE message or HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoResAlloInterFail.cause

Where cause identifies the failure cause of the handover resource allocations.

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)   One usage of this performance measurements is for performance assurance.

Next change

###### 5.1.1.6.1.X	Number of requested handover executions

a)	This measurement provides the number of outgoing handover executions requested by the source gNB.

b)	CC.

c)	On transmission of RRC ConnectionReconfiguration message to the UE triggering the handover from the source NRCellCU to the target NRCellCU, indicating the attempt of an outgoing inter-gNB handover (see TS 38.331 [20]).

d)	A single integer value.

e)	MM.HoExeInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.Y	Number of successful handover executions

a)	This measurement provides the number of successful handover executions received by the source gNB.

b)	CC

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB following a successful handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF following a successful handover.

d)	A single integer value.

e)	MM.HoExeInterSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.Z	Number of failed handover executions

a)	This measurement provides the number of failed handover executions received by the source gNB. This measurement is split into subcounters per failure cause.

b)	CC

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB indicating an unsuccessful handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF indicating an unsuccessful handover.

The failure causes are listed for the UE CONTEXT RELEASE in [13] and for UE CONTEXT RELEASE COMMAND in [11]. Each received message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoExeInterFail.cause.

Where cause identifies the failure cause of the UE CONTEXT RELEASE or UE CONTEXT RELEASE COMMAND message.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

End of changes