3GPP TSG-SA5 Meeting #125 	S5-193286

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0513 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Rate-Control information and triggers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | APN rate control is not introduced for List of Traffic Volume container for IP-CAN session level charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new fields and triggers to List of Traffic Volume container. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for this function is not complete in 3GPP and it is not possible to do proper implementation in case IP-CAN session level charging is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.10.1, 5.2.3.10.2, 5.3.1.6.2, 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.298 CR 0717, TS 32.299 CR 0821 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.1.10.1	IP-CAN bearer charging

For the purpose of interoperator charging, the P-GW collects charging information per user per IP-CAN bearer. 
In case the P-GW is not aware of IP-CAN bearers, i.e. in case of PMIP based connectivity, P-GW collects charging information per IP-CAN session as it would be one IP-CAN bearer. IP-CAN bearer charging allows the P-GW to collect charging information related to data volumes sent to and received by the UE/MS, categorised by the QCI and ARP applied to the IP-CAN bearer. The user can be identified by MSISDN and/or IMSI, while the IP-CAN bearer can be determined by a unique identifier generated by the P-GW when creating an IP-CAN bearer. This identifier is forwarded to the S-GW/ePDG/ TWAG/SGSN so as to allow correlation of S-GW/ePDG/ TWAG/SGSN IP-CAN bearer CDRs with the matching P-GW charging information in the BD.

The amount of data counted in P-GW shall be based on:

-	in case of GTP based tunnelling and interworking with external IP networks; the inner IP packets in the GTP-U packet (T-PDU, see TS 29.281 [217])

-	in case of GTP based tunnelling and interworking with external networks handling other data services (e.g. non-IP, PPP); the T-PDU packets (see TS 29.281 [217])

-	In case of PMIP based protocol; the payload of the GRE tunnel

-	In case of DSMIPv6 based protocol; the payload of the tunnelling layer.

As minimum behaviour, the full payload shall be included. Time metering is started when IP-CAN bearer is activated.

NOTE 1:	The control plan address of the P-GW, together with the unique charging identifier assigned by the P-GW, enables the correlation of charging information. The control plane IP address of SGSN or P-GW (acting as GGSN) is the IP address used at Gn/Gp interface. 
The control plane IP address of S-GW or P-GW is the IP address used at S5/S8 interface. 
The control plane IP address of ePDG or P-GW is the IP address used at S2b interface. 
The control plane IP address of TWAG or P-GW is the IP address used at S2a interface.

When Charging per IP-CAN session is active and measurements for IP-CAN bearers are captured in the same CDR as FBC measurements, the following chargeable events are defined:

-	Start of the default bearer for an IP-CAN session when single access is used or start of the first default bearer for a multi-access PDN connection (i.e., when NBIFOM is accepted by PCRF). 
Upon encountering this event, a new CDR for the IP-CAN session is created and the data volume counts 
(i.e., uplink and downlink) are started and captured for the IP-CAN bearer.

NOTE 2:	Start of the default bearer or start of the first default bearer for an IP-CAN session is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Addition of access to a PDN connection. 
Additional volume counts are started and captured for IP-CAN bearers of the access. New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Start of a dedicated bearer for an IP-CAN session. 
Additional volume counts are started and captured for the dedicated bearer.

-	End of dedicated bearer in the P-GW. 
The counters and time stamps for the IP-CAN bearer are closed and resulting container added to the CDR.

-	Removal of access from a multi-access PDN connection. 
The counters and time stamps for the IP-CAN bearers of the removed access are closed and resulting containers added to CDR.

-	Access of a multi-access PDN connection becomes unusable. 
The counters and time stamps for the IP-CAN bearers of the unusable access are closed and resulting containers added to the CDR.

-	Access of a multi-access PDN connection becomes usable. 
New volume counts are started and captured for all bearers of the access that has become usable. These may not be the same as those that were previously active when the access became unusable due to changes in PCC Rules.

-	End of IP-CAN session (i.e. end of the default bearer for a single access PDN connection or end of the last default bearer for a multi-access PDN connection) in the P-GW. 
The counters and time stamps for all IP-CAN bearers and the resulting containers added to the CDR. The CDR is closed.

NOTE 3:	The End of IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Serving node (e.g. SGSN/S-GW/ePDG/TWAG) change in the P-GW. 
New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR.

-	Expiry of an operator configured time limit for keeping a CDR open. 
This event closes all counters. The resulting containers are added to the CDR and the CDR is closed. 
A new CDR is opened if the IP-CAN session is still active.

NOTE 4:	The expiry of an operator configured time limit for keeping a CDR open event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured time limit per IP-CAN bearer. 
The counters and time stamps for the IP-CAN bearer are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Expiry of an operator configured data volume limit per IP-CAN session. 
This event closes the CDR and a new one is opened if the IP-CAN session is still active.

NOTE 5:	The expiry of an operator configured data volume limit per IP-CAN session event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured data volume limit per IP-CAN bearer. 
The counters and time stamps are closed and added to the CDR. A new IP-CAN bearer traffic volume container is opened if the IP-CAN bearer is still active.

-	Change of charging condition specific to APN-AMBR change. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

-	Change of charging condition specific to IP-CAN bearer modification QoS change. 
When this event is encountered, all counts and time stamps for the modified bearer are captured and new counts and time stamps for the specific bearer are started.

-	Change of charging condition. 
IP-CAN bearer modification except QoS change (e.g. SGSN change, S-GW change, user location change, user CSG information change, change of UE presence in Presence Reporting Area(s) , change of 3GPP PS Data Off status, Serving PLMN Rate Control change, APN Rate Control change), or tariff time change. When this event is encountered, all current configured counts and time stamps are captured and new counts and time stamps for all active bearers are started. 
IP-CAN bearer modification except QoS change events are associated with one access of a multi-access PDN connection and all counts associated with affected access are identified with the specific event. All counts associated with the other access of a multi-access PDN connection are identified as an indirect change of charging condition.

NOTE 7:	The change of charging condition event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Editor’s Note: whether the special case of user location reporting on dedicated bearer release triggers a change of charging condition for the IP-CAN session is ffs.

-	Intersystem change (e.g. change of radio interface from GSM to UMTS, RAT change) for any connected access visible in the P-GW. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 8:	The intersystem change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	PLMN change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 9:	The PLMN change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	MS Timezone change visible in the P-GW. This event closes the CDR. 
A new one is opened if the IP-CAN session is still active.

NOTE 10:	The MS Timezone change event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

-	Expiry of an operator configured limit of number of charging condition changes per IP-CAN session. 
This event closes the CDR, and a new one is opened if the IP-CAN session is still active.

NOTE 11:	The expiry of an operator configured limit of number of charging condition changes event is a shared event for FBC in clause 5.2.1.10.2 for the single shared CDR.

Management intervention may also force trigger a chargeable event.

NOTE 12:	For non-IP type PDN connection, the following do not apply:

-	Addition of access to a PDN connection;

-	Start of a dedicated bearer for an IP-CAN session;

-	End of dedicated bearer in the P-GW;

-	Removal of access from a multi-access PDN connection;

-	Access of a multi-access PDN connection becomes unusable;

-	Access of a multi-access PDN connection becomes usable.

Second change

##### 5.2.3.10.2	Triggers for PGW-CDR charging information addition when IP-CAN session charging is active

The "List of Traffic Data Volumes" attribute of the PGW-CDR consists of a set of containers, which are added when specific trigger conditions are met, and identify the volume counter per QCI/ARP pair, separated for uplink and downlink traffic, on encountering that trigger conditions. Table 5.2.3.10.2.1 identifies which conditions are supported to trigger PGW-CDR charging information addition for the "List of Traffic Data Volumes" attribute.

The "List of Service Data" attribute of the PGW-CDR consists of a set of containers, which are added when specific trigger conditions are met. Each container identifies the configured counts (volume separated for uplink and downlink, elapsed time or number of events) per rating group or combination of the rating group and service id within the same 
IP-CAN bearer, on encountering that trigger condition. For envelope reporting, the containers represent complete and closed time envelopes determined by mechanisms defined in TS 32.299 [50]. Table 5.2.3.10.2.2 identifies conditions that may be supported as recording triggers under consideration of additional Debit / Reserve Units triggers.

Some of the triggers are non-exclusive (e.g. IP-CAN bearer modification with a couple of reasons, IP-CAN bearer modification reasons that cause PGW-CDR closure).

Table 5.2.3.10.2.1: Triggers for PGW-CDR charging information addition for List of Traffic Volumes when charging per IP-CAN session is active

| Trigger Conditions | Description/Behaviour |
| --- | --- |
| End of dedicated bearer in P-GW | The end of a dedicated bearer in P-GW shall result that open "List of Traffic Data Volumes" containers for the dedicated bearer are closed and added to the CDR. |
| Serving node change | A serving node (e.g., SGSN/S-GW/ePDG/TWAG) change in the P-GW shall result that all open "List of Traffic Data Volumes" containers are closed and added to the CDR. New containers are opened for each bearer. |
| QoS Change | A change in the QoS shall result that open "List of Traffic Data Volumes" containers for the effected bearer being closed and added to the CDR and new IP-CAN bearer specific container is opened if the IP-CAN bearer is still active. |
| Tariff Time Change | On reaching the Tariff Time Change open "List of Traffic Data Volumes" containers shall be closed and added to the CDR. |
| User Location Change | A change in the User Location Info (e.g. ECGI, TAI, RAI, SAI or CGI) shall result that open "List of Traffic Data Volumes" containers being closed and added to the CDR, if location reporting is required, and a report of User Location Change is received. |
| User CSG Information change | A change in the User CSG info (e.g. CSG ID, access mode or CSG membership indication) shall result that open "List of Traffic Data Volumes" containers being closed and added to the CDR, if CSG information reporting is required, and a report of User CSG information change is received. |
| Change of UE presence in Presence Reporting Area | A change of UE presence in Presence Reporting Area(s) shall result that open "List of Traffic Data Volumes" containers being closed and added to the CDR, if such reporting is required, and a report that user enters/leaves the area or the Presence Reporting Area(s) is set to inactive by the serving node are received. |
| Addition of access to a PDN connection | New SGSN/S-GW/ePDG/TWAG address is added to data for the IP-CAN bearer in the CDR. |
| Removal of access from a multi-access PDN connection | The counters and time stamps for the IP-CAN bearers of the removed access are closed and resulting containers added to CDR |
| Access of a multi-access PDN connection becomes unusable | The counters and time stamps for the IP-CAN bearers of the unusable access are closed and resulting containers added to the CDR. |
| Traffic volume report | A "List of Traffic Data Volumes" container for an IP-CAN bearer shall be added when: - expiry of time limit per IP-CAN bearer; - expiry of data volume limit per IP-CAN bearer. |
| Rate Control Chane | A "List of Traffic Data Volumes" container for an IP-CAN bearer shall be added when: - Serving PLMN Rate Control Change - APN Rate Control Change. |
| CDR Closure | Open "List of Traffic Data Volumes" containers shall be closed and added to the PGW-CDR. |

Editor’s Note: The individual events associated with change of charging condition must be re-evaluated in the context of NBIFOM. Changes to this event are FFS.

Table 5.2.3.10.2.2: Triggers for PGW-CDR charging information addition for "List of Service Data" when charging per IP-CAN session is active

| Trigger Conditions | Description/Behaviour |
| --- | --- |
| IP-CAN bearer modification except QoS change | A change of IP-CAN bearer conditions (e.g. SGSN/S-GW/ePDG/TWAG change, user location change, user CSG information change, change in UE presence in Presence Reporting Area, Serving PLMN Rate Control Change, APN Rate Control Change) shall result in a set of "List of Service Data" containers, i.e. all active service data flow containers, being added to the CDR as described in clause 5.2.1.10.2. In a tight interworking between online and offline charging the specified quota re-authorisation triggers armed by OCS are supported. |
| Tariff Time Change | On reaching the Tariff Time Change a set of "List of Service Data" containers, i.e. all active service data flow containers, shall be added to the CDR. In a tight interworking between online and offline charging the Debit / Reserve Units tariff time change from OCS is supported. |
| ASR or Failure Handling procedure triggering | When the Debit / Reserve Unitssession is terminated with ASR or Failure Handling mechanism is triggered a "List of Service Data'”, i.e. all active service data flow containers shall be added to the CDR. The causes are only relevant in case of simultaneously usage of an active Debit / Reserve Units session. |
| Service data flow report | In case of independent online and offline charging a "List of Service Data" container for the service data flow shall be added when: - expiry of time limit per rating group; - expiry of volume limit per rating group; - expiry of unit or data event limit per rating group; - access change of service data flow and all service data flows for the rating group, combination of rating group and service identifier, or combination of rating group, sponsor identity and application service provider identity are moved from one access to another; - termination of service data flow and this is the last service data flow for the rating group or combination of rating group and service identifier, or combination of rating group, sponsor identity and application service provider identity. |
|  | In case of tight interworking online and offline charging a "List of Service Data" container for the service data flow shall be added when: - time threshold reached; - volume threshold reached; - unit threshold reached; - time quota exhausted; - volume quota exhausted; - unit quota exhausted; - expiry of quota validity time; - access change of service data flow; - termination of service data flow: - re-authorization request by OCS. |
| CDR Closure | All active "List of Service Data" containers shall be added to the PGW-CDR. Note: The trigger condition is a common value that has to be used for CDR closure together with detailed reason. |

Editor’s Note: The individual events associated with change of charging condition must be re-evaluated in the context of NBIFOM. Changes to this event are FFS.

The first traffic container identifies the data traffic since the IP-CAN session was opened. 
Subsequent data traffic containers store the configured counts accrued since the closure of the last container.

For envelope reporting, each envelope contains information about the data volume transferred in both uplink and downlink and / or the number of events that occurred for the duration that envelope is open. Only completed time envelopes shall be added to the PGW-CDR. The determination of completed envelopes are defined in TS 32.299 [50]. The triggers listed in the previous table shall not apply to envelope reporting. Envelopes that are not complete when a partial PGW-CDR is closed shall be added to the next PGW-CDR.

The "Serving node Address" attribute of the PGW-CDR consists of a list of SGSN/S-GW/ePDG/TWAG addresses. 
New SGSN/S-GW/ePDG/TWAG address is added to the list when SGSN/S-GW/ePDG/TWAG changes.

When Charging Event (ACR) is triggered by table 5.2.3.10.2.1 or 5.2.3.10.2.2 conditions, the Change-Condition sub-field associated to the added container, indicating the appropriate condition, shall be present, excluding CDR Closure case.

When Charging Event (ACR) is triggered by CDR Closure condition, this Change-Condition sub-field associated to the added container shall be omitted, except when CDR closure is due to "maximum number of charging condition changes", where it shall be present with the original condition change.

When Charging Event (ACR) is triggered by "User CSG Information change" as a Change condition, the following shall apply for the added volume container:

# -	When User enters in a CSG cell or a hybrid cell: the CSG ID, access mode and CSG membership indication (when hybrid), shall be provided together with this "User CSG Information change" Change-Condition.

# -	User leaves a CSG cell or a hybrid cell: this "User CSG Information change" Change-Condition shall be provided without any CSG ID, access mode and CSG membership indication, unless the user is entering a new CSG cell or hybrid cell.

Third change

##### 5.3.1.6.2	Flow Based Charging (FBC)

FBC is supported by the P-GW by the integration of a PCEF. With PCEF, charging is enhanced by the capability to categorise the service data flows within IP-CAN session data traffic by rating group or combination of the rating group and service id. For online charging, FBC provides credit management for each rating group and may provide reporting (i.e., counts) per each rating group or combination of the rating group and service id. The level of the reporting is defined per PCC rule. Details of this functionality are specified in TS 23.203 [215] and TS 32.240 [1].

NOTE 1:	Even though an individual service data flow template is bound to a specific IP-CAN bearer, the assigned rating group or combination of rating group and service id applies to the entire IP-CAN session. 
As a result, data traffic from multiple bearers can be included in the count maintained for the rating group or combination of the rating group and service id. This implies if an operator wishes to be able to separate usage according to IP-CAN bearer within their charging system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

NOTE 2:	The P-GW can only include one QoS Information occurrence per Multiple Unit Operation. 
This implies if an operator wishes to be able to separate usage according to QCI and ARP within their charging system they will need to ensure that services having different QCI and ARP do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

NOTE 2a: The P-GW can only include one RAT type per service data container.
This implies if an operator wishes to be able to separate usage according to RAT type within their billing system they will need to ensure that services having different RAT type do not have the same:

-	rating group in cases where rating group reporting is used;

-	rating group/service id where rating group/service id reporting is used.

Extended packet inspection can be done in the PCEF with pre-defined PCC rules. The PCEF also have the possibility to output service specific information related to the packet inspection in the online charging information.

The capability of P-GW to support ABC is achieved with PCRF providing appropriate PCC Rules to the P-GW. Such PCC Rule shall be defined with service data flow template including an Application Identifier for the application which needs to be detected, enforced and charged.

According to TS 23.203 [215], FBC shall support different charging models per PCC rule. These charging models may be based on volume and/or time and on number of events matching a specific service data flow template in PCC rule. 
In online charging the PCEF shall request the reservation of units  prior to service delivery.

The following chargeable events are defined for FBC when online charging and Charging per IP-CAN session is activated:

-	Start of IP-CAN session. Upon encountering this event, a Debit / Reserve Units Request[Initial], indicating the start of the IP-CAN session, is sent towards the OCS to authorize the IP-CAN session. PCEF may request quota later when service usage is started.

-	Start of service data flow. If no valid quota has been granted for the rating group, a Debit / Reserve UnitsRequest[Update]  is generated to request quota. In case charging session does not exist, the request is a Debit / Reserve Units Request[Initial] sent towards the OCS. The type of requested quota shall depend on measurement method configured for the PCC rule in case of decentralized unit determination. When event based charging applies, the first occurrence of an event matching a service data flow template in PCC rule shall be considered as the start of a service.

-	Access change of service data flow. 
If service identifier level reporting is required by the PCC rule and all service data flows for this combination of the rating group and service id are changing from one access to another, or if rating group level reporting is required by the PCC rule and all service data flows for this rating group are changing from one access to another, corresponding counts are closed and Debit / Reserve Units Request[Update] is triggered.

-	Termination of service data flow. If reporting is per rating group and this is the last service data flow utilizing that specific rating group or if reporting is per combination of the rating group and service id and this is the last service data flow utilizing that specific rating group and service id, the required counters are updated. Termination of the service data flow itself does not trigger Debit / Reserve Units Request[Update].

- 	Expiry of Unused Quota timer for the charging session in PCEF, a Debit / Reserve Units Request[Terminate] is sent to OCS indicating that charging session is terminated, and the IP-CAN bearer is still active. In this case the OCS shall allow for a new session to be started with the same Charging Id.

-	End of IP-CAN session. Upon encountering this event, a Debit / Reserve Units Request[Terminate], indicating the end of the IP-CAN session, is sent towards the OCS together with the final counts.

-	Ro specific chargeable events (e.g. threshold reached, QHT expires, quota exhaustion, validity time reached, forced re-authorization). Corresponding counts for the rating group(s) are closed and Debit / Reserve Units Request[Update] is triggered according the rules defined in TS 32.299 [50].

-	Change of charging condition specific to APN-AMBR change: When this event is encountered and the corresponding re-authorization trigger is armed, all current counts are captured and sent towards the OCS with a  Debit / Reserve Units Request[Update].

-	Change of charging condition other than QoS change: E.g. user location change, user CSG information change, change of UE presence in Presence Reporting Area(s) , change of 3GPP PS DataOff status, Serving PLMN Rate Control change, APN Rate Control Change. When this event is encountered and the corresponding re-authorization trigger is armed, all current counts are captured and sent towards the OCS with a Debit / Reserve Units Request[Update]. 
Change of charging condition other than QoS change events are associated with one access of a multi-access PDN connection and all counts associated with affected access are identified with the specific event. All counts associated with the other access of a multi-access PDN connection are identified as an indirect change of charging condition.

-	Tariff time change. When this event is encountered, all current counts are captured and new counts are started. 
The counts are sent to the OCS in next Debit / Reserve Units Request.

Management intervention may also force trigger a chargeable event.

If the PCEF supports the Unused Quota timer it may send its locally configured value to the OCS. The OCS may in this case respond with a new value of the Unused Quota timer to the PCEF, to be used instead of the PCEF configured value.

PCC rules can be activated, deactivated and modified any time during the IP-CAN session lifetime. PCC rule activation, deactivation and modification are not chargeable events. However these PCC rule changes may lead to "start of service data flow'" and "termination of service data flow'" chargeable events.

According to TS 23.203 [215], the PCRF can modify the following charging information in a dynamic PCC rule which is active in the PCEF: Charging key, Service identifier, Measurement method, Service identifier level reporting. The PCRF can also modify the allowed access type in the case of NBIFOM.
A change of allowed access type in the case of NBIFOM will trigger an "access change for service data flow" chargeable event when all service data flows for the associated counter are changed from one access to another and takes priority over change of charging information.
Change of Charging key, ServiceIdentifier, IP-CAN type or measurement method will trigger a "start of service data flow'" chargeable event when valid quota does not exist corresponding to that changed PCC rule. Change of Charging key, Service Identifier, IP-CAN type or measurement method, or Service identifier level reporting will trigger a "termination of service data flow" chargeable event when this is the last service data flow utilizing the quota used for the original PCC rule.

Removal of an access from a PDN connection is not a trigger for FBC. Subsequent PCC rule handling, may lead to the "access change for a service data flow" or "termination of service data flow" events for the service data flows that were active on the removed access.

For non-IP type PDN connection:

-	non-3GPP access types are not supported;

-	multi-access PDN connections do not apply;

-	A single PCC Rule is used with the service data flow descriptor matching the whole non-IP data traffic, and can contain e.g.:

-	service data flow template matching all the packets;

-	charging method to identify whether online/offline/both/neither charging interface is used;

-	measurement method for offline charging to identify whether time/data volume/events are measured for this service data flow;

-	Charging key (i.e. rating group) for that service data flow;

-	service identifier for that service data flow;

-	reporting level for that service data flow:

-	rating group level in case a specific rating group is used for non-IP traffic charging;

-	combination of the rating group and service id in case a specific rating group/service Id is used for non-IP traffic charging.

Fourth change

### 6.1.3	FBC IP-CAN bearer charging data in P-GW (PGW-CDR)

If FBC is enabled and the collection of CDR data is enabled then the P-GW data specified in table 6.1.3.1 shall be available for each IP-CAN bearer when charging per IP-CAN session is not active, or for each IP-CAN session when charging per IP-CAN session is active.

Table 6.1.3.1: P-GW IP-CAN bearer data (PGW-CDR)

| Field | Category | Description |  |
| --- | --- | --- | --- |
| Record Type | M | P-GW IP-CAN bearer record. |  |
| Retransmission | OC | This parameter, when present, indicates that information from retransmitted Charging Data Request has been used in this CDR. |  |
| Served IMSI | C | IMSI of the served party, if available. |  |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEI | OC | The IMEI or IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |  |
| Served 3GPP2 MEID | OC | MEID of the served party's terminal equipment for 3GPP2 access. |  |
| Served MN NAI | OC | Mobile Node Identifier in NAI format (based on IMSI), if available. |  |
| P-GW Address used | M | The control plane IP address of the P-GW used. |  |
| P-GW Address IPv6 | OC | The control plane IPv6 address, in case of IPv4v6 dual stack, of the P-GW used. |  |
| Charging ID | M | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. This field holds the unique Charging Id in the PMIP case. When charging per IP-CAN session is active, this field holds the Charging Id of the EPS default bearer. It is the same as the PDN Connection Charging Id. |  |
| Charging per IP-CAN Session Indicator | Oc | This field indicates whether charging per IP-CAN session is active. If this field is not present, charging per IP-CAN session is not active. |  |
| PDN Connection Charging Id | OM | This field holds the explicit Charging Id for the PDN connection. When NBIFOM is supported, the PGW will assign an explicit PDN Connection Charging ID for the PDN connection. Otherwise the field holds the Charging Id of the EPS default bearer. This field is used to identify different records belonging to same PDN connection. |  |
| Serving node Address | M | List of SGSN/S-GW/TWAG control plane IP addresses, or the ePDG address used during this record. |  |
| Serving node IPv6 Address | OC | List of SGSN/S-GW/TWAG control plane IPv6 addresses, in case of IPv4v6 dual stack, used during this record. |  |
| Serving node Type | M | List of serving node types in control plane (SGSN, SGW, ePDG, AGW, TWAG). The serving node types listed here map to the serving node addresses listed in the field "Serving node Address" in sequence. |  |
| PGW PLMN Identifier | Oc | PLMN identifier (MCC MNC) of the P-GW. |  |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |  |
| PDP/PDN Type | OM | PDP/PDN type, i.e. IPv4, IPv6, IPv4v6 , or PDP type PPP. |  |
| PDP/PDN Type extension | OM | This field indicates Non-IP PDN type. |  |
| SGi PtP Tunnelling Method | OC | This field indicates whether SGi PtP tunnelling method based on UDP/IP or other methods are used for this PDN connection when non-IP PDN connection. |  |
| SCS/AS Address | OC | This field holds the Address of SCS/AS for SGi PtP tunnelling. |  |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, i.e. IPv4 address when PDP/PDN Type is IPv4 or IPv6 prefix when PDP/PDN Type is IPv6 or IPv4v6, or assigned IP address if any when PDP/PDN Type extension is Non-IP. This parameter shall be present except: - when both the PDP type is PPP and dynamic IP-CAN bearer address assignment is used or - when PDP/PDN Type extension is Non-IP, and no IP address has been assigned by the PGW for the PDN connection. |  |
| Served PDP/PDN Address prefix length | OC | PDP/PDN Address prefix length of an IPv6 typed Served PDP Address. The field needs not available for prefix length of 64 bits. |  |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDP/PDN type is IPv4v6. |  |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if IPv4 address is static when PDN Type is IPv4, or if IPv6 address is static when PDN Type is IPv6 or IPv4v6. |  |
| Dynamic Address Flag extension | OC | Indicates whether served IPv4 PDP/PDN address is dynamic, which is allocated during IP-CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity with PDP/PDN type IPv4v6. This field is missing if IPv4 address is static. |  |
| List of Traffic Data Volumes | OC | A list of changes in charging conditions for all bearers within the IP-CAN session. Each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Applicable only for IP-CAN bearer charging when IP-CAN session charging is active. See table 6.1.3.1 for a description of List of Traffic Data Volumes handling. |  |
| List of Service Data | OM | A list of changes in charging conditions for all service data flows within this IP-CAN bearer categorized per rating group or per combination of the rating group and service id or per combination of rating group, sponsor identity and application service provider identity. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. Online charging information (PS Furnish Charging Information) may be added per each service data flow container in case it is sent by the OCS. External charging identifiers may be added per each service data flow container in case sent by the PCRF. Failure-Handling: This field shall be present in case P-GW triggers the Failure-Handling procedure. It shall indicate the Failure Handling scenario and the instant the Failure Action is triggered (see annex B). Scenarios: Continue/New Session; Continue/Ongoing Session; Retry&Terminate/Ongoing Session; Terminate/Ongoing Session. When charging per IP-CAN session is active, see table 6.1.3.2 for a description of List of Service Data handling. |  |
| Record Opening Time | M | Time stamp when IP-CAN bearer is activated in this P-GW or record opening time on subsequent partial records. |  |
| MS Time Zone | OC | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |  |
| Last MS Time Zone | OC | This field contains the UE Time Zone the UE is located at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| Duration | M | Duration of this record in the P-GW. |  |
| Cause for Record Closing | M | The reason for the release of record from this P-GW. |  |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a single cause is applicable. . |  |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. When charging per IP-CAN session is active, this field holds a more detailed reason for the release of the IP-CAN session, when a set of causes is applicable. |  |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |  |
| Node ID | OM | Name of the recording entity. |  |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |  |
| APN Selection Mode | OM | An index indicating how the APN was selected. |  |
| Served MSISDN | OC | The primary MSISDN of the subscriber, if available. |  |
| User Location Information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |  |
| Last User Location Information | OC | This field contains the User Location Information of the UE at IP-CAN bearer deactivation, or at default bearer deactivation when IP-CAN session charging is active, when available. |  |
| User Location Information Time | OC | This field contains the time at which the user location information was acquired, if available. |  |
| User CSG information | OC | This field contains the User CSG Information of the UE, if available, including CSG ID, access mode and CSG membership indication. |  |
| 3GPP2 User Location information | OC | This field contains the User Location Information of the MS when the CDR is opened as defined in TS 29.212 [71] for 3GPP2 access, if available. |  |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN) , if available: - "TWAN identifier" as defined in TS 29.274 [210] for S2a GTP-based case, or, -"Access Network Identifier Option IE" as defined in 29.275 [211] for S2a PMIP-based case. |  |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information (SSID and, when available, BSSID of the access point) the ePDG may have received from the 3GPP AAA server about the UE as defined in TS 29.274 [210]. |  |
| Presence Reporting Area Information | OC | This grouped field contains the Presence Reporting Area Identifier for the UE PDN connection, additional the list(s) of elements comprising the Presence Reporting Area for a UE-dedicated Presence Reporting Area, and the initial status of the UE presence in the Presence Reporting Area, if available. This field is not applicable to multiple PRA(s). |  |
| Charging Characteristics | M | The Charging Characteristics applied to the IP-CAN bearer. |  |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |  |
| IMS Signalling Context | OC | Included if the IP-CAN default bearer IM-CN Subsystem Signalling Flag is set, see TS 23.060 [201] is used for IMS signalling. |  |
| Serving node PLMN Identifier | OM | This fields holds the PLMN Identifier (MCC and MNC) serving the UE during this record, as received in the "Serving Network” IE over S5/S8 or retrieved from the RAI received over Gn/Gp. When the Serving node is TWAG, this field includes the PLMN identifier used for UE authentication, i.e. the VPLMN in roaming case, and the HPLMN in non-roaming case. |  |
| CN Operator Selection Entity | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |  |
| PS Furnish Charging Information | OC | Online charging session specific information |  |
| CAMEL Information | OC | Set of CAMEL information related to IP-CAN bearer, if available. This field applies only for GPRS. When charging per IP-CAN session is active, this field is not applicable. |  |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. When NBIFOM is accepted by PCRF, this field holds the first default bearer’s RAT type when charging per IP-CAN session is active. |  |
| Start Time | OC | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session starts. |  |
| Stop Time | OC | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. When charging per IP-CAN session is active, this field holds the time when User IP-CAN session is terminated. |  |
| Low Priority Indicator | OC | This field indicates if this IP-CAN session has a low priority, i.e. for Machine Type Communication. |  |
| QoS Information | OC | This field indicates the APN-AMBR uplink and downlink information for the IP-CAN session. Applicable only when charging per IP-CAN session is active. |  |
| NBIFOM Support | OC | This field indicates that NBIFOM was requested by the UE, supported and accepted by the network for the IP-CAN session or if NBIFOM is not supported for the IP-CAN session. |  |
| NBIFOM Mode | OC | This field indicates the NBIFOM mode selected for the PDN connection, when NBIFOM is supported for the IP-CAN session. This field is only present when NBIFOM is accepted. |  |
| UNI PDU CP Only Flag | OC | This field indicates whether this PDN connection is applied with "Control Plane Only Flag" for UNI PDU transfer, i.e. transfer only using Control Plane NAS PDU (Control Plane CIoT EPS optimisation). |  |
| Serving PLMN Rate Control | Oc | This field holds the Serving PLMN Rate Control used by the MME during this record |  |
| APN Rate Control | Oc | This field holds the APN Rate Controls enforced in the PGW during this record. |  |
| MO exception data counter | Oc | This field holds the MO exception data counter value with the timestamp indicating the time at which the counter value increased from 0 to 1, as defined in TS 29.274 [210]. |  |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |  |
| 3GPP PS Data Off Status | Oc | This field holds the 3GPP PS Data Off Status. |  |
| List of RAN Secondary RAT Usage Reports | OC | A list of containers with volumes reported from RAN for secondary RAT usage, each container is time stamped. |  |

Table 6.1.3.2 contains special handling for the fields in the List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active and IP-CAN bearer charging is performed.

Table 6.1.3.2: List of Traffic Data Volumes in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| Data Volume Uplink | OC | This field includes the number of octets transmitted during the use of the packet data services in the uplink direction. |
| Data Volume Downlink | OC | This field includes the number of octets transmitted during the use of the packet data services in the downlink direction. |
| Change Condition | M | This field defines the reason for closing the container as specified in clause 5.2.1.10.1. |
| Change Time | M | This field is a time stamp, which defines the moment when the volume container is closed. |
| User Location Information | OC | This field contains the location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE is located and used during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "user location change". |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP or TCP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information the ePDG may have received from the 3GPP AAA server about the UE as "WLAN location information" defined in TS 29.274 [210]. |
| TWAN User Location Information | OC | This field contains the UE location in a Trusted WLAN Access Network (TWAN), if available: - as "TWAN identifier" defined in TS 29.274 [210] for S2a GTP-based case, or, - as "Access Network Identifier Option IE" defined in 29.275 [211] for S2a PMIP-based case. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area. This field is not applicable to multiple PRA(s). |
| List of Presence Reporting Area Information | OC | This field contains a list of Presence Reporting Area Identifier(s) for the UE PDN connection, and the Presence Reporting Area(s) status (UE presence or not, inactive), differentiated between PCRF and OCS subscription. |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer. First container for each QCI/ARP pair includes this field. In following containers this field is present if previous change condition is "QoS change". This field includes: QCI and ARP. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Charging Id | OC | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PCNs. Charging Id is generated by P-GW at IP-CAN bearer activation and is included in all containers in order to identify the containers which pertain to the IP-CAN bearer. |
| RAT Type | OC | This field contains the RAT type for the current IP-CAN bearer. |
| Access Availability Change Reason | OC | This field indicates the reason why the availability of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable. This field is only present when one access of a multi-access PDN connection becomes unusable. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| Diagnostics | OM | A more detailed reason for the release of the IP-CAN bearer, when a single cause is applicable. |
| Enhanced Diagnostics | OC | This field holds a more detailed reason for the release of the IP-CAN bearer, when a set of causes is applicable. |
| 3GPP PS Data Off Status | OC | This field contains the status of the UE’s 3GPP PS Data off Status. |
| Serving PLMN Rate Control | OC | This field holds the Serving PLMN Rate Control used by the MME |
| APN Rate Control | OC | This field holds the APN Rate Controls enforced in the PGW |

Table 6.1.3.3 contains special handling for fields in the List of Service Data when charging per IP-CAN session is active. The complete set of fields in the List of Service Data is defined in TS 32.298 [51].

Table 6.1.3.3: List of Service Data in PGW-CDR when charging per IP-CAN session is active

| Field | Category | Description |
| --- | --- | --- |
| AF Record Information | OC | As specified in TS 32.298 [51]. |
| Charging Rule Base Name | OC | As specified in TS 32.298 [51]. |
| ADC Rule Base Name | OC | Not applicable to PGW-CDR. |
| Data Volume Downlink | OC | As specified in TS 32.298 [51]. |
| Data Volume Uplink | OC | As specified in TS 32.298 [51]. |
| Event Based Charging Information | OC | As specified in TS 32.298 [51]. |
| Local Sequence Number | OC | As specified in TS 32.298 [51]. |
| PS Furnish Charging Information | OC | As specified in TS 32.298 [51]. |
| EPC QoS Information | OC | This field contains the authorized QoS for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. This field includes: QCI and ARP. For non-GBR QCI values, this field also includes the MBR for the downlink and uplink direction. For GBR QCI values, this field also includes GBR for the downlink and uplink direction. If the MBR is different than the GBR, then this field also includes the MBR for the downlink and uplink direction. This field does not include the APN-AMBR, which is contained in a separate field in the CDR for the IP-CAN session. |
| Rating Group | M | As specified in TS 32.298 [51]. |
| Report Time | M | As specified in TS 32.298 [51]. |
| Result Code | OC | As specified in TS 32.298 [51]. |
| Service Condition Change | M | As specified in TS 32.298 [51]. Only those values specified for the events in clause 5.3.2.X apply. |
| Service Identifier | OC | As specified in TS 32.298 [51]. |
| Service Specific Info | OC | As specified in TS 32.298 [51]. |
| Serving Node Address | OC | As specified in TS 32.298 [51]. |
| Time of First Usage | OC | As specified in TS 32.298 [51]. |
| Time of Last Usage | OC | As specified in TS 32.298 [51]. |
| Tme Usage | OC | As specified in TS 32.298 [51]. |
| User Location Information | OC | As specified in TS 32.298 [51]. |
| 3GPP2 User Location Information | OC | As specified in TS 32.298 [51]. |
| UWAN User Location Information | OC | This field contains the UE location in an Untrusted Wireless Access Network (UWAN) which includes the UE local IP address and optionally UDP or TCP source port number (if NAT is detected) as defined in TS 29.274 [210]. It may also include WLAN location information the ePDG may have received from the 3GPP AAA server about the UE as "WLAN location information" defined in TS 29.274 [210]. |
| Presence Reporting Area Status | OC | This field contains the status of the UE presence in Presence Reporting Area as defined in TS 29.274[210]. This field is not applicable to multiple PRA(s). |
| List of Presence Reporting Area Information | OC | This field contains a list of Presence Reporting Area Identifier(s) for the UE PDN connection, and the Presence Reporting Area(s) status (UE presence or not, inactive), differentiated between PCRF and OCS subscription. |
| RAT Type | OC | This field contains the RAT type for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included. |
| Sponsor Identity | OC | As specified in TS 32.298 [51]. |
| Application Service Provider Identity | OC | As specified in TS 32.298 [51]. |
| Time Quota Mechanism | OC | As specified in TS 32.298 [51]. |
| Related Change Condition Information | OC | This field indicates the change condition associated with an alternate access of a multi-access PDN connection that indirectly caused closure of the container. This field includes optional supplemental information associated with the change event (e.g., user location information). |
| 3GPP PS Data Off Status | OC | This field contains the status of the UE’s 3GPP PS Data Off Status. |
| Traffic Steering Policy Identifier Uplink | OC | This field contains the traffic steering policy identifier in the uplink direction. |
| Traffic Steering Policy Identifier Downlink | OC | This field contains the traffic steering policy identifier in the downlink direction. |
| Serving PLMN Rate Control | OC | This field holds the Serving PLMN Rate Control used by the MME |
| APN Rate Control | OC | This field holds the APN Rate Controls enforced in the PGW |

End of change