3GPP TSG-SA5 Meeting #125 	S5-193289

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0047 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of termination handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correcting the missing termination handling on Converged Charging Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding how termination should be handled in on the Converged Charging Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The termination handling on for the Converged Charging Service would be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.5.x	Termination handling

The termination action only applies when quota management is active. It indicates the action, which the NF Consumer (CTF) should perform when no quota is granted. If it applies to a specific session, event or rating group level depends on the error or results type, defined in clause 5.5.x. The termination actions are defined in table 5.5.x-1

Table 5.5.x-1: Termination Actions

| Termination action | Description |
| --- | --- |
| Session or event level actions |  |
| Continue | The session or event shall be allowed to continue without quota management. |
| Terminate | The session or event shall be terminated |
| Redirect | The session or event shall be redirected to a defined destination |
| Service level errors or results |  |
| Continue | The service shall be allowed to continue without quota management. |
| Terminate | The service or event shall be terminated |
| Redirect | The service or event shall be redirected to a defined destination |
| NOTE: Redirect or continue on session or event level shall, if charging without quota management is needed, trigger a new charging session. |  |

The NF Consumer (CTF) shall have a Default Termination Action for all types of errors and results and may have for rating groups, which shall apply if none is provided form the CHF. The CHF may provide a termination action for each rating group and shall apply for the duration of the charging session. Any previously provided termination action may be overwritten by the CHF.

The termination action shall continue to be enforced until a notification with reauthorization is received from the CHF, or the charging session is terminated.

End of change