3GPP TSG-SA5 Meeting #125 	S5-193291

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0048 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of QBC roaming definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | QBC can be used both in VPLMN and in HPLMN as well as alone or together with FBC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding that the QBC and FBC can be used at the same time as well as that QBC can be used in HPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the spec may lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the SMF as used for 5G data connectivity converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case SUPI is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Notify URI | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Container Information | OC | This field holds the 5G data connectivity PDU session container specific information described in clause 6.2. |
| UPF ID | OC | This field holds the UPF identifier used to identify the UPF. These fields shall only be included when either quota is requested per UPF, or used units are reported per UPF |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4 |

Second change

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G data connectivity converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Result Code | OC | Described in TS 32.290 [57] |
| Failed Parameter | OC | Described in TS 32.290 [57] |
| Failure Handling | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | Described in TS 32.290 [57] |
| Multiple Unit Information | OC | Described in TS 32.290 [57] This field is not applicable to QBC. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | M | Described in TS 32.290 [57] |
| UPF ID | OC | This field holds the UPF identifier used for quota granted per UPF by CHF |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| Final Unit Indication | OC | Described in TS 32.290 [57] |
| Time Quota Threshold | OC | Described in TS 32.290 [57] |
| Volume Quota Threshold | OC | Described in TS 32.290 [57] |
| Unit Quota Threshold | OC | Described in TS 32.290 [57] |
| Quota Holding Time | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in clause 6.2. |
| Roaming QBC Information | OM | This field holds the roaming QBC specific information defined in clause 6.2.1.4. |

End of change