3GPP TSG-SA5 Meeting #125 	S5-193085

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0060 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline only charging service API resource definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The offline only charging service API is not specified in TS 32.291. Resource name and resource URIs definition for offline only charging service API are not addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resource names and resource URIs definition of offline only charging service API are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource names and resource URIs definition of offline only charging service API is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.3 (new), 6.x.3.1 (new), 6.x.3.2 (new), 6.x.3.2.1 (new), 6.x.3.2.2 (new), 6.x.3.2.3 (new), 6.x.3.2.3.1 (new), 6.x.3.2.4 (new), 6.x.3.3 (new), 6.x.3.3.1 (new), 6.x.3.3.2 (new), 6.x.3.3.3 (new), 6.x.3.3.4 (new), 6.x.3.3.4.1 (new), 6.x.3.3.4.2 (new), 6.x.3.3.4.2.1 (new), 6.x.3.3.4.2.2 (new), 6.x.3.3.4.3 (new), 6.x.3.3.4.3.1 (new), 6.x.3.3.4.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.291

6.x.3	Resources

6.x.3.1	Overview

Figure 6.x.3.1-1: Resource URI structure of the Nchf_OfflineOnlyCharging API

Offline Only Charging Data Ref is a unique identifier for a charging data resource in a PLMN. It’s created in CHF when CHF receives a Nchf_ OfflineOnlyCharging_Create request and provided to NF (CTF) in the Location header field in the Nchf_ OfflineOnlyCharging_Create response. The NF (CTF) shall use the Offline Only Charging Data Ref received in subsequent requests to the CHF for the same charging data resource.

Table 6.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description | Corresponding service operation |
| --- | --- | --- | --- | --- |
| Offline Only Charging Data | {apiRoot}/ Nchf_OfflineOnlyCharging/ v1/chargingdata/ | POST | Create a new Offline Only Charging Data resource | Nchf_ OfflineOnlyCharging_Create |
| Individual Offline Only Charging Data | {apiRoot}/ Nchf_OfflineOnlyCharging/v1/ offlineonlychargingdata/{OfflineOnlyChargingDataRef }/update | POST | Update an existing Offline Only Charging Data resource. | Nchf_ OfflineOnlyCharging_Update |
|  | {apiRoot}/ Nchf_OfflineOnlyCharging/v1/ chargingdata /{ChargingDataRef}/release | POST | Update and release an existing Offline Only Charging Data resource. | Nchf_ OfflineOnlyCharging_Release |

6.x.3.2	Resource: Charging Data

6.x.3.2.1	Description

Offline Only Charging Data resource represents a collection of the different charging data resources created by the CHF for offline only charging as defined in 3GPP TS 32.290 [58].

6.x.3.2.2	Resource Definition

Resource URI: {apiRoot}/nchf-OfflineOnlyCharging/v1/chargingData/

This resource shall support the resource URI variables defined in table 6.x.3.2.2-1.

Table 6.x.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.X.1 |

6.x.3.2.3	Resource Standard Methods

6.x.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.x.3.2.3.1-1.

Table 6.x.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.3.2.3.1-2 and the response data structures and response codes specified in table 6.x.3.2.3.1-3.

Table 6.x.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ChargingDataRequest | M | 1 | Parameters to create a new Offline Only Charging Data resource. |

Table 6.x.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ChargingDataResponse | M | 1 | 201 Created | The creation of a Offline Only Charging Data resource is confirmed and a representation of that resource is returned. The Offline Only Charging Data resource which is created and returned successfully. The representation of created resource is identified via Location header field in the 201 response. |
|  |  |  | 307 Temporary Redirect | (NOTE 2) |
| ChargingDataResponse | M | 1 | 400 Bad Request | (NOTE 2) |
| ChargingDataResponse | M | 1 | 403 Forbidden | (NOTE 2) |
| ChargingDataResponse | M | 1 | 404 Not Found | (NOTE 2) |
|  | M | 1 | 405 Method Not Allowed | (NOTE 2) |
|  | M | 1 | 408 Request Timeout | (NOTE 2) |
|  | M | 1 | 500 Internal Server Error | (NOTE 2) |
|  | M | 1 | 503 Service Unavailable | (NOTE 2) |
|  | M | 1 | 508 Gateway Timeout | (NOTE 2) |
| NOTE 1: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [299] for the POST method also apply. NOTE 2: Failure cases are described in subclause 6.x.7. |  |  |  |  |

6.x.3.2.4	Resource Custom Operations

None.

6.x.3.3	Resource: Individual Offline Only Charging Data

6.x.3.3.1	Description

Individual Offline Only Charging Data resource represents a offline only Charging data resource created in the CHF.

6.x.3.3.2	Resource Definition

Resource URI: {apiRoot}/nchf-offlineonlycharging/v1/offlineonlychargingdata/{OfflineDataChargingDataRef}

This resource shall support the resource URI variables defined in table 6.x.3.3.2-1.

Table 6.x.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 6.x.1 |
| OfflineOnlyChargingDataRef | Offline only charging data resource reference assigned by the CHF during the Nchf_ OfflineOnlyCharging_Create operation, |

6.x.3.3.3	Resource Standard Methods

None.

6.x.3.3.4	Resource Custom Operations

6.x.3.3.4.1	Overview

Table 6.x.3.3.4.1-1: Custom operations

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/ nchf-offlineonlyncharging/v1/ offlineonlychargingdata/{OfflineOnlyChargingDataRef }/update | POST | Update an existing Offline Only Charging Data resource. |
| {apiRoot}/ nchf-offlineonlycharging/v1/ offlineonlychargingdata /{OfflineOnlyChargingDataRef}/release | POST | Update and release an existing Offline Only Charging Data resource. |

6.x.3.3.4.2	Operation: update

6.x.3.3.4.2.1	Description

This operation updates an existing Offline Only Charging Data resource.

6.x.3.3.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.x.3.3.4.2.2-1 and the response data structures and response codes specified in table 6.x.3.3.4.2.2-2.

Table 6.x.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ChargingDataRequest | M | 1 | Parameters to modify an existing Offline Only Charging Data resource matching the OfflineOnlyChargingDataRef according to the representation in the OfflineOnlyChargingData. The request URI is the representation in the Location header field in the 201 response of resource creation. |

Table 6.x.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ChargingDataResponse | M | 1 | 200 OK | The modification of a Offline Only Charging Data resource is confirmed and a representation of that resource is returned. The Offline Only Charging Data resource which is modified and returned successfully. |
|  |  |  | 307 Temporary Redirect | (NOTE 2) |
| ChargingDataResponse | M | 1 | 400 Bad Request | (NOTE 2) |
| ChargingDataResponse | M | 1 | 403 Forbidden | (NOTE 2) |
| ChargingDataResponse | M | 1 | 404 Not Found | (NOTE 2) |
|  | M | 1 | 405 Method Not Allowed | (NOTE 2) |
|  | M | 1 | 408 Request Timeout | (NOTE 2) |
|  | M | 1 | 500 Internal Server Error | (NOTE 2) |
|  | M | 1 | 503 Service Unavailable | (NOTE 2) |
|  | M | 1 | 508 Gateway Timeout | (NOTE 2) |
| NOTE 1: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [299] for the POST method also apply. NOTE 2: Failure cases are described in subclause 6.x.7. |  |  |  |  |

6.x.3.3.4.3	Operation: release

6.x.3.3.4.3.1	Description

This operation update and release an existing charging session

6.x.3.3.4.3.2	Operation Definition

This operation shall support the request data structures specified in table 6.x.3.3.4.3.2-1 and the response data structures and response codes specified in table 6.x.3.3.4.3.2-2.

Table 6.x.3.3.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ChargingDataRequest | M | 1 | Parameters to modify and then release the Offline Only Charging Data resource matching the OfflineOnlyChargingDataRef according to the representation in the OfflineOnlyChargingData. The request URI is the representation in the Location header field in the 201 response of resource creation. |

Table 6.x.3.3.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | Successful case: The Offline Only Charging Data resource matching the OfflineOnlyChargingDataRef is modified and then released. |
| ChargingDataResponse | M | 1 | 404 Not Found | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in subclause 6.x.7. |  |  |  |  |

6.x.4	Custom Operations without associated resources

None.

End of change