3GPP TSG-SA5 Meeting #125 	S5-193283

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0045 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Failure and Retry handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correcting the failure and retry handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding how failure and retry should be handled by CHF and NF (CTF) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interworking between CHF and any NF (CTF) in case of failure and retry would be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.5      Error handling

### 5.5.1	Failure handling

#### 5.5.1.1	CTF detected failure

An application level error can be detected by the NF consumer (CTF) based on the response from the CHF (i.e. Invocation Result Code in Invocation Result) or when a Charging Data Request times out (i.e. receives no answer with in a configurable time frame).

The NF Consumer shall have a locally configured failure handling procedure, which the CHF may override by using Failure Handling field in Invocation Result, to indicate how the NF consumer (CTF) shall behave in case of a failure. The latest value for Failure Handling received from CHF shall be applicable until a new value is received.

The NF consumer (CTF) shall support three types of Failure Handling:

-	Retry and Continue: the charged session or event shall be allowed to continue without charging, based on locally configured limits (e.g. time, volume). The NF consumer (CTF) may retry the request;

-	Terminate: the charged session or event shall not be allowed to continue, the NF consumer (CTF) shall try to send a Termination to the CHF in the case the failed request was an Initial or Update;

-	Retry and Terminate: the request may be retried towards the CHF before the NF consumer (CTF) follows the Terminate failure handling procedure.

The application level errors in the response from the CHF are classified as either transient or permanent, NF consumer (CTF) handling shall depend on the classification.

For protocol level errors, refer to applicable protocol failure handling mechanisms.

#### 5.5.1.2	CHF detected failure

An application level error for requests from NF consumer (CTF) can be detected by the CHF based on missing invocations (i.e. missing Invocation Sequence Numbers) or a request that times out (i.e. receives no update when expected). In this case the CHF closes a CDR and all the reserved resources are freed for the charging session.

An application level error for notifications to NF consumer (CTF) can be detected by the CHF when a request times out (i.e. receives no answer with in a configurable time frame). In this case the Charging session shall be kept, and the CHF may retry the notification.

### 5.5.2	Retry handling

In the case of a transient failure or a request time out the NF consumer (CTF) may retry the message. The number of retries and delay between retries shall be locally configured in the NF consumer (CTF).

If a request is retried it shall have the same Invocation Sequence Number as the retried message i.e. it shall not be incremented. The NF consumer (CTF) may send it to an alternative CHF if the Session Failover indication is received from the CHF.

In the case of a notification request time out the CHF may retry the message. The number of retries and delay between retries shall be locally configured in the CHF.

End of change