3GPP TSG-SA5 Meeting #125Adhoc 	S5-194043

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0073 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce offline only determination by SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-06-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Solve Editor's Note on how SMF determines PDU session is "offline only" during the PDU session lifetime. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce "offline only" charging method applicable at PDU session level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for the SMF to determine when to invoke "offline only" charging service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.8, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.8	CHF selection

The CHF selection, i.e. CHF address determination, by the SMF is done at the PDU session establishment, this selection shall be based on the following and with this priority order (highest to lowest):

-	CHF address(es) provided by the PCF for the PDU session.

-	UDM provided charging characteristics.

-	NRF based discovery.

-	SMF locally provisioned charging characteristics.

This means that if there are PCF provided CHF address(es) these shall be used, otherwise if the UDM provides charging characteristics these shall be used. If neither of these results in CHF address(es) the NRF can be used to discover CHF instance(s), and as a last resource the SMF locally provisioned charging characteristics shall be used.

When NRF is used for the CHF selection, and the applicable charging method for the PDU session is "offline only", CHF instance(s) supporting "offline only" charging service instances may be selected.

Next change

Annex A (normative):
Charging Characteristics

# A.1	General

A subscriber may have Charging Characteristics assigned to his subscription and/or his subscribed DNNs. Default Charging Characteristics may also be pre-provisioned on the SMF.

During UE PDU session establishment, when the SMF retrieves subscription data from the UDM, if a subscribed Charging Characteristics for the requested DNN is identified, it may override the SMF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as Behaviours (B), freely defined by Operators, as shown in TS 32.298 [51]. Each bit corresponds to a specific charging behaviour which is configured on a per operator basis and pointed when bit is set to "1" value.

A charging behaviour is defined as an association to a specific usage design:

One usage may consist of a set of trigger profiles associated to the PDU session as described in the Table A.1 example:

Table A.1: Example of Charging Characteristics behaviours for SMF

|  |  |  |  |  |  |  |  | PDU session charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behaviour index | Default charging method |  | PDU session charging method |  |  | Charging service |  |  | Primary and Secondary CHF addresses |  | Active |  | Time Limit Per PDU session |  | Vol Limit Per PDU session |  | Change Cond. Per PDU session |  | Tariff times |  | … |  |
| 0 | Online |  | - |  |  | Converged charging |  |  | URI 1 URI 2 |  | Yes |  | 10 min |  | 1Mb |  | 2 |  | 0-7, 7-12 |  | ... |  |
| 1 | Offline |  | - |  |  | Converged charging |  |  | URI 1 URI 2 |  | Yes |  | 15 min |  | 5Mb |  | 3 |  | 0-24 |  | … |  |
| 2 | Online |  | - |  |  | Converged charging |  |  | URI 1 URI 2 |  | Yes |  | 30 min |  | 2Mb |  | 2 |  | 0-7, 7-12 |  | ... |  |
| 3 | - |  | Offline only |  |  | Offline only charging |  |  | URI 1 URI 2 |  | Yes |  | 15 min |  | 1MB |  | 1 |  | 0-24 |  | ... |  |
| 4 | - | Offline only |  | Converged charging |  | URI 1 URI 2 | No | 15 min | 1MB | 1 | 0-24 | … |
| … | … |  |  |  |  |  |  |  | … |  | … |  | … |  | … |  | … |  | … |  | … |  |

Associated to the behaviour, the following may also be configured:

-	the CHF addresses to be used by the SMF;

-	the "Default charging method" (online or offline) to be used as the default one for every PCC rules of the PDU Session;

-	the "PDU session charging method" indicating whether the charging method for the PDU session is "offline only".

-	charging service.

The "Default charging method", "PDU session charging method" and CHF addresses configured in the applicable Charging Characteristics behaviour, are superseded by "Default charging method", "PDU session charging method"  and CHF addresses supplied by the PCF if any, during SMF interaction with the PCF at PDU session establishment, as described in TS 23.503 [202].

End of changes