3GPP TSG-SA5 Meeting #125Adhoc 	S5-194112

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0077 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Simple data types and enumerations for offline only charging service API data model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The offline only charging service API is not specified in TS 32.291. 5G Simple data types and enumerations of data model for offline only charging service API is not addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G Simple data types and enumerations of definitions of offline only charging service API are added. Add a new clause (6.2.5.x (new)) for pending content potential for SMF specified data types before clause (6.2.5.y (new)) for added 5G Simple data types and enumerations definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G Simple data types and enumerations of offline only charging service API is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5.x (new), 6.2.5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.291

6.2.5.x	Void

6.2.5.y	Simple data types and enumerations

6.2.5.y.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses. Following definitions of data types and enumerations for offline charging are same as definitions of data types and enumerations for converged charging specified in clause 6.1.6.3 of this specification:

- Definition of simple data types;

- Enumeration: NotificationType;

- Enumeration: ChargingCharacteristicsSelectionMode;

- Enumeration: FinalUnitAction;

- Enumeration: RedirectAddressType;

- Enumeration: TriggerCategory;

- Enumeration: 3GPPPSDataOffStatus;

- Enumeration: ResultCode;

- Enumeration: PartialRecordMethod;

- Enumeration: RoamerInOut;

- Enumeration: SubscriberIdentityType;

- Enumeration: SMMessageType;

- Enumeration: SMPriority;

- Enumeration: DeliveryReportRequested;

- Enumeration: InterfaceType;

- Enumeration: ClassIdentifier;

- Enumeration: SMAddressType;

- Enumeration: SMAddresseeType;

- Enumeration: ReplyPathRequested.

6.2.5.y.2	Enumeration: NodeFunctionality

Table 6.2.5.x.2-1: Enumeration NodeFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SMF | This field identifies that NF is a SMF. |  |

6.2.5.y.3	Enumeration: TriggerType

Table 6.2.5.x.3-1: Enumeration TriggerType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FINAL | a service termination has happened |  |
| ABNORMAL_RELEASE | PDU session has abnormal released. |  |
| QOS_CHANGE | In request message, this value is used to indicate that OoS change has happened. |  |
| VOLUME_LIMIT | Volume limit has been reached. |  |
| TIME_LIMIT | Time limit has been reached |  |
| EVENT_LIMIT | Event limit has been reached |  |
| PLMN_CHANGE | PLMN has been changed. |  |
| USER_LOCATION_CHANGE | In request message, this value is used to indicate that User location has been changed. |  |
| RAT_CHANGE | In request message, this value is used to indicate that RAT type has been changed. |  |
| UE_TIMEZONE_CHANGE | In request message, this value is used to indicate that UE timezone has been changed. |  |
| TARIFF_TIME_CHANGE | Tariff time change has happened. |  |
| MAX_NUMBER_OF_CHANGES_IN CHARGING_CONDITIONS | Max number of change has been reached |  |
| MANAGEMENT_INTERVENTION | Management intervention |  |
| CHANGE_OF_UE_PRESENCE_IN PRESENCE_REPORTING_AREA | In request message, this value is used to indicate that Change of UE presence in PRA has happened. In response message, this value is used to indicate a request of reporting the event that the user enters/leaves the area(s) as indicated in the presenceReportingArea Attribute |  |
| CHANGE_OF_3GPP_PS_DATA_OFF_STATUS | In request message, this value is used to indicate that Change of 3GPP PS Data off status has happened. |  |
| SERVING_NODE_CHANGE | A serving node (e.g., AMF) change in the NF Consumer |  |
| REMOVAL_OF_UPF | A used UPF is removed |  |
| ADDITION_OF_UPF | A new UPF is added. |  |
| HANDOVER_CANCEL | The handover is cancelled. |  |
| HANDOVER_START | The handover is start. |  |
| HANDOVER_COMPLETE | The handover is completed. |  |

6.2.5.y.4	Enumeration: FailureHandling

Table 6.2.5.x.4-1: Enumeration FailureHandling

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TERMINATE | the service shall only be granted for as long as there is a connection between NF consumer and the CHF. |  |
| CONTINUE | the NF consumer should re-send and continue the request to an alternative server in the case of transport temporary failures, provided that a failover procedure is supported in the CHF and the NF consumer, and that an alternative server is available. Otherwise, the service SHOULD be granted, even if charging data request can't be delivered. |  |
| RETRY_AND_TERMINATE | the NF consumer should re-send the request to an alternative server in the case of transport temporary failures, provided that a failover procedure is supported in the CHF and NF consumer, and that an alternative server is available. Otherwise, the service should not be granted when the charging data request can't be delivered. |  |

6.2.5.y.5	Enumeration: SessionFailover

Table 6.2.5.x.5-1: Enumeration SessionFailover

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FAILOVER_NOT_SUPPORTED | The Nchf_OfflineOnlyCharging messages could not be moved to an alternative destination in the case of communication failure. This is the default behaviour if the attribute is not present in the response. |  |
| FAILOVER_SUPPORTED | The Nchf_ OfflineOnlCharging messages should be moved to an alternative destination in the case of communication failure. |  |

6.2.5.x.6	Enumeration: ResultCode

Table 6.1.6.3.6-1: Enumeration ResultCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | The CHF grants the service to the end-user. This applies to the rating group. |  |
| END_USER_SERVICE_DENIED | The CHF denies the service request due to end-user service restrictions or limitations related to the end-user (i.e. terminate services for the rating group). If the request contained used units they shall be deducted, if applicable. This applies to the rating group. |  |

End of change