3GPP TSG-SA5 Meeting #125Adhoc 	S5-194160

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0019 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on kbits abbreviation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some undefined kbits used in KPI definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unify the use of kbit in the corresponding KPIs and add a clear abbreviation of kbit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use of the undefined kbits may lead to interoperation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.3.2, 6.3.3, 6.3.4, 6.3.5, 6.3.6.1, 6.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

kbit	kilobit (1000 bits)

RTT	Round Trip Time

Next change

### 6.3.2	Upstream throughput for network and Network Slice Instance

a)	Upstream throughput for network and network slice instance.

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by upstream throughput provided by N3 interface from NG-RAN to all UPFs which are related to the single network slice instance.

d)	UTSNSI=UPFGTP.InDataOctN3UPF

e)	GTP.InDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	CUM.

### 6.3.3	Downstream Throughput for Single Network Slice Instance

a)	Downstream throughput for network and network slice instance.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by downstream throughput provided by N3 interface from all UPFs to NG-RAN which are related to the single network slice instance.

d)

e)	GTP.OutDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	CUM.

6.3.4	Upstream Throughput at N3 interface

a)	Upstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all incoming GTP data packets on the N3 interface (measured at UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate upstream GTP throughput integrity performance at the N3 interface.

c)	This KPI is obtained by measuring the GTP data upstream throughput provided by N3 interface from NG-RAN to UPF, during the granularity period.

d)	UGTPTS=SUM (GTP.InDataOctN3UPF)/timeperiod) at UPF

e)	GTP.InDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

6.3.5	Downstream Throughput at N3 interface

a)	Downstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all downstream GTP data packets on the N3 interface (transmitted downstream from UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate integrity performance at N3 interface.

c)	This KPI is obtained by measuring the GTP data downstream throughput provided by N3 interface from UPF to NG-RAN, during the granularity period.

d)	DGTPTS=SUM (GTP.OutDataOctN3UPF)/timeperiod) at UPF

e)	GTP.OutDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

6.3.6	RAN UE Throughput

6.3.6.1	Definition

a)	RAN UE Throughput.

b)	A KPI that shows how NG-RAN impacts the service quality provided to an end-user.

c)	Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface.

d)	RAN UE Throughput DL = DRB.UEThpDl and

RAN UE Throughput UL = DRB.UEThpUl

or optionally RAN UE Throughput DL for single mapped 5QI or QCI = DRB.UEThpDl.QoS and

RAN UE Throughput UL for single mapped 5QI or QCI = DRB.UEThpUl.QoS

e)	DRB.UEThpDl, DRB.UEThpUl, DRB.UEThpDl.QoS, DRB.UEThpUl.QoS

f)	 NG-RAN

g)	Integrity

h)	kbit/s

i)	MEAN

6.3.6.2	Extended definition

To achieve a Throughput measurement (below examples are given for DL) that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in figure 1).

Time (slots)

Data arrives to

empty DL buffer

First data is

transmitted to the UE

The send buffer is

again empty

calulations since it

can be impacted

by packet size of

User Plane (UP) packets.

ThpTimeDl

Failed transmission (

”

Block

error

”

)

Successful transmission,

buffer not empty

Successful transmission,

buffer empty

ThpVolDl =

∑

Total DL transferred volume =

∑

(kbit)

+

(kbit)

UE Throughput in DL =

ThpVolDl / ThpTimeDl (kbit/s)

No transmission, buffer not

empty (e.g. due to contention)

The last slot shall always be removed from

Figure 1

To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

End of changes