3GPP TSG-SA5 Meeting #125Adhoc 	S5-194175

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.426 | CR | 0074 | rev | - | Current version: | 10.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify measurement KPIs for paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM8 |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurement KPIs for paging in TS 32.426 missed some precise information on how to judge successfully received reponse to paging. In TS 32.426 v10.5.0, the service request can be triggered by UE when reciving a paging reqeust and UE is in EMM-IDLE mode. See 5.6.1.1 in TS 24.301 v10.15.0. The service request as the response to the paging request toward to MME, the call type equel to "terminating calls". The cases that a SERVICE REQUEST is a response to paging, where the CN domain indicator is set to "PS" or where service type set to "mobile terminating CS fallback or 1xCS fallback". Current description in TS 32.426 v.10.5.0 when set the criteria for MME increments count for successful paging is not clear to state that "Incremented when a paging_response is received by the MME from the UE as response to an EPS PS paging procedure (Receipt of "SERVICE REQUEST" message with Service Type = Paging Response from the UE (TS 24.301 [3]))". At least, Paging Response is not an existing value of Service Type in a SERVICE REQUEST. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the exact value of Service Type within a SERVICE REQUEST that is a response to paging request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The criteria for MME to successful paging request is incorrectly described in TS 32.426 v10.5.0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start change

4.1.9.2         Successful EPS paging procedures

- This measurement provides the number of successful PS paging procedures initiated at the MME. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a "SERVICE REQUEST" as a response to paging request is received by the MME from the UE where call type is "terminating calls", and, Service Type is the CN domain indicator is set to "PS" or "mobile terminating CS fallback or 1xCS fallback" (TS 24.301 [3]). which are counted when paging area is smaller than or equal to one TA.

- A single integer value.

- MM.PagingEpsSucc

- TA, specified by a concatenation of the MCC (Mobile Country Code), MNC (Mobile Network Code), TAC (Tracking Area Code).

- Valid for packet switching.

- EPS.

End of change