3GPP TSG-SA5 Meeting #125Adhoc 	S5-194236

# Sapporo, Japan 25-28 June 2019												        Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR |  | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure of NSI performance assurance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the conclusion for Key Issue 14 of TR 23.791, SA2 concluded that SLA fulfilment evaluation is performed by OAM. The slice QoE analytics granularity when consumed by OAM will be per slice ID per user. ************* Extracted from TS 23.791 V16.2.0 ********************** 8.14A Key Issue 14: How to ensure that slice SLA is guaranteed The following conclusions are agreed for normative work related to Key Issue 14: - NWDAF provides analytics for slice QoE. - The slice QoE analytics granularity when consumed by OAM will be per slice ID per user. - SLA fulfilment evaluation is performed by OAM as indicated in TR 28.805 [26]. Any slice admission control mechanism in 5GC, based on NWDAF analytics and OAM triggers, requires further SA2 investigation and will not be progressed in Rel-16. No interaction between 5GC and NG-RAN on slice SLA or slice QoE will be progressed in Rel-16. ****************** End of extraction from TS 23.791 ******************** |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure of NSI performance assurance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of SA2 will not be implemented in R16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, x(new), x.1(new), x.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modified section in TS 28.550

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	ITU-T Recommendation X.721 (1992): "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	ISO 8601:2000(E) Data elements and interchange formats – Information interchange – Representation of dates and times".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[9]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[10]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[11]	W3C REC-xml-names-19990114: "Namespaces in XML".

[12]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[13]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 32.158: "Management and orchestration; Design rules for Representational State Transfer (REST) Solution Sets (SS)".

[x]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[y]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[z]	3GPP TS 22.288: " Technical Specification Group Services and System Aspects; Architecture enhancements for 5G System (5GS) to support network data analytics services".

Start of 2nd modified section in TS 28.550

X.	Network slice performance assurance

X.1	General

After NSI creation all resources to the NSI have been created and configured to satisfy the network slice requirements, including the performance of NSI requested according to network slice SLA. In order to guarantee the performance of network slice, NSI modification can be triggered by NSI supervision automatically. During the NSI performance supervision, the end to end KPIs defined in TS 28.554 [y], performance measurements defined in TS 28.552[2] and optionally Slice QoE provided by NWDAF [z] should be used for SLA fulfilment evaluation. According to the evaluation result.  NSI modification defined in TS 23.531[x] can be triggered for network slice performance assurance.

X.2	Procedure of NSI performance assurance

Figure X.2-1 depicts the procedure of performance assurance for a network slice instance to satisfy the NSI performance request received from an authorized consumer.

Figure X.2-1: Network slice instance performance assurance procedure

- Authorized consumer requests NSMS_ Provider to create a new NSI. After NSI allocation, the NSMS_Provider sends NSI allocation result to consumer. NSMS_ Provider obtains slice provisioning requirements including the requested network slice performance defined by perfReq in TS 23.541 [3]. NSMS_Provider performs NSI performance supervision according to the requested network slice performance.

- NSMS_Provider may consume the services and get Slice QoE provided by NWDAF [z].

- NSMS_Provider may get performance measurements of constituent NSSIs by consume NSSI PM services.

- Based on the KPIs of NSI, performance measurements of constituent NSSIs and with Slice QoE, NSMS_Provider evalutates SLA fulfilments for the NSI with the assistance of MDAS_Provider.

- According to the result of SLA fulfilments evaluation, NSMS_Provider decides whether to modify the NSI.

- If NSI modification is needed, the NSMS_Provider triggers the NSI modification procedure defined in TS 23.531 [x]. NSSMS_ Provider of CN modifies virtrual resources and configuration of 5GC NFs to guarantee the performance requirements. NSSMS_ Provider of AN reconfigures RRMPolicy to optimize performance.

NOTE1: Step 2 to 6 will be performed during the whole NSI operation phase.

NOTE2: If there is a potential improvement of the performance, NSMS_Provider may also trigger NSI modification according to the recommendation from MDAS_Provider.

End of modification in TS 28.550