3GPP TSG-SA5 Meeting #126	S5-195224

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0027 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.550 NSI and NSSI performance assurance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conclusion of TR 23.791, the slice QoE analytics per slice ID provided by NWDAF can be consumed by 3GPP management system. The analytics from NWDAF may be used for NSI and NSSI performance assurance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure of NSI and NSSI performance assurance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no NSI and NSSI performance procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new), X.1(new), X.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modified section in TS 28.550

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	ITU-T Recommendation X.721 (1992): "Information technology - Open Systems Interconnection - Structure of management information: Definition of management information".

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	ISO 8601:2000(E) Data elements and interchange formats – Information interchange – Representation of dates and times".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[9]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[10]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[11]	W3C REC-xml-names-19990114: "Namespaces in XML".

[12]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[13]	3GPP TS 28.628: "Telecommunication management; Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	3GPP TS 32.158: "Management and orchestration; Design rules for Representational State Transfer (REST) Solution Sets (SS)".

[x]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[y]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[z]	3GPP TS 23.288: " Technical Specification Group Services and System Aspects; Architecture enhancements for 5G System (5GS) to support network data analytics services".

Start of 2nd modified section in TS 28.550

Annex X (Normative)

NSI and NSSI performance assurance

x.1	General

After network slice instance is created, 3GPP management system creates all resources to the NSI and configurations satisfy the network slice requirements, including the performance of NSI requested according to network slice SLA. In order to guarantee the performance of network slice, NSI modification can be triggered by NSI provisioning service. During the NSI performance supervision, the end to end KPIs defined in TS 28.554 [y], performance measurements defined in TS 28.552[x] and optionally slice QoE provided by NWDAF in 3GPP TS 23.288 [z] should be used for SLA fulfilment evaluation. According to the evaluation result, NSI modification defined in TS 23.531[8] can be triggered for network slice performance assurance.

x.2	Procedure of NSI and NSSI performance assurance

This Figure x.2-1 illustrates the procedure of performance assurance for NSI(s) or NSSI(s).

Figure x.2-1: Example of procedure for NSI and NSSI performance assurance

1.	The authorized consumer requests NSMS_ Provider to allocate a new NSI. NSMS_ Provider consumes provisioning services provided by NSSMS_ Provider to create the NSI. After NSI allocation, the NSMS_Provider and NSSMS_Provider perform NSI and NSSI performance supervision.

2.	The NSMS_Provider or NSSMS_Provider may get Slice QoE provided by NWDAF [x].

3.	The NSMS_Provider or NSSMS_Provider check whether the performance requirements can be met by NSI or NSSI.

4a. If the performance requirements of NSI cannot be met, the NSMS_Provider triggers the NSI modification procedure. NSMS_ Provider modifies the capacity of the network slice, or modify the network slice configuration to guarantee the performance requirements.

4b. If the performance requirements of NSSI cannot be met, the NSSMS_ Provider of CN modifies virtual resources and configuration of 5GC NFs to guarantee the performance requirements. NSSMS_ Provider of AN reconfigures RRMPolicy to optimize performance.

End of modification in TS 28.550