3GPP TSG-SA5 Meeting #126	S5-195229

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.426 | CR | 0083 | rev | - | Current version: | 11.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Rel-11 32.426 Correct description of measurement KPI of paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM8 |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The criteria for MME to increment the count of successful paging is not reflected in TS 32.426 v15.2.0 description of measurement for paging,: "Incremented when a paging response is received by the MME from the UE as response to an EPS PS paging procedure (Receipt of "SERVICE REQUEST" message with Service Type = Paging Response from the UE (TS 24.301 [3]))". Since the "Paging Response" in a SERVICE REQUEST does not represents the value of Service Type, but only the indication of paging response to MME, the measurement description for paging in TS 32.426 is missing the information indicating the successfully received response to paging. The missing information can be seen in SERVICE REQUEST message indicates a paging response where the CN domain indicator set to "PS". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the exact value of Service Type within a SERVICE REQUEST of the response to paging request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The criteria for MME to successful paging request is incorrectly described in TS 32.426 v11.4.0 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start change

4.1.9.2         Successful EPS paging procedures

- This measurement provides the number of successful PS paging procedures initiated at the MME. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a paging_response is received by the MME from the UE as response to an EPS PS paging procedure (Receipt of "SERVICE REQUEST" message indicaiting a paging response from the UE (TS 24.301 [3])) , which are counted when paging area is smaller than or equal to one TA.

NOTE:	SERVICE REQUEST message indicates a paging response where the CN domain indicator set to "PS".

- A single integer value.

- MM.PagingEpsSucc

- TA, specified by a concatenation of the MCC (Mobile Country Code), MNC (Mobile Network Code), TAC (Tracking Area Code).

- Valid for packet switching.

- EPS.

End of change