3GPP TSG-SA5 Meeting #126	S5-195267

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0113 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the measurement related to Number of PDU session creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/8/9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 28.552 (16.2.0), the number of PDU session creation does not consider HR roaming scenario. Therefore it needs to clearly identify for the current number to be only applicable to Non-roaming/LBO roaming scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update number of PDU session creation: - Number of PDU session creation requests - Number of successful PDU session creations - Number of failed PDU session creations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of PDU session creation depending on the different roaming scenarios cannot be classified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.3, 5.3.1.4, 5.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.1.3	Number of PDU session creation requests

a)	This measurement provides the number of PDU sessions requested to be created by the (V-)SMF in Non-roaming/LBO roaming scenario.

b)	CC

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_CreateSMContext Request (see 3GPP TS 23.502 [7]). Each PDU session requested to be created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationReq.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationReq.ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	(V-)SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.3.1.4	Number of successful PDU session creations

a)	This measurement provides the number of PDU sessions successfully created by the SMF in Non-roaming/LBO roaming scenario.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a successful PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session successfully created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationSucc.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationSucc. ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	(V-)SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.3.1.5	Number of failed PDU session creations

a)	This measurement provides the number of PDU sessions failed to be created by the SMF in Non-roaming/LBO roaming scenario.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a rejected PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session rejected to be created is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationFail.cause

Where cause indicates the rejection cause for the PDU session.

f)	(V-)SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

End of Changes