3GPP TS 28.552 V16.2.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Management and orchestration;

5G performance measurements

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G,management,orchestration,performance,measurements

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions, abbreviations and measurement family	12

3.1	Definitions	12

3.2	Abbreviations	13

3.4	Measurement family	13

4	Concepts and overview	13

4.1	Performance indicators	13

5	Performance measurements for 5G Network Functions	14

5.1	Performance measurements for gNB	14

5.1.1	Performance measurements valid for all gNB deployment scenarios	14

5.1.1.1	Packet Delay	14

5.1.1.1.1	Average delay DL air-interface	14

5.1.1.1.2	Distribution of delay DL air-interface	14

5.1.1.2	Radio resource utilization	15

5.1.1.2.1	DL Total PRB Usage	15

5.1.1.2.2	UL Total PRB Usage	15

5.1.1.2.3	Distribution of DL Total PRB Usage	16

5.1.1.2.4	Distribution of UL total PRB usage	16

5.1.1.2.5	DL PRB used for data traffic	17

5.1.1.2.6	DL total available PRB	17

5.1.1.2.7	UL PRB used for data traffic	18

5.1.1.2.8	UL total available PRB	18

5.1.1.3	UE throughput	18

5.1.1.3.1	Average DL UE throughput in gNB	18

5.1.1.3.2	Distribution of DL UE throughput in gNB	19

5.1.1.3.3	Average UL UE throughput in gNB	21

5.1.1.3.4	Distribution of UL UE throughput in gNB	21

5.1.1.3.5	Percentage of unrestricted DL UE data volume in gNB	23

5.1.1.3.6	Percentage of unrestricted UL UE data volume in gNB	24

5.1.1.4	RRC connection number	24

5.1.1.4.1	Mean number of RRC Connections	24

5.1.1.4.2	Max number of RRC Connections	25

5.1.1.5	PDU Session Management	25

5.1.1.5.1	Number of PDU Sessions requested to setup	25

5.1.1.5.2	Number of PDU Sessions successfully setup	25

5.1.1.5.3	Number of PDU Sessions failed to setup	26

5.1.1.6	Mobility Management	26

5.1.1.6.1	Inter-gNB handovers	26

5.1.1.6.1.1	Number of requested handover preparations	26

5.1.1.6.1.2	Number of successful handover preparations	27

5.1.1.6.1.3	Number of failed handover preparations	27

5.1.1.6.1.4	Number of requested handover resource allocations	27

5.1.1.6.1.5	Number of successful handover resource allocations	28

5.1.1.6.1.6	Number of failed handover resource allocations	28

5.1.1.6.1.7	Number of requested handover executions	29

5.1.1.6.1.8	Number of successful handover executions	29

5.1.1.6.1.9	Number of failed handover executions	29

5.1.1.6.1.10	Mean Time of requested handover executions	30

5.1.1.6.2	Intra-gNB handovers	31

5.1.1.6.2.1	Number of requested handover executions	31

5.1.1.6.2.2	Number of successful handover executions	31

5.1.1.7	TB related Measurements	31

5.1.1.7.1	Total number of DL initial TBs	31

5.1.1.7.2	Intial error number of DL TBs	32

5.1.1.7.3	Total number of DL TBs	32

5.1.1.7.4	Total error number of DL TBs	33

5.1.1.7.5	Residual error number of DL TBs	33

5.1.1.7.6	Total number of UL initial TBs	33

5.1.1.7.7	Error number of UL initial TBs	34

5.1.1.7.8	Total number of UL TBs	34

5.1.1.7.9	Total error number of UL TBs	34

5.1.1.7.10	Residual error number of UL TBs	35

5.1.1.8	Void	35

5.1.1.9	Void	35

5.1.1.10	DRB related measurements	35

5.1.1.10.1	Number of DRBs attempted to setup	35

5.1.1.10.2	Number of DRBs successfully setup	36

5.1.1.10.3	Number of released active DRBs	36

5.1.1.10.4	In-session activity time for DRB	37

5.1.1.11	CQI related measurements	38

5.1.1.11.1	Wideband CQI distribution	38

5.1.1.12	MCS related Measurements	38

5.1.1.12.1	MCS Distribution in PDSCH	38

5.1.1.12.2	MCS Distribution in PUSCH	38

5.1.1.13	QoS flow related measurements	39

5.1.1.13.1	QoS flow release	39

5.1.1.13.1.2	Number of QoS flows attempted to release	40

5.1.1.13.2	QoS flow activity	40

5.1.1.13.3	QoS flow setup	41

5.1.1.13.3.1	Number of QoS flow attempted to setup	41

5.1.1.13.3.2	Number of QoS flow successfully established	41

5.1.1.13.3.3	Number of QoS flow failed to setup	42

5.1.1.14	Void	42

5.1.1.15	RRC connection establishment related measurements	42

5.1.1.15.1	Attempted RRC connection establishments	42

5.1.1.15.2	Successful RRC connection establishments	43

5.1.1.16	UE-associated logical NG-connection related measurements	43

5.1.1.16.1	Attempted UE-associated logical NG-connection establishment from gNB to AMF	43

5.1.1.16.2	Successful UE-associated logical NG-connection establishment from gNB to AMF	44

5.1.1.17	RRC Connection Re-establishment	44

5.1.1.17.1	Number of RRC connection re-establishment attempts	44

5.1.1.17.2	Successful RRC connection re-establishment with UE context	44

5.1.1.17.3	Successful RRC connection re-establishment without UE context	45

5.1.1.18	RRC Connection Resuming	45

5.1.1.18.1	Number of RRC connection resuming attempts	45

5.1.1.18.2	Successful RRC connection resuming	45

5.1.1.18.3	Successful RRC connection resuming with fallback	46

5.1.1.18.4	RRC connection resuming followed by network release	46

5.1.1.18.5	RRC connection resuming followed by network suspension	46

5.1.1.19	Power, Energy and Environmental (PEE) measurements	47

5.1.1.19.1	Applicability of measurements	47

5.1.1.19.2	PNF Power Consumption	47

5.1.1.19.2.1	Average Power	47

5.1.119.2.2	Minimum Power	47

5.1.1.19.2.3	Maximum Power	47

5.1.1.19.3	PNF Energy consumption	48

5.1.1.19.4	PNF Temperature	48

5.1.1.19.4.1	Average Temperature	48

5.1.1.19.4.2	Minimum Temperature	48

5.1.1.19.4.3	Maximum Temperature	48

5.1.1.19.5	PNF Voltage	49

5.1.1.19.6	PNF Current	49

5.1.1.19.7	PNF Humidity	49

5.1.2	Performance measurements valid only for non-split gNB deployment scenario	50

5.1.2.1	PDCP Data Volume	50

5.1.2.1.1	DL PDCP SDU Data Volume Measurements	50

5.1.2.1.2	UL PDCP SDU Data Volume Measurements	51

5.1.3	Performance measurements valid for split gNB deployment scenario	53

5.1.3.1	Packet Loss Rate	53

5.1.3.1.1	UL PDCP SDU Loss Rate	53

5.1.3.1.2	UL F1-U Packet Loss Rate	54

5.1.3.1.3	DL F1-U Packet Loss Rate	54

5.1.3.2	Packet Drop Rate	55

5.1.3.2.1	DL PDCP SDU Drop rate in gNB-CU-UP	55

5.1.3.2.2	DL Packet Drop Rate in gNB-DU	55

5.1.3.3	Packet delay	56

5.1.3.3.1	Average delay DL in CU-UP	56

5.1.3.3.2	Average delay DL on F1-U	56

5.1.3.3.3	Average delay DL in gNB-DU	57

5.1.3.3.4	Distribution of delay DL in CU-UP	57

5.1.3.3.5	Distribution of delay DL on F1-U	58

5.1.3.3.6	Distribution of delay DL in gNB-DU	58

5.1.3.4	IP Latency measurements	59

5.1.3.4.1	General information	59

5.1.3.4.2	Average IP Latency DL in gNB-DU	59

5.1.3.4.3	Distribution of IP Latency DL in gNB-DU	59

5.1.3.5	UE Context Release	60

5.1.3.5.1	UE Context Release Request (gNB-DU initiated)	60

5.1.3.5.2	Number of UE Context Release Requests (gNB-CU initiated)	60

5.1.3.6	PDCP data volume measurements	61

5.1.3.6.1	PDCP PDU data volume Measurement	61

5.1.3.6.2	PDCP SDU data volume Measurement	62

5.1.3.6.2.4	UL PDCP SDU Data Volume per interface	63

5.1.3.7	Handovers measurements	64

5.1.3.7.1	Intra-gNB handovers	64

5.1.3.7.1.1	Number of requested handover preparations	64

5.1.3.7.1.2	Number of successful handover preparations	64

5.1.3.8	Distribution of Normally Released Call (5QI 1 QoS Flow) Duration	65

5.1.3.9	Distribution of Abnormally Released Call (5QI 1 QoS Flow) Duration	65

5.2	Performance measurements for AMF	65

5.2.1	Registered subscribers measurement	65

5.2.1.1	Mean number of registered subscribers	65

5.2.1.2	Maximum number of registered subscribers	66

5.2.2	Registration procedure related measurements	66

5.2.2.1	Number of initial registration requests	66

5.2.2.2	Number of successful initial registrations	67

5.2.2.3	Number of mobility registration update requests	67

5.2.2.4	Number of successful mobility registration updates	67

5.2.2.5	Number of periodic registration update requests	68

5.2.2.6	Number of successful periodic registration updates	68

5.2.2.7	Number of emergency registration requests	68

5.2.2.8	Number of successful emergency registrations	69

5.2.2.9	Mean time of Registration procedure	69

5.2.2.10	Max time of Registration procedure	70

5.2.3	Service Request procedure related measurements	70

5.2.3.1	Number of attempted network initiated service requests	70

5.2.3.2	Number of successful network initiated service requests	70

5.2.3.3	Total number of attempted service requests (including both network initiated and UE initiated)	71

5.2.3.4	Total number of successful service requests (including both network initiated and UE initiated)	71

5.2.4	Measurements related to registration via untrusted non-3GPP access	71

5.2.4.1	Number of initial registration requests via untrusted non-3GPP access	71

5.2.4.2	Number of successful initial registrations via untrusted non-3GPP access	72

5.2.4.3	Number of mobility registration update requests via untrusted non-3GPP access	72

5.2.4.4	Number of successful mobility registration updates via untrusted non-3GPP access	73

5.2.4.5	Number of periodic registration update requests via untrusted non-3GPP access	73

5.2.4.6	Number of successful periodic registration updates via untrusted non-3GPP access	73

5.2.4.7	Number of emergency registration requests via untrusted non-3GPP access	74

5.2.4.8	Number of successful emergency registrations via untrusted non-3GPP access	74

5.2.5	Mobility related measurements	74

5.2.5.1	Inter-AMF handovers	74

5.2.5.1.1	Number of PDU sessions requested for inter-AMF incoming handovers	74

5.2.5.1.2	Number of PDU sessions failed to setup for inter-AMF incoming handovers	75

5.2.5.1.3	Number of QoS flows requested for inter-AMF incoming handovers	75

5.2.5.1.4	Number of QoS flows failed to setup for inter-AMF incoming handovers	76

5.2.5.2	Measurements for 5G paging	76

5.2.5.2.1	 Number of 5G paging procedures	76

5.2.5.2.2	Number of successful 5G paging procedures	76

5.2.6	Measurements related to Service Requests via Untrusted non-3GPP Access	77

5.2.6.1	Number of attempted service requests via Untrusted non-3GPP Access	77

5.2.6.2	Number of successful service requests via Untrusted non-3GPP Access	77

5.2.7	Measurements related to SMS over NAS	77

5.2.7.1	Registration of SMS over NAS	77

5.2.7.1.1	Number of registration requests for SMS over NAS via 3GPP access	77

5.2.7.1.2	Number of successful registrations allowed for SMS over NAS via 3GPP access	78

5.2.7.1.3	Number of registration requests for SMS over NAS via non-3GPP access	78

5.2.7.1.4	Number of successful registrations allowed for SMS over NAS via non-3GPP access	78

5.2.7.2	MO SMS over NAS	79

5.2.7.2.1	Number of attempted MO SMS messages over NAS via 3GPP access	79

5.2.7.2.2	Number of MO SMS messages successfully transported over NAS via 3GPP access	79

5.2.7.2.3	Number of attempted MO SMS messages over NAS via non-3GPP access	79

5.2.7.2.4	Number of MO SMS messages successfully transported over NAS via non-3GPP access	80

5.2.7.3	MT SMS over NAS	80

5.2.7.3.1	Number of attempted MT SMS messages over NAS via 3GPP access	80

5.2.7.3.2	Number of MT SMS messages successfully transported over NAS via 3GPP access	80

5.2.7.3.3	Number of attempted MT SMS messages over NAS via non-3GPP access	81

5.2.7.3.4	Number of MT SMS messages successfully transported over NAS via non-3GPP access	81

5.2.8	UE Configuration Update procedure related measurements	81

5.2.8.1	Number of UE Configuration Update	81

5.2.8.2	Number of successful UE Configuration Update	82

5.3	Performance measurements for SMF	82

5.3.1	Session Management	82

5.3.1.1	Number of PDU sessions (Mean)	82

5.3.1.2	Number of PDU sessions (Maximum)	82

5.3.1.3	Number of PDU session creation requests	83

5.3.1.4	Number of successful PDU session creations	83

5.3.1.5	Number of failed PDU session creations	84

5.3.1.6	PDU session modifications	84

5.3.1.6.1	Number of requested PDU session modifications (UE initiated)	84

5.3.1.6.2	Number of successful PDU session modifications (UE initiated)	84

5.3.1.6.3	Number of failed PDU session modifications (UE initiated)	85

5.3.1.6.4	Number of requested PDU session modifications (SMF initiated)	85

5.3.1.6.5	Number of successful PDU session modifications (SMF initiated)	85

5.3.1.6.6	Number of failed PDU session modifications (SMF initiated)	86

5.3.1.7	PDU session releases	86

5.3.1.7.1	Number of released PDU sessions (AMF initiated)	86

5.3.1.8	Number of PDU session creation requests in HR roaming scenario	87

5.3.1.9	Number of successful PDU session creations in HR roaming scenario	87

5.3.1.10	Number of failed PDU session creations in HR roaming scenario	87

5.3.1.11	Mean time of PDU session establishment	88

5.3.1.12	Max time of PDU session establishment	88

5.3.2	QoS flow monitoring	89

5.3.2.1	QoS flow monitoring	89

5.3.2.1.1	Number of QoS flows requested to create	89

5.3.2.1.2	Number of QoS flows successfully created	89

5.3.2.1.3	Number of QoS flows failed to create	90

5.3.2.1.4	Number of QoS flows requested to modify	90

5.3.2.1.5	Number of QoS flows successfully modified	90

5.3.2.1.6	Number of QoS flows failed to modify	91

5.3.2.1.7	Mean number of QoS flows	91

5.3.2.1.8	Peak number of QoS flows	91

5.3.3	Performance measurement for N4 interface	92

5.3.3.1	Number of N4 session modifications	92

5.3.3.2	Number of failed N4 session modifications	92

5.3.3.3	Number of N4 session deletions	92

5.3.3.4	Number of failed N4 session deletions	93

5.4	Performance measurements for UPF	93

5.4.1	N3 interface related measurements	93

5.4.1.1	Number of incoming GTP data packets on the N3 interface, from (R)AN to UPF	93

5.4.1.2	Number of outgoing GTP data packets of on the N3 interface, from UPF to (R)AN	94

5.4.1.3	Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF	94

5.4.1.4	Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN	94

5.4.1.5	Data volume of incoming GTP data packets per QoS level on the N3 interface, from (R)AN to UPF	95

5.4.1.6	Data volume of outgoing GTP data packets per QoS level on the N3 interface, from UPF to (R)AN	95

5.4.1.7	Incoming GTP Data Packet Loss	95

5.4.1.8	Outgoing GTP Data Packet Loss	96

5.4.1.9	Round-trip GTP Data Packet Delay	96

5.4.1.9.1	Average round-trip N3 delay on PSA UPF	96

5.4.1.9.2	Distribution of round-trip N3 delay on PSA UPF	96

5.4.1.9.3	Average round-trip N3 delay on I-UPF	97

5.4.1.9.4	Distribution of round-trip N3 delay on I-UPF	97

5.4.2	N6 related measurements	97

5.4.2.1	N6 incoming link usage	97

5.4.2.2	N6 outgoing link usage	98

5.4.3	N4 interface related measurements	98

5.4.3.1	Session establishments	98

5.4.3.1.1	Number of requested N4 session establishments	98

5.4.3.1.2	Number of failed N4 session establishments	99

5.4.3.2	N4 Session reports	99

5.4.3.2.1	Number of requested N4 session reports	99

5.4.3.2.2	Number of successful N4 session reports	99

5.4.4	N9 interface related measurements	100

5.4.4.1	Round-trip GTP Data Packet Delay on N9 interface	100

5.4.4.1.1	Average round-trip N9 delay on PSA UPF	100

5.4.4.1.2	Distribution of round-trip N9 delay on PSA UPF	100

5.4.4.1.3	Average round-trip N9 delay on I-UPF	100

5.4.4.1.4	Distribution of round-trip N9 delay on I-UPF	101

5.4.5	GTP packets delay in UPF	101

5.4.5.1	DL GTP packets delay in UPF	101

5.4.5.1.1	Average DL GTP packets delay in PSA UPF	101

5.4.5.1.2	Distribution of DL GTP packets delay in PSA UPF	102

5.4.5.1.3	Average DL GTP packets delay in I-UPF	102

5.4.5.1.4	Distribution of DL GTP packets delay in I-UPF	102

5.4.5.2	UL GTP packets delay in UPF	103

5.4.5.2.1	Average UL GTP packets delay in PSA UPF	103

5.4.5.2.2	Distribution of UL GTP packets delay in PSA UPF	103

5.4.5.2.3	Average UL GTP packets delay in I-UPF	104

5.4.5.2.4	Distribution of UL GTP packets delay in I-UPF	104

5.4.6	Round-trip packet delay between PSA UPF and UE	104

5.4.6.1	Average round-trip packet delay between PSA UPF and UE	104

5.4.6.2	Distribution of round-trip packet delay between PSA UPF and UE	105

5.5	Performance measurements for PCF	106

5.5.1	AM policy association related measurements	106

5.5.1.1	Number of AM policy association requests	106

5.5.1.2	Number of successful AM policy associations	106

5.5.2	SM policy association related measurements	107

5.5.2.1	Number of SM policy association requests	107

5.5.2.2	Number of successful SM policy associations	107

5.6	Performance measurements for UDM	107

5.6.1	Mean number of registered subscribers through UDM	107

5.6.2	Maximum number of registered subscribers through UDM	108

5.7	Common performance measurements for NFs	108

5.7.1	VR usage of NF	108

5.7.1.1	Virtual CPU usage	108

5.7.1.1.1	Mean virtual CPU usage	108

5.7.1.2	Virtual memory usage	109

5.7.1.2.1	Mean virtual memory usage	109

5.7.1.3	Virtual disk usage	109

5.7.1.3.1	Mean virtual disk usage	109

5.8	Performance measurements for N3IWF	110

5.8.1	PDU Session Resource management	110

5.8.1.1	PDU Session Resource setup	110

5.8.1.1.1	Number of PDU Sessions requested to setup	110

5.8.1.1.2	Number of PDU Sessions successfully setup	111

5.8.1.1.3	Number of PDU Sessions failed to setup	111

5.8.1.2	PDU Session Resource modification	111

5.8.1.2.1	Number of PDU Sessions requested to modify	111

5.8.1.2.2	Number of PDU Sessions successfully modified	112

5.8.1.2.3	Number of PDU Sessions failed to modify	112

5.9	Performance measurements for NEF	113

5.9.1	Measurements related to application triggering	113

5.9.1.1	Number of application trigger requests	113

5.9.1.2	Number of application trigger requests accepted for delivery	113

5.9.1.3	Number of application trigger requests rejected for delivery	113

5.9.1.4	Number of application trigger delivery reports	114

6	Measurements related to end-to-end 5G network and network slicing	114

6.1	Void	114

6.2	Virtualised resource usage measurement	114

Annex A (informative):	Use cases for performance measurements	116

A.1	Monitoring of UL and DL user plane latency in NG-RAN	116

A.2	Monitoring of UL and DL packet loss in NG-RAN	116

A.3	Monitoring of DL packet drop in NG-RAN	116

A.4	Monitoring of UL and DL user plane delay in NG-RAN	116

A.5	Monitoring of UE Context Release Request (gNB-DU initiated)	117

A.6	Monitoring of physical radio resource utilization	117

A.7	Monitoring of RRC connection number	117

A.8	Monitoring of UE Context Release	117

A.9	Monitoring of UE Throughput in NG-RAN	118

A.10	Monitoring of Unrestricted volume in NG-RAN	118

A.11	N3 data volume related measurements	118

A.12	N6 related measurements	118

A.13	Registration related measurements	118

A.14	PDU session establishment related measurements	119

A.15	Policy association related measurements	119

A.16	Monitoring of PDU session resource setup in NG-RAN	119

A.17	Monitoring of handovers	120

A.18	Monitor of BLER performance	120

A.19	Monitor of ARQ and HARQ performance	120

A.20	Monitoring of PDU session modifications	120

A.21	Monitoring of PDU session releases	121

A.22	Monitoring of N4 session management	121

A.23	Use case of VR measurements for NF	121

A.24	Monitoring of DRB Setup in NG-RAN	121

A.25	Monitoring of PDCP data volume measurements	121

A.26	Monitoring of RF performance	122

A.27	Monitoring of RF measurements	122

A.28	Monitor of QoS flow release	122

A.29	Monitor of call (/session) setup performance	123

A.30	Monitor of QoS flow release	123

A.31	Monitoring of QoS flows for SMF	123

A.32	Monitoring of service requests	123

A.33	Monitoring of DL PDCP UE buffered throughput	124

A.34	Monitoring of RRC connection setup in NG-RAN	124

A.35	Monitoring of UE associated NG signalling connection setup in NG-RAN	124

A.36	Monitoring of PDCP data volume per interface	124

A.37	Monitoring of RRC connection re-establishment	125

A.38	Monitoring of RRC connection resuming	125

A.39	Monitoring of inter-AMF handovers	125

A.40	Monitoring of incoming/outgoing GTP packet loss on N3	125

A.41	Monitoring of round-trip GTP packet delay on N3	125

A.42	Monitoring of PDU session resource management for untrusted non-3GPP access	125

A.43	Monitor of DRB release	126

A.44	Monitoring of application triggering	126

A.45	Monitoring of SMS over NAS	127

A.46	Monitoring of round-trip GTP packet delay on N9	127

A.47	Monitoring of GTP packets delay in UPF	127

A.48	Monitoring of round-trip delay between PSA UPF and UE	127

A.49	Monitoring of Power, Energy and Environmental (PEE) parameters	128

A.50	Monitoring of UE configuration update	128

Annex B (informative):	Change history	129

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document specifies the performance measurements for 5G networks including network slicing. Performance measurements for NG-RAN (clause 5.1) as well as for 5GC (clause 5.2 to 5.6) are defined in this document. Related KPIs are defined to those measurements in TS 28.554 [8].

The performance measurements for NG-RAN applies also to NR option 3 in many cases, but not to the RRC connection related measurements which are handled by E-UTRAN for NR option 3 (those are measured according to TS 32.425 [9] and related KPIs in TS 32.451 [10]).

The performance measurements are defined based on the measurement template as described in TS 32.404 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Definitions, abbreviations and measurement family

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IP Latency:  the time it takes to transfer a first/initial packet in a data burst from one point to another.

Mapped 5QI:   5QI that is used for a DRB within the gNB when a single 5QI is assigned to the DRB.

NOTE1:	In this case the mapped 5QI is used for separating certain measurements per QoS class.

NOTE 2:	Individual QoS  flows into a common 5QI is specified in TS 38.473 [6].

Packet Delay: the time it takes to transfer any packet from one point to another.

Packet Drop Rate: share of packets that were not sent to the target due to high traffic load or traffic management and should be seen as a part of the packet loss rate.

Packet Loss Rate: share of packets that could not be received by the target,. including packets droped, packets lost in transmission and packets received in wrong format.

Performance Indicators: The performance data aggregated over a group of NFs which is derived from the performance measurements collected at the NFs that belong to the group, according to the aggregation method identified in the Performance Indicator definition.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.501 [4].

PI	Performance Indicator.

kbit	kilobit (1000 bits)

MN	Master Node.

NG-RAN	Next Generation Radio Access Network

NSI	Network Slice Instance

SN	Secondary Node.

## 3.4	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	AT (measurements related to Application Triggering).

-	SMS (measurements related to Short Message Service).

-	PEE (measurements related to Power, Energy and Environment).

# 4	Concepts and overview

## 4.1	Performance indicators

Performance Indicators are the performance data aggregated over a group of NFs, such as, for example, average latency along the NSI. The Performance Indicators can be derived from the performance measurements collected at the NFs that belong to the group. The aggregation method is identified in the Performance Indicator definition

Performance Indicators at the NSSI level can be derived from the performance measurements collected at the NFs that belong to the NSSI or to the constituent NSSIs. The Performance Indicators at the NSSI level can be made available via the corresponding performance management service for NSSI.

The Performance Indicators at the NSI level, can be derived from the NSSI level Performance Indicators collected at the constituent NSSIs and/or NFs. The NSI level Performance Indicators can be made available via the corresponding performance management service for NSI.

# 5	Performance measurements for 5G Network Functions

## 5.1	Performance measurements for gNB

### 5.1.1	Performance measurements valid for all gNB deployment scenarios

#### 5.1.1.1	Packet Delay

##### 5.1.1.1.1	Average delay DL air-interface

a)	This measurement provides the average (arithmetic mean) time it takes to get a reponse back on a HARQ transmission in the downlink direction. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or time when the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode, minus time when the last part of the same packet was transmitted over the aircorresponding RLC SDUs arriving at MAC lower SAP) divided by total number of RLC SDUs transmitted to UE successfully. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3) and for each S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one. If the optional QoS level subcounters and S-NSSAI subcounters are perfomed, the number of measurements is equal to the sum of number of mapped 5QIs and the number of S-NSSAIs.

e)	The measurement name has the form DRB.AirIfDelayDl, 
optionally DRB.AirIfDelayDl.QOS, where QOS identifies the target quality of service class, and
optionally DRB.AirIfDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.1.2	Distribution of delay DL air-interface

a)	This measurement provides the distribution of the time it takes to get a reponse back on a HARQ transmission in the downlink direction. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by 1) calculating the DL delay for an RLC SDU packet by: the time when the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information, minus the time when the last part of the same packet was transmitted over the air; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement.

d)	Each measurement is an integer representing the number of RLC SDU packets measured with the delay within the range of the bin.

e)	DRB.AirIfDelayDist.Bin.QOS, where QOS identifies the target quality of service class, and Bin indicates a delay range which is vendor specific;
DRB.AirIfDelayDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and Bin indicates a delay range which is vendor specific.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.1.2	Radio resource utilization

##### 5.1.1.2.1	DL Total PRB Usage

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose.

b)	SI

c)	This measurement is obtained as: M(T)=M1(T)P(T)∗100, where M(T)is the DL total PRB usage, which is percentage of PRBs used, averaged during time period T with value range: 0-100%; M1(T)is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; P(T)is total number of PRBs available for DL traffic transmission during time period T; and Tis the time period during which the measurement is performed.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotDl, which indicates the DL PRB Usage for all traffic

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.2	UL Total PRB Usage

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose.

b)	SI

c)	This measurement is obtained as: M(T)=M1(T)P(T)∗100, where M(T)is the UL total PRB usage, which is percentage of PRBs used, averaged during time period T with value range: 0-100%; M1(T)is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included; P(T)is total number of PRBs available for UL traffic transmission during time period T; and Tis the time period during which the measurement is performed

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotUl, which indicates the UL PRB Usage for all traffic

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.3	Distribution of DL Total PRB Usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the downlink in different ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the downlink may experience high load in certain short times (e.g. in a second) and recover to normal very quickly.

b)	CC

c)	Each measurement sample is obtained as: , where  is total PRB usage at sample n for DL, which is a percentage of PRBs used, averaged during time period tn with value range: 0-100%;  is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included;is the total number of PRBs available for DL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

d)	Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period T.

e)	Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is provided by the operator.

f)	A set of integers. Each representing the (integer) number of samples with a DL total PRB percentage usage in the range represented by that bin.

g)	RRU.PrbTotDlDist.BinX, which indicates the distribution of DL PRB Usage for all traffic.

h)	NRCellDU

i)	Valid for packet switched traffic

j)	5GS

k)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.4	Distribution of UL total PRB usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the uplink in different usage ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the uplink may experience high load in certain short times (e.g. in a second) and recover to normal very quickly.

b)	CC

c)	Each measurement sample is obtained as: , where  is total PRB usage at sample n for UL, which is a percentage of PRBs used, averaged during time period tn with value range: 0-100%;  is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included;is the total number of PRBs available for UL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period T.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is provided by the operator.

d)	A set of integers, each representing the (integer) number of samples with a UL PRB percentage usage in the range represented by that bin.

e)	RRU.PrbTotUlDist.BinX, which indicates the distribution of UL PRB Usage for all traffic.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.5	DL PRB used for data traffic

a)	This measurement provides the number of physical resource blocks (PRBs) in average used in downlink per S-NSSAI.

b)	SI.

c)	Each measurement is obtained as the averagenumber (arithmetic mean)  of all PRBs used for DL data traffic transmission per S-NSSAI during a time period T.

d)	Each measurement is a single integer value. The number of measurements are equal to the number of S-NSSAI(s).

e)	RRU.PrbUsedDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring t DL PRB he load of the radio physical layer per S-NSSAI.

##### 5.1.1.2.6	DL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in average available downlink.

b)	SI.

c)	The measurement is obtained as the average (arithmetic mean) of total availible  count of PRBs available for DL traffic transmission during time period T.

d)	One measurement, (average number of DL PRBs) is a single integer value. e)	RRU.PrbAvailDl.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average for DL traffic.

##### 5.1.1.2.7	UL PRB used for data traffic

a)	This measurement provides the number of physical resource blocks (PRBs) in average used in uplink per S-NSSAI.

b)	SI

c)	Each measurementis obtained as the average number (arithmetic mean) of all PRBs used for UL data traffic transmission per S-NSSAI during a time period T.

d)	Each measurement (number of PRBs) is a single integer value. The number of measurements are equal to the number of S-NSSAI(s).

e)	RRU.PrbUsedUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the UL PRB load of the radio physical layer perS-NSSAI .

##### 5.1.1.2.8	UL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) available uplink.

b)	SI.

c)	The measurement is obtained as the average number (arithmetic mean) of total available  count of PRBs available for UL traffic transmission during time period T.

d)	One measurement, (average of total number of UL PRBs) that is a single integer value.

e)	RRU.PrbAvailUl, which indicates the UL PRB available.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average UL

#### 5.1.1.3	UE throughput

##### 5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. It is optionally split into subcounters for each QoS level. 
IfUEsThpTimeDl>0,   UEsThpVolDlUEsThpTimeDlx1000 [kbitss]If UEsThpTimeDl=0,   0  [kbitss]

If , ×1000 [kbit/s]

If , 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,  ThpTimeDl=0, otherwise ThpTimeDl = T1−T2 ms

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
| T2 | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
| ThpVolDl | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEThpDl, or optionally DRB.UEThpDl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.3.2	Distribution of DL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers.

b)	CC

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample is obtained by the following formula for a measurement period tn:

IfThpTimeDl>0,   ThpVolDlThpTimeDlx1000 [kbitss]If ThpTimeDl=0,   0  [kbitss]If , ×1000 [kbit/s]

If , 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission, ThpTimeDl=0, otherwise ThpTimeDl = T1−T2 ms

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
| T2 | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
| ThpVolDl | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time ( may be counted and obtained by the formula:

ThpTimeDl = slot×TBVol−PaddingVolTBVol ms

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample, the bin corresponding to the DL throughput experienced by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a DL UE throughput in the range represented by that bin.

e)	The measurement name has the form 
DRB.UEThpDlDist.BinX where BinX represents the bin.

NOTE: Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

B)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. It is optionally split into subcounters for each QoS level. 
IfUEsThpTimeUl>0,   UEsThpVolUlUEsThpTimeUlx1000 [kbitss]If UEsThpTimeUl=0,   0  [kbitss]

If , ×1000 [kbit/s]

If , 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission ThpTimeUl=0otherwise:

ThpTimeUl = T1−T2 ms

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
| T2 | The point in time when transmission is started for the first data in data burst for a particular DRB. |
| ThpVolUl | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEThpUl, or optionally DRB.UEThpUl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.3.4	Distribution of UL UE throughput in gNB

a)	This measurement provides the distribution of the UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers.

b)	CC

c)	Considering there are n samples during measurement time T and each sample has the same time period tn, the measurement of one sample is obtained by the following formula for a measurement period tn: 
IfUEsThpTimeUl>0,   UEsThpVolUlUEsThpTimeUlx1000 [kbitss]If UEsThpTimeUl=0,   0  [kbitss]

If , ×1000 [kbit/s]

If , 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission ThpTimeUl=0otherwise:

ThpTimeUl = T1−T2 ms

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
| T1 | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
| T2 | The point in time when transmission is started for the first data in data burst for a particular DRB. |
| ThpVolUL | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Alternatively, for small data bursts, that are successfully transmitted in any given slot (i.e. the requirement that data bursts need to span across several slots excluding transmission of the last piece of the data in a data burst does not apply). where all buffered data is included in one initial HARQ transmission, fraction of the slot time ( may be counted and obtained by the formula:

ThpTimeUl = slot×TBVol−PaddingVolTBVol ms

| slot | Duration of the slot |
| --- | --- |
| TBVol | Volume of the TB related to one slot burst |
| PaddingVol | Volume of padding bits added into Transport Block related to one slot burst. |

For each measurement sample, the bin corresponding to the UL throughput experienced by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a UL UE throughput in the range represented by that bin.

e)	The measurement name has the form 
DRB.UEThpUlDist.BinX where BinX represents the bin.

NOTE: Number of bins and the range for each bin is left to implementation

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.3.5	Percentage of unrestricted DL UE data volume in gNB

a)	This measurement provides the percentage of DL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI.

c)	For periods when no data is transferred at all Percentage Unrestricted Volume DL = 0, otherwise:

| ThpUnresVolDl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolDl is the data volume counted on RLC SDU level in kbits sent in DL for one DRB. |
| --- | --- |
| ThpVolDl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume counted on RLC SDU level in kbits sent in DL for one DRB. |

d)	Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEUnresVolDl or optionally DRB.UEUnresVolDl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.3.6	Percentage of unrestricted UL UE data volume in gNB

a)	This measurement provides the percentage of UL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	For periods when no data is transferred at all Percentage Unrestricted Volume UL = 0, otherwise:

| ThpUnresVolUl | The volume of a data burst that is transmitted in the slot when the buffer is emptied (which could be the only slot needed to transmit the data burst) and not included in the UE throughput measurement. A sample for ThpUnresVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB. |
| --- | --- |
| ThpVolUl | The volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbits received in UL for one DRB. |

d)	Each measurement is a single integer value from 0 to 100. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form 
DRB.UEUnresVolUl or optionally DRB.UEUnresVolUl.QOS, where QOS identifies the target quality of service class.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.1.4	RRC connection number

##### 5.1.1.4.1	Mean number of RRC Connections

a)	This measurement provides the mean number of users in RRC connected mode during each granularity period.

b)	SI.

c)	This measurement is obtained by sampling at a pre-defined interval, the number of users in RRC connected mode for each NR cell and then taking the arithmetic mean.

d)	A single integer value.

e)	RRC.ConnMean

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the number of RRC connections in connected mode during the granularity period.

##### 5.1.1.4.2	Max number of RRC Connections

a)	This measurement provides the maximum number of users in RRC connected mode during each granularity period.

b)	SI.

c)	This measurement is obtained by sampling at a pre-defined interval, the number of users in RRC connected mode for each NR cell and then taking the maximum.

d)	A single integer value.

e)	RRC.ConnMax

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the number of RRC connections in connected mode during the granularity period.

#### 5.1.1.5	PDU Session Management

##### 5.1.1.5.1	Number of PDU Sessions requested to setup

a)	This measurement provides the number of PDU Sessions by the gNB. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	On receipt of PDU SESSION RESOURCE SETUP REQUEST message, INITIAL CONTEXT SETUP REQUEST message (see 3GPP TS 38.413 [11]) by the gNB from the AMF. Each PDU Session requested to setup increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupReq.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

##### 5.1.1.5.2	Number of PDU Sessions successfully setup

a)	This measurement provides the number of PDU Sessions successfully setup by the gNB from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	On transmission of PDU SESSION RESOURCE SETUP RESPONSE message, INITIAL CONTEXT SETUP RESPONSE message containing the "PDU Session Resource Setup Response List" IE (see 3GPP TS 38.413 [11]) by the gNB to the AMF. Each PDU Session listed in the "PDU Session Resource Setup Response List" IE increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupSucc.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

##### 5.1.1.5.3	Number of PDU Sessions failed to setup

a)	This measurement provides the number of PDU Sessions failed to setup by the gNB. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission of PDU SESSION RESOURCE SETUP RESPONSE message, INITIAL CONTEXT SETUP FAILURE message containing the "PDU Session Resource Failed to Setup List" IE (see 3GPP TS 38.413 [11]) by the gNB to the AMF. Each PDU Session listed in the "PDU Session Resource Failed to Setup List" IE increments the relevant subcounter per failure cause (see clause 9.3.1.2 of 3GPP TS 38.413 [11]) by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupFail.Cause.

Where Cause identifies the cause of the PDU Sessions Resource Setup failure, per the "PDU Session Resource Setup Unsuccessful Transfer" IE. Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11].

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

#### 5.1.1.6	Mobility Management

##### 5.1.1.6.1	Inter-gNB handovers

###### 5.1.1.6.1.1	Number of requested handover preparations

a)	This measurement provides the number of handover preparations requested by the source gNB.

b)	CC.

c)	On transmission of HANDOVER REQUIRED message (see 3GPP TS 38.413 [11]) by the NR cell CUto the AMF, or transmission of HANDOVER REQUEST message (see 3GPP TS 38.423 [13]) by the source NR cell CUto target NR cell CU, for requesting the preparation of resources at the target NR cell CU.

d)	A single integer value.

e)	MM.HoPrepInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.2	Number of successful handover preparations

a)	This measurement provides the number of successful handover preparations received by the source NR cell CU.

b)	CC

c)	On receipt of HANDOVER COMMAND message by the NR cell CUfrom the AMF (see 3GPP TS 38.413 [11]), or receipt of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.423 [13]) by the source NR cell CUfrom the target NR cell CU, for informing that the resources for the handover have been prepared at the target NR cell CU.

d)	A single integer value.

e)	MM.HoPrepInterSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.3	Number of failed handover preparations

a)	This measurement provides the number of failed handover preparations received by the source NR cell CU. This measurement is split into subcounters per failure cause.

b)	CC

c)	On receipt of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.413 [11]) by the NR cell CUfrom the AMF, or receipt of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.423 [13]) by the source NR cell CUfrom the target NR cell CU, for informing that the preparation of resources at the target NR cell CUhas failed. Each received HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoPrepInterFail.cause.

Where cause identifies the failure cause of the handover preparations.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.4	Number of requested handover resource allocations

a)	This measurement provides the number of handover resource allocation requests received by the target NR cell CU.

b)	1CC

c)	On receipt of HANDOVER REQUEST message (see 3GPP TS 38.413 [1]) by the NR cell CUfrom the AMF, or receipt of HANDOVER REQUEST message (see 3GPP TS 38.423 [13]) by the target NR cell CUfrom the source NR cell CU, for requesting the preparation of resources for handover.

d)	A single integer value.

e)	MM.HoResAlloInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.5	Number of successful handover resource allocations

a)	This measurement provides the number of successful handover resource allocations at the target NR cell CUfor the handover.

b)	CC.

c)	On transmission of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.413 [11]) by the NR cell CUto the AMF, or transmission of HANDOVER REQUEST ACKNOWLEDGE message (see 3GPP TS 38.423 [13]) by the target NR cell CUto the source NR cell CU, for informing that the resources for the handover have been prepared.

d)	A single integer value.

e)	MM.HoResAlloInterSucc

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.6	Number of failed handover resource allocations

a)	This measurement provides the number of failed handover resource allocations at the target NR cell CUfor the handover. This measurement is split into subcounters per failure cause.

b)	CC

c)	On transmission of HANDOVER FAILURE message (see 3GPP TS 38.413 [11]) by the NR cell CUto the AMF, or transmission of HANDOVER PREPARATION FAILURE message (see 3GPP TS 38.423 [13]) by the target NR cell CUto the source NR cell CU, for informing that the preparation of resources has failed. Each transmitted HANDOVER FAILURE message or HANDOVER PREPARATION FAILURE message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoResAlloInterFail.cause

Where cause identifies the failure cause of the handover resource allocations.

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

###### 5.1.1.6.1.7	Number of requested handover executions

a)	This inter gNB handover measurement provides the number of outgoing handover executions requested by the source gNB.

b)	CC.

c)	On transmission of RRC ConnectionReconfiguration message to the UE triggering the inter gNB handover from the source NRCellCU to the target NRCellCU, indicating the attempt of an outgoing inter-gNB handover (see TS 38.331 [20]), the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeInterReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.1.8	Number of successful handover executions

a)	This inter gNB handover measurement provides the number of successful handover executions received by the source gNB.

b)	CC

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB following a successful handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF following a successful inter gNB handover, the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeInterSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.1.9	Number of failed handover executions

a)	This inter gNB handover measurement provides the number of failed handover executions received by the source gNB. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On receipt at the source gNB of UE CONTEXT RELEASE [13] over Xn from the target gNB indicating an unsuccessful inter gNB handover, or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND [11] from AMF indicating an unsuccessful inter gNB handover.

The failure causes are listed for the UE CONTEXT RELEASE in [13] and for UE CONTEXT RELEASE COMMAND in [11]. Each received message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoExeInterFail.cause.

Where cause identifies the failure cause of the UE CONTEXT RELEASE or UE CONTEXT RELEASE COMMAND message.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.1.10	Mean Time of requested handover executions

a)	This measurement provides the mean time of Inter-gNB handover executions during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by accumulating the time interval for every successful Inter-gNB handover executions procedure per S-NSSAI between the receipt by the Source NG-RAN from the Target NG-RAN of a " Release Resource" and the sending of a " N2 Path Switch Request " message from Source NG-RAN to the Target NG-RAN over a granularity period using DER. The end value of this time will then be divided by the number of Inter-gNB handovers observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value.(in milliseconds)

e)	MM.HoExeInterReq.TimeMean.SNSSAI

f)	NRCellCU.

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of Inter-gNB handovers during the granularity period.

##### 5.1.1.6.1.11	Max Time of requested handover executions

a)	This measurement provides the max time of Inter-gNB handover executions during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by measuring the time interval for every successful Inter-gNB handover executions procedure per S-NSSAI between the receipt by the Source NG-RAN from the Target NG-RAN of a “Release Resource" and the sending of a " N2 Path Switch Request " message from Source NG-RAN to the Target NG-RAN over a granularity period using DER. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value.(in milliseconds)

e)	MM.HoExeInterReq.TimeMax.SNSSAI

f)	NRCellCU.

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of Inter-gNB handovers during the granularity period.

##### 5.1.1.6.2	Intra-gNB handovers

###### 5.1.1.6.2.1	Number of requested handover executions

a)	This measurement provides the number of outgoing intra-gNB handover executions requested by the source NRCellCU.

b)	CC.

c)	On transmission of RRC ConnectionReconfiguration message to the UE triggering the handover from the source NRCellCU to the target NRCellCU, indicating the attempt of an outgoing intra-gNB handover (see 3GPP TS 38.331 [20]), the counter is steped by 1.

d)	A single integer value.

e)	MM.HoExeIntraReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.1.6.2.2	Number of successful handover executions

a)	This measurement provides the number of successful intra-gNB handover executions received by the source NRCellCU.

b)	CC.

c)	On reception of RRC ConnectionReconfigurationComplete message from the UE to the target NRCellCU indicating a successful intra-gNB handover (see 3GPP TS 38.331 [20]), the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeIntraSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

#### 5.1.1.7	TB related Measurements

##### 5.1.1.7.1	Total number of DL initial TBs

a)	This measurement provides the total number of initial TBs transmitted on the downlink in a cell. HARQ re-transmissions are excluded from this measurement.This measurement is optionally split into subcounters per modulation schema.

b)	 CC.

c)	 On transmission by the gNB of TB to UE during the period of measurement.This measurement is optionally split into subcounters per modulation schema.

d)	 A single integer value.

e)	 The measurement name has the form TB.TotNbrDlInitial, TB.TotNbrDlInitial.Qpsk, TB.TotNbrDlInitial.16Qam.

TB.TotNbrDlInitial.64Qam, TB.TotNbrDlInitial.256Qam.

f)	NRCellDU.

g)	Valid for packet switched traffic .

h)	5GS.

##### 5.1.1.7.2	Intial error number of DL TBs

a) This measurement provides the number of initial faulty TBs transmitted on the downlink in a cell.This measurement is optionally split into subcounters per modulation schema.

b) CC.

c) On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE at first HARQ feedback during the period of measurement. This measurement is optionally split into subcounters per modulation schema.

d) A single integer value.

e) The measurement name has the form TB.IntialErrNbrDl, TB.IntialErrNbrDl.Qpsk, TB.IntialErrNbrDl.16Qam

TB.IntialErrNbrDl.64Qam, TB.IntialErrNbrDl.256Qam.

f)	NRCellDU.

g)	Valid for packet switched traffic .

h)	5GS.

##### 5.1.1.7.3	Total number of DL TBs

a) This measurement provides the total number of TBs transmitted on the downlink in a cell.The measurement is split into subcounters per layer at MU-MIMO case. This measurement includes all transmitted TBs (including the successful and failed TBs during initial transmission and HARQ re-transmission).

b) CC.

c)  On transmission by the gNB of TB to UE during the period of measurement.The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. .

d)	 Each measurement is an integer.

e)	 TB.TotNbrDl.X

Where X identified by DL MU-MIMO maximum layer.

f)	NRCellDU.

g)	Valid for packet switched traffic .

h)	5GS.

##### 5.1.1.7.4	Total error number of DL TBs

a)	 This measurement provides the number of total faulty TBs transmitted on the downlink in a cell .The measurement is split into subcounters per layer at MU-MIMO case.This measurement include all transmitted  faulty TBs  of initial transmission and re-transmission .

b)	 CC.

c)	 On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE during the period of measurement. The measurement is split into subcounters per Layer at MU-MIMO case.

d)	 Each measurement is an integer.

e)	 TB.ErrToltalNbrDl.X.

Where X identified by DL MU-MIMO maximum layer.

f)	NRCellDU.

g)	Valid for packet switched traffic .

h)	5GS.

##### 5.1.1.7.5	Residual error number of DL TBs

a)	 This measurement provides the number of final faulty TBs transmitted on the downlink in a cell at last HARQ re-transmissions.

b)	 CC.

c)	 On receipt by the gNB of a NACK or DTX from UE which indicates a faulty reception of TB by UE at the last HARQ feedback during the period of measurement.

d) A single integer value.

e)  TB.ResidualErrNbrDl.

f)	NRCellDU.

g)	Valid for packet switched traffic

h)	5GS.

##### 5.1.1.7.6	Total number of UL initial TBs

a) This measurement provides the total number of initial TBs on the uplink in a cell.This measurement is optionally split into subcounters per modulation schema.

b)  CC

c)  On receipt by the gNB of TB from UE during the period of measurement.This measurement is optionally split into subcounters per modulation schema.

d) A single integer value.

e)	 The measurement name has the form TB.TotNbrUlInit, TB.TotNbrUlInit.Qpsk, TB.TotNbrUlInit.16Qam,

f)	TB.TotNbrUlInit.64Qam, TB.TotNbrUlInit.256Qam.

g) NRCellDU.

h)	Valid for packet switched traffic .

i)	5GS.

##### 5.1.1.7.7	Error number of UL initial TBs

a)	 This measurement provides the number of initial faulty TBs on the uplink in a cell. This measurement is optionally split into subcounters per modulation schema.

b)	 CC

c)	 On receipt by the gNB of a initial TB on which CRC fails or DTX from UE during the period of measurement.This measurement is optionally split into subcounters per modulation schema.

d)	 A single integer value.

e) The measurement name has the form TB.ErrNbrUlInitial, TB.ErrNbrUlInitial.Qpsk, TB.ErrNbrUlInitial.16Qam

TB.ErrNbrUlInitial.64Qam, TB.ErrNbrUlInitial.256Qam.

f)	NRCellDU.

g)	Valid for packet switched traffic .

h)	5GS.

##### 5.1.1.7.8	Total number of UL TBs

a)	This measurement provides the total number of TBs on the uplink in a cell.The measurement is split into subcounters per layer at MU-MIMO case.This measurement includes all transmitted TBs (including the successful and failed TBs during initial transmission and  HARQ re-transmission).

b)	 CC

c)	 On receipt by the gNB of TB from UE during the period of measurement.The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value. The sum value identified by the .sum suffix.

d)	 Each measurement is an integer.

e)	 TB.TotNbrUl.X

Where X identified by UL MU-MIMO maximum layer.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.1.1.7.9	Total error number of UL TBs

a)	This measurement provides the number of total faulty TBs on the uplink in a cell.  The measurement is split into subcounters per layer at MU-MIMO case.This measurement include all transmitted faulty TBs of initial  and re-transmission .

b)	 CC

c)	 On receipt by the gNB of a TB on which CRC fails or DTX from UE during the period of measurement. The measurement is split into subcounters per Layer at MU-MIMO case.A single integer value.

d)	Each measurement is an integer.

e)	TB.ErrToltalNbrUl.X

Where X identified by UL MU-MIMO maximum layer.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.1.1.7.10	Residual error number of UL TBs

a)	This measurement provides the number of final faulty TBs on the uplink in a cell.

b)	 CC

c)	On receipt by the gNB of a TB on which CRC fails or DTX at last HARQ re-transmissions from UE during the period of measurement.

d)	A single integer value.

e)	TB.ResidualErrNbrUl .

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.1.1.8	Void

#### 5.1.1.9	Void

#### 5.1.1.10	DRB related measurements

##### 5.1.1.10.1	Number of DRBs attempted to setup

a)	This measurement provides the number of DRBs attempted to setup to support all requested QoS flows in the PDU sessions to be setup by the INITIAL CONTEXT SETUP REQUESTs, PDU SESSION RESOURCE SETUP REQUESTs and PDU SESSION RESOURCE MODIFY REQUEST message received by the gNB from AMF. This measurement is split into subcounters per mapped 5QI and per S-NSSAI.

b)	CC.

c)	On receipt of "PDU Session Resource Setup Request List" IE in a INITIAL CONTEXT SETUP REQUEST message, PDU SESSION RESOURCE SETUP REQUEST message (see 3GPP TS 38.413 [11]) or a by the PDU SESSION RESOURCE MODIFY REQUEST message to gNB from the AMF. Each DRB that is needed to setup in the transmitted RRCReconfiguration message increments the relevant subcounter per mapped 5QI by 1, and the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	DRB.EstabAtt.5QI, where 5QI identifies mapped 5QI and

DRB.EstabAtt.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

##### 5.1.1.10.2	Number of DRBs successfully setup

a)	This measurement provides the number of DRBs successfully setup to support all requested QoS flows in the PDU sessions to be setup by the INITIAL CONTEXT SETUP REQUESTs, PDU SESSION RESOURCE SETUP REQUESTs and PDU SESSION RESOURCE MODIFY REQUEST message received by the gNB from AMF. This measurement is split into subcounters per mapped 5QI and per S-NSSAI.

b)	CC.

c)	On transmission of INITIAL CONTEXT SETUP RESPONSE, PDU SESSION RESOURCE SETUP RESPONSE message containing the "PDU Session Resource Setup Response List" IE (see 3GPP TS 38.413 [11]) or by the PDU SESSION RESOURCE MODIFY REQUEST message from the gNB to the AMF.The counter increases by the number of DRBs that was successfully setup indicated by the  RRCReconfigurationComplete message from the UE, as the response to the transmitted RRCReconfiguration message that contains the DRBs to add (see 3GPP TS 38.331[20]). Each DRB that was successfully setup to the UE increments the relevant subcounter per mapped 5QI by 1, and the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	DRB.EstabSucc.5QI, where 5QI identifies mapped 5QI and

DRB.EstabSucc.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance.

##### 5.1.1.10.3	Number of released active DRBs

a)	This measurement provides the number of abnormally released DRBs that were active at the time of release. DRBs with bursty flow are seen as being active if any DRB data on a Data Radio Bearer (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are always seen as active DRBs in the context of this measurement. DRBs used in 3GPP option 3 shall not be covered in this measurement
The measurement is split into sub counters per mapped 5QI and per S-NSSAI.

b)	CC

c)	On

-	transmission by the NG-RAN of a PDU SESSION RESOURCE RELEASE RESPONSE message for the PDU release initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE RELEASE COMMAND message with "Cause" equal to "Normal Release" or "User inactivity", "Load balancing TAU required", "Release due to CN-detected mobility", "O&M intervention", or-

-	transmission by the NG-RAN of a PDU SESSION RESOURCE MODIFY RESPONSE message for the PDU modification initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE MODIFY REQUEST message with the "Cause" equal to "Normal Release", or

-	transmission by the NG-RAN of a UE CONTEXT RELEASE COMPLETE for the UE context release initiated by the NG-RAN with the exception of the corresponding UE CONTEXT RELEASE REQUEST message with the cause equal to "Normal Release" or "User inactivity", "Partial handover", "Successful handover", or

-	transmission by the NG-RAN of a UE CONTEXT RELEASE COMPLETE message for the UE context release initiated by the AMF with the exception of the corresponding UE CONTEXT RELEASE COMMAND message with "Cause" equal to "Normal Release", "Handover Cancelled" or a successful mobility activity (e.g., cause "Successful Handover", or "NG Intra system Handover triggered"), or

-	receipt by the NG-RAN of a PATH SWITCH REQUEST ACKNOWLEDGE or PATH SWITCH REQUEST FAILED message by which some or all DRBs in the corresponding PATH SWITCH REQUEST need to be released, or

-	transmission of a NG RESET ACKNOWLEDGE message to AMF; or

-	receipt of a NG RESET ACKNOWLEDGE message from AMF,

Any of the UL or DL DRBs release using the RRCReconfiguration message (see 3GPP TS 38.331[20]) sent to the UE, triggers the corresponding counter to increment by 1.

DRBs with bursty flow are considered active if any data (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are always seen as active DRBs in the context of this measurement. Each corresponding DRB to release is added to the relevant measurement per mapped 5QI and S-NSSAI. 

A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

d)	Each measurement is an integer value. The number of measurements is equal to the number of mapped 5QI levels plus the number of S-NSSAIs.

e)	The measurements name has the form DRB.RelActNbr.5QI, where 5QI identifies the mapped 5QI and DRB.RelActNbr.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	This measurement is to support the Retainability KPI "DRB Retainability" defined in TS 28.554 [8].

##### 5.1.1.10.4	In-session activity time for DRB

a)	This measurement provides the aggregated active session time for DRBs in a cell. The measurement is split into sub counters per mapped 5QI and per S-NSSAI. DRBs used in 3GPP option 3 shall not be covered in this measurement.

b)	CC

c)	Number of "in session" seconds aggregated for DRBs with a certain mapped 5QI level or for a certain S-NSSAI, where "in session" has the following definitions:

- 	DRBs with bursty flow is said to be "in session" if any data (UL or DL) has been transferred during the   last 100 ms for that DRB.

- 	DRBs with continuous flow are always seen as being "in session" in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

d)	Each measurement is an integer value. The number of measurements is equal to the number of mapped 5QI levels plus the number of S-NSSAIs.

e)	The measurement name has the form DRB.SessionTime.5QI, where 5QI identifies the mapped 5QI and DRB.SessionTime.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	This measurement is to support the Retainability KPI "DRB Retainability" defined in TS 28.554 [8].

#### 5.1.1.11	CQI related measurements

##### 5.1.1.11.1	Wideband CQI distribution

a)	This measurement provides the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell.

b)	CC.

c)	This measurement is obtained by incrementing the appropriate measurement bin when a wideband CQI value is reported by a UE in the cell. When spatial multiplexing is used, CQI for both rank indicator should be considered. When different CSI-ReportConfig is used, different 4-bit CQI tables defined in TS 38.214 [19] should be considered.

d)	Each measurement is a single integer value.

e)	CARR.WBCQIDist.BinX.BinY.BinZ, where X represents the index of the CQI value (0 to 15). Y represents the index of rank value (1 to 8), Z represents the index of table value (1 to 3).

f)	NRCellDU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.12	MCS related Measurements

##### 5.1.1.12.1	MCS Distribution in PDSCH

a)	This measurement provides the distribution of the MCS scheduled for PDSCH RB by NG-RAN.

b)	CC

c)	This measurement is obtained by incrementing the appropriate measurement bin with the number of the PDSCH RBs according to the MCS scheduled by NG-RAN. When spatial multiplexing is used, MCS for both rank indicator should be considered. Different MCS index tables for PDSCH should be considered when the configuration is different as defined in clause 5.1.3.1, TS 38.214 [19]. The RBs used for broadcast should be excluded.

d)	Each measurement is a single integer value.

e)	CARR.PDSCHMCSDist.BinX.BinY.BinZ, where X represents the index of rank value (1 to 8), Y represents the index of table value (1 to 3), and Z represents the index of the MCS value (0 to 31).

f)	NRCellDU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.12.2	MCS Distribution in PUSCH

a)	This measurement provides the distribution of the MCS scheduled for PUSCH RB by NG-RAN.

b)	CC.

c)	This measurement is obtained by incrementing the appropriate measurement bin with the number of the PUSCH RBs according to the MCS scheduled by NG-RAN. When spatial multiplexing is used, MCS for both rank indicator should be considered. Different MCS index tables for PUSCH with transform precoding and 64QAM should be considered when the configuration is different as defined in clause 6.1.4.1, TS 38.214 [19].

d)	Each measurement is a single integer value.

e)	CARR.PUSCHMCSDist.BinX.BinY.BinZ, , where X represents the index of rank value (1 to 8), Y represents the index of table value (1 to 2), and Z represents the index of the MCS value (0 to 31).

f)	NRCellDU.

g)	Valid for packet switching.

h)	5GS.

#### 5.1.1.13	QoS flow related measurements

##### 5.1.1.13.1	QoS flow release

5.1.1.13.1.1	Number of released active QoS flows

a)	This measurement provides the number of released QoS flows that were active at the time of release. QoS flows with bursty flow are seen as being active when there is user data in the queue in any of the directions. QoS flows with continuous flow are always seen as active QoS flows in the context of this measurement.
The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	CC.

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE RELEASE RESPONSE message for the PDU release initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE RELEASE COMMAND message with "Cause" equal to "Normal Release" or "User inactivity", "Load balancing TAU required", "Release due to CN-detected mobility", "O&M intervention", or on transmission by the PDU SESSION RESOURCE MODIFY RESPONSE message for the PDU modification initiated by the AMF with the exception of corresponding PDU SESSION RESOURCE MODIFY REQUEST message with the "Cause" equal to "Normal Release", or on transmission by the NG-RAN of UE CONTEXT RELEASE COMPLETE for the UE context release initiated by the NG-RAN with the exception of the corresponding UE CONTEXT RELEASE REQUEST message with the cause equal to "Normal Release" or "User inactivity", "Partial handover", "Successful handover", or on transmission by the NG-RAN of UE CONTEXT RELEASE COMPLETE message for the UE context release initiated by the AMF with the exception of the corresponding UE CONTEXT RELEASE COMMAND message with "Cause" equal to "Normal Release", "Handover Cancelled" or a successful mobility activity (e.g., cause "Successful Handover", or "NG Intra system Handover triggered"), or on receipt by the NG-RAN of a PATH SWITCH REQUEST ACKNOWLEDGE or PATH SWITCH REQUEST FAILED message by which some or all QoS flows in the corresponding PATH SWITCH REQUEST need to be released , or on transmission of a NG RESET ACKNOWLEDGE message to AMF; or on receipt of a NG RESET ACKNOWLEDGE message from AMF, if any of the UL or DL are considered active in 3GPP TS 38.413 [11].

QoS flows with bursty flow are considered active when there is still data transmission in the DL or UL. QoS flows with continuous flow are always seen as active QoS flows in the context of this measurement. Each corresponding QoS flows to release is added to the relevant measurement per QoS level (mapped 5QI or QCI in NR option 3), the possible 5QIs are described in TS 23.501 [4]. The sum of all supported per QoS flow measurements shall equal the total number of QoS flows attempted to release when the QoS flows is active according to the definition of bursty flow/continuous flow. In case only a subset of per QoS flows measurements is supported, a sum subcounter will be provided first. 

How to define for a particular 5QI if the QoS flow is of type bursty flow or continuous flow is outside the scope of this document.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS flows plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form QF.RelActNbr.QoS.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	This measurement is to support the Retainability KPI "QoS flow Retainability" defined in TS 28.554 [8].

###### 5.1.1.13.1.2	Number of QoS flows attempted to release

a)	This measurement provides the number of QoS flows attempted to release. The measurement is split into subcounters per QoS level and per S-NSSAI.

b)	CC.

c)	On receipt by the gNB of an PDU SESSION RESOURCE RELEASE COMMAND or PDU SESSION RESOURCE MODIFY REQUEST message, or on gNB send the message of UE CONTEXT RELEASE REQUEST or PDU SESSION RESOURCE NOTIFY to AMF, each requested QoS Flow release Item in the message is release to the relevant measurement per QoS level, the possible QoS levels are included in TS 38.413.  The sum of all supported per QoS level measurements shall equal the total number of Qos FlowS attempted to setup plus the number of S-NSSAI. In case only a subset of per QoS level measurements is supported, a sum subcounter will be provided first. Measurements are subcounters per 5QI and subcounters per S-NSSAI.

d)	A single integer value.

e)	The measurement name has the form:

QF.ReleaseAttNbr.5QI where 5QI identifies the 5QI and

QF.ReleaseAttNbr.SNSSAI identifies the S-NSSAI

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.1.1.13.2	QoS flow activity

5.1.1.13.2.1	In-session activity time for QoS flow

a)	This measurement provides the aggregated active session time for QoS flow in a cell. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	CC.

c)	Number of "in session" seconds aggregated for QoS flows with a certain QoS level. , where "in session" has the following definitions: 
-  QoS flows with bursty flow  is said to be "in session" for a UE if any QoS flow data (UL or DL) has been transferred during the last 100 ms for that 5QI 
- QoS flows with continuous flow are always seen as being "in session" in the context of this measurement, and the session  time is increased from the first data transmission on the QoS flow until 100 ms after the last data transmission on the QoS flow.

The sum of all supported per QoS flow measurements shall equal the total session seconds. In case only a subset of per QoS flow measurements is supported, a sum subcounter will be provided first. 

How to decide for a particular QoS level if the QoS flow is of type continuous flow is outside the scope of this document.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form QF.SessionTimeQoS.QoS.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	This measurement is to support the Retainability KPI "QoS flow Retainability" defined in TS 28.554 [8].

5.1.1.13.2.2	In-session activity time for UE

a)	This measurement provides the aggregated active session time for UEs in a cell.

b)	CC.

c)	Number of session seconds aggregated for UEs in a cell. 
For QoS flows with bursty flow, a  UE is said to be "in session" if any QoS flow data on a Data Radio Bearer (UL or DL) has been transferred during the last 100 ms. 
For QoS flows with continuous flow, the  QoS flows (and the UE) is always seen as being "in session" in the context of this measurement, and the session  time is increased from the first data transmission on the QoS flow until 100 ms after the last data transmission on the QoS flow.

d)	Each measurement is an integer value.

e)	The measurement name has the form QF.SessionTimeUE

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	This measurement is to support the Retainability KPI "QoS flow Retainability" defined in TS 28.554 [8].

##### 5.1.1.13.3	QoS flow setup

###### 5.1.1.13.3.1	Number of QoS flow attempted to setup

a)	This measurement provides the number of QoS flows attempted to setup. The measurement is split into subcounters per QoS level (5QI).

b)	CC.

c)	On receipt by the NG-RAN of a PDU SESSION RESOURCE SETUP REQUEST message, or receipt by the NG-RAN of a INITIAL CONTEXT SETUP REQUEST message, or receipt by the NG-RAN of a PDU SESSION RESOURCE MODIFY REQUEST message, each requested QoS flow in the message is added to the relevant measurement per QoS level (5QI) and per S-NSSAI, the possible 5QIs are included in TS 23.501 [4]. The sum of all supported per QoS level measurements shall equal the total number of QoS flows attempted to setup. In case only a subset of per QoS level measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels plus the number of S-NSSAIs, plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form.

QF. EstabAttNbr.5QI where 5QI identifies the 5QI and

QF. EstabAttNbr.SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

###### 5.1.1.13.3.2	Number of QoS flow successfully established

a)	This measurement provides the number of QoS flows successfully established. The measurement is split into subcounters per QoS level and per S-NSSAI.

b)	CC.

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE SETUP RESPONSE message, or transmission by the NG-RAN of a INITIAL CONTEXT SETUP RESPONSE message, or transmission by the NG-RAN of a PDU SESSION RESOURCE MODIFY RESPONSE message, each QoS flow successfully established is added to the relevant measurement per QoS level (5QI) and per S-NSSAI, the possible 5QIs are included in TS 23.501 [4]. The sum of all supported per QoS level measurements shall equal the total number of QoS flows successfully setup. In case only a subset of per QoS level measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form:

QF.EstabSuccNbr.5QI where 5QI identifies the 5QI and

QF. EstabSuccNbr.SNSSAI identifies the S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

###### 5.1.1.13.3.3	Number of QoS flow failed to setup

a)	This measurement provides the number of QoS flows failed to setup. The measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission by the NG-RAN of a PDU SESSION RESOURCE SETUP RESPONSE message, or transmission by the NG-RAN of a INITIAL CONTEXT SETUP RESPONSE message, or transmission by the NG-RAN of a PDU SESSION RESOURCE MODIFY RESPONSE message, each QoS flow failed to establish is added to the relevant measurement per cause, the possible causes are included in TS 38.413 [18]. The sum of all supported per cause measurements shall equal the total number of additional QoS flows failed to setup. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form QF. EstabFailNbr.Cause
where Cause identifies the cause resulting in the QoS flow setup failure.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.1.1.14	Void

##### 5.1.1.15	RRC connection establishment related measurements

##### 5.1.1.15.1	Attempted RRC connection establishments

a)	This measurement provides the number of RRC connection establishment attempts for each establishment cause.

b)	CC

c)	Receipt of an RRCSetupRequest message by the gNB from the UE. Each RRCSetupRequest message received is added to the relevant per establishment cause measurement. RRCSetupRequests that are received while a setup procedure is already ongoing for this UE are excluded. RRCSetupRequests that are received during AMF Overload action (see clause 9.3.1.105 in TS 38.413) are effectively to be excluded from the measurement. The possible establishmentCause are included in TS 38.331 [20] (clause 6.2.2). The sum of all supported per cause measurement values shall be equal the total number of RRCSetupRequest.

d)	Each measurement is an integer value. The number of measurements is equal to the number of establishment causes.

e)	RRC.ConnEstabAtt.Cause where Cause identifies the establishment cause.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance within accessibility area.

##### 5.1.1.15.2	Successful RRC connection establishments

a)	This measurement provides the number of successful RRC establishments for each establishment cause.

b)	CC

c)	Receipt by the gNB of an RRCSetupComplete message following a RRC connection setup request. Each RRCSetupComplete message received is added to the relevant per establishment cause measurement. The possible causes are included in TS 38.331 [20] (clause 6.2.2). The sum of all supported per cause measurements shall be equal the total number of RRCSetupComplete messages.

d)	Each measurement is an integer value. The number of measurements is equal to the number of establishment causes.

e)	RRC.ConnEstabSucc.Cause where Cause identifies the establishment cause.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance within accessibility area.

#### 5.1.1.16	UE-associated logical NG-connection related measurements

##### 5.1.1.16.1	Attempted UE-associated logical NG-connection establishment from gNB to AMF

a)	This measurement provides the number of attempted UE-associated logical NG-connection establishments from gNB to AMF, for each RRCSetupRequest establishment cause. The possible causes are included in TS 38.331 [20] (clause 6.2.2).

b)	CC.

c)	On transmission of an INITIAL UE MESSAGE by the gNodeB to the AMF (See 38.413 [11], clause 8.6.1), the relevant per RRCSetupRequest establishment cause measurement is incremented by 1.

d)	Each subcounter is an integer value. The number of measurements is equal to the number of establishment causes.

e)	UECNTX.ConnEstabAtt.Cause  where Cause identifies the establishment cause.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance within accessibility area.

##### 5.1.1.16.2	Successful UE-associated logical NG-connection establishment from gNB to AMF

a)	This measurement provides the number of successful UE-associated logical NG-connection establishments from gNB to AMF, for each RRCSetupRequest establishment cause. The possible causes are included in TS 38.331 [20] (clause 6.2.2).

b)	CC.

c)	On receipt by the gNB of first message from AMF which succeeds INITIAL UE MESSAGE message on an UE-associated logical NG-connection (See 36.413 11], clause 8.6.1), the relevant per RRCSetupRequest establishment cause measurement is incremented by 1.

d)	Each subcounter is an integer value. The number of measurements is equal to the number of establishment causes.

e)	UECNTX.ConnEstabSucc.Cause  where Cause identifies the establishment cause.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurements is for performance assurance within accessibility area.

#### 5.1.1.17	RRC Connection Re-establishment

##### 5.1.1.17.1	Number of RRC connection re-establishment attempts

a)	This measurement provides the number of RRC connection re-establishment attempts.

b)	CC.

c)	On Receipt of RRCReestablishmentRequest message from UE (see TS 38.331[20]).

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ReEstabAtt.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.17.2	Successful RRC connection re-establishment with UE context

a)	This measurement provides the successful number of RRC connection re-establishment when UE context can be retrieved.

b)	CC.

c)	On Transmission of a RRCReestablishmentComplete message to UE for RRC connection re-establishment (see TS 38.331[20]).

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ReEstabSuccWithUeContext.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.17.3	Successful RRC connection re-establishment without UE context

a)	This measurement provides the successful number of RRC connection re-establishment when UE context can not be retrieved.

b)	CC.

c)	On Transmission of a RRCSetupComplete message to UE for RRC connection re-establishment (see TS 38.331[20]).

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ReEstabSuccWithoutUeContext.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

#### 5.1.1.18	RRC Connection Resuming

##### 5.1.1.18.1	Number of RRC connection resuming attempts

a)	This measurement provides the number of RRC connection resuming attempts.

b)	CC.

c)	On Receipt of the RRCResumeRequest message or RRCResumeRequest1 from UE.Each RRCResumeRequest is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeAtt.cause

Where cause indicates the resume cause defined in clause 6.2.2 of TS 38.331 [20].

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.18.2	Successful RRC connection resuming

a)	This measurement provides the total successful number of RRC connection resuming.

b)	CC.

c)	On Receipt of a RRCResumeComplete message from UE for RRC connection resuming. Each successful RRC connection resuming is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeSucc.cause

Where cause indicates the resume cause defined in clause 6.2.2 of TS 38.331 [20].

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.18.3	Successful RRC connection resuming with fallback

a)	This measurement provides the successful number of RRC connection resuming by fallback to RRC connection establishment.

b)	CC.

c)	On Receipt of a RRCSetupComplete message from UE for RRC connection resuming by fallback to RRC connection establishment. Each successful RRC connection resuming is added to the relevant subcounter per resume cause.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeSuccByFallback.cause.

Where cause indicates the resume cause defined in clause 6.2.2 of TS 38.331 [20].

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.18.4	RRC connection resuming followed by network release

a)	This measurement provides the number of RRC connection resuming followed by network release.

b)	CC.

c)	On Transmission of a RRCRelease message to UE after RRC connection resuming request.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ResumeFollowedbyNetworkRelease.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.18.5	RRC connection resuming followed by network suspension

a)	This measurement provides the number of RRC connection resuming followed by network suspension.

b)	CC.

c)	On Transmission of a RRCRelease with suspension configuration message to UE after RRC connection resume request.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ResumeFollowedbySuspension.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

#### 5.1.1.19	Power, Energy and Environmental (PEE) measurements

##### 5.1.1.19.1	Applicability of measurements

The PEE related measurements defined here are valid for a 5G Physical Network Function (PNF). The NR NRM is defined in TS 28.541 [26].

##### 5.1.1.19.2	PNF Power Consumption

###### 5.1.1.19.2.1	Average Power

a)	This measurement provides the average power consumed over the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.AvgPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.119.2.2	Minimum Power

a)	This measurement provides the minimum power consumed during the measurement period

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.MinPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.19.2.3	Maximum Power

a)	This measurement provides the maximum power consumed during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in watts (W).

e)	The measurement name has the form PEE.MaxPower

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.19.3	PNF Energy consumption

a)	This measurement provides the energy consumed.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clauses 4.4.3.1, 4.4.3.4, Annex A.

d)	A real value in kilowatt-hours (kWh).

e)	The measurement name has the form PEE.Energy

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.19.4	PNF Temperature

###### 5.1.1.19.4.1	Average Temperature

a)	This measurement provides the average temperature over the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.AvgTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.19.4.2	Minimum Temperature

a)	This measurement provides the minimum temperature during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.MinTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

###### 5.1.1.19.4.3	Maximum Temperature

a)	This measurement provides the maximum temperature during the measurement period.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clause 4.4.3.4, Annex A.

d)	A real value in degrees Celsius (°C).

e)	The measurement name has the form PEE.MaxTemperature

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.19.5	PNF Voltage

a)	This measurement provides the voltage.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – Clauses 4.4.3.3, 4.4.3.4, Annex B.

d)	A real value in volts (V).

e)	The measurement name has the form PEE.Voltage.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.19.6	PNF Current

a)	This measurement provides the current.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – Clauses 4.4.3.3, 4.4.3.4, Annex B.

d)	A real value in amperes (A).

e)	The measurement name has the form PEE.Current.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.19.7	PNF Humidity

a)	This measurement provides the percentage of humidity during the measurement period

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [25] – clause 4.4.3.3, Annex B.

d)	An integer value from 0 to 100.

e)	The measurement name has the form PEE.Humidity.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

### 5.1.2	Performance measurements valid only for non-split gNB deployment scenario

#### 5.1.2.1	PDCP Data Volume

##### 5.1.2.1.1	DL PDCP SDU Data Volume Measurements

5.1.2.1.1.1	DL Cell PDCP SDU Data Volume

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeDL_Filter.

Where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS .

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

5.1.2.1.1.2	DL Cell PDCP SDU Data Volume on X2 Interface

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered on X2 interface in DC-scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits transferred in the downlink through X2 interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeX2DL_Filter.

Where filter is a combination of PLMN ID and QoS level.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level.

f)	NRCellCU.

g)	Valid for packet switched traffic..

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

5.1.2.1.1.3	DL Cell PDCP SDU Data Volume on Xn Interface

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered on Xn interface in DC-scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits transferred in the downlink through Xn interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeXnDL_Filter.
Where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

##### 5.1.2.1.2	UL PDCP SDU Data Volume Measurements

5.1.2.1.2.1	UL Cell PDCP SDU Data Volume

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits delivered from PDCP layer to higher layers. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeUL_Filter.
Where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

5.1.2.1.2.2	UL Cell PDCP SDU Data Volume on X2 Interface

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered on X2 interface in NSA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits transferred in the uplink through X2 interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeX2UL_Filter.

Where filter is a combination of PLMN ID and QoS level.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

5.1.2.1.2.3	UL Cell PDCP SDU Data Volume on Xn Interface

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered on Xn interface in SA scenarios. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits transferred in the uplink through Xn interface. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeXnUL_Filter.

Where filter is a combination of PLMN ID and QoS level and S-NSSAI.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU.

g)	Valid for packet switched traffic..

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

NRCellCU in non-split NG-RAN deployment scenarios represents NRCell.

### 5.1.3	Performance measurements valid for split gNB deployment scenario

#### 5.1.3.1	Packet Loss Rate

##### 5.1.3.1.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface.  Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI.

c)	This measurement is obtained as:  1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3).

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurements is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.PacketLossRateUl or optionally DRB.PacketLossRateUl.QOS where QOS identifies the target quality of service class.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

##### 5.1.3.1.2	UL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss on the F1-U interface.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	This measurement is obtained as:  1000000* Number of missing UL GTP sequence numbers (TS 29.281), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by gNB-DU to gNB-CU-UP until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for option 3).

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.F1UPacketLossRateUl or optionally DRB.F1UPacketLossRateUl.QOS where QOS identifies the target quality of service class.

f)	GNBCUUPFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.1.3	DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI

c)	This measurement is obtained as:  1000000* Number of missing DL GTP sequence numbers (TS 29.281), representing packets that are not delivered to lower layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by gNB-CU-UP to gNB-DU until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3).

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.F1UPacketLossRateDl or optionally DRB.F1UPacketLossRateDl.QOS where QOS identifies the target quality of service class.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.3.2	Packet Drop Rate

##### 5.1.3.2.1	DL PDCP SDU Drop rate in gNB-CU-UP

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

NOTE: this measurement may include packets that were supposed to be sent via the eUtran air interface if using NR split bearer option 3, 4 or 7.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of DL packets, for which no part has been transmitted over the F1-U or Xn-U or X2-U interface, of a data radio bearer, that are discarded in the PDCP layer, divided by Number of DL packets for data radio bearers that has entered PDCP upper SAP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3).

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.PdcpPacketDropRateDl or optionally DRB.PdcpPacketDropRateDl.QOS 
where QOS identifies the target quality of service class.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

##### 5.1.3.2.2	DL Packet Drop Rate in gNB-DU

a)	This measurement provides the fraction of RLC SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-DU. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-DU without any part of it having been transmitted on the air interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

b)	SI.

c)	This measurement is obtained as: 1000000*Number of DL packets, for which no part has been transmitted over the air, of a data radio bearer, that are discarded in the gNB-DU divided by Number of DL packets for data radio bearers that were received from gNB-CU-UP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3).

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the measurements are equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.RlcPacketDropRateDl or optionally DRB.RlcPacketDropRateDl.QOS 
where QOS identifies the target quality of service class.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.3.3	Packet delay

##### 5.1.3.3.1	Average delay DL in CU-UP

a)	This measurement provides the average (arithmetic mean) PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when sending a PDCP SDU to the gNB-DU at the egress PDCP layer on F1-U/Xn-U, minus time of arrival of the same packet at NG-U ingress IP termination) divided by total number of PDCP SDUs arriving at NG-U ingress IP termination. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3) and for each S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one. If the optional QoS level subcounters and S-NSSAI subcounters are, the number of measurements is equal to sum of the number of mapped 5QIs and S-NSSAIs.

e)	The measurement name has the form DRB.PdcpSduDelayDl, 
optionally DRB.PdcpSduDelayDl.QOS where QOS identifies the target quality of service class, and
optionally DRB.PdcpSduDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI..

f)	GNBCUUPFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.3.2	Average delay DL on F1-U

a)	This measurement provides the average (arithmetic mean) GTP packet delay DL on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: the time when receiving a GTP packet delivery status message from the gNB-DU at the egress GTP termination, minus time when sending the same packet to gNB-DU at the GTP ingress termination, minus feedback delay time in gNB-DU, obtained result is divided by two. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3) and for each S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.PdcpF1DelayDl, 
optionally DRB.GtpF1DelayDl.QOS where QOS identifies the target quality of service, and
optionally DRB.GtpF1DelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.3.3	Average delay DL in gNB-DU

a)	This measurement provides the average (arithmetic mean) RLC SDU delay on the downlink within the gNB-DU, for initial transmission of all RLC packets. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when the last part of an RLC SDU was scheduled and sent to the MAC layer for transmission over the air, minus time of arrival of the same packet at the RLC ingress F1-U termination) divided by total number of RLC SDUs arriving at the RLC ingress F1-U termination. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3) and for each S-NSSAI. Each measurement is an integer representing the mean delay in microseconds.

d)	The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs.

e)	The measurement name has the form DRB.RlcSduDelayDl, 
optionally DRB.RlcSduDelayDl.QOS where QOS identifies the target quality of service class, and
optionally DRB.RlcSduDelayDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.3.4	Distribution of delay DL in CU-UP

a)	This measurement provides the distribution of PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by 1) calculating the DL delay within the gNB-CU-UP for a PDCP SDU packet by: the time when sending a PDCP SDU to the gNB-DU at the egress PDCP layer on F1-U/Xn-U, minus time of arrival of the same packet at NG-U ingress IP termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI.

d)	Each measurement is an integer representing the number of PDCP SDU packets measured with the delay within the range of the bin.

e)	DRB.PdcpSduDelayDlDist.Bin.QOS, where QOS identifies the target quality of service class, and Bin indicates a delay range which is vendor specific;
DRB.PdcpSduDelayDlDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and Bin indicates a delay range which is vendor specific.

f)	GNBCUUPFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.3.5	Distribution of delay DL on F1-U

a)	This measurement provides the distribution of GTP packet delay DL on the F1-U interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by 1) calculating the DL delay on F1-U for a GTP packet by: the time when receiving a GTP packet delivery status message from the gNB-DU at the egress GTP termination, minus time when sending the same packet to gNB-DU at the GTP ingress termination, minus feedback delay time in gNB-DU, obtained result is divided by two; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI.

d)	Each measurement is an integer representing the number of GTP packets measured with the delay within the range of the bin.

e)	DRB.GtpF1DelayDlDist.Bin.QOS where QOS identifies the target quality of service class, and Bin indicates a delay range which is vendor specific;
DRB.GtpF1DelayDlDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and Bin indicates a delay range which is vendor specific.

f)	GNBCUUPFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.3.6	Distribution of delay DL in gNB-DU

a)	This measurement provides the distribution of RLC SDU delay on the downlink within the gNB-DU, for initial transmission of all RLC packets. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by 1) calculating the delay on the downlink within the gNB-DU for a RLC SDU packet by: the time when the last part of an RLC SDU was scheduled and sent to the MAC layer for transmission over the air, minus time of arrival of the same packet at the RLC ingress F1-U termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement.

d)	Each measurement is an integer representing the number of RLC SDU packets measured with the delay within the range of the bin.

e)	DRB.RlcSduDelayDlDist.Bin.QOS, where QOS identifies the target quality of service class, and Bin indicates a delay range which is vendor specific;
DRB.RlcSduDelayDlDist.Bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and Bin indicates a delay range which is vendor specifics.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.3.4	IP Latency measurements

##### 5.1.3.4.1	General information

This subclause  defines the DL latency in gNB-DU. DL latency measurements for CU-UP and F1-U are not defined.

##### 5.1.3.4.2	Average IP Latency DL in gNB-DU

a)	This measurement provides the average IP Latency in DL (arithmetic mean) within the gNB-DU, when there is no other prior data to be transmitted to the same UE in the gNB-DU. The measurement is optionally split into subcounters per QoS level and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when the first piece of an RLC SDU transmitted on the air interface, minus time of arrival of the same packet at the RLC ingress F1-U termination, for IP packets arriving when there is no other prior data to be transmitted to the same UE in the gNB-DU) divided by total number of RLC SDUs arriving at the RLC ingress F1-U termination when there is no other prior data to be transmitted to the same UE in the gNB-DU. Separate counters are optionally maintained for each mapped 5QI (or QCI for option 3) and for each S-NSSAI.

d)	Each measurement is an integer representing the average latency in microseconds. The number of measurements is equal to one. If the optional QoS level subcounters and S-NSSAI subcounters are measurement is performed, the number of measurements is equal to the sum of number of supported mapped 5QIs and the number of S-NSSAIs.

e)	The measurement name has the form DRB.RlcSduLatencyDl, 
optionally DRB.RlcSduLatencyDl.QOS where QOS identifies the target quality of service class, and
optionally DRB.RlcSduLatencyDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.3.4.3	Distribution of IP Latency DL in gNB-DU

a)	This measurement provides the distribution of IP Latency in DL within the gNB-DU, when there is no other prior data to be transmitted to the same UE in the gNB-DU. The measurement is split into subcounters per QoS level and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is obtained by 1) calculating the latency on the downlink within the gNB-DU for a RLC SDU packet by: time when the first piece of an RLC SDU transmitted on the air interface, minus time of arrival of the same packet at the RLC ingress F1-U termination, for IP packets arriving when there is no other prior data to be transmitted to the same UE in the gNB-DU; and 2) incrementing the corresponding bin with the latency range where the result of 1) falls into by 1 for the subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcunters per S-NSSAI.

d)	Each measurement is an integer representing the number of RLC SDU packets measured with the latency within the range of the bin.

e)	DRB.RlcSduLatencyDlDist.bin.QOS, where QOS identifies the target quality of service class, and Bin indicates a latency range which is vendor specific;
DRB.RlcSduLatencyDlDist.bin.SNSSAI, where SNSSAI identifies the S-NSSAI, and Bin indicates a latency range which is vendor specifics.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

#### 5.1.3.5	UE Context Release

##### 5.1.3.5.1	UE Context Release Request (gNB-DU initiated)

a)	This measurement provides the number of UE CONTEXT Release initiated by gNB-DU for each release cause.

b)	SI

c)	Transmission of an UE CONTEXT RELEASE REQUEST message initiated by gNB-DU. Each release request is to be added to the relevant cause measurement. The possible causes are defined in 38.473 [6]. The sum of all supported per causes measurements shall equal the total number of UE CONTEXT Release initiated by gNB-DU. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form UECNTX.RelReq.Cause 
	where Cause identifies the release cause.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

##### 5.1.3.5.2	Number of UE Context Release Requests (gNB-CU initiated)

a)	This measurement provides the number of UE CONTEXT RELEASE initiated by gNB-CU for each release cause.

b) 	SI

c)	Transmission of an UE CONTEXT RELEASE  COMMAND message initiated by gNB-CU. Each release request is to be added to the relevant cause measurement. The possible causes are defined in 38.473 [6]. The sum of all supported per causes measurements shall equal the total number of UE CONTEXT Release initiated by gNB-CU. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

d)	Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form UECNTX.RelCmd.Cause where Cause identifies the release cause.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

#### 5.1.3.6	PDCP data volume measurements

##### 5.1.3.6.1	PDCP PDU data volume Measurement

5.1.3.6.1.1	DL PDCP PDU Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from GNB-CU to GNB-DU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

b)	CC.

c)	This measurement is obtained by counting the number of DL PDCP PDU bits sent to GNB-DU. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

The measurements of DL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpPduVolumeDL_Filter.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.6.1.2	UL PDCP PDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from GNB-DU to GNB-CU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). The unit is Mbit (1MBits=1000*1000 bits).

b)	CC

c)	This measurement is obtained by counting the number of bits entering the GNB-CU. The measurement is performed at the PDCP PDU level.  The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

The measurements of UL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUl_Filter.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

##### 5.1.3.6.2	PDCP SDU data volume Measurement

5.1.3.6.2.1	DL PDCP SDU Data Volume

This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeDl_Filter.

f)	 GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.6.2.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to SDAP layer or UPF. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3). 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits leaving the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUL_Filter.

f)   GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.6.2.3	DL PDCP SDU Data Volume per interface

- This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered from GNB-CU-UP to GNB-DU (F1-U interface), to external gNB-CU-UP (Xn-U interface) and to external eNB (X2-U interface). The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

b)	CC

c)	This measurement is obtained by counting the number of DL PDCP SDU bits sent to GNB-DU (F1-U interface), sent to external gNB-CU-UP (Xn-U interface) and sent to external eNB (X2-U interface). The measurement is performed in GNB-CU-UP per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per interface (F1-U, Xn-U, X2-U).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels per interface plus the number of S-NSSAIs per interface.

e)	The measurement names have the form:

- DRB.F1uPdcpSduVolumeDl.QoS and DRB.F1uPdcpSduVolumeDl.SNSSAI (F1-U interface measurements)

- DRB.XnuPdcpSduVolumeDl.QoS and DRB.XnuPdcpSduVolumeDl.SNSSAI (Xn-U interface measurements)

- DRB.X2uPdcpSduVolumeDl.QoS and DRB.X2uPdcpSduVolumeDl.SNSSAI (X2-U interface measurements)

where QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	EP_F1U (F1-U interface), EP_XnU (Xn-U interface), EP_X2U (X2-U interface).

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

#### 5.1.3.6.2.4	UL PDCP SDU Data Volume per interface

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered to GNB-CU-UP from GNB-DU (F1-U interface), from external gNB-CU-UP (Xn-U interface) and from external eNB (X2-U interface). The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

b)	CC.

c)	This measurement is obtained by counting the number of UL PDCP SDU bits entering the GNB-CU-UP from GNB-DU (F1-U interface), from external gNB-CU-UP (Xn-U interface) and from external eNB (X2-U interface). The measurement is performed in GNB-CU-UP per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI, and reported per Interface (F1-U, Xn-U, X2-U).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels per interface plus the number of S-NSSAIs per interface.

e)	The measurement names have the form:

- DRB.F1uPdcpSduVolumeUl.QoS and DRB.F1uPdcpSduVolumeUl.SNSSAI (F1-U interface measurements).

- DRB.XnuPdcpSduVolumeUl.QoS and DRB.XnuPdcpSduVolumeUl.SNSSAI (Xn-U interface measurements).

- DRB.X2uPdcpSduVolumeUl.QoS (X2-U interface measurements).

where QoS representes the mapped 5QI or the QCI level, and SNSSAI represents S-NSSAI.

f)	EP_F1U (F1-U interface), EP_XnU (Xn-U interface), EP_X2U (X2-U interface).

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality) and in the energy efficency (EE) area.

##### 5.1.3.7	Handovers measurements

##### 5.1.3.7.1	Intra-gNB handovers

###### 5.1.3.7.1.1	Number of requested handover preparations

a)	This measurement provides the number of outgoing intra-gNB handover preparations requested by the source NRCellCU for split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending UE CONTEXT MODIFY REQUEST message (see 3GPP TS 38.473 [6]) to gNB-DU to initiate an intra-gNB handover.

d)	A single integer value.

e)	MM.HoPrepIntraReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.2	Number of successful handover preparations

a)	This measurement provides the number of successful intra-gNB handover preparations received by the source NRCellCU, for split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and steped by 1 when gNB-CUCP receives UE CONTEXT MODIFY RESPONSE message (see 3GPP TS 38.473 [6]) from gNB-DU to initiate a successful intra-gNB handover.

d)	A single integer value.

e)	MM.HoPrepIntraSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

#### 5.1.3.8	Distribution of Normally Released Call (5QI 1 QoS Flow) Duration

a)	This measurement provides the histogram result of the samples related to normally released call (5QI 1 QoS Flow) duration collected during measurement period duration.

b)	CC

c)	Each sample is measured from the point in time the 5QI 1 QoS Flow has been successfully established via initial Context setup or additional 5QI 1 QoS Flow setup procedure or incoming handover till the point in time the 5QI 1 QoS Flow is released via gNB or AMF initiated release procedure or successful outgoing handover according to 3GPP TS 37.340 due to normal release cause. Triggering is done for the bin the given sample falls in.

d)	Each measurement is an integer value.

e)	The measurement name has the form 5QI1QoSflow.Rel.NormCallDurationBinX where X denotes the X-th bin from total number of N configured bins. X-th bin stands for the normal call duration which is within the range from tx-1 to tx.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	Each histogram function is represented by the configured number of bins with configured bin width by operator.

#### 5.1.3.9	Distribution of Abnormally Released Call (5QI 1 QoS Flow) Duration

a)	This measurement provides the histogram result of the samples related to abnormally released call (5QI 1 QoS Flow) duration collected during measurement period duration.

b)	CC

c)	Each sample is measured from the point in time the 5QI 1 QoS Flow has been successfully established via initial Context setup or additional 5QI 1 QoS Flow setup procedure or incoming handover till the point in time the 5QI 1 QoS Flow is released via gNB or AMF initiated release procedure according to 3GPP TS 37.340 due to abnormal release cause. Triggering is done for the bin the given sample falls in.

d)	Each measurement is an integer value.

e)	The measurement name has the form 5QI1QoSflow.Rel.AbnormCallDurationBinX where X denotes the X-th bin from total number of N configured bins. X-th bin stands for the abnormal call duration which is within the range from tx-1 to tx.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	Each histogram function is represented by the configured number of bins with configured bin width by operator.

## 5.2	Performance measurements for AMF

### 5.2.1	Registered subscribers measurement

#### 5.2.1.1	Mean number of registered subscribers

a)	This measurement provides the mean number of registered state subscribers per AMF

b)	GAUGE

c)	This measurement is obtained by sampling at a pre-defined interval the number of registered subscribers in an AMF and then taking the arithmetic mean. The measurement can be split into subcounters per NSI identifier (S-NSSAI).

d)	A single integer value

- e)	RM.RegisteredSubNbrMean.SNSSAI

Where SNSSAI identifies the NSI

f)	AMFFunction

g)	Valid for packet switching

h)	5GS

#### 5.2.1.2	Maximum number of registered subscribers

a)	This measurement provides the maximum number of registered state subscribers per AMF

b)	GAUGE

c)	 This measurement is obtained by sampling at a pre-defined interval the number of registered subscribers in an AMF and then taking the maximum. The measurement can be split into subcounters per NSI identifier (S-NSSAI).

d)	A single integer value

e)	RM.RegisteredSubNbrMax.SNSSAI

Where SNSSAI identifies the NSI

f)	AMFFunction

g)	Valid for packet switching

h)	5GS

### 5.2.2	Registration procedure related measurements

#### 5.2.2.1	Number of initial registration requests

a)	This measurement provides the number of initial registration requests received by the AMF.

b)	CC

c)	On receipt by the AMF from the UE of Registration Request with the registration type indicating an initial registration (see clause 4.2.2.2.2 of 3GPP TS 23.502 [7]). Each initial registration request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegInitReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.2	Number of successful initial registrations

a)	This measurement provides the number of successful initial registrations at the AMF.

b)	CC

c)	On transmission of Registration Accept by the AMF to the UE that sent the initial registration request (see 3GPP TS 23.502 [7]). Each accepted initial registration is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegInitSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.3	Number of mobility registration update requests

a)	This measurement provides the number of mobility registration update requests received by the AMF.

b)	CC

c)	On receipt by the AMF from the UE of Registration Request with the registration type indicating a Mobility Registration Update (see clause 4.2.2.2.2 of 3GPP TS 23.502 [7]). Each mobility registration update request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegMobReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.4	Number of successful mobility registration updates

a)	This measurement provides the number of successful mobility registration updates at the AMF.

b)	CC

c)	On transmission of Registration Accept by the AMF to the UE that sent the mobility registration update request (see 3GPP TS 23.502 [7]). Each accepted mobility registration update is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegMobSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.5	Number of periodic registration update requests

a)	This measurement provides the number of periodic registration update requests received by the AMF.

b)	CC

c)	On receipt by the AMF from the UE of Registration Request with the registration type indicating a Periodic Registration Update (see clause 4.2.2.2.2 of 3GPP TS 23.502 [7]). Each periodic registration update request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegPeriodReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.6	Number of successful periodic registration updates

a)	This measurement provides the number of successful mobility registration updates at the AMF.

b)	CC

c)	On transmission of Registration Accept by the AMF to the UE that sent the periodic registration update request (see 3GPP TS 23.502 [7]). Each accepted periodic registration update is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegPeriodSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.7	Number of emergency registration requests

a)	This measurement provides the number of emergency registration requests received by the AMF.

b)	CC

c)	On receipt by the AMF from the UE of Registration Request with the registration type indicating an Emergency Registration (see clause 4.2.2.2.2 of 3GPP TS 23.502 [7]). Each emergency registration request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegEmergReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.8	Number of successful emergency registrations

a)	This measurement provides the number of successful emergency registrations at the AMF.

b)	CC

c)	On transmission Registration Accept by the AMF to the UE that sent the emergency registration request (see 3GPP TS 23.502 [7]). Each accepted emergency registration is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	RM.RegEmergSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.2.9	Mean time of Registration procedure

a)	This measurement provides the mean time of registration procedure during each granularity period. The measurement is split into subcounters per S-NSSAI per registration type.

b)	DER(n=1)

c)	This measurement is obtained by accumulating the time interval for every successful registration procedure per S-NSSAI per registration type between the receipt by the AMF from the UE of a " REGISTRATION REQUEST " and the sending of a " REGISTRATION ACCEPT " message to the UE over a granularity period using DER. The end value of this time will then be divided by the number of successful registration procedures observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period. The measurement is split into subcounters per registration type, see TS 24.501 [24].

d)	Each measurement is an integer value.(in milliseconds)

e)	RM.RegTimeMean.SNSSAI. InitialReg
RM.RegTimeMean.SNSSAI. MobilityRegUpdate
RM.RegTimeMean.SNSSAI. PeriodicRegUpdate
RM.RegTimeMean.SNSSAI. EmergencyReg

Where SNSSAI identifies the S-NSSAI, InitialReg identifies the registration type "Initial Registration ", MobilityRegUpdate identifies the registration type "Mobility Registration Update", PeriodicRegUpdate identifies the registration type "Periodic Registration Update", EmergencyReg identifies the registration type "Emergency Registration".

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of registration procedure during the granularity period.

#### 5.2.2.10	Max time of Registration procedure

a)	This measurement provides the max time of registration procedure during each granularity period. The measurement is split into subcounters per S-NSSAI per registration type.

b)	DER(n=1)

c)	This measurement is obtained by monitoring the time interval for every successful registration procedure per S-NSSAI per registration type between the receipt by the AMF from the UE of a " REGISTRATION REQUEST " and the sending of a " REGISTRATION ACCEPT " message to the UE over a granularity period using DER. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period. The measurement is split into subcounters per registration type, see TS 24.501 [24].

d)	Each measurement is an integer value.(in milliseconds)

e)	RM.RegTimeMax.SNSSAI. InitialReg
RM.RegTimeMax.SNSSAI. MobilityRegUpdate
RM.RegTimeMax.SNSSAI. PeriodicRegUpdate
RM.RegTimeMax.SNSSAI. EmergencyReg

f)	Where SNSSAI identifies the S-NSSAI, InitialReg identifies the registration type "Initial Registration ", MobilityRegUpdate identifies the registration type "Mobility Registration Update", PeriodicRegUpdate identifies the registration type "Periodic Registration Update", EmergencyReg identifies the registration type "Emergency Registration".

g)	AMFFunction

h)	Valid for packet switched traffic

i)	5GS

j)	One usage of this measurement is for monitoring the max time of registration procedure during the granularity period.

### 5.2.3	Service Request procedure related measurements

#### 5.2.3.1	Number of attempted network initiated service requests

a)	This measurement provides the number of attempted network initiated service requests.

b)	CC.

c)	Receipt of Namf_Communication_N1N2MessageTransfer indicating a network initiated service request from SMF or another NF by the AMF (see 3GPP TS 23.502 [7]).

d)	An integer value.

e)	MM.ServiceReqNetInitAtt.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.3.2	Number of successful network initiated service requests

a)	This measurement provides the number of successful network initiated service requests.

b)	CC.

c)	Transmission of N2 request that contains “MM NAS Service Accept” by the AMF to (R)AN (see 3GPP TS 23.502 [7]), corresponding to the received Namf_Communication_N1N2MessageTransfer that indicated a network initiated service request.

d)	An integer value.

e)	MM.ServiceReqNetInitSucc.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.3.3	Total number of attempted service requests (including both network initiated and UE initiated)

a)	This measurement provides the total number of the attempted service requests, including both network initiated and UE initiated service requests.

b)	CC.

c)	Receipt of Service Request by the AMF from (R)AN (see 3GPP TS 23.502 [7]).

d)	An integer value.

e)	MM.ServiceReqTotalAtt.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.3.4	Total number of successful service requests (including both network initiated and UE initiated)

a)	This measurement provides the total number of the successful service requests, including both network initiated and UE initiated service requests.

b)	CC.

c)	Transmission of N2 request that contains “MM NAS Service Accept” by the AMF to (R)AN (see 3GPP TS 23.502 [7]).

d)	An integer value.

e)	MM.ServiceReqTotalSucc.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.2.4	Measurements related to registration via untrusted non-3GPP access

#### 5.2.4.1	Number of initial registration requests via untrusted non-3GPP access

a)	This measurement provides the number of initial registration requests via untrusted non-3GPP access received by the AMF.

b)	CC.

c)	Receipt by the AMF from N3IWF of an N2 message that contains Registration Request with the registration type indicating an initial registration (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each initial registration request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegInitReqNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.2	Number of successful initial registrations via untrusted non-3GPP access

a)	This measurement provides the number of successful initial registrations via untrusted non-3GPP access at the AMF.

b)	CC.

c)	Transmission by the AMF to N3IWF of an N2 message that contains Registration Accept corresponding to an initial registration request (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each accepted initial registration is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegInitSuccNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.3	Number of mobility registration update requests via untrusted non-3GPP access

a)	This measurement provides the number of mobility registration update requests via untrusted non-3GPP access received by the AMF.

b)	CC.

c)	Receipt by the AMF from N3IWF of an N2 message that contains Registration Request with the registration type indicating a Mobility Registration Update (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each mobility registration update request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegMobReqNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.4	Number of successful mobility registration updates via untrusted non-3GPP access

a)	This measurement provides the number of successful mobility registration updates via untrusted non-3GPP access at the AMF.

b)	CC.

c)	Transmission by the AMF to N3IWF of an N2 message that contains Registration Accept corresponding to a mobility registration update request (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each accepted mobility registration update is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegMobSuccNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.5	Number of periodic registration update requests via untrusted non-3GPP access

a)	This measurement provides the number of periodic registration update requests via untrusted non-3GPP access received by the AMF.

b)	CC.

c)	Receipt by the AMF from N3IWF of an N2 message that contains Registration Request with the registration type indicating a Periodic Registration Update (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each periodic registration update request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegPeriodReqNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.6	Number of successful periodic registration updates via untrusted non-3GPP access

a)	This measurement provides the number of successful mobility registration updates via untrusted non-3GPP access at the AMF.

b)	CC.

c)	Transmission by the AMF to N3IWF of an N2 message that contains Registration Accept corresponding to a periodic registration update request (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each accepted periodic registration update is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegPeriodSuccNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.7	Number of emergency registration requests via untrusted non-3GPP access

a)	This measurement provides the number of emergency registration requests via untrusted non-3GPP access received by the AMF.

b)	CC.

c)	Receipt by the AMF from N3IWF of an N2 message that contains Registration Request with the registration type indicating an Emergency Registration (see clause 4.2.2.2.2 of 3GPP TS 23.502 [7]). Each emergency registration request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegEmergReqNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.4.8	Number of successful emergency registrations via untrusted non-3GPP access

a)	This measurement provides the number of successful emergency registrations via untrusted non-3GPP access Transmission by the AMF to N3IWF of an N2 message that contains Registration Accept corresponding to at the AMF.

b)	CC.

c)	Transmission by the AMF to N3IWF of an N2 message that contains Registration Accept corresponding to an emergency registration request (see clause 4.12.2.2 of 3GPP TS 23.502 [7]). Each accepted emergency registration is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value.

e)	RM.RegEmergSuccNon3GPP.SNSSAI.

Where SNSSAI identifies the NSI;

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.2.5	Mobility related measurements

#### 5.2.5.1	Inter-AMF handovers

##### 5.2.5.1.1	Number of PDU sessions requested for inter-AMF incoming handovers

a)	This measurement provides the number of PDU sessions requested for the inter-AMF incoming handovers received by target AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	Receipt by the target AMF from source AMF of Namf_Communication_CreateUEContext Request (see clause 4.9.1.3 of 3GPP TS 23.502 [7]). Each PDU session requested in the Namf_Communication_CreateUEContext Request (see 3GPP TS 29.518 [21]) increments the relevant subcounter per S-NSSAI by 1.

d)	Each measurement is an integer value.

e)	MM.NbrPDUReqInterAMFHOInc.SNSSAI.

Where the SNSSAI identifies the S-NSSAI.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.2.5.1.2	Number of PDU sessions failed to setup for inter-AMF incoming handovers

a)	This measurement provides the number of PDU sessions failed to setup for inter-AMF incoming handover requests by target AMF. This measurement is split into subcounters per S-NSSAI and subcounters per failure cause.

b)	CC.

c)	Transmission by the target AMF to the source AMF of Namf_Communication_CreateUEContext Response (see clause 4.9.1.3 of 3GPP TS 23.502 [7]) that contains the PDU Sessions failed to be setup list (including List Of PDU Sessions failed to be setup received from target RAN and the Non-accepted PDU session List generated by the T-AMF). Each PDU session failed to setup increments the relevant subcounter per S-NSSAI and the relevant subcounter per failure cause by 1 respectively.

d)	Each measurement is an integer value.

e)	MM.NbrPDUFailInterAMFHOInc.SNSSAI,
MM.NbrPDUFailInterAMFHOInc.cause,

Where the SNSSAI identifies the S-NSSAI, and cause identifies the failure cause (Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11]).

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.2.5.1.3	Number of QoS flows requested for inter-AMF incoming handovers

a)	This measurement provides the number of QoS flows requested for inter-AMF incoming handover requests by target AMF. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	CC.

c)	Receipt by the target AMF from source AMF of Namf_Communication_CreateUEContext Request (see clause 4.9.1.3 of 3GPP TS 23.502 [7]). Each QoS flow requested in the Namf_Communication_CreateUEContext Request (see 3GPP TS 29.518 [21]) increments the relevant subcounter per S-NSSAI and the relevant subcounter per 5QI by 1 respectively.

d)	Each measurement is an integer value.

e)	MM.NbrQoSFlowReqInterAMFHOInc.SNSSAI,
	MM.NbrQoSFlowReqInterAMFHOInc.5QI,

Where the SNSSAI identifies the S-NSSAI, and 5QI identifies the 5QI.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.2.5.1.4	Number of QoS flows failed to setup for inter-AMF incoming handovers

a)	This measurement provides the number of PDU sessions failed to setup for inter-AMF incoming handover requests by target AMF. This measurement is split into subcounters per S-NSSAI, subcounters per 5QI, and subcounters per failure cause.

b)	CC.

c)	Transmission by the target AMF to the source AMF of Namf_Communication_CreateUEContext Response (see clause 4.9.1.3 of 3GPP TS 23.502 [7]) that includes 1) the PDU Sessions failed to be setup list (including List Of PDU Sessions failed to be setup received from target RAN and the Non-accepted PDU session List generated by the T-AMF) and/or 2) the PDU sessions successfully setup but with the QoS flow failed to setup List. Each QoS flow corresponding to the PDU Session failed to be setup, or in the QoS flow failed to setup List of the PDU sessions successfully setup increments the relevant subcounter per S-NSSAI, the subcounter per 5QI and the subcounter per failure cause by 1 respectively.

d)	Each measurement is an integer value.

e)	MM.NbrQoSFlowFailInterAMFHOInc.SNSSAI,
MM.NbrQoSFlowFailInterAMFHOInc.5QI,
MM.NbrQoSFlowFailInterAMFHOInc.cause,

Where the SNSSAI identifies the S-NSSAI, 5QI identifies the 5QI and cause identifies the failure cause (Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11]).

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.5.2	Measurements for 5G paging

##### 5.2.5.2.1		Number of 5G paging procedures

a)	This measurement provides the number of 5G paging procedures initiated at the AMF. The initial paging procedures as well as the repeated paging procedures are counted.

b)	CC.

c)	Incremented when a 5G paging request is sent i.e. at the transmission of the first paging request (TS 23.502 [16] and TS 24.501 [24]).

d)	A single integer value.

e)	MM.Paging5GReq

f)	AMFFunction

g)	Valid for packet switching.

h)	5GS.

##### 5.2.5.2.2	Number of successful 5G paging procedures

a)	This measurement provides the number of successful 5G paging procedures initiated at the AMF. The initial paging procedures as well as the repeated paging procedures are counted.

b)	CC.

c)	When a service request from UE that with service type value equal "mobile terminated service" is received at the AMF (see TS 23.502 [7] and TS 24.501 [24]), the AMF increments the count by 1.

d)	A single integer value.

e)	MM.Paging5GSucc

f)	AMFFunction

g)	Valid for packet switching.

h)	5GS.

### 5.2.6	Measurements related to Service Requests via Untrusted non-3GPP Access

#### 5.2.6.1	Number of attempted service requests via Untrusted non-3GPP Access

a)	This measurement provides the number of attempted service requests via Untrusted non-3GPP Access.

b)	CC.

c)	Receipt of an N2 Message indicating the Service Request by the AMF from N3IWF (see 3GPP TS 23.502 [7]).

d)	An integer value.

e)	MM.ServiceReqNon3GPPAtt.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.2.6.2	Number of successful service requests via Untrusted non-3GPP Access

a)	This measurement provides the number of successful service requests via Untrusted non-3GPP Access.

b)	CC

c)	Transmission of N2 request that contains "MM NAS Service Accept" by the AMF to N3IWF (see 3GPP TS 23.502 [7]).

d)	An integer value.

e)	MM.ServiceReqNon3GPPSucc.

f)	AMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.2.7	Measurements related to SMS over NAS

#### 5.2.7.1	Registration of SMS over NAS

##### 5.2.7.1.1	Number of registration requests for SMS over NAS via 3GPP access

a)	This measurement provides the number of registration requests for SMS over NAS received by the AF from UEs via 3GPP access.

b)	CC

c)	Receipt of a Registration Request message containing the "SMS supported" indication indicating that the UE supports SMS delivery over NAS by the AMF from UE via 3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasReg3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.1.2	Number of successful registrations allowed for SMS over NAS via 3GPP access

a)	This measurement provides the number of successful registrations allowed for SMS over NAS sent by the AF to UEs via 3GPP access.

b)	CC

c)	Transmission of a Registration Accept message containing the "SMS allowed" indication by the AMF to UE via 3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasReg3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.1.3	Number of registration requests for SMS over NAS via non-3GPP access

a)	This measurement provides the number of registration requests for SMS over NAS received by the AF from UEs via non-3GPP access.

b)	CC

c)	Receipt of a Registration Request message containing the "SMS supported" indication indicating that the UE supports SMS delivery over NAS by the AMF from UE via non-3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasRegNon3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.1.4	Number of successful registrations allowed for SMS over NAS via non-3GPP access

a)	This measurement provides the number of successful registrations allowed for SMS over NAS sent by the AF to UEs via non-3GPP access.

b)	CC

c)	Transmission of a Registration Accept message containing the "SMS allowed" indication by the AMF to UE via non-3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasRegNon3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.2.7.2	MO SMS over NAS

##### 5.2.7.2.1	Number of attempted MO SMS messages over NAS via 3GPP access

a)	This measurement provides the number of NAS messages encapsulating the SMS messages received by the AF from UEs via 3GPP access.

b)	CC

c)	Receipt of an NAS message with an indication of SMS transportation by the AMF from UE via 3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMo3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.2.2	Number of MO SMS messages successfully transported over NAS via 3GPP access

a)	This measurement provides the number of MO SMS messages successfully transported over NAS via 3GPP access.

b)	CC

c)	Transmission, by the AMF to UE via 3GPP access, of an NAS message that contains the “submit report” indicating the MO SMS message has been successfully delivered (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMo3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.2.3	Number of attempted MO SMS messages over NAS via non-3GPP access

a)	This measurement provides the number of NAS messages encapsulating the SMS messages received by the AF from UEs via non-3GPP access.

b)	CC

c)	Receipt of an NAS message with an indication of SMS transportation by the AMF from UE via non-3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMoNon3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.2.4	Number of MO SMS messages successfully transported over NAS via non-3GPP access

a)	This measurement provides the number of MO SMS messages successfully transported over NAS via non-3GPP access.

b)	CC

c)	Transmission, by the AMF to UE via non-3GPP access, of an NAS message that contains the “submit report” indicating the MO SMS message has been successfully submitted (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMoNon3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.2.7.3	MT SMS over NAS

##### 5.2.7.3.1	Number of attempted MT SMS messages over NAS via 3GPP access

a)	This measurement provides the number of NAS messages encapsulating the SMS messages sent by the AF to UEs via 3GPP access.

b)	CC

c)	Transmission of an NAS message with an indication of SMS transportation by the AMF to UE via 3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMt3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.3.2	Number of MT SMS messages successfully transported over NAS via 3GPP access

a)	This measurement provides the number of MT SMS messages successfully transported over NAS via 3GPP access.

b)	CC

c)	Receipt, by the AMF from UE via 3GPP access, of an NAS message that contains the “delivery report” indicating the MT SMS message has been successfully delivered (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMt3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.3.3	Number of attempted MT SMS messages over NAS via non-3GPP access

a)	This measurement provides the number of NAS messages encapsulating the SMS messages sent by the AF to UEs via non-3GPP access.

b)	CC

c)	Transmission of an NAS message with an indication of SMS transportation by the AMF to UE via non-3GPP access (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMtNon3GPPReq

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.2.7.3.4	Number of MT SMS messages successfully transported over NAS via non-3GPP access

a)	This measurement provides the number of MT SMS messages successfully transported over NAS via non-3GPP access.

b)	CC

c)	Receipt, by the AMF from UE via non-3GPP access, of an NAS message that contains the “delivery report” indicating the MT SMS message has been successfully delivered (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	SMS.SmsOverNasMtNon3GPPSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

### 5.2.8	UE Configuration Update procedure related measurements

#### 5.2.8.1	Number of UE Configuration Update

a)	This measurement provides the number of UE Configuration Update requested by the AMF.

b)	CC

c)	On transmission of Configuration Update Command from the AMF to UE (see 3GPP TS 23.502 [7]).

d)	Each counter is an integer value

e)	MM.ConfUpdate

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.2.8.2	Number of successful UE Configuration Update

a)	This measurement provides the number of UE Configuration Update successfully completed by the UE.

b)	CC

c)	On receipt by the AMF from the UE of Configuration Update Complete (see 3GPP TS 23.502 [7]).

NOTE: Configuration Update Complete shall be requested for all parameters included in Configuration Update Command except when only NITZ is included.

d)	Each counter is an integer value

e)	MM.ConfUpdateSucc

f)	AMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

## 5.3	Performance measurements for SMF

### 5.3.1	Session Management

#### 5.3.1.1	Number of PDU sessions (Mean)

- a)	This measurement provides the mean number of PDU sessions.

- b)	SI

- c)	The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then taking the arithmetic mean. The measurement is optionally split into subcounters per NSI identifier (S-NSSAI).

- d)	A single integer value

- e)	SM.SessionNbrMean.SNSSAI
Where SNSSAI identifies the NSI

- f)	SMFFunction

- g)	Valid for packet switched traffic

- h)	5GS

#### 5.3.1.2	Number of PDU sessions (Maximum)

a)	This measurement provides the max number of PDU sessions.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then selecting the maximum value. The measurement is optionally split into subcounters per NSI identifier (S-NSSAI).

d)	A single integer value

e)	SM.SessionNbrMax.SNSSAI
Where SNSSAI identifies the NSI

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.3.1.3	Number of PDU session creation requests

a)	This measurement provides the number of PDU sessions requested to be created by the SMF.

b)	CC

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_CreateSMContext Request (see 3GPP TS 23.502 [7]). Each PDU session requested to be created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationReq.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationReqReqType.

WhereReqType  indicates the request type (e.g., initial request, initial emergency request)  cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.4	Number of successful PDU session creations

a)	This measurement provides the number of PDU sessions successfully created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a successful PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session successfully created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationSucc.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationSuccReqType.

WhereReqType  indicates the request type (e.g., initial request, initial emergency request)  cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.5	Number of failed PDU session creations

a)	This measurement provides the number of PDU sessions failed to be created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a rejected PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session rejected to be created is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationFail.cause

Where cause indicates the rejection cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.6	PDU session modifications

##### 5.3.1.6.1	Number of requested PDU session modifications (UE initiated)

a)	This measurement provides the number of PDU session modification requests (initiated by UE) received by the SMF.

b)	CC.

c)	On receipt of Nsmf_PDUSession_UpdateSMContext Request which includes the N1 SM container IE indicating the "PDU Session Modification Request" (see 3GPP TS 23.502 [7]) by the SMF from AMF.

d)	A single integer value.

e)	SM.PduSessionModUeInitReq.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.1.6.2	Number of successful PDU session modifications (UE initiated)

a)	This measurement provides the number of successful PDU session modifications (initiated by UE) acknowledged by the SMF.

b)	CC.

c)	On transmission of Nsmf_PDUSession_UpdateSMContext Response indicating a successful PDU session modification by the SMF to AMF as reply to a smf_PDUSession_UpdateSMContext Request that includes the N1 SM container IE indicating the "PDU Session Modification Complete" (see 3GPP TS 23.502 [7]) for a PDU session modification request (initiated by the UE).

d)	A single integer value.

e)	SM.PduSessionModUeInitSucc.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.1.6.3	Number of failed PDU session modifications (UE initiated)

a)	This measurement provides the number of failed PDU session modifications (initiated by UE) responded by the SMF. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission of Nsmf_PDUSession_UpdateSMContext Response indicating a failed PDU session modification by the SMF to AMF (see TS 23.502 [7]) for a PDU session modification request (initiated by the UE). Each transmitted Nsmf_PDUSession_UpdateSMContext Response indicating the failed PDU session modification triggers the relevant subcounter per failure cause (see the causes listed in table 6.1.3.3.4.2.2-2 of TS 29.502 [14]) to increment by 1.

d)	A single integer value.

e)	SM.PduSessionModUeInitFail.Cause.

Where Cause identifies the cause of the PDU session modification failure. Encoding of the Cause is defined in in table 6.1.3.3.4.2.2-2 of TS 29.502 [14].

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.1.6.4	Number of requested PDU session modifications (SMF initiated)

a)	This measurement provides the number of PDU session modification requests (initiated by SMF) sent by the SMF to AMF.

b)	CC.

c)	On transmission of Namf_Communication_N1N2MessageTransfer which includes the N2 SM information IE and N1 SM container IE indicating the "PDU Session Modification Command" (see 3GPP TS 23.502 [7]) by the SMF to AMF.

d)	A single integer value.

e)	SM.PduSessionModSmfInitReq.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.1.6.5	Number of successful PDU session modifications (SMF initiated)

a)	This measurement provides the number of successful PDU session modifications (initiated by SMF) acknowledged by the SMF.

b)	CC.

c)	On transmission of Nsmf_PDUSession_UpdateSMContext Response indicating a successful PDU session modification by the SMF to AMF as reply to a smf_PDUSession_UpdateSMContext Request that includes the N1 SM container IE indicating the "PDU Session Modification Complete" (see 3GPP TS 23.502 [7]) for a PDU session modification request (initiated by the SMF).

d)	A single integer value.

e)	SM.PduSessionModSmfInitSucc.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.1.6.6	Number of failed PDU session modifications (SMF initiated)

a)	This measurement provides the number of failed PDU session modifications (initiated by SMF) responded by the SMF. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On transmission of Nsmf_PDUSession_UpdateSMContext Response indicating a failed PDU session modification by the SMF to AMF (see TS 23.502 [7]) for a PDU session modification request (initiated by the SMF). Each transmitted Nsmf_PDUSession_UpdateSMContext Response indicating the failed PDU session modification triggers the relevant subcounter per failure cause (see the causes listed in table 6.1.3.3.4.2.2-2 of TS 29.502 [14]) to increment by 1.

d)	A single integer value.

e)	SM.PduSessionModSmfInitFail.Cause.

Where Cause identifies the cause of the PDU session modification failure. Encoding of the Cause is defined in in table 6.1.3.3.4.2.2-2 of TS 29.502 [14].

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.3.1.7	PDU session releases

##### 5.3.1.7.1	Number of released PDU sessions (AMF initiated)

a)	This measurement provides the number of released PDU sessions (initiated by AMF) at the SMF. There could be several reasons for the AMF to request release of PDU sessions, for instance the mismatch of PDU Session status between UE and AMF. This step may also be invoked due to a change of the set of network slices for a UE where a network slice instance is no longer available, as described in TS 23.501 clause 5.15.5.2.2, or the PDU Session(s) is not accepted by the T-AMF (e.g. S-NSSAI associated with the PDU Session is not available in the T-AMF). This measurement is split into subcounters per S-NSSAI and subcounters per cause.

b)	CC.

c)	On transmission of Nsmf_PDUSession_ReleaseSMContext Response indicating a successful PDU session release from the SMF to AMF, as a reply to the received Nsmf_PDUSession_ReleaseSMContext Request from the AMF (see 3GPP TS 23.502 [7]). Each transmitted Nsmf_PDUSession_ReleaseSMContext Response triggers the relevant subcounter per S-NSSAI and the relevant subcounter per cause (the cause, ngApCause or 5GMmCauseValue as indicated in the received Nsmf_PDUSession_ReleaseSMContext Request, see Table 6.1.6.2.6-1 of TS 29.502 [14]) to increment by 1 respectively.

d)	A single integer value.

e)	SM.PduSessionRelAmfInit.SNSSAI and SM.PduSessionRelAmfInit.cause.

Where the SNSSAI identifies the S-NSSAI; and the cause identifies the cause, ngApCause or 5GMmCauseValue as indicated in the received Nsmf_PDUSession_ReleaseSMContext Request, see Table 6.1.6.2.6-1 of TS 29.502 [14]).

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance.

#### 5.3.1.8	Number of PDU session creation requests in HR roaming scenario

a)	This measurement provides the number of PDU sessions requested to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On receipt by the H-SMF from V-SMF of Nsmf_PDUSession_Create Request (see 3GPP TS 23.502 [7]). Each PDU session requested to be created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroam.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationHRroam.ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.9	Number of successful PDU session creations in HR roaming scenario

a)	This measurement provides the number of PDU sessions successfully created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response that indicates a successful PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session successfully created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroamSucc.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationHRroamSucc.ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.10	Number of failed PDU session creations in HR roaming scenario

a)	This measurement provides the number of PDU sessions failed to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response that indicates a rejected PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session rejected to be created is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroamFail.cause

Where cause indicates the rejection cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.3.1.11	Mean time of PDU session establishment

a)	This measurement provides the mean time of PDU session establishment during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by measuring the time interval for every successful PDU session establishment procedure per S-NSSAI between the receipt by SMF from AMF of " Nsmf_PDUSession_UpdateSMContext Request ", which includes N2 SM information received from (R)AN to the SMF and the sending of a " Nsmf_PDUSession_CreateSMContext Request or Nsmf_PDUSession_UpdateSMContext Request " message from AMF to the SMF over a granularity period using DER. The end value of this time will then be divided by the number of successful PDU session establishment observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value.(in milliseconds)

e)	SM.PduSessionTimeMean.SNSSAI

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of registration procedure during the granularity period.

#### 5.3.1.12	Max time of PDU session establishment

a)	This measurement provides the max time of PDU session establishment during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by measuring the time interval for every successful registration procedure per S-NSSAI per registration type between the receipt by SMF from AMF of " Nsmf_PDUSession_UpdateSMContext Request", which includes N2 SM information received from (R)AN to the SMF and the sending of a " Nsmf_PDUSession_CreateSMContext Request or Nsmf_PDUSession_UpdateSMContext Request PDU Session Establishment Request " message from AMF to the SMF over a granularity period using DER. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value.(in milliseconds)

e)	SM.PduSessionTimeMax.SNSSAI

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the max time of registration procedure during the granularity period.

### 5.3.2	QoS flow monitoring

#### 5.3.2.1	QoS flow monitoring

##### 5.3.2.1.1	Number of QoS flows requested to create

a)	This measurement provides the number of QoS flows requested to create. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request which includes the N1 SM container IE containing the QoS flows requested to create (see 3GPP TS 23.502 [7]) from AMF by the SMF; or transmission of Namf_Communication_N1N2MessageTransfer which includes N1 SM container IE containing the QoS flows requested to create to AMF by the SMF (see 3GPP TS 23.502 [7]). Each QoS flow requested to create in the message triggers the relevant subcounter per S-NSSAI and the relevant subcounter per 5QI to increment by 1 respectively (the S-NSSAI is the S-NSSAI that the PDU session belongs to, or the new S-NSSAI if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request, see clause 6.1.6.2.4 in TS 29.502 [15]).

d)	Each measurement is an integer value.

e)	SM.QoSflowCreateReq.SNSSAI and SM.QoSflowCreateReq.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.2	Number of QoS flows successfully created

a)	This measurement provides the number of QoS flows successfully created. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request that includes the N2 SM information IE containing the successfully created (set up or added) QoS flows from AMF by the SMF (see 3GPP TS 23.502 [7]). Each successfully created QoS flow triggers the relevant subcounter per S-NSSAI and the relevant subcounter per 5QI to increment by 1 respectively (the S-NSSAI is the S-NSSAI that the PDU session belongs to, or the new S-NSSAI if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request, see clause 6.1.6.2.4 in TS 29.502 [15]).

d)	Each measurement is an integer value.

e)	SM.QoSflowCreateSucc.SNSSAI and SM.QoSflowCreateSucc.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.3	Number of QoS flows failed to create

a)	This measurement provides the number of QoS flows failed to create. This measurement is split into subcounters per cause.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request that includes the N2 SM information IE containing the QoS flows failed to create (set up or add)  from AMF by the SMF (see 3GPP TS 23.502 [7]). Each QoS flow failed to create triggers the relevant subcounter per cause (see clause 9.3.1.13 in TS 38.413 [11]).

d)	Each measurement is an integer value..

e)	SM.QoSflowCreateFail.cause.

Where the cause identifies the cause that resulted in the QoS flow setup failure (see clause 9.3.1.2 in TS 38.413 [11]).

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.4	Number of QoS flows requested to modify

a)	This measurement provides the number of QoS flows requested to modify. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request which includes the N1 SM container IE containing the QoS flows requested to modify (see 3GPP TS 23.502 [7]) from AMF by the SMF; or transmission of Namf_Communication_N1N2MessageTransfer which includes N1 SM container IE containing the QoS flows requested to modify to AMF by the SMF (see 3GPP TS 23.502 [7]). Each QoS flow requested to modify in the message triggers the relevant subcounter per S-NSSAI and the relevant subcounter per 5QI to increment by 1 respectively (the S-NSSAI is the S-NSSAI that the PDU session belongs to, or the new S-NSSAI if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request, see clause 6.1.6.2.4 in TS 29.502 [15]).

d)	Each measurement is an integer value.

e)	SM.QoSflowModReq.SNSSAI and SM.QoSflowModReq.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.5	Number of QoS flows successfully modified

a)	This measurement provides the number of QoS flows successfully modified. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request that includes the N2 SM information IE containing the successfully modified QoS flows from AMF by the SMF (see 3GPP TS 23.502 [7]). Each successfully modified QoS flow triggers the relevant subcounter per S-NSSAI and the relevant subcounter per 5QI to increment by 1 respectively (the S-NSSAI is the S-NSSAI that the PDU session belongs to, or the new S-NSSAI if the S-NSSAI for the serving PLMN derived from the S-NSSAI of the home PLMN differs from the S-NSSAI provided in the Create SM Context Request, see clause 6.1.6.2.4 in TS 29.502 [15]).

d)	Each measurement is an integer value.

e)	SM.QoSflowModSucc.SNSSAI and SM.QoSflowModSucc.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.6	Number of QoS flows failed to modify

a)	This measurement provides the number of QoS flows failed to modify. This measurement is split into subcounters per cause.

b)	CC.

c)	Receipt of Nsmf_PDUSession_UpdateSMContext Request that includes the N2 SM information IE containing the QoS flows failed to modify from AMF by the SMF (see 3GPP TS 23.502 [7]). Each QoS flow failed to modify triggers the relevant subcounter per cause (see clause 9.3.1.13 in TS 38.413 [11]).

d)	Each measurement is an integer value.

e)	SM.QoSflowModFail.cause.

Where the cause identifies the cause that resulted in the QoS flow modification failure (see clause 9.3.1.2 in TS 38.413 [11]).

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.7	Mean number of QoS flows

a)	This measurement provides the mean number of QoS flows at the SMF. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	 SI

c)	This measurement is obtained by sampling at a pre-defined interval, the number of QoS flows per S-NSSAI and per 5QI, and then taking the arithmetic mean.

d)	Each measurement is a real value.

e)	SM.QoSflowNbrMean.SNSSAI and SM.QoSflowNbrMean.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.3.2.1.8	Peak number of QoS flows

a)	This measurement provides the peak number of QoS flows at the SMF. This measurement is split into subcounters per S-NSSAI and subcounters per 5QI.

b)	 SI

c)	This measurement is obtained by sampling at a pre-defined interval, the number of QoS flows per S-NSSAI and per 5QI, and then taking the maximum.

d)	Each measurement is a real value.

e)	SM.QoSflowNbrPeak.SNSSAI and SM.QoSflowNbrPeak.5QI.

Where the SNSSAI identifies the S-NSSAI, and the 5QI identifies the 5QI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.3.3	Performance measurement for N4 interface

#### 5.3.3.1	Number of N4 session modifications

a)	This measurement provides the number of attempted N4 session modifications.

b)	 CC

c)	Transmission of "N4 Session Modification Request" message from SMF, this counter is cumulated by different N4 Session Modification Request messages sent by SMF as specified in TS 23.502 [7] and TS 29.244 [16].

d)	A single integer value.

e)	SM.N4SessionModify.

f)	SMFFunction

g)	Valid for packet switching

h) 	5GS.

#### 5.3.3.2	Number of failed N4 session modifications

A	This measurement provides the number of failed N4 session modifications.

b)	CC

c)	Receipt of "N4 Session Modification Response" message with appropriate error cause value from UPF, SMF identifies a failed N4 session modification as defined in TS 23.502 [7] and TS 29.244 [16]. Each rejected N4 Session Modification Request increments the relevant subcounter by 1.

d)	A single integer value.

e)	SM.N4SessionModifyFail.Cause.

Where Cause identifies the reject cause of N4 session modification request, per the encoding of the cause specified in TS 29.244 [16].

f)	SMFFunction

g)	Valid for packet switching

h)	5GS.

#### 5.3.3.3	Number of N4 session deletions

a)	This measurement provides the number of attempted N4 session deletions.

b)	CC

c)	Transmission of "N4 Session Deletion Request" message from SMF, this counter is cumulated by different N4 Session Deletion Request messages sent by SMF as specified in TS 23.502 [7] and TS 29.244 [16].

d)	A single integer value.

e)	SM.N4SessionDelete.

f)	 SMFFunction

g)	Valid for packet switching

h)	5GS.

#### 5.3.3.4	Number of failed N4 session deletions

a)	This measurement provides the number of failed N4 session deletions.

b)	CC

c)	Receipt of "N4 Session Deletion Response" message with appropriate error cause value from UPF, SMF identifies a failed N4 session deletion as defined in TS 23.502 [7] and TS 29.244 [16]. Each rejected N4 Session Deletion Request increments the relevant subcounter by 1.

d)	A single integer value.

e)	SM. N4SessionDeleteFail.Cause.

Where Cause identifies the reject cause of N4 session deletion request, per the encoding of the cause specified in TS 29.244 [16].

f)	SMFFunction

g)	Valid for packet switching

h)	5GS.

## 5.4	Performance measurements for UPF

### 5.4.1	N3 interface related measurements

#### 5.4.1.1	Number of incoming GTP data packets on the N3 interface, from (R)AN to UPF

a)	This measurement provides the number of GTP data PDUs on the N3 interface which have been accepted and processed by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Reception by the UPF of a GTP-U data PDU on the N3 interface from the (R)AN. See TS 23.501 [4].

d)	A single integer value.

e)	GTP.InDataPktN3UPF

f)	EP_N3

g)	Valid for packet switching.

h)	5GS

#### 5.4.1.2	Number of outgoing GTP data packets of on the N3 interface, from UPF to (R)AN

a)	This measurement provides the number of GTP data PDUs on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Transmission by the UPF of a GTP-U data PDU of on the N3 interface to the (R)AN. See TS 23.501 [4].

d)	A single integer value.

e)	GTP.OutDataPktN3UPF

f)	EP_N3

g)	Valid for packet switching.

h)	5GS

#### 5.4.1.3	Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF

a)	This measurement provides the number of octets of incoming GTP data packets on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Reception by the UPF of a GTP-U data PDU on the N3 interface from (R)AN. See TS 23.501 [4].

d)	A single integer value

e)	GTP.InDataOctN3UPF

f)	EP_N3

g)	Valid for packet switching

h)	5GS

#### 5.4.1.4	Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN

a)	This measurement provides the number of octets of outgoing GTP data packets on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Transmission by the UPF of a GTP-U data PDU on the N3 interface to the(R)AN, .See TS 23.501 [4].

d)	A single integer value

e)	GTP.OutDataOctN3UPF

f)	EP_N3

g)	Valid for packet switching

h)	5GS

#### 5.4.1.5	Data volume of incoming GTP data packets per QoS level on the N3 interface, from (R)AN to UPF

a)	This measurement provides the data volume of the incoming GTP data packets per QoS level which have been accepted and processed by the GTP-U protocol entity on the N3 interface.  The measurement is calculated and split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by counting the number of GTP PDU bits sent from GNB to UPF on the N3 interface. The measurement is performed per configured QoS level (5QI).

d)	Each measurement is an integer value representing the number of bits measured in kbit . The number of measurements is equal to the number of QoS levels.

e)	GTP.InDataVolumeQoSLevelN3UPF.

f)	EP_N3.

g)	Valid for packet switching.

h)	5GS.

#### 5.4.1.6	Data volume of outgoing GTP data packets per QoS level on the N3 interface, from UPF to (R)AN

a)	This measurement provides the data volume of the outgoing GTP data packets per QoS level which have been generated by the GTP-U protocol entity on the N3 interface.  The measurement is calculated and split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by counting the number of GTP PDU bits sent from UPF to GNB on the N3 interface. The measurement is performed per configured QoS level (5QI).

d)	Each measurement is an integer value representing the number of bits measured in kbitk . The number of measurements is equal to the number of QoS levels.

e)	GTP.OutDataVolumeQoSLevelN3UPF

f)	EP_N3.

g)	Valid for packet switching.

h)	5GS.

##### 5.4.1.7	Incoming GTP Data Packet Loss

a)	This measurement provides the numer of GTP data packets which are not successfully received at UPF. It is a measure of the incoming GTP data packet loss per N3 on an UPF interface.  The measurement is split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by a counter: Number of missing incoming GTP sequence numbers (TS 29.281) among all GTP packets delivered by a gNB to an UPF interface. Separate counter is maintained for each 5QI.

d)	Each measurement is an integer value representing the number of the lost GTP pakets. If the QoS level measurement is perfomed, the measurements are equal to the number of 5QIs.

e)	The measurement name has the form GTP.InDataPktPacketLossN3UPF or GTP.InDataPktPacketLossN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.1.8	Outgoing GTP Data Packet Loss

a)	This measurement provides the number of GTP data packets which are not successfully received at gNB over N3. It is a measure of the outgoing GTP data packet loss per N3 on an UPF interface.  The measurement is split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by a counter: Number of missing outgoing GTP sequence numbers (TS 29.281) among all GTP packets delivered by an UPF interface to a gNB. Separate counter is maintained for each 5QI.

d)	Each measurement is an integer value representing the lost GTP packets.. If the QoS level measurement is perfomed, the measurements are equal to the number of 5QIs.

e)	The measurement name has the form GTP.OutDataPktPacketLossN3UPF or GTP.OutDataPktPacketLossN3UPF.QoS where QoS identifies the target quality of service class.

f)	EP_N3.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.1.9	Round-trip GTP Data Packet Delay

##### 5.4.1.9.1	Average round-trip N3 delay on PSA UPF

a)	This measurement provides the average round-trip delay on a N3 interface on PSA UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained as: the sum (the time when receiving a GTP echo reply message from the gNB-CU-UP at PSA UPF’s ingress GTP termination, minus time when sending the associated echo request message to gNB-CU-UP at the PSA UPF’s GTP egress termination) divided by total number of GTP echo reply message received at PSA UPF’s ingress GTP termination. This measurement is calculated for each DSCP.

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form GTP.RttDelayN3DlPsaUpfMean.DSCP
Where DSCP identifies the DSCP.

f)	EP_N3 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.1.9.2	Distribution of round-trip N3 delay on PSA UPF

a)	This measurement provides the distribution of delay on a N3 interface on PSA UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained by 1) calculating the RTT N3 delay by:  the time when receiving a GTP echo reply message from the gNB-CU-UP at PSA UPF’s ingress GTP termination, minus time when sending the associated echo request message to gNB-CU-UP at the PSA UPF’s GTP egress termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per DSCP.

d)	Each measurement is an integer representing the number of GTP echo messages measured with the delay within the range of the bin.

e)	The measurement name has the form GTP.RttDelayN3PsaUpfDist.Bin.DSCP
Where Bin indicates a delay range which is vendor specific, and DSCP identifies the DSCP.

f)	EP_N3 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.1.9.3	Average round-trip N3 delay on I-UPF

a)	This measurement provides the average round-trip delay on a N3 interface on I-UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained as: the sum (the time when receiving a GTP echo reply message from the gNB-DU at I-UPF’s ingress GTP termination, minus time when sending the associated echo request message to gNB-DU at the I-UPF’s GTP egress termination) divided by total number of GTP echo reply message received at I-UPF’s ingress GTP termination. This measurement is calculated for each DSCP.

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form GTP.RttDelayN3IUpfMean.DSCP
Where DSCP identifies the DSCP.

f)	EP_N3 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.1.9.4	Distribution of round-trip N3 delay on I-UPF

a)	This measurement provides the distribution of delay on a N3 interface on I-UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained by 1) calculating the RTT N3 delay by:  the time when receiving a GTP echo reply message from the gNB-DU at I-UPF’s ingress GTP termination, minus time when sending the associated echo request message to gNB-DU at the I-UPF’s GTP egress termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per DSCP.

d)	Each measurement is an integer representing the number of GTP echo messages measured with the delay within the range of the bin.

e)	The measurement name has the form GTP.RttDelayN3IUpfsDist.Bin.DSCP
Where Bin indicates a delay range which is vendor specific, and DSCP identifies the DSCP.

f)	EP_N3 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

### 5.4.2	N6 related measurements

#### 5.4.2.1	N6 incoming link usage

a)	This measurement provides the PDU-layer incoming link usage of N6 interface.

b)	CC

c)	See clause 2.3.4 for IP packet. Definition: IP-type-P (broad spectrum of packet types) Link Usage in IETF RFC 5136 [5].

NOTE:	 How to measure the unstructured data type is not specified in the present document.

d)	Each measurement is an integer value.

e)	IP.N6IncLinkUsage.N6RP
where N6RP identifies the N6 reference point of this UPF, the format of N6RP is vendor specific.

f)	EP_N6

g)	Valid for packet switched traffic.

h)	5GS

#### 5.4.2.2	N6 outgoing link usage

a)	This measurement provides the PDU-layer outcoming link usage of N6 interface.

b)	CC

c)	See clause 2.3.4 for IP packet. Definition: IP-type-P (broad spectrum of packet types) Link Usage in IETF RFC 5136 [5].

NOTE: How to measure the unstructured data type is not specified in the present document.

d)	Each measurement is an integer value.

e)	IP.N6OutLinkUsage.N6RP
where N6RP identifies the N6 reference point of this UPF, the format of N6RP is vendor specific.

f)	EP_N6

g)	Valid for packet switched traffic.

h)	5GS

### 5.4.3	N4 interface related measurements

#### 5.4.3.1	Session establishments

##### 5.4.3.1.1	Number of requested N4 session establishments

a)	This measurement provides the number of N4 session establishment requests received by the UPF.

b)	CC.

c)	On receipt of N4 session establishment request message (see 3GPP TS 23.502 [7]) by the UPF from SMF.

d)	A single integer value.

e)	SM.N4SessionEstabReq.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.3.1.2	Number of failed N4 session establishments

a)	This measurement provides the number of failed N4 session establishments at the UPF. This measurement is split into subcounters per rejection cause.

b)	CC.

c)	On transmission of N4 session establishment response message that contains the cause indicating the rejection of N4 session establishment request (see 3GPP TS 23.502 [7]) by the UPF to SMF. Each N4 session establishment response message indicating the rejection of N4 session establishment request triggers the relevant subcounter per rejection cause to increment by 1.

d)	A single integer value.

e)	SM.N4SessionEstabFail.cause
where the cause identities the cause of the rejection of N4 session establishment request, per the encoding of the cause defined in section 8.2.1 of TS 29.224 [16].

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.3.2	N4 Session reports

##### 5.4.3.2.1	Number of requested N4 session reports

a)	This measurement provides the number of N4 session reports sent by the UPF.

b)	CC.

c)	When UPF sends N4 session report message (see 3GPP TS 23.502 [7]) to SMF.

d)	A single integer value.

e)	SM.N4SessionReport.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.3.2.2	Number of successful N4 session reports

a)	This measurement provides the number of successful N4 session report at the UPF.

b)	CC.

c)	On receipt of N4 session report ACK message (see 3GPP TS 23.502 [7] by the UPF. Each N4 session report ACK message indicating the successful N4 session report request triggers the counter to increment by 1.

d)	A single integer value.

e)	SM.N4SessionReportSucc

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.4.4	N9 interface related measurements

#### 5.4.4.1	Round-trip GTP Data Packet Delay on N9 interface

##### 5.4.4.1.1	Average round-trip N9 delay on PSA UPF

a)	This measurement provides the average round-trip delay on a N9 interface on PSA UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained as: the sum (the time when receiving a GTP echo reply message from the I-UPF at PSA UPF’s ingress GTP termination, minus time when sending the associated echo request message to I-UPF at the PSA UPF’s GTP egress termination) divided by total number of GTP echo reply message received at PSA UPF’s ingress GTP termination. This measurement is calculated for each DSCP.

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form GTP.RttDelayN9PsaUpfMean.DSCP
Where DSCP identifies the DSCP.

f)	EP_N9.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.4.1.2	Distribution of round-trip N9 delay on PSA UPF

a)	This measurement provides the distribution of delay on a N9 interface on PSA UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained by 1) calculating the RTT N9 delay by:  the time when receiving a GTP echo reply message from the I-UPF at PSA UPF’s ingress GTP termination, minus time when sending the associated echo request message to I-UPF at the PSA UPF’s GTP egress termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per DSCP.

d)	Each measurement is an integer representing the number of GTP echo messages measured with the delay within the range of the bin.

e)	The measurement name has the form GTP.RttDelayN9PsaUpfDist.Bin.DSCP
Where Bin indicates a delay range which is vendor specific, and DSCP identifies the DSCP.

f)	EP_N9.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.4.1.3	Average round-trip N9 delay on I-UPF

a)	This measurement provides the average round-trip delay on a N9 interface on I-UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained as: the sum (the time when receiving a GTP echo reply message from the PSA UPF at I-UPF’s ingress GTP termination, minus time when sending the associated echo request message to PSA UPF at the I-UPF’s GTP egress termination) divided by total number of GTP echo reply message received at I-UPF’s ingress GTP termination. This measurement is calculated for each DSCP.

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form GTP.RttDelayN9IUpfMean.DSCP
Where DSCP identifies the DSCP.

f)	EP_N9.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.4.1.4	Distribution of round-trip N9 delay on I-UPF

a)	This measurement provides the distribution of delay on a N9 interface on I-UPF. This measurement is split into subcounters per DSCP (Differentiated Services Code Point).

b)	DER (n=1).

c)	This measurement is obtained by 1) calculating the RTT N9 delay by:  the time when receiving a GTP echo reply message from the PSA UPF at I-UPF’s ingress GTP termination, minus time when sending the associated echo request message to PSA UPF at the I-UPF’s GTP egress termination; and 2) incrementing the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per DSCP.

d)	Each measurement is an integer representing the number of GTP echo messages measured with the delay within the range of the bin.

e)	The measurement name has the form GTP.RttDelayN9IUpfDist.Bin.DSCP
Where Bin indicates a delay range which is vendor specific, and DSCP identifies the DSCP.

f)	EP_N9.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.4.5	GTP packets delay in UPF

#### 5.4.5.1	DL GTP packets delay in UPF

##### 5.4.5.1.1	Average DL GTP packets delay in PSA UPF

a)	This measurement provides the average (arithmetic mean) DL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the DL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled DL GTP PDU to the gNB-CU-UP or I-UPF at the PSA UPF’s egress GTP termination, minus time of arrival of the same packet at PSA UPF’s ingress IP termination for N6 interface) divided by total number of sampled DL GTP PDUs sent to the gNB-CU-UP or I-UPF. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayDlInPsaUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayDlInPsaUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.1.2	Distribution of DL GTP packets delay in PSA UPF

a)	This measurement provides the distribution of DL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the DL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the DL delay for the sampled GTP PDU in I-UPF by:  time when sending the sampled DL GTP PDU to the gNB-CU-UP or I-UPF at the PSA UPF’s egress GTP termination, minus time of arrival of the same packet at PSA UPF’s ingress IP termination for N6 interface; and 3) incrementing the corresponding bin with the delay range where the result of 2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled DL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayDlInPsaUpfDist.Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayDlInPsaUpfDist.Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.1.3	Average DL GTP packets delay in I-UPF

a)	This measurement provides the average (arithmetic mean) DL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the DL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled DL GTP PDU to the gNB-CU-UP at the I-UPF’s egress GTP termination, minus time of arrival of the same packet at I-UPF’s ingress GTP termination for N9 interface) divided by total number of sampled DL GTP PDUs sent to the gNB-CU-UP. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayDlInIUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayDlInIUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.1.4	Distribution of DL GTP packets delay in I-UPF

a)	This measurement provides the distribution of DL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the DL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the DL delay for the sampled GTP PDU in I-UPF by:  time when sending the DL GTP PDU to the gNB-CU-UP at the I-UPF’s egress GTP termination, minus time of arrival of the same packet at I-UPF’s ingress GTP termination for N9 interface; and 3) incrementing the corresponding bin with the delay range where the result of 2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled DL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayDlInIUpfDist. Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayDlInIUpfDist. Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.5.2	UL GTP packets delay in UPF

##### 5.4.5.2.1	Average UL GTP packets delay in PSA UPF

a)	This measurement provides the average (arithmetic mean) UL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled UL data packet at the PSA UPF’s egress IP termination for N6 interface, minus time of arrival of the corresponding GTP SDU from N3 or N9 interface at PSA UPF’s ingress GTP termination) divided by total number of sampled UL data packets sent to N6 interface. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayUlInPsaUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayUlInPsaUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.2	Distribution of UL GTP packets delay in PSA UPF

a)	This measurement provides the distribution of UL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the UL delay for a GTP PDU in I-UPF by:  time when sending the sampled UL data packet at the PSA UPF’s egress IP termination for N6 interface, minus time of arrival of the corresponding GTP SDU from N3 or N9 interface at PSA UPF’s ingress GTP termination; and 3) incrementing the corresponding bin with the delay range where the result of 2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled UL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlInPsaUpfDist.Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayUlInPsaUpfDist.Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.3	Average UL GTP packets delay in I-UPF

a)	This measurement provides the average (arithmetic mean) UL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled UL GTP PDU to the PSA UPF at the I-UPF’s egress GTP termination, minus time of arrival of the same packet from N3 interface at I-UPF’s ingress GTP termination) divided by total number of sampled UL GTP PDUs sent to the PSA UPF. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayUlInIUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayUlInIUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.4	Distribution of UL GTP packets delay in I-UPF

a)	This measurement provides the distribution of UL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the UL delay for a GTP PDU in I-UPF by:  time when sending the sampled UL GTP PDU to the PSA UPF at the I-UPF’s egress GTP termination, minus time of arrival of the same packet from N3 interface at I-UPF’s ingress GTP termination; and 3) incrementing the corresponding bin with the delay range where the result of2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled UL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlInIUpfDist.Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayUlInIUpfDist.Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

### 5.4.6	Round-trip packet delay between PSA UPF and UE

#### 5.4.6.1	Average round-trip packet delay between PSA UPF and UE

a)	This measurement provides the average round-trip GTP packet delay between PSA UPF and UE (including UE processing time). This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are not time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

For each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4]):

- 	T1 that the DL GTP PDU was sent by the PSA UPF;

-	T6 that the corresponding UL GTP PDU (with QMP indicator and the corresponding sequence number) was received by the PSA UPF.

-	The 5QI and S-NSSAI associated to the DL GTP PDU.

The PSA UPF counts the number (N) of DL GTP PDUs encapsulated with QFI, TEID, sequence number and QMP indicator for each 5QI and each S-NSSAI respectively, and takes the following calculation for each 5QI and each S-NSSAI:

d)	Each measurement is a real representing the average delay in microseconds.

e)	GTP.RttDelayPsaUpfUeMean.5QI, where 5QI identifies the 5QI; 
GTP.RttDelayPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.6.2	Distribution of round-trip packet delay between PSA UPF and UE

a)	This measurement provides the distribution of round-trip GTP packet delay between PSA UPF and UE (including UE processing time). This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are not time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

For each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4]):

- 	T1 that the DL GTP PDU was sent by the PSA UPF;

-	T6 that the corresponding UL GTP PDU (with QMP indicator and the corresponding sequence number) was received by the PSA UPF.

-	The 5QI and S-NSSAI associated to the DL GTP PDU.

The PSA UPF 1) takes the following calculation for each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for each 5QI and each S-NSSAI respectively, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of DL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.RttDelayPsaUpfUeDist.5QI.Bin.DSCP, Where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.RttDelayPsaUpfUeDist.Bin.SNSSAI, Where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

## 5.5	Performance measurements for PCF

### 5.5.1	AM policy association related measurements

#### 5.5.1.1	Number of AM policy association requests

a)	This measurement provides the number of AM policy association requests received by the visiting PCF ((V-)PCF).

b)	CC

c)	On receipt by the PCF from the AMF of Npcf_AMPolicyControl_Create (see 3GPP TS 23.502 [7]). Each AM policy association request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	PA.PolicyAMAssoReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.5.1.2	Number of successful AM policy associations

a)	This measurement provides the number of successful AM policy associations at the visiting PCF ((V-)PCF).

b)	CC

c)	On transmission by the PCF to the AMF of Npcf_AMPolicyControl_Create response (see 3GPP TS 23.502 [7]). Each successful AM policy association is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	PA.PolicyAMAssoSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

### 5.5.2	SM policy association related measurements

#### 5.5.2.1	Number of SM policy association requests

a)	This measurement provides the number of SM policy association requests received by the PCF.

b)	CC

c)	On receipt by the PCF from the SMF of Npcf_SMPolicyControl_Create (see 3GPP TS 23.502 [7]). Each SM policy association request is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	PA.PolicySMAssoReq.SNSSAI

Where SNSSAI identifies the NSI;

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

#### 5.5.2.2	Number of successful SM policy associations

a)	This measurement provides the number of successful SM policy associations at the PCF.

b)	CC

c)	On transmission by the PCF to the SMF of Npcf_SMPolicyControl_Create response (see 3GPP TS 23.502 [7]). Each successful SM policy association is added to the relevant subcounter per NSI identifier (S-NSSAI).

d)	Each subcounter is an integer value

e)	PA.PolicySMAssoSucc.SNSSAI

Where SNSSAI identifies the NSI;

f)	PCFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

## 5.6	Performance measurements for UDM

### 5.6.1	Mean number of registered subscribers through UDM

a)	This measurement provides the mean number of registered  subscribers to UDM .

b)	SI

c)	This measurement is obtained by sampling at a unified interval the number of registered subscribers in a UDM and then taking the arithmetic mean.

d)	A single integer value

e)	RM.RegisteredSubUDMNbrMean

f)	UDMFunction

g)	Valid for packet switching

h)	5GS

### 5.6.2	Maximum number of registered subscribers through UDM

a)	This measurement provides the maximum number of registered  subscribers to UDM .

b)	SI

c)	This measurement is obtained by sampling at a unified interval the number of registered subscribers in the UDM and then taking the maximum.

d)	A single integer value

e)	RM.RegisteredSubUDMNbrMax

f)	UDMFunction

g)	Valid for packet switching

h)	5GS

## 5.7	Common performance measurements for NFs

### 5.7.1	VR usage of NF

#### 5.7.1.1	Virtual CPU usage

##### 5.7.1.1.1	Mean virtual CPU usage

a)	This measurement provides the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VcpuUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VcpuUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VCpuUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
GNBCUFunction (for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.7.1.2	Virtual memory usage

##### 5.7.1.2.1	Mean virtual memory usage

a)	This measurement provides the mean usage of the underlying virtualized memories for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VmemoryUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VmemoryUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VMemoryUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
GNBCUFunction (for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.7.1.3	Virtual disk usage

##### 5.7.1.3.1	Mean virtual disk usage

a)	This measurement provides the mean usage of the underlying virtualized disks for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the VdiskUsageMeanVnf.vComputeId measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of each received measurement from VNFC instance to the MOI(s) of NF(s). The measurement is generated by taking the weighted average of the values of the VdiskUsageMeanVnf.vComputeId measurement(s) whose measured object(s) are mapped to the MOI of the measured NF. The algorithm of the weighted average is vendor specific.

d)	A single integer value (Unit: %).

e)	VR.VDiskUsageMean

f)	GNBCUCPFunction (for 3 split scenario)
GNBCUUPFunction (for 3 split scenario)
GNBCUFunction (for 2 split scenario)
AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

g)	Valid for packet switched traffic.

h)	5GS.

## 5.8	Performance measurements for N3IWF

### 5.8.1	PDU Session Resource management

#### 5.8.1.1	PDU Session Resource setup

##### 5.8.1.1.1	Number of PDU Sessions requested to setup

a)	This measurement provides the number of PDU Sessions in the PDU SESSION RESOURCE SETUP REQUESTs received by the N3IWF from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	Receipt of PDU SESSION RESOURCE SETUP REQUEST message (see 3GPP TS 29.413 [22]) by the N3IWF from the AMF. Each PDU Session requested to setup increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupNon3GPPReq.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.8.1.1.2	Number of PDU Sessions successfully setup

a)	This measurement provides the number of PDU Sessions successfully setup by the N3IWF for the PDU SESSION RESOURCE SETUP REQUESTs received from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	Transmission of PDU SESSION RESOURCE SETUP RESPONSE message containing the "PDU Session Resource Setup Response List" IE (see 3GPP TS 38.413 [11]) by the N3IWF to the AMF. Each PDU Session listed in the "PDU Session Resource Setup Response List" IE increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupNon3GPPSucc.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.8.1.1.3	Number of PDU Sessions failed to setup

a)	This measurement provides the number of PDU Sessions failed to setup by the N3IWF for the PDU SESSION RESOURCE SETUP REQUESTs received from AMF. This measurement is split into subcounters per failure cause.

b)	CC.

c)	Transmission of PDU SESSION RESOURCE SETUP RESPONSE message containing the "PDU Session Resource Failed to Setup List" IE (see 3GPP TS 38.413 [11]) by the N3IWF to the AMF. Each PDU Session listed in the "PDU Session Resource Failed to Setup List" IE increments the relevant subcounter per failure cause (see clause 9.3.1.2 of 3GPP TS 38.413 [11]) by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionSetupNon3GPPFail.Cause.

Where Cause identifies the cause of the PDU Sessions Resource Setup failure, per the "PDU Session Resource Setup Unsuccessful Transfer" IE. Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11].

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.8.1.2	PDU Session Resource modification

##### 5.8.1.2.1	Number of PDU Sessions requested to modify

a)	This measurement provides the number of PDU Sessions in the PDU SESSION RESOURCE MODIFY REQUESTs received by the N3IWF from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	Receipt of PDU SESSION RESOURCE MODIFY REQUEST message (see 3GPP TS 29.413 [22]) by the N3IWF from the AMF. Each PDU Session requested to modify increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionModifyNon3GPPReq.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.8.1.2.2	Number of PDU Sessions successfully modified

a)	This measurement provides the number of PDU Sessions successfully modified by the N3IWF for the PDU SESSION RESOURCE MODIFY REQUESTs received from AMF. This measurement is split into subcounters per S-NSSAI.

b)	CC.

c)	Transmission of PDU SESSION RESOURCE MODIFY RESPONSE message containing the "PDU Session Resource Modify Response Item" IE (see 3GPP TS 38.413 [11]) by the N3IWF to the AMF. Each PDU Session listed in the "PDU Session Resource Modify Response Item" IE increments the relevant subcounter per S-NSSAI by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionModifyNon3GPPSucc.SNSSAI.

Where SNSSAI identifies the S-NSSAI.

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.8.1.2.3	Number of PDU Sessions failed to modify

a)	This measurement provides the number of PDU Sessions failed to modify by the N3IWF for the PDU SESSION RESOURCE MODIFY REQUESTs received from AMF. This measurement is split into subcounters per failure cause.

b)	CC.

c)	Transmission of PDU SESSION RESOURCE MODIFY RESPONSE message containing the "PDU Session Resource Failed to Modify List" IE (see 3GPP TS 38.413 [11]) by the N3IWF to the AMF. Each PDU Session listed in the "PDU Session Resource Failed to Modify List" IE increments the relevant subcounter per failure cause (see clause 9.3.1.2 of 3GPP TS 38.413 [11]) by 1.

d)	Each subcounter is an integer value.

e)	SM.PDUSessionModifyNon3GPPFail.Cause.

Where Cause identifies the cause of the PDU Sessions Resource modification failure, per the "PDU Session Resource Modify Unsuccessful Transfer" IE. Encoding of the Cause is defined in clause 9.3.1.2 of 3GPP TS 38.413 [11].

f)	N3IWFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

## 5.9	Performance measurements for NEF

### 5.9.1	Measurements related to application triggering

#### 5.9.1.1	Number of application trigger requests

a)	This measurement provides the number of application trigger requests received by the NEF from AF.

b)	CC

c)	Receipt of an Nnef_Trigger_Delivery request by the NEF from AF (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	AT.AppTriggerReq

f)	NEFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.9.1.2	Number of application trigger requests accepted for delivery

a)	This measurement provides the number of application trigger requests accepted for delivery to the UE.

b)	CC

c)	Transmission of Nnef_Trigger_Delivery response by the NEF to AF indicating the application trigger request has been accepted for delivery to the UE (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	AT.AppTriggerAcc

f)	NEFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.9.1.3	Number of application trigger requests rejected for delivery

a)	This measurement provides the number of application trigger requests rejected for delivery to the UE. This measurement is split into subcounters per error code (i.e., the response code as specified in clause 5.2.6 of TS 29.122 [23]).

b)	CC

c)	Transmission of an Nnef_Trigger_Delivery response by the NEF to AF indicating the application trigger request has been rejected for delivery to the UE (see 3GPP TS 23.502 [7]). Each said Nnef_Trigger_Delivery response increments the relevant subcounter per error code (i.e., the response code as specified in clause 5.2.6 of TS 29.122 [23]) by 1.

d)	Each subcounter is an integer value

e)	AT.AppTriggerRej.ErrorCode
Where the ErrorCode identifies the error code (i.e., response code as specified in clause 5.2.6 of TS 29.122 [23]) causing the rejection.

f)	NEFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.9.1.4	Number of application trigger delivery reports

a)	This measurement provides the number of application trigger delivery reports indicating the delivery results (e.g., success or failure) sent by the NEF to AF. This measurement is split into subcounters per delivery result (see the DeliveryResult specified in clause 5.7.2.2.3 of TS 29.122 [23]).

b)	CC

c)	Transmission of an Nnef_Trigger_DeliveryNotify message by the NEF to AF indicating the delivery result of the application trigger (see 3GPP TS 23.502 [7]). Each said Nnef_Trigger_DeliveryNotify message increments the relevant subcounter per delivery result by 1 (see the DeliveryResult specified in clause 5.7.2.2.3 of TS 29.122 [23]).

d)	Each subcounter is an integer value

e)	AT.AppTriggerRej.DeliveryResult
Where the DeliveryResult identifies the delivery result (i.e., the DeliveryResult specified in clause 5.7.2.2.3 of TS 29.122 [23]).

f)	NEFFunction

g)	Valid for packet switched traffic

h)	5GS

# 6	Measurements related to end-to-end 5G network and network slicing

## 6.1	Void

## 6.2	Virtualised resource usage measurement

a) This measurement provides the mean usage of virtualised resource (e.g. processor, memory, disk) in single network slice instance during the granularity period.

b) OM

c) This measurement is generated with .sum suffix for the usage of each virtualised NF (see 3GPP TS 32.426 [1]) related to single network slice instance by taking the weighted average. The algorithm of the weighted average is vendor specific.

d) Each measurement is an real value (Unit:%).

e) MeanProcessorUsage

MeanMemoryUsage

MeanDiskUsage

f) Performance measurement service.

g) Packet Switched.

h) 5GS

NOTE: The name of service in f) needs to align with the TS (e.g., 28.550) defining the management service.

Annex A (informative):
Use cases for performance measurements

# A.1	Monitoring of UL and DL user plane latency in NG-RAN

Satisfying low latency expectations for 5G services, such as URLLC, is one of the key tasks for the operator to meet service performance expectations. As the performance in UL and DL differs, it is important for operators to be able to monitor the UL and DL user plane latencies separately. With performance measurements allowing the operator to obtain or derive the UL and DL user plane latency information separately, the operators can pinpoint the services performance problems to specific problems in UL or DL.

The DL IP latency monitoring in NG-RAN refers to the transmission within gNB of IP packets arriving when there is no other prior data to be transmitted to the same UE in the gNB.

The average DL latency needs to be measured to give an general indication of the latency performance; further more the latency distributions (into bins with latency ranges) need to be measured, to tell the occurrences about the packets with each certain range of latency and better reflect the user experience.

Different network slices may have different requirements on the delay, so the delay needs to be measured for each S-NSSAI.

To further pinpoint performance problem detected, separate counters may be provided per mapped 5QI (which are particularly useful when the mapped 5QI is used by few services and users and the packet size does not vary much).

# A.2	Monitoring of UL and DL packet loss in NG-RAN

Keeping track of UL and DL packet loss in the NG-RAN is essential, since for certain services packets that are lost along the way through the system may have a noticeable impact on the end user. UL and DL packet loss measurements can be useful for evaluation, optimization and for performance assurance within the integrity area (user plane connection quality).

UL packet loss is a measure of packets dropped in the UE and the packets lost on the interfaces (air interface and F1-U interface). If parts of the gNB are deployed in a virtualized environment, it is important to measure also the F1-U UL interface packet loss in a separate measurement, to be able to pinpoint the reason for high packet loss.

# A.3	Monitoring of DL packet drop in NG-RAN

Keeping track of DL packet drops in the NG-RAN is essential, since for certain services packets that are dropped along the way through the system may have a noticeable impact on the end user. DL packet drop measurements can be useful for evaluation, optimization and for performance assurance of the network.

For gNBs that are deployed in a split architecture, e.g. when parts of a gNB are deployed in a virtualized environment, the DL packet drops may occur in two parts; the gNB CU-UP and the gNB DU. Therefore, it is important to measure this separately.

# A.4	Monitoring of UL and DL user plane delay in NG-RAN

Satisfying low packet delay is of prime concern for some services, particularly conversational services like speech and instant messaging. As the performance in UL and DL differs, it is important for operators to be able to monitor the UL and DL user plane delay separately. With performance measurements allowing the operator to obtain or derive the UL and DL user plane delay information separately, the operators can pinpoint the services performance problems to specific problems in UL or DL.

The DL delay monitoring in gNB refers to the  delay of any packet within NG-RAN, including air interface delay until the UE receives the packet. A gNB deployed in a split architecture, the user plane delay will occur in gNB-CU-UP, on the F1 interface, in gNB-DU and on the air interface. Therefore, the delay measurements related to the four segments needs to be monitored for the DL delay to pinpoint where end user impact from packet delay occurs.

The average DL delay needs to be measured to give an general indication of the delay performance; further more the delay distributions (into bins with delay ranges) need to be measured, to tell the occurrences about the packets with each certain range of delay and better reflect the user experience.

Different network slices may have different requirements on the delay, so the delay needs to be measured for each S-NSSAI.

To further pinpoint a detected delay performance problem, the packet delay measurement separation may be based on mapped 5QI (or for QCI in case of NR option 3).

# A.5	Monitoring of UE Context Release Request (gNB-DU initiated)

In order to monitor the stability of the network and detect the service/connection interruption caused by NGRAN, monitoring the UE Context Release Request initiated by gNB-DU is an effective method. Collecting the measurement information of the message and analysing the releasing cause conveyed in the message, operators could detect the stability of NG-RAN, and could decide a specific means to improve the NG-RAN performance.

# A.6	Monitoring of physical radio resource utilization

The physical radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. The physical radio resource utilization measurements should reflect the average usage and the usage distribution of the radio resource of the physical layer. The measurements can make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing.

Network slicing is an important feature in 5GS, monitoring physical radio resource utilization per S-NSSAI is helpful for opeators to be aware of the load.

# A.7	Monitoring of RRC connection number

The number of the users in RRC connected and inactive mode need to be monitored as it reflects the load of the radio network, the operators can use this information for dynamic frequency resource allocation or load balance purpose. Moreover, it is an important factor to be evaluated in the radio network capacity enhancement decision-making.

# A.8	Monitoring of UE Context Release

In order to monitor the stability of the network and detect the service/connection interruption caused by NG-RAN, monitoring the UE Context Release Request initiated by gNB-DU and UE Context Release Command initiated by gNB-CU is an effective method. Collecting the measurement information of the message and analysing the releasing cause conveyed in the message, operators could detect the stability of NG-RAN, and could decide a specific means to improve the NG-RAN performance.

# A.9	Monitoring of UE Throughput in NG-RAN

Keeping track of UL and DL UE throughput in the NG-RAN is essential, to ensure end user satisfaction and well-functioning and well configured cells and scheduling features.

The restricted UE throughput per mapped 5QI will show the scheduling efficiency and QoS priority handling in the gNB and the ratio between unrestricted and restricted volume will show the gNB ability to handle small data transfers efficiently.

To be able to monitor the spread of throughput within the cell, and estimate the ratio of satisfied users, the throughput distribution measurement can be used.

When network slicing is supported by the NG-RAN, multiple NSIs may be supported. The UL and DL UE throughput for each NSI is then of importance to the operator to pinpoint a specific performance problem.

# A.10	Monitoring of Unrestricted volume in NG-RAN

Measuring the share of unrestricted user data volume in the NG-RAN is important, to show the gNB ability to handle small data transfers efficiently and to see how large share of the volume that is part of the UE throughput measurement. It is not meaningful to measure throughput for data transfers so small that they fit in one single slot but it is still important to know how much such transfers can be handled by the gNB.

When network slicing is supported by the NG-RAN, multiple NSIs may be supported. The share of unrestricted volume for each NSI is then of importance to the operator to pinpoint a specific performance problem.

# A.11	N3 data volume related measurements

N3 related measurements are used to measure data volume on N3 interface including incoming and outgoing of GTP data packets in total and per QoS level without counting the mandatory part of the GTP-U header

It is useful to analyse transport bandwidth usage of N3 interface in total and QoS level granularity. If the transport bandwidth usage is too high, more bandwidth should be deployed, or load balance should be considered according to core network dimension if there are multiple UPFs connected to multiple gNodeBs. Decision on the additional bandwidth provisioning or load balancing can be made in more fine grained level due to the QoS level measurement.

So it is necessary to define N3 related measurements.

# A.12	N6 related measurements

N6 related measurements are used to measure data volume on N6 interface including incoming and outgoing of IP data packets.

It is useful to analyse transport bandwidth usage of N6 interface. If the transport bandwidth usage is too high, more bandwidth should be deployed.

So it is necessary to define N6 related measurements.

# A.13	Registration related measurements

A UE needs to register with the 5GS to get authorization to receive services, to enable mobility tracking and to enable reachability. The following registration types are defined:

-	Initial Registration to the 5GS;

-	Mobility Registration Update (upon changing to a new Tracking Area (TA) outside the UE's Registration Area in both CM-CONNECTED and CM-IDLE state, or when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure with or without changing to a new TA);

-	Periodic Registration Update (due to a predefined time period of inactivity); and

-	Emergency Registration (i.e. the UE is in limited service state).

The registration may be via 3GPP access, or via non-3GPP access.

The performance of registration for each registration type needs to be monitored by the operator since it is relevant to whether the end user can use the service of 5GS or a specific network slice. The performance of registration via 3GPP access and non-3GPP access needs to be monitored respectively.

# A.14	PDU session establishment related measurements

The PDU session establishment is one of essential procedures for 5G network. The performance of PDU session establishment directly impacts the QoS of the network and the QoE of the end users. Therefore, the performance measurements are needed to reflect the performance of the PDU session establishment.

The PDU sessions are created in two scerarios; Non-roaming/LBO-roaming and HR-roaming, and are created by (V-)SMF and H-SMF respectively.

On receipt by the (V-)SMF from AMF of Nsmf_PDUSession_CreateSMContex Request, the PDU session is created in non-roaming/LBO-roaming scenario and HR-roaming scenario.

On receipt by the H-SMF from V-SMF of Nsmf_PDUSession_Create Request, the PDU session is created in HR-roaming scenario.

The number and success rate of PDU session creations, the number of PDU sessions running on the SMF are some of the basic performance measurements to monitor the performance of the PDU session establishment. And the performance measurements of failed PDU session creations are helpful to solve the network issues in case the performance is below the expectation.

# A.15	Policy association related measurements

To ensure the UE properly use the services provided by 5GS, the UE needs to be associated with a policy. The policies are categorized into AM policy and SM policy, and are executed by AMF and SMF respectively. Both kinds of policies are provisioned by PCF.

The AM policy association needs to be established in case the UE initially registers to the network or the UE needs the AMF re-allocation.

The SM policy association needs to be established when the UE requests a PDU Session Establishment.

The policy association establishment is the essential steps allowing the UE to be served by the 5GS under the designed policies, therefore it needs to be monitored.

# A.16	Monitoring of PDU session resource setup in NG-RAN

The PDU Session Resource Setup procedure in NG-RAN is to assign resources on Uu and NG-U for one or several PDU session resources and the corresponding QoS flows, and to setup corresponding Data Radio Bearers for a given UE.

The PDU Session Resource Setup is one of the most key procedure to allocate resources in the NG-RAN to the UE per the QoS requirements for the NSSAI(s). Whether or not the PDU Session Resource is successfully setup for the NSSAI(s) has direct impact to the user experience. The failed PDU Session Resource Setup may directly cause the service failure for an end user. So, the performance related to the PDU Session Resource Setup for the gNB needs to be monitored.

# A.17	Monitoring of handovers

Mobility is one of the most significant feature of the mobile networks, and handover is one typical action of the mobility. The handover failure would cause service discontinuation, thus the performance of the handover has direct impact to the user experience.The handover procedure includes handover preparation, handover resource allocation and handover execution, and the performance related to handover needs to be monitored for each phase. The resources (e.g., PDU Session Resource) need to be prepared and allocated for a handover according to the QoS requirements for each NSI identifier (S-NSSAI).

The handover may occur intra-gNB and inter-gNB for 5G networks, and for inter-gNB case the handover could happen via NG or Xn interface.

For the handover failures, the measurements with specific causes are required for trouble shooting.

# A.18	Monitor of BLER performance

The TB Error Rate in UL/DL can directly reflect the BLER, and has an influence on MCS selection and user throughput. It can be helpful to estimate the performance of radio resource management like radio resource schedulein transport layer and be helpful in trouble shooting. Furthermore, they should be taken into account to optimize the system performance. To obtain TB Error Rate by calculating, the number of total and error TBs transmitted in a cell should be monitored.

# A.19	Monitor of ARQ and HARQ performance

Reliable Packet Delivery is one of the important Performance factor for a better User experience. HARQ re-transmissions at the MAC layer ensure reliable packet delivery

In addition, RLC can be configured to operate in acknowledged mode for those applications that need very low packet drops and can tolerate a slightly higher delay from RLC re-transmissions.

If a MAC PDU is not delivered, HARQ takes care of re-transmitting (up to a maximum configurable number). If all the re-transmissions fail at MAC layer, and if RLC is configured to operate in acknowledged mode, RLC’s ARQ mechanism will take care of any residual packet errors.

It is important to:

a)	 Maintain the block error rate or packet error rate within tolerable limits.

b)	 Ensure that HARQ re-transmissions take care of most packet errors, instead of relying on RLC layer re-transmissions (which would increase the delay).

So, it is important to monitor the performance of these schemes.

HARQ Performance if viewed at MCS (Modulation Coded Scheme) can help in monitoring the MCS Performance also.

# A.20	Monitoring of PDU session modifications

The PDU session may need to be modified by various causes (see TS 29.502 [14]), whether a PDU session can be successful modified may impact the communication services supported by the PDU session. Therefore the performance of PDU session modification procedures need to be monitored. Besides PDU session modification requests and successes, the PDU session modification failures with specific causes need to be monitored for trouble shooting.

# A.21	Monitoring of PDU session releases

The PDU session release may be released by the UE, SMF or AMF. When a PDU session is released by an unexpected reason, the user service would be impacted. The PDU session releases initiated by the UE and SMF are usually triggered by normal reason (e.g., UE deregistration, under request from DN, etc.). The PDU session releases initiated by AMF may be due to an abnormal reason (e.g., mismatch of PDU Session status between UE and AMF). Therefore the PDU session releases initiated by AMF need to be monitored.

# A.22	Monitoring of N4 session management

UPF handles the user plane path of PDU Sessions. UPF selection is performed by SMF, and deployments where a UPF is controlled either by a single SMF or multiple SMFs (for different PDU Sessions) are supported.

The SMF uses N4 session management procedures to control the traffic detection, traffic reporting, QoS enforcement and traffic routing of the UPF.

The N4 session management procedures include N4 Session Establishment procedure, N4 session Modification procedure and N4 session release procedure.

If PDF fails to handle the user plane path for a PDU session, the user service will be impacted. So the performance about N4 session management needs to be monitored.

# A.23	Use case of VR measurements for NF

In case the NF is virtualized, the performance of an NF may be impacted by the underlying VRs (i.e., virtual CPUs, virtual memories and virtual storages). To enable the operator to analyze the impact of the VRs to the performance of the NF, the performance of the virtual compute, virtual memory and virtual disk also needs to be monitored. The usage is the key measurement for the performance of the VR, it can tell whether the VR is overloaded and whether the VR is efficiently utilized. By correlating the VR related measurements with the performance measurement of the NF, the operator can know whether the NF performance issue is caused by the VRs or not. When necessary, the operator may take appropriate action to solve the performance issue of the NF, for example, to scale in/out the VNF instance(s) that realizes the NF, or switch on/off the auto-scaling for the VNF instance(s).

# A.24	Monitoring of DRB Setup in NG-RAN

The DRB setup procedure in NG-RAN is to assign resources in gNB and on the air interface (Uu) for one or several DRBs that will handle the QoS flows requested to setup by the core network. The gNB may map several QoS flows to the same DRB so there is no one-to-one mapping between the flows and DRBs.

At DRB setup, the gNB will handle all QoS flows mapped to one DRB the same (mapped 5QI). A QoS flow that is at a later stage mapped to an already setup existing DRB will not increment the DRB setup counters.

The DRB setup is one of the most key procedures to allocate resources in the NG-RAN to the UE per the QoS requirements. Whether or not the DRB is successfully setup has direct impact to the user experience. A failed DRB setup may directly cause service failure or degradation for an end user. So, the performance related to the DRB setup for the gNB needs to be monitored per supported mapped 5QI and per S-NSSAI.

# A.25	Monitoring of PDCP data volume measurements

In 5GS, Cell PDCP data volume is a useful measurement which represents the real data traffic for each cell. The monitor of the Cell PDCP data volume could provide operators the traffic information and is useful for operators to do cell load evaluation, load balancing and cell capacity planning.

In addition, in scenarios of dual connectivity, for split bearers, PDCP data is transferred between the MN and the SN via the MN-SN user plane interface. To monitor the real PDCP data volume transmitted in MN and SN in MR-DC scenarios, the data volume transferred between MN and SN should be counted.

# A.26	Monitoring of RF performance

RF Performance includes performance of Power Resource Utilization, RF signal, TA, interference and etc.  Monitoring of the performance measurements can help to reflect the cell loading information and abnormal conditions.

Monitoring of the quality of RF signal in the cell is useful for the purpose of network planning and overall service quality assessment. Measurements of Channel Quality Indicator (CQI) reported by UEs is a useful metric reflecting RF signal quality and service quality.

# A.27	Monitoring of RF measurements

MCS represents the modulation and coding schemes scheduled for the physical resources by NG-RAN. The measurements of MCS distribution is a useful metric reflecting the efficiency for PDSCH and PUSCH RBs. It is helpful for operator to optimize the scheduling of physical resources to improve the network efficiency and overall service quality.

# A.28	Monitor of QoS flow release

QoS flow is the key and limited resource for NG-RAN to deliver services. The release of the QoS flow needs to be monitored as:

-	an abnormal release of the QoS flow will cause the call(/session) drop, which directly impacts the QoS delivered by the networks, and the satisfaction degree of the end user;

-	a successfully released QoS flow can be used to setup other requested calls(/sessions). The QoS flow failed to be released will still occupy the limited resource and hence it can not be used to admit other requested calls(/sessions).

From a retainability measurement aspect, QoS flows do not need to be released because they are inactive, they can be kept to give fast access when new data arrives.

To define (from a QoS flow release measurement point of view) if a QoS flow is considered active or not, the QoS flow can be divided into two groups:

For QoS flows with bursty flow, a  UE is said to be "in session" if any QoS flow data on a Data Radio Bearer (UL or DL) has been transferred during the last 100 ms. 
For QoS flows with continuous flow, the  QoS flow (and the UE) is always seen as being "in session" in the context of this measurement, and the session  time is increased from the first data transmission on the QoS flow until 100 ms after the last data transmission on the QoS flow.

How to decide for a particular QoS flow if the QoS flow is of type bursty flow or continuous flow is outside the scope of this document.

The specific reason causing the abnormal and failed release of the QoS flow is required in order to find out the problem and ascertain the solutions. And due to different priority and tolerance for different service type with different QoS level in the networks, the monitor needs to be opened on each service type with QoS level.

The QoS flow can be released by PDU Session Resource Release procedure, UE Context Release procedure, Reset procedure either initiated by NG-RAN or AMF and NG Path Switch procedure (see 3GPP TS 38.413 [11]).

So performance measurements related to QoS flow Release (see 3GPP TS 38.413 [11]) and UE Context Release (see 3GPP TS 38.413 [11]) procedure for each service type with QoS level are necessary to support the monitor of QoS flow release.

# A.29	Monitor of call (/session) setup performance

Call(/session) setup is one of most important step to start delivering services by the networks to users.

The success or failure of a call(/session) setup directly impacts the quality level for delivering the service by the networks, and also the feeling of the end user. So the success or failure of call(/session) setup needs be monitored, this can be achieved by the calculation of call setup success rate which gives a direct view to evaluate the call setup performance, and the analysis of the specific reason causing the failure to find out the problem and ascertain the solutions.

In addition, the time duration of the call(/session) setup need to be monitored as it impacts the end user experience, and by comparison with operator’s benchmark requirements, the optimization may be required according the performance.

To support the monitor of success or failure of the call(/session) setup, the performance measurements related to QoS flow setup (See 3GPP TS 38.413[18]) and Initial UE Context Setup (See 3GPP TS 38.413[18]) procedure for each QoS level are required.

# A.30	Monitor of QoS flow release

QoS flow is the key and limited resource for NG-RAN to deliver services. The release of the QoS flow needs to be monitored:

-	an abnormal release of the QoS flow will cause the call(/session) drop, which directly impacts the QoS delivered by the networks, and the satisfaction degree of the end user;

-	a successfully released QoS flow can be used to setup other requested calls(/sessions). The QoS flow failed to be released will still occupy the limited resource and hence it can not be used to admit other requested calls(/sessions).

The specific reason causing the abnormal and failed release of the QoS flow is required in order to find out the problem and ascertain the solutions.

# A.31	Monitoring of QoS flows for SMF

To support a service for a UE, the QoS flow supporting the specific QoS needs to be added or modified. If the QoS flow fails to be added or modified, the user service cannot be conducted or the QoS cannot be met. So the QoS flow addition and modification need to be monitored.

Furthermore, in order to know the UE traffic pattern at SMF, it is necessary to monitor the peak and mean number of ongoing QoS flows for each granularity period.

# A.32	Monitoring of service requests

The Service Request procedure is initiated via 3GPP access::

by the UE in CM-IDLE state in order to send uplink signalling messages, user data, or as a response to a network paging request; or

by the network when the network needs to signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane connection activation for PDU Session(s) to deliver mobile terminating user data) with a UE.

The Service Request procedure via Untrusted non-3GPP Access is used by a UE:

-	in CM-IDLE state over non-3GPP access to request the re-establishment of the NAS signalling connection and the re-establishment of the user plane for all or some of the PDU Sessions which are associated to non-3GPP access; and

-	in CM-CONNECTED state over non-3GPP access to request the re-establishment of the user plane for one or more PDU Sessions which are associated to non-3GPP access.

The Service Request procedures via 3GPP access and via Untrusted non-3GPP Access need to be monitored respectively in order to know the performance of the 5G network in terms of providing services to the UEs.

# A.33	Monitoring of DL PDCP UE buffered throughput

To monitor DL PDCP buffered throughput per UE and bearer is essential, to ensure end user satisfaction and well functioning and well configured cells.  If an end user often experiences low quality during use of a service, the end-user might change wireless subscription provider, i.e. loss of income for the network operator.

# A.34	Monitoring of RRC connection setup in NG-RAN

RRC connection setup is one of most important step to start delivering services by the networks to users, (see 3GPP TS 38.331 [20]).

Whether or not the RRC connection is successfully setup has direct impact to the user experience. A failed RRC connection setup may cause service failure or failure in updating tracking area information for an end user. So, the performance related to the RRC connection setup for the gNB needs to be monitored. This can be achieved by the calculation of RRC connection setup success rate (number of successful / number of attempt) which gives a direct view to evaluate the RRC connection setup performance, and the analysis of the specific reason causing the failure to find out the problem and ascertain the solutions.

Since the intended service is not yet know when establishing the RRC connection, it is not possible to do separation between QoS classes or S-NSSAIs.

# A.35	Monitoring of UE associated NG signalling connection setup in NG-RAN

The NG signalling connection setup procedure in NG-RAN is to establish signalling connection between gNB and AMF for a given UE.

Whether or not the NG signalling connection is successfully setup has direct impact on the user experience. A failed NG signalling connection setup may directly cause service failure or failure in updating tracking area information for an end user. So, the performance related to the NG signalling connection setup for the gNB needs to be monitored. This can be achieved by the calculation of success rate for UE associated NG signalling connection (number of successful / number of attempted) which gives a direct view to evaluate the setup performance.

Since the intended service is not yet know when establishing the UE associated NG signalling connection, it is not possible to do separation between QoS classes or S-NSSAI.

# A.36	Monitoring of PDCP data volume per interface

In 5GS, PDCP data volume is a useful measurement which represents the real data traffic towards each GNBDUFunction (F1-U interface), each external gNB-CU-UP (Xn-U interface) and each external eNB (X2-U interface). The monitoring of the PDCP data volume could provide operators with traffic information and is a useful measure in performance assurance within integrity area (user plane connection quality) and in energy efficiency evaluation.

# A.37	Monitoring of RRC connection re-establishment

The failed RRC connection re-establishment will cause the call (/session) drop, which directly impacts the QoS delivered by the networks.

# A.38	Monitoring of RRC connection resuming

RRC connection resuming is one of important step to start delivering services by the networks to users or for RNA update.

The success or failure of a RRC connection resuming directly impacts the quality level for delivering the service by the networks, and also the feeling of the end user. So the success or failure of RRC connection resuming needs be monitored, this can be achieved by the calculation of RRC connection resuming success rate which gives a direct view to evaluate the resume performance, and the analysis of the specific reason causing the failure to find out the problem and ascertain the solutions.

# A.39	Monitoring of inter-AMF handovers

The handover could occur from a source NG-RAN to the target NG-RAN that are served by different AMFs. During the handover, the PDU sessions and QoS flows need to be setup in the target side. The failure of the PDU session setup or QoS flow setup during the inter-AMF handover has direct impact to the user’s experience. Therefore, it is necessary to monitor the performance related to PDU session setup or QoS flow setup for the Inter-AMF handover.

# A.40	Monitoring of incoming/outgoing GTP packet loss on N3

Keeping track of GTP data packet loss over N3 is essential, since for certain services packets that are lost along the way through the system may have a noticeable impact on the end user. Incoming/outgoing GTP data packet loss measurements can be useful for evaluation, optimization, and performance assurance between gNB and UPF in the core.  It is also important for the performance measurement of end-to-end point of view from UE to UPF. Performance degradation can happen any point although the focus is centered more at UE and RAN.

# A.41	Monitoring of round-trip GTP packet delay on N3

Keeping track of GTP data packet delay over N3 is essential, since for certain services packet delay along the way through the system may have a noticeable impact on the end user. Incoming/outgoing GTP data packet delay measurements can be useful for evaluation, optimization, and performance assurance between gNB and UPF in the core.  It is also important for the performance measurement of end-to-end point of view from UE to UPF. Performance degradation can happen any point although the focus is centered more at UE and RAN.

# A.42	Monitoring of PDU session resource management for untrusted non-3GPP access

The PDU Session Resource management procedure for Untrusted non-3GPP Access is to manage resources in Untrusted non-3GPP Access for the PDU sessions.

The PDU Session Resource needs to be setup or modified via the Untrusted non-3GPP Access for the UE per the QoS requirements for the NSSAI(s).

The PDU Session Resource setup and modification via the Untrusted non-3GPP Access have direct impact to the user experience. So, the performance related to the PDU Session Resource setup and modification via the Untrusted non-3GPP Access needs to be monitored.

# A.43	Monitor of DRB release

DRB is the key and limited resource for NG-RAN to deliver services. Once a QoS flow reaches a gNB it will trigger setup of a new DRB or it will be mapped to an existing DRB. The decision on how to map QoS flows into new or existing DRBs is taken at the CU-CP. CU-CP also defines one set of QoS parameters (one 5QI) for the DRB. If a QoS flow is mapped to an existing DRB, the packets belonging to that QoS flow are not treated with the 5QI of the QoS flow, but they are treated with the mapped 5QI of the DRB.

The release of the DRB needs to be monitored as:

-	an abnormal release of the DRB will cause the call(/session) drop, which directly impacts the QoS and slice delivered by the network, and the satisfaction degree of the end user;

-	a successfully released DRB can be used to setup other requested calls(/sessions). The DRB failed to be released will still occupy the limited resource and hence it can not be used to admit other requested calls(/sessions).

From a retainability measurement aspect, DRBs do not need to be released because they are inactive, they can be kept to give fast access when new data arrives.

To define (from a DRB release measurement point of view) if a DRB is considered active or not, the DRB can be divided into two groups:

- For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms.

- For DRBs with continuous flow, the DRB is always seen as being active in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

Due to different priority and tolerance for different service type with different QoS level in the DRB, the measurement needs to be performed per mapped 5QI, to be able to judge the result.

Similarly, the abnormal and failed DRB releases will affect different Service Level Agreements in the networks. Therefore, each S-NSSAI needs to be monitored.

The DRB can be released by PDU Session Resource Release procedure, UE Context Release procedure, Reset procedure either initiated by NG-RAN or AMF and NG Path Switch procedure (see 3GPP TS 38.413 [11]).

Therefore, performance measurements related to DRB Release (see 3GPP TS 38.413 [11]) and UE Context Release (see 3GPP TS 38.413 [11]) procedure for each QoS level (mapped 5QI) and each S-NSSAI are necessary to support the monitor of DRB release.

# A.44	Monitoring of application triggering

When the AF needs to trigger the UE for some actions, the AF requests the NEF to send an application trigger to the UE.

The application in the UE may perform actions as indicated by the Trigger payload when the Triggered payload is received at the UE. For example initiation of immediate or later communication with the application server based on the information contained in the Trigger payload, which includes the PDU Session Establishment procedure if the related PDU Session is not already established.

Therefore, to ensure the application run normally, the network needs to successfully deliver the application trigger to the UE.

# A.45	Monitoring of SMS over NAS

The SMS can be transferred over NAS in 5G networks. To enable SMS over NAS transporting, the UE includes an "SMS supported" indication in Registration Request indicating the UE's capability for SMS over NAS transport, and in the Registration Accept the networks indicates to the UE whether the network allows the SMS message delivery over NAS. If the SMS over NAS is allowed by the network, the SMS messages can be originated or terminated by the UE, via 3GPP or non-3GPP access.

The performance of SMS over NAS, as a service provided to the end users, has direct impact to user experience and user satisfaction, and thus needs to be monitored. For this purpose, the measurements for the following aspects are needed:

- 	registration procedure for SMS over NAS to reflect whether the UEs are allowed or disallowed to send or receive SMS messages over NAS;

- 	the number of SMS messages requested to be sent or received over NAS and the number of SMS messages successfully delivered over NAS, which can directly reflect whether the services can be successfully delivered to the users.

# A.46	Monitoring of round-trip GTP packet delay on N9

When I-UPF exists, the delay over N9 interface for DL/UL data packets is essential as part of the e2e packet delay, since for certain services packet delay along the way through the system may have a noticeable impact on the end user. RTT GTP data packet delay measurements for DL/UL data packets can be useful for evaluation, optimization, and performance assurance for the N9 interface between PSA UPF and I-UPF.  It is also important for the performance measurement of end-to-end point of view from UE to PSA UPF. Performance degradation can happen any point, so the RTT N9 delay needs to be measured at PSA UPF and I-UPF respectively. The GTP packets are prioritized for transmitting using DSCPs, so it is necessary to measure the round-trip GTP packet delay per DSCP.

# A.47	Monitoring of GTP packets delay in UPF

The GTP packets may be delayed on the interfaces and in the NFs. For some services (such as URLLC sevices), the end to end packet delay in the network has clear impact to users’ experience. The GTP packets delay in the UPF, as one segment of the end to end delay in the network, needs to be measured in order to indicate where the performance degradation happens. The GTP packets are prioritized for transmitting for different 5QIs and S-NSSAIs, so the measurements per 5QI and S-NSSAI are needed.

# A.48	Monitoring of round-trip delay between PSA UPF and UE

The end to end delay in 5G networks between UE and PSA UPF has direct impact to users’ experience for some types of services (e.g., URLLC). In case the PSA UPF and NG-RAN are not time synchronised, the round-trip delay between PSA UPF and UE can be measured at PSA UPF.

The measurements on the round-trip delay between PSA UPF and NE can be used to evaluate the user plane delay performance in 5G networks and users’ experience.

# A.49	Monitoring of Power, Energy and Environmental (PEE) parameters

Power, Energy and Environmental (PEE) parameters, combined with data volume measurements, are valuable information for operators to measure the energy efficiency of their 5G network. Hence it is necessary to define PEE parameters related to 5G network such as power, energy, temperature, voltage, current and humidity.

# A.50	Monitoring of UE configuration update

To ensure the UE properly use the services provided by 5GS, the UE needs to update access and mobility management related parameters decided and provided by the AMF.

The UE configuration update is the essential steps allowing the UE to be served by the 5GS under the changed configuration, therefore it needs to be monitored.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-12 | SA#82 | SP-181047 | 0002 | 1 | F | Remove the redundant measurement of end-to-end latency KPI | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0024 | 1 | F | Correction of the Packet loss measurements | 15.1.0 |
| 2018-12 | SA#82 | SP-181048 | 0004 | 2 | B | Add PDU Session Resource setup related measurements for gNB | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0005 | 2 | B | Add inter-gNB handover related measurements | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0011 | 1 | B | Add use case and definitions of TB related measurements | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0016 | 1 | F | PM terms for NSI and NSSI | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0017 | 1 | B | Add PDU session modification related measurements for SMF | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0018 | 1 | B | Add PDU session release related measurements for SMF | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0019 | - | B | Add N4 Session Establishment related measurements for UPF | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0020 | 1 | B | Add NF performance measurements related to VR | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0034 | 1 | B | Add DRB setup related measurements and UC for gNB | 16.0.0 |
| 2018-12 | SA#82 | SP-181048 | 0036 | - | B | Add use case and definitions of PDCP data volume measurements | 16.0.0 |
| 2019-03 | SA#83 | SP-190111 | 0043 | 1 | B | Add measurements of CQI distribution | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0044 | 1 | B | Add measurements of MCS distribution | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0045 | 1 | B | Add measurements related to QoS flow retainability | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0046 | 1 | B | Add measurements of PDCP data volume in DC-scenarios | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0047 | 1 | B | Add QoS flow related performance measurements | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0049 | 1 | F | Clean-up | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0050 | 1 | B | Add QoS flow related measurements for SMF | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0051 | 1 | B | Add service requests related measurements for AMF | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0052 | 1 | B | Add use case for PDCP end user throughput measurements | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0053 | 1 | B | Add measurements and UC related to RRC connection establishment | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0054 | - | B | Add measurements and UC related to setup of UE associated NG signalling connection | 16.1.0 |
| 2019-03 | SA#83 | SP-190119 | 0055 | 1 | B | Add PDCP data volume measurements for EE | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0060 | 1 | B | Add measurements of RRC connection re-establishment | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0061 | 1 | B | Add measurements of RRC connection resuming | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0065 | - | B | Add use case and definitions of QoS level measurement over N3 | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0067 | - | B | Add measurements related to registration via untrusted non-3GPP Access for AMF | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0068 | 1 | B | Add measurements related to inter-AMF handover | 16.1.0 |
| 2019-03 | SA#83 | SP-190111 | 0070 | 2 | B | Add radio resource utilization of network slice instance measurements | 16.1.0 |
| 2019-03 | SA#83 | SP-190122 | 0072 | 1 | A | Correction of percentage unrestricted volume measurements | 16.1.0 |
| 2019-06 | SA#84 | SP-190371 | 0074 | 1 | B | Add use case and definitions of packet loss measurement over N3 | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0075 | 1 | B | Add use case and definitions of packet delay measurement over N3 | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0076 | 1 | B | Add measurements related to Service Requests via Untrusted non-3GPP Access | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0077 | - | B | Add measurements related to PDU session resource management via Untrusted non-3GPP Access | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0079 | 1 | B | Add measurements related to inter gNB Handover | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0080 | 1 | B | Add measurements related to intra gNB Handover | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0082 | - | F | Correct DRBs successfully setup measurement | 16.2.0 |
| 2019-06 | SA#84 | SP-190375 | 0084 | - | A | Correction of F1 measurements | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0085 | 1 | F | Correction of monitoring of PDCP data volume measurements | 16.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0086 | 2 | F | Correction of PRB measurements | 16.2.0 |
| 2019-09 | SA#85 | SP-190746 | 0081 | 3 | B | Add measurements related to DRB retainability | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0088 | - | B | Add measurements related to application triggering | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0089 | 1 | B | Add measurements related to SMS over NAS | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0090 | - | F | Correction of clause titles | 16.3.0 |
| 2019-09 | SA#85 | SP-190748 | 0092 | - | A | Correct the definition of Average delay DL air-interface measurement | 16.3.0 |
| 2019-09 | SA#85 | SP-190751 | 0094 | 1 | A | Correction on kbits abbreviation | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0095 | 1 | B | Add measurement of SMF for N4 interface | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0096 | - | B | Add measurement of UPF for N4 interface | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0097 | 1 | B | Add measurement of paging | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0098 | 1 | F | Update performance measurements for UDM | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0101 | 2 | B | Update and add delay related measurements for NG-RAN | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0102 | 2 | B | Update and add latency related measurements for NG-RAN | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0104 | 1 | B | Update and add measurements related to RTT N3 delay for DL data packets on UPF | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0105 | 2 | B | Add measurements related to RTT N9 delay for DL and UL data packets | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0106 | 2 | B | Add measurements related to GTP packet delay within UPF | 16.3.0 |
| 2019-09 | SA#85 | SP-190746 | 0109 | 1 | B | Add measurements related to round-trip delay between PSA UPF and UE | 16.3.0 |
| 2019-09 | SA#85 | SP-190755 | 0111 | 2 | B | Add Power, Energy and Environmental (PEE) measurements and related use case description | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0112 | 1 | B | Add Number of PDU session creation in HR roaming scenario | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0113 | 1 | F | Update the measurement related to Number of PDU session creation | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0114 | 1 | B | Add UE Configuration Update procedure related measurements | 16.3.0 |
| 2019-09 | SA#85 | SP-190748 | 0117 | - | F | Correction of QoS flow monitoring for SMF | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0122 | 1 | F | Modify PM definition for non-split NG-RAN scenario | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0123 | 1 | F | Modify DRB setup management related measurements | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0124 | 1 | F | Modify PDU Sessions setup related measurements | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0127 | 2 | B | Add a description of Inter-gNB handover Execution time measurement | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0128 | 2 | B | Add a description of PDU session establishment time measurement | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0129 | 2 | B | Add measurements related to extended monitoring of the retainability for the 5QI 1 QoS Flow services | 16.3.0 |
| 2019-09 | SA#85 | SP-190747 | 0131 | - | B | Add new definition of measurements related to AMF registration procedure set-up time measurement | 16.3.0 |