3GPP TS 28.554 V16.2.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;Management and orchestration;

5G end to end Key Performance Indicators (KPI)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G,KPI,management,orchestration

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	End to end KPI concept and overview	6

5	KPI definitions template	7

6	End to end KPI definitions	8

6.1	KPI Overview	8

6.2	Accessibility KPI	8

6.2.1	Registered Subscribers of Network and Network Slice Instance through AMF	8

6.2.2	Registered Subscribers of Network and Network Slice Instance through UDM	8

6.2.3	Registration success rate of one single network slice instance	9

6.2.4	DRB Accessibility for UE services	9

6.2.5	PDU session Establishment Success Rate of one network slice (S-NSSAI)	10

6.3	Integrity KPI	10

6.3.1	Latency of 5G Network	10

6.3.1.0	KPI categories	10

6.3.1.1	Downlink latency in gNB-DU	10

6.3.1.2	Integrated downlink delay in RAN	11

6.3.2	Upstream throughput for network and Network Slice Instance	11

6.3.3	Downstream throughput for Single Network Slice Instance	11

6.3.4	Upstream Throughput at N3 interface	12

6.3.5	Downstream Throughput at N3 interface	12

6.3.6	RAN UE Throughput	13

6.3.6.1	Definition	13

6.3.6.2	Extended definition	13

6.4	Utilization KPI	14

6.4.1	Mean number of PDU sessions of network and network Slice Instance	14

6.4.2	Virtualised Resource Utilization of Network Slice Instance	14

6.5	Retainability KPI	15

6.5.1	QoS flow Retainability	15

6.5.1.1	Definition	15

6.5.1.2	Extended definition	16

6.5.2	DRB Retainability	17

6.5.2.1	Definition	17

6.5.2.2	Extended definition	17

6.6	Mobility KPI	17

6.6.1	NG-RAN handover success rate	17

6.6.2	Mean Time of Inter-gNB handover Execution of Network Slice	18

Annex A (informative):	 Use cases for end to end KPIs	19

A.1	Use case for end-to-end latency measurements of 5G network-related KPI	19

A.2	Use case for number of registered subscribers of single network-slice-instance-related KPI	19

A.3	Use case for upstream/downstream throughput for one-single-network-slice-related KPI	19

A.4	Use case for mean PDU sessions number in network slice instance	19

A.5	Use case for virtualised resource utilization of network-slice-instance related KPI	20

A.6	Use case for 5GS registration success rate of one single-network-slice instance-related KPI	20

A.7	Use case for RAN UE throughput-related KPI	20

A.8	Use case for QoS flow retainability-related KPI	20

A.9	Use case for DRB accessibility-related KPI	20

A.10	Use case for mobility KPI	21

A.11	Use case for DRB retainability related KPI	21

A.12	Use case for PDU session establishment success rate of one network slice (S-NSSAI) related KPI	21

A.13	Use case for integrated downlink latency in RAN	21

A.14	Use case for PDU session Establishment success rate of one single-network-slice instance-related KPI	22

A.15	Use case for QoS flow retainability-related KPI	22

Annex B (informative):	Change history	23

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies end-to-end Key Performance Indicators (KPIs) for the 5G network and network slicing.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	ITU-T Recommendation E.800: "Definitions of terms related to quality of service".

[4]	3GPP TS 24.501: " Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[7]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

kbit	kilobit (1000 bits)

RTT	Round Trip Time

# 4	End to end KPI concept and overview

The following KPI categories are included in the present document:

-	Accessibility (see the definition in [3]).

-	Integrity (see the definition in [3]).

-	Utilization.

-	Retainability (see the definition in [3]).

-	Mobility.

Editor's note:	For future update of the document it will also include:

-	Availability.

# 5	KPI definitions template

a)	Long name (Mandatory): This field shall contain the long and descriptive name of the KPI.

b)	Description (Mandatory): This field shall contain the description of the KPI. 
Within this field it should be given if the KPI is focusing on network or user view.

c)	Logical formula definition (Mandatory): 
The logical formula should describe what the KPI formula is in logical way. The description of the formula is given in a written textual format without any measurement or counter names. E.g. a success rate KPI’s logical formula is the successful event divided by all event.

d)	Physical formula definition (Optional): 
This field should contain the KPI formula description using the 3GPP defined counter names. 
This field can be used only if the counters needed for the KPI formula is defined in any of the 3GPP TS for performance measurements (TS 28.552 [6]).

e)	Measurement names used for the KPI (Optional): 
This clause should list the measurement names used for the KPI. 
This clause can be filled out only when the underlying measurements for the KPI formula can be defined, 
i.e. physical formula definition is available.

f)	KPI Object (mandatory)
This clause shall describe the object of the KPI. The object of the KPI is one or some of the following:

-	NR and NG-RAN;

-	5GC

-	5GS

g)	KPI category (mandatory)
This clause contains the classification of the KPI into one of the KPI categories listed in clause 4.

h)	Unit of the KPI (mandatory)
This clause describes the unit of the KPI. The unit can be one of the following:

-	percentage;

-	time interval (second or millisecond or microsecond);

-	Erlang;

-	kbit/s.

i)	Type of the KPI (Mandatory)
This clause describes the type of the KPI. The KPI type can be one of the following:

-	MEAN: This KPI is produced to reflect a mean measurement value based on a number of sample results.

-	RATIO: KPI is produced to reflect the percentage of a specific case occurrence to all the cases.

-	CUM: This KPI is produced to reflect a cumulative measurement which is always increasing.

j)	Remark: (Optional)
This field is for any further information that is needed for the KPI definition. 
Here it is proposed to define any additional information that would be needed for the KPI definition; e.g. the definition of a call in UTRAN.

# 6	End to end KPI definitions

## 6.1	KPI Overview

The KPI categories defined in [2] will be reused by the present document.

## 6.2	Accessibility KPI

## 6.2.1	Registered Subscribers of Network and Network Slice Instance through AMF

a)	Registered Subscribers of Single Network Slice Instance through AMF.

b)	This KPI describe the total number of subscribers that are registered to a network slice instance.

c)	This KPI is obtained by counting the subscribers in AMF that are registered to a network slice instance.

d)

e)	RegisteredAMFSubNbrMean

f)	5GS

g)	Accessibility

h)	Interger

i)	CUM

## 6.2.2	Registered Subscribers of Network and Network Slice Instance through UDM

a)	Registered Subscribers of Single Network Slice Instance through UDM

b)	This KPI describe the total number of subscribers that are registered to a network slice instance.

c)	This KPI is corresponding to the measurement RM.RegisteredSubUDMNbrMean that counts subscribers registered in UDM..

d)

e)	RegisteredSubUDMNbrMean

f)	5GS

g)	Accessibility

h)	Integer

i)	CUM

## 6.2.3	Registration success rate of one single network slice instance

a)	Registration success rate of one single network slice instance.

b)	This KPI describes the ratio of the number of successfully performed registration procedures to the number of attempted registration procedures for the AMF set which related to one single network slice instance and is used to evaluate accessibility provided by the end-to-end network slice instance and network performance.

c)	This KPI is obtained by successful registration procedures divided by attempted registration procedures.

d)

e)	AMF.5GSRegisAtt.Type
AMF.5GSRegisAttachSucc.Type

Note:	Above measurements with subcounter .Type should be defined in 3GPP TS 24.501 [4].

f)	5GS

g)	Accessibility

h)	Percentage

i)	RATIO

### 6.2.4	DRB Accessibility for UE services

a)	DRB Accessibility

b)	This KPI describes the DRBs setup success rate, including the success rate for setting up RRC connection and NG signalling connection.

c)	This KPI is obtained as the succeess rate for RRC connection setup multiplied by the success rate for NG signalling connection setup multiplied by the success rate for DRB setup. The success rate for RRC connection setup and for NG signalling connection setup shall exclude setups with establishment cause mo-Signalling [5].

d)	DRB Accessibility Success Rate for mapped 5QI = (∑RRC.ConnEstabSucc.Cause/∑RRC.ConnEstabAtt.Cause) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.5QI/DRB.EstabAtt.5QI) * 100 and  DRB Accessibility Success Rate for SNSSAI = (∑RRC.ConnEstabSucc.Cause/∑RRC.ConnEstabAtt.Cause) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.SNSSAI/DRB.EstabAtt.SNSSAI) * 100.

Note that the sum over causes shall exclude the establishment cause mo-Signalling [5].

e)	RRC.ConnEstabSucc.Cause, RRC.ConnEstabAtt.Cause, UECNTXT.ConnEstabSucc.Cause, UECNTXT.ConnEstabAtt.Cause, DRB.EstabSucc.5QI, DRB.EstabAtt.5QI, DRB.EstabSucc.SNSSAI, DRB.EstabAtt.SNSSAI.

f)	5GS.

g)	Accessibility.

h)	Percentage.

i)	RATIO.

### 6.2.5	PDU session Establishment Success Rate of one network slice (S-NSSAI)

a)	PDU session establishment success rate of one network slice (S-NSSAI).

b)	This KPI describes the ratio of the number of successful PDU session establishment request to the number of PDU session establishment request attempts for 5G network for the SMF which related to one network slice (S-NSSAI) and is used to evaluate accessibility provided by the end-to-end network slice and network performance.

c)	This KPI is obtained by the number of successful PDU session requests divided by the number of attempted PDU session requests.

d)	
SM.PduSessionCreationSuccNSI.SNSSAI 
SM.PduSessionCreationReqNSI.SNSSAI

e)	5G System

f)	Accessibility.

g)	Percentage.

h)	RATIO.

## 6.3	Integrity KPI

### 6.3.1	Latency of 5G Network

#### 6.3.1.0	KPI categories

a)	End-to-end latency of 5G network.

b)	This KPI describes the end to end packet transmission latency through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network.

c)	This KPI is the RTT end to end latency of UE IP packets transmitted from UE to the N6 interface in the 5G network. The N6 interface is the reference point between UPF and DN.

d)	E2ELatency

e)	End-to-end latency

f)	5GS

g)	Integrity

h)	Time interval (millisecond)

i)	MEAN

#### 6.3.1.1	Downlink latency in gNB-DU

a)	Downlink latency for IP packets through gNB in split scenario.

b)	This KPI describes the gNB-DU part of the packet transmission latency experienced by an end-user. It is used to evaluate the gNB latency contribution to the total packet latency.

c)	This KPI is the average (arithmetic mean) of the time from reception of IP packet to gNB-DU until transmission of first part of that packet over the air interface, for a packet arriving when there is no previous data in queue for transmission to the UE.

d)	DownlinkLat = DRB.RlcSduLatencyDl or optionally DownlinkLat.QoSx = DRB.RlcSduLatencyDl.QoSx where QOS identifies the target quality of service class.

e)	DRB.RlcSduLatencyDl, DRB.RlcSduLatencyDl.QoS,

f)	NG-RAN

g)	Integrity

h)	Time interval (microsecond)

i)	MEAN

#### 6.3.1.2	Integrated downlink delay in RAN

a)	Integrated downlink latency of NR-RAN.

b)	This KPI describes the average packet transmission latency through the RAN part to the UE. It is used to evaluate latency performance of NG-RAN in downlink. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

c)	This KPI is the average packets latency transmitted from RAN to UE.

d)	DownlinkLat = DRB.PdcpSduDelayDl + DRB.PdcpF1Delay + DRB.RlcSduDelayDl + DRB.AirIfDelayDl.

e)	DRB.PdcpSduDelayDl, DRB.PdcpF1Delay, DRB.RlcSduDelayDl, DRB.AirIfDelayDl

f)	5GS

g)	Integrity

h)	Time interval (microsecond)

i)	MEAN

j)	In non-split gNB scenario, the value of DRB.PdcpF1Delay is set to zero for there are no F1-interfaces in this scenario.

### 6.3.2	Upstream throughput for network and Network Slice Instance

a)	Upstream throughput for network and network slice instance.

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by upstream throughput provided by N3 interface from NG-RAN to all UPFs which are related to the single network slice instance.

d)

e)	GTP.InDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	CUM.

### 6.3.3	Downstream throughput for Single Network Slice Instance

a)	Downstream throughput for network and network slice instance.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by downstream throughput provided by N3 interface from all UPFs to NG-RAN which are related to the single network slice instance.

d)

e)	GTP.OutDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	CUM.

### 6.3.4	Upstream Throughput at N3 interface

a)	Upstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all incoming GTP data packets on the N3 interface (measured at UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate upstream GTP throughput integrity performance at the N3 interface.

c)	This KPI is obtained by measuring the GTP data upstream throughput provided by N3 interface from NG-RAN to UPF, during the granularity period.

d)	UGTPTS=SUM (GTP.InDataOctN3UPF)/timeperiod) at UPF

e)	GTP.InDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

### 6.3.5	Downstream Throughput at N3 interface

a)	Downstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all downstream GTP data packets on the N3 interface (transmitted downstream from UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate integrity performance at N3 interface.

c)	This KPI is obtained by measuring the GTP data downstream throughput provided by N3 interface from UPF to NG-RAN, during the granularity period.

d)	DGTPTS=SUM (GTP.OutDataOctN3UPF)/timeperiod) at UPF

e)	GTP.OutDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

### 6.3.6	RAN UE Throughput

#### 6.3.6.1	Definition

a)	RAN UE Throughput.

b)	A KPI that shows how NG-RAN impacts the service quality provided to an end-user.

c)	Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface.

d)	RAN UE Throughput DL = DRB.UEThpDl and

RAN UE Throughput UL = DRB.UEThpUl

or optionally RAN UE Throughput DL for single mapped 5QI or QCI = DRB.UEThpDl.QoS and

RAN UE Throughput UL for single mapped 5QI or QCI = DRB.UEThpUl.QoS

e)	DRB.UEThpDl, DRB.UEThpUl, DRB.UEThpDl.QoS, DRB.UEThpUl.QoS

f)	 NG-RAN

g)	Integrity

h)	kbit/s

i)	MEAN

#### 6.3.6.2	Extended definition

To achieve a Throughput measurement (below examples are given for DL) that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in figure 1).

Time (slots)

Data arrives to

empty DL buffer

First data is

transmitted to the UE

The send buffer is

again empty

calulations since it

can be impacted

by packet size of

User Plane (UP) packets.

ThpTimeDl

Failed transmission (

”

Block

error

”

)

Successful transmission,

buffer not empty

Successful transmission,

buffer empty

ThpVolDl =

∑

Total DL transferred volume =

∑

(kbits)

+

(kbits)

UE Throughput in DL =

ThpVolDl / ThpTimeDl (kbits/s)

No transmission, buffer not

empty (e.g. due to contention)

The last slot shall always be removed from

Figure 1

To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

## 6.4	Utilization KPI

### 6.4.1	Mean number of PDU sessions of network and network Slice Instance

a)	Mean number of PDU sessions of Single Network Slice Instance.

b)	This KPI describes the mean number of PDU sessions that are successfully established in a network slice instance.

c)	This KPI is obtained by successful PDU session establishment procedures of SMFs which is related to the network slice instance.

d)	PDUSesMeanNbr=Sum (SM.SessionNbrMean.SNSSAI) over SMFs.

e)	PDUSessionNum

f)	5GS

g)	Utilization

h)	Integer

i)	MEAN

### 6.4.2	Virtualised Resource Utilization of Network Slice Instance

a)	Virtualised resource utilization of single network slice instance.

b)	This KPI describes utilization of virtualised resource (e.g. processor, memory, disk) that are allocated to a network slice instance.

Note: 	In the present document, this KPI is for the scenario when NF is not shared between different network slice instances.

c)	This KPI is obtained by the usage of virtualised resource (e.g. processor, memory, disk) divided by the system capacity that allocated to the network slice instance.

d)

e)	MeanProcessorUsage

MeanMemoryUsage

MeanDiskUsage

System capacity indicates amount of virtualised resource which allocated to the network slice instance.

f)	5GS

g)	Utilization KPI

h)	Percentage

i)	 Ratio

- 6.4.3	PDU session establishment time of network slice

a)	PDU session establishment time one single network slice.

b)	This KPI describes the time of successful PDU session establishment which related to one single network slice and is used to evaluate utilization provided by the end-to-end network slice and network performance.

c)	This KPI is obtained by measuring the time between the receipt by SMF from AMF of " Nsmf_PDUSession_UpdateSMContext Request ", which includes N2 SM information received from (R)AN to the SMF and the sending of a " Nsmf_PDUSession_CreateSMContext Request or Nsmf_PDUSession_UpdateSMContext Request PDU Session Establishment Request " message from AMF to the SMF.

d)	 SM.PduSessionTimeMean.SNSSAI

e)	5GS

f)	Utilization.

g)	Time interval (millisecond).

h)	MEAN.

## 6.5	Retainability KPI

### 6.5.1	QoS flow Retainability

#### 6.5.1.1	Definition

a)	QoS flow Retainability

b)	A measurement that shows how often an end-user abnormally loses a QoS flow during the time the QoS flow is used.

c)	Number of QoS flows with data in a buffer that was abnormally released, normalized with number of data session time units.

d)	To measure QoS flow Retainability for a single QoS level (R1) is fairly straight forward.



However to measure the QoS flow Retainability for UEs is not as straight forward.  The measurement R1 is defined to look at the activity level of just one QoS level at the time, so to use this formula and measurements in an aggregated way to get QoS flow Retainability on UE level will not be accurate (e.g. for an UE with multiple QoS flows there might be QoS flows that are active at the same time, hence aggregating the QoS level measurements for session time will give a larger session time than the total UE session time. See picture below).


Hence a measurement QoS flow Retainability on UE level is defined (R2) to provide a measurement for the overall QoS flow Retainability.

e)	QF.RelActNbr.QoS

QF.SessionTimeUE

QF.SessionTimeQoS.QoS

f)	5GS

g)	Retainability.

h)	Active release/second.

i)	MEAN.

j)	The definition of the service provided by 5GS is QoS flows.

#### 6.5.1.2	Extended definition

The retainability rate is defined as:

Number of abnormally released QoS flow with data in any of the buffers

[Releases/Session time]

Active QoS flow Time

As for defining an abnormal QoS flow release with end-user impact, it shall only be considered an abnormal release of the QoS flows if the NG-RAN considers there to be data waiting for transfer in any of the buffers.

As for defining a QoS flow as active, a QoS flow shall be considered active if there recently has been any data transmission in any direction.

### 6.5.2	DRB Retainability

#### 6.5.2.1	Definition

a)	DRB Retainability

b)	A measurement that shows how often an end-user abnormally loses a DRB during the time the DRB is active.

c)	Number of DRBs that were abnormally released and that were active at the time of release, normalized with number of data session time units.

d)	DRB Retainability for a single mapped 5QI level (R1) and for a single S-NSSAI (R1) are defined as:

and

e)	DRB.RelActNbr.5QI and DRB.RelActNbr.SNSSAI 

DRB.SessionTime.5QI and DRB.SessionTime.SNSSAI

f)	5GS

g)	Retainability

h)	Active release/second

i)	MEAN

j)	The definition of the service provided by 5GS is DRBs.

#### 6.5.2.2	Extended definition

To define (from a DRB Retainability point of view) if a DRB is considered active or not, the DRB can be divided into two groups:

-	For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms.

-	For DRBs with continuous flow, the DRB (and the UE) is always seen as being active in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

## 6.6	Mobility KPI

### 6.6.1	NG-RAN handover success rate

a)	NG-RAN Handover Success Rate.

b)	A KPI that shows how often a handover within NR-RAN is successful, regardless if the handover was made due to bad coverage or any other reason.

c)	This KPI is obtained by successful handovers to the same or another gNB divided by attempted handovers to the same or another gNB.

d)e)	MM.HoExeInterSucc, MM.HoExeIntraSucc, MM.HoExeInterReq, MM.HoExeIntraReq, MM.HoPrepInterSucc, MM.HoPrepIntraSucc, MM.HoPrepInterReq and MM.HoPrepIntraReq.

f)	5GS.

g)	Mobility.

h)	Percentage.

i)	RATIO.

### 6.6.2	Mean Time of Inter-gNB handover Execution of Network Slice

-	Mean Time of Inter-gNB handover of one single network slice.

-	This KPI describes the time of successful Mean Time of Inter-gNB handover which related to one single network slice and is used to evaluate utilization provided by the end-to-end network slice and network performance.

-	This KPI is obtained by measuring the time between the receipt by the Source NG-RAN from the Target NG-RAN of a " Release Resource" and the sending of a " N2 Path Switch Request " message from Source NG-RAN to the Target NG-RAN over a granularity period

-	MM.HoExeInterReq.TimeMean.SNSSAI

-	5GS

-	Mobility.

-	Time interval (millisecond).

-	MEAN.

Annex A (informative): 
Use cases for end to end KPIs

# A.1	Use case for end-to-end latency measurements of 5G network-related KPI

The end-to-end latency is an important performance parameter for operating 5G network. In some scenarios (e.g. uRLLC), if end-to-end latency is insufficient, the 5G network customer cannot obtain guaranteed network performance provided by the network operator. So it is necessary to define end-to-end latency of network related measurement to evaluate whether the end-to-end latency that network customer requested has been satisfied. A procedure is invoked by network management system and is used:

-	to update the CSMF/NSMF with the end-to-end latency parameter for monitoring;

-	to inform the network customer/network operator the end-to-end latency;

-	to make CSMF/NSMF aware if the end-to-end latency can meet network customer’s service requirement.

If high end-to-end latency are measured, it is also of benefit to pinpoint where in the chain from application to UE that the latency occurs.

# A.2	Use case for number of registered subscribers of single network-slice-instance-related KPI

Number of registered subscribers of single network slice instance can be used to describe the amount of subscribers that are successfully registered, it can reflect the usage of network slice instance, It is useful to evaluate accessibility performance provided by one single network slice instance which may trigger the lifecycle management of the network slice, this kind of KPI is valuable especially when network functions (e.g. AMF) are shared between different network slice instances. This KPI is focusing on both network and user view.

# A.3	Use case for upstream/downstream throughput for one-single-network-slice-related KPI

Measuring throughput is useful to evaluate system load of end to end network slice. If the throughput of the specific network slice instance cannot meet the performance requirement, some actions need to be performed to the network slice instance e.g. reconfiguration, capacity relocation. So it is necessary to define the IP throughput for one single network slice instance. This KPI is focusing on network and user view.

# A.4	Use case for mean PDU sessions number in network slice instance

It is necessary to evaluate the mean PDU session number in the network slice instance to indicate system load level. For example, if the mean value of the PDU sessions is high, maybe the system capacity should be increased. This KPI is focusing on network view.

# A.5	Use case for virtualised resource utilization of network-slice-instance related KPI

It is necessary to evaluate the current utilization of virtualised resources (e.g. memory and storage utilization) that a network slice instance is occupied. If the utilization is larger or smaller than the threshold, maybe some scale in/out operations will be made by the management system. This KPI is focusing on network and user view.

# A.6	Use case for 5GS registration success rate of one single-network-slice instance-related KPI

It is necessary to evaluate accessibility performance provided by 5GS. 5GS registration for a UE is important when they have registered to the network slice instance. If users or subscribers cannot register to the network slice instance, they cannot access any network services in the network slice instance. This KPI is focusing on network view.

# A.7	Use case for RAN UE throughput-related KPI

The UE perceived throughput in NG-RAN is an important performance parameter for operating 5G network. If the UE throughput of the NR cell cannot meet the performance requirement, some actions need to be performed to the network, e.g. reconfiguration or capacity increase. So it is necessary to define UE throughput KPI to evaluate whether the end-users are satisfied. The KPI covers volume large enough to make the throughput measurement relevant, i.e. excluding data volume of the last or only slot.

The UE throughput KPI covers also "NR option 3" scenarios. Then the gNB is "connected" towards the EPC, and not towards 5GC.

It is proposed to allow the KPI separated based on mapped 5QI (or for QCI in case of NR option 3).

When network slicing is supported by the NG-RAN, multiple NSIs may be supported. The UL and DL UE throughput for each NSI is then of importance to the operator to pinpoint a specific performance problem.

# A.8	Use case for QoS flow retainability-related KPI

QoS flow is the key and limited resource for 5GS to deliver services. The release of the QoS flow needs to be monitored. QoS flow retainability is a key performance indicator of how often an end-user abnormally losing a QoS flow during the time the QoS flow is used. This key performance indicator is of great importance to estimate the end users’ experiences.

# A.9	Use case for DRB accessibility-related KPI

In providing services to end-users, the first step is to get access to the service. First after access to the service has been performed, the service can be used.

The service provided by NG-RAN is the DRB. For the DRB to be successfully setup it is also necessary to setup an RRC connection and an NG signalling connection.

If an end user cannot access a service, it is hard to charge for the service. Also, if it happens often that an end-user cannot access the provided service, the end-user might change wireless subscription provider, i.e. loss of income for the network operator. Hence, to have a good accessibility of the services is important from a business point of view.

A DRB accessibility KPI requires the following 3 measurements:

-	RRC connection setup success rate.

-	NG signalling connection setup success rate.

-	DRB setup success rate.

The success rate for RRC connection setup and for NG signalling connection setup shall exclude setups with establishment cause mo-Signalling, since these phases/procedures occur when there is no request to setup a DRB.

This KPI is available per mapped 5QI and per S-NSSAI, and it assists the network operator with information about the accessibility provided to their 5G network customers.

# A.10	Use case for mobility KPI

When a service is used it is important that it is not interrupted or aborted. One of the fault cases in a radio system for this is handovers/mobility.

If a mobility KPI is not considered OK, then the network operator can investigate which steps that are required to improve the mobility towards their services.

This KPI can be used for observing the impact of mobility in NG-RAN on end-users.

# A.11	Use case for DRB retainability related KPI

DRB is the key and limited resource for 5GS to deliver services. Once a QoS flow reaches a gNB it will trigger setup of a new DRB or it will be mapped to an existing DRB. The decision on how to map QoS flows into new or existing DRBs is taken at the CU-CP. CU-CP also defines one set of QoS parameters (one 5QI) for the DRB. If a QoS flow is mapped to an existing DRB, the packets belonging to that QoS flow are not treated with the 5QI of the QoS flow, but they are treated with the mapped 5QI of the DRB.

The release of the DRB needs to be monitored, so that abnormal releases while the UE is considered in an active transfer shall be logged. DRB retainability is a key performance indicator of how often an end-user abnormally loses a DRB during the time the DRB is actively used. This key performance indicator is of great important to estimate the end users’ experiences. DRBs with bursty flow are considered active if any data (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are always seen as active DRBs in the context of this measurement. A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

The key performance indicator shall monitor the DRB retainability for each used mapped 5QI value, as well as for the used S-NSSAI(s). DRBs used in 3GPP option 3 shall not be covered by this KPI. For the case when a DRB have multiple QoS flows mapped and active, when a QoS flow is released it will not be counted as a DRB release (DRB still active) in this KPI.

# A.12	Use case for PDU session establishment success rate of one network slice (S-NSSAI) related KPI

It is necessary to evaluate accessibility performance provided by 5GS. PDU session Establishment for a UE is important when it has registered to the network slice. If users or subscribers cannot establish PDU sessions in slice instance, they cannot access any network services in the network slice. This KPI is focusing on network view.

# A.13	Use case for integrated downlink latency in RAN

Following figure captured in clause 4.2.3, 3GPP TS 23.501[x] illustrates the 5G system architecture. The end to end downlink latency should be measured from Data Network to UE, of which the latency from RAN to UE is an important part for the latency of this section is closely related to NG-RAN.

The integrated downlink latency in RAN is a key performance parameter for evaluating the packet delay in RAN for QoS monitoring. This KPI is also an important part of the end-to-end network latency for SLA assurance.

# A.14	Use case for PDU session Establishment success rate of one single-network-slice instance-related KPI

It is necessary to evaluate PDU session establishment time, it can be used to analyse the network service difference between different RAN locations in one area, which can be used for management area division. This KPI is focusing on network view.

# A.15	Use case for QoS flow retainability-related KPI

QoS Flow is the key and limited resource for 5GS to deliver services. The release of the QoS flow needs to be monitored. QoS Flow drop ratio is a key performance indicator of how often an end-user is abnormally losing a bearer. This key performance indicator is of great importance to estimate the end users’ experiences.

The KPI shall be available per QoS group.

From QoS perspective it is important to focus also on call duration as in some cases wrong quality perceived by the end user is not fully reflected by drop ratio nor retainability KPI. Typical case is when due to poor radio conditions the end user redials (the call was terminated normally) to the same party to secure the quality. But in this case the drop ratio KPI will not show any degradation. Secondly, although the call is dropped the end user may or may not redial depending on dropped call duration compared to the case when the call would be normally released. It is therefore highly recommended to monitor distribution of duration of normally and abnormally released calls.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EditHelp fix | 15.0.1 |
| 2018-12 | SA#82 | SP-181041 | 0001 | - | F | Align title with TS database | 15.1.0 |
| 2019-03 | SA#83 | SP-190122 | 0005 | 2 | F | Update KPI subscribers of single network slice instance through UDM | 15.2.0 |
| 2019-03 | SA#83 | SP-190122 | 0011 | 2 | F | Update definition of mean number of PDU sessions KPI | 15.2.0 |
| 2019-03 | SA#83 | SP-190111 | 0007 | 1 | B | Add KPI of QoS flow Retainability | 16.0.0 |
| 2019-03 | SA#83 | SP-190111 | 0009 | 1 | B | Add DRB Accessibility KPI and Use Case | 16.0.0 |
| 2019-06 | SA#84 | SP-190371 | 0013 | - | B | Add KPI for NG-RAN Handover Success Rate | 16.1.0 |
| 2019-06 | SA#84 | SP-190375 | 0015 | 1 | A | Correction of Throughput KPI | 16.1.0 |
| 2019-09 | SA#85 | SP-190747 | 0016 | 2 | B | Add KPI for DRB Retainability | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0017 | 3 | B | Add a new KPI definition of PDU session Establishment Success Rate of one network slice (S-NSSAI) | 16.2.0 |
| 2019-09 | SA#85 | SP-190751 | 0020 | - | A | Correction on kbits abbreviation | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0021 | 1 | F | Correction of Flow Retainability KPI | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0022 | 1 | F | Correction of DRB Accessibility KPI | 16.2.0 |
| 2019-09 | SA#85 | SP-190748 | 0024 | 2 | A | Correct the title of KPI | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0025 | 1 | B | Add definition of integrated downlink latency in RAN | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0028 | 2 | B | Add a new KPI definition of Inter-gNB handover Execution time of one single network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0029 | 2 | B | Add a new KPI definition of PDU session Establishment Time of one single network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190747 | 0030 | 2 | B | Add new specification requirement related to extended 5QI 1 QoS Flow Retainability monitoring | 16.2.0 |