3GPP TSG-SA5 Meeting #126	S5-195077

# Bruges (Belgium), 19 - 23 August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0024 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add informative annex to describe a network slice journey |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-PRO_NS |  |  |  |  |  | Date: |  |  | 2019-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A description of how network slice instances can be ordered by a Network Slice Customer (NSC) to a Network Slice Provider (NSP) and how such an order is handled by the NSP OSS is missing to give an illustration of how the 3GPP network slice provisioning management service can be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The concept of Generic Slice Template and its attributes, defined by GSMA, is introduced, as well as the concept of NEST (NEtwork Slice Type). The notion of products defined by the NSP based on NEST is introduced. A description of how network slice instances can be ordered by a Network Slice Customer (NSC) to a Network Slice Provider (NSP) and how such an order is handled by the NSP OSS is introduced to give an illustration of how the 3GPP network slice provisioning management service can be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A description of how network slice instances can be ordered by a Network Slice Customer (NSC) to a Network Slice Provider (NSP) and how such an order is handled by the NSP OSS would be missing to give an illustration of how the 3GPP network slice provisioning management service can be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V2.4.1) (2018-02): "Network Function Virtualisation (NFV); Release 2; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[x]	GSMA NG.116 Generic Network Slice Template v1.0 (2019-05-23) - https://www.gsma.com/newsroom/wp-content/uploads//NG.116-v1.0-4.pdf

Next change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GAA	5G Automative Association

5G-ACIA	5G Association for Connected Industries and Automation

5GC	5G Core Network

AMF	Access and Mobility Management Function

CSC	Communication Service Customer

CSMF	Communication Service Management Function

CSP	Communication Service Provider

CP	Control Plane

GST	Generic Slice Template

IOC	Information Object Class

MANO	Management and Orchestration

MnS	Management Service

NEST	Network Slice Type

NF	Network Function

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NSaaS	Network Slice as a Service

NSC	Network Slice Customer

NSI	Network Slice Instance

NSMF	Network Slice Management Function

NSP	Network Slice Provider

NSSI	Network Slice Subnet Instance

NSSMF	Network Slice Subnet Management Function

NSST	Network Slice Subnet Template

NST	Network Slice Template

P-NEST	Private NEST

S-NEST	Standardized NEST

TN	Transport Network

VNF	Virtualized Network Function

UP	User Plane

Next change

Annex A (informative):
A network slice journey example

The Generic Slice Template (GST) (cf. [x]) provides a standardized list of attributes that can be used to characterise different types of network slice. GST is generic and is not tied to any:

- Type of network slice

- Agreement between a Network Slice Customer (NSC) and a Network Slice Provider (NSP).

A Network Slice Type (NEST) is a GST filled with (ranges of) values. There may be two kinds of NESTs:

- Standardized NESTs (S-NEST), i.e. NESTs which character attributes are assigned (ranges of) values by SDOs, working groups, foras, etc. such as e.g. 3GPP, GSMA, 5GAA, 5G-ACIA, etc.;

- Private NESTs (P-NEST), i.e. NESTs which character attributes are assigned (ranges of) values by the Network Slice Providers, which are different from those assigned in S-NESTs.

Network Slice Providers can build their network slice product offering based on S-NESTs and/or their P-NESTs. For example, a Standardized Network Slice Type (S-NEST) NST-A, for which the attribute ‘Packet delay budget’ (cf. [X] – clause 3.4.26) value range is between 1 ms and 100 ms, is specified by 3GPP. Netwok Slice Provider Y may offer e.g. 3 products based on NST-A:

- Platinum NST-A based network slice product, where the attribute ‘Packet delay budget’ (cf. [X] – clause 3.4.26) value range is between 1 ms and 10 ms;

- Gold NST-A based network slice product, where the attribute ‘Packet delay budget’ (cf. [X] – clause 3.4.26) value range is between 11 ms and 50 ms;

- Silver NST-A based network slice product, where the attribute ‘Packet delay budget’ (cf. [X] – clause 3.4.26) value range is between 51 ms and 100 ms.

NSP Y’s customers can then order network slice instances from Y’s product catalog, as depicted by Figure A.1.

Figure A.1: From GST to S-(/P-)NEST based product ordering

1)	A Network Slice Customer orders network slice instance to a Network Slice Provider, based on NSP’s product offering, by sending a request to NSP’ BSS

2)	NSP’s BSS receives the request from NSC and translates NSC requirements into NSP technical parameter values

3) NSP’s BSS sends a request to NSP’s OSS to provide a network slice instance which fits NSC’s requirements (cf. Table 6.1-1)

4) NSP’s Network Slice Provisioning MnS either creates a new network slice instance or assigns an already existing network slice instance to NSC’s request

5) NSP’s Network Slice Provisioning MnS may have to request the Network Slice Subnet Provisioning MnS to provide a network slice subnet instance corresponding to the network slice instance (cf. Table 6.2-1)

6) NSP’s Network Slice Subnet Provisioning MnS may have to create one or more inner network slice subnet instances

7) NSP’s Network Function Provisioning MnS configures VNFs and/or PNFs (cf. Table 6.3-1).

Figure A.2: Network Slice journey (NSaaS model) – high-level call flow

Depending on NSP’s product offering, NSC can order network slice instances spanning one or more network domains, e.g.:

- network slice instances composed of a RAN-only network slice subnet instance

Figure A.3: RAN-only network slice subnet exposed as network slice

- network slice instances composed of a CN-only network slice subnet instance

- network slice instances composed of a RAN network slice subnet instance + Transport network slice subnet + CN network slice subnet instance

Figure A.4: RAN+TN+CN network slice subnets exposed as network slice

- network slice instances composed of a RAN network slice subnet instance + CN network slice subnet instance + IMS network slice subnet instance

- or more, as shown below..

Figure A.5: RAN+TN+CN+TN+IMS+TN+AS network slice subnets exposed as network slice

Next change

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-12 | SA#82 | SP-181043 | 0001 | 1 | F | Complete the reference information and reword the note | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0002 | - | F | Update operation names in the procedures of NSI provisioning | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0003 | - | F | Update operation names in the procedures of NSSI provisioning | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0004 | 1 | F | Update operation names in the procedures of NF provisioning | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0005 | 1 | F | Remove release specific information from clause 7.9.1 | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0006 | 1 | F | Correct procedures with reference to TS 28.541 | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0009 | 1 | B | Add Network slice subnet management use case with assigned priority | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0010 | 2 | B | Add network slice management interactions with NFV MANO for network service priority | 15.1.0 |
| 2018-12 | SA#82 | SP-181044 | 0011 | 2 | D | Implement minor corrections | 16.0.0 |
| 2019-03 | SA#83 | SP-190123 | 0013 | - | A | Update management services tables | 16.1.0 |
| 2019-03 | SA#83 | SP-190123 | 0015 | - | A | Correction on procedure of Network Slice Subnet Instance Deallocation | 16.1.0 |
| 2019-03 | SA#83 | SP-190123 | 0017 | 1 | A | Correct management service term | 16.1.0 |
| 2019-05 | SA#84 | SP-190370 | 0019 | 1 | A | Editor's change for configuration management service | 16.2.0 |

End