3GPP TSG-SA5 Meeting #126	S5-195294

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0135 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.541 Correction of Network slice NRM relationship |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Network slice NRM relationship in 6.2.1 figure 6.2.1-1 shows that the mapping relationship between NetworkSlice and NetworkSliceSubnet is 1:1. In fact, a NetworkSlice Instance can contain multiple NetworkSliceSubnet instances(such as CN/TN/RAN). Even for the core network, there is a case where one NetworkSlice Instance contains multiple CN NetworkSliceSubnet instances. On the other hand, one NetworkSliceSubnet instance can also belong to multiple NetworkSlice instances. Therefore, it is not appropriate to map relationship between NetworkSlice and NetworkSliceSubnet by 1:1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the mapping relationship between NetworkSlice and NetworkSliceSubnet to N:M(or *:*). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to build the mapping relationship between NetworkSlice and NetworkSliceSubnet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

End of change