3GPP TSG-SA5 Meeting #127	S5-196254

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019                    Revision of S5-196xxx

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0067 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Post Event Charging flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2019-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Post Event Charging (PEC) case is specified in TS 32.290 and corresponding scenarios are missing for SMS Converged Event based charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PEC scenario for SMS submission and SMS delivery in SMS Converged Charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for Operators to obtain CDRs for SMS submission and delivery with final status |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.1.2.1, 5.4.2.x(New), 5.4.2.y(New), 5.4.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.4.1.1	General

Converged charging may be performed by the SMSF interacting with CHF using Nchf specified in TS 32.290 [19] and TS 32.291 [20]. In order to provide the data required for the management activities outlined in TS 32.240 [2] (Credit-Control, accounting, billing, statistics etc.), the SMSF shall be able to perform converged charging for each of the SMS transactions.

The SMSF shall be able to perform convergent charging by interacting with CHF, for charging data related to SMS over NAS. The Charging Data Request and Charging Data Response are exchanged between the SMSF and the CHF, based on PEC, IEC or ECUR scenarios specified in TS 32.290 [19]. The Charging Data Request is issued by the SMSF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging IEC specified in TS 32.290 [19].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following sub-clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [20].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [3].

The selection of the CHF can be configured in the SMSF but may also rely on NRF.

The chargeable events or messages exchanged between the SMSF and the other nodes are described with generic names 
(i.e SMS submit, SMS deliver), to reflect SMS reception or sending by/from the SMSF, independently from the protocol conveying the SMS.

Next change

##### 5.4.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the SMS over NAS converged charging functionality, is specified with the associated behaviour when they are met.

When a MO or MT SMS is sent through the SMSF, and the converged charging is activated, the SMSF invokes a Charging Data Request [Initial] towards the CHF to get authorization to start in ECUR mode. In IEC mode, the Charging Data Request [Event] is sent towards the CHF.

Table 5.4.1.2.1 summarizes the set of default trigger conditions and their category which shall be supported by the SMSF. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Event, Termination] message sent from SMSF towards the CHF.

Table 5.4.1.2.1: Default Trigger conditions in SMSF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| SMS Submit | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| SMS to deliver | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| SMS Submit Answer | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| SMS Deliver Answer | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

For converged charging, the following details of chargeable events and corresponding actions in the SMSF are defined in Table 5.4.1.2.2:

Table 5.4.1.2.2: Chargeable events and their related actions in SMSF

| Chargeable event | Conditions | SMSF action |
| --- | --- | --- |
| SMS Submit |  | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] with a possible request quota for later use |
| SMS to deliver |  | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination], indicating that charging session is terminated |
| SMS Submit Answer |  | PEC: Charging Data Request [Event] |
| SMS Deliver Answer |  | PEC: Charging Data Request [Event] |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event, Initial, Termination] issued by the SMSF for these chargeable events, is specified in clause 5.4.3.

.

Next change

#### 5.4.2.x	SMS Submission - PEC

Figure 5.4.2.x.1 describes the scenario where a SMS is submitted to the SMSF for PEC mode

Figure 5.4.2.x.1: SMS submission to SMSF - PEC

1.	Initial procedures: see applicable flows in table 5.4.2.2.1.

2.	The SMSF receives a "SMS Submit" incoming message originated by a UE.

3. 	The SMSF returns "SMS Submit Answer" with appropriate result successful or unsuccessful reception of the SM by the SMSF.

3ch-a. The SMSF sends Charging Data Request [Event] to CHF for the received MO SMS.

3ch-b. The CHF creates a CDR for this MO SMS.

3ch-c.  The CHF acknowledges by sending Charging Data Response [Event] to the SMSF.

4.	Forward SMS per applicable flows table 5.4.2.2.1.

Next change

#### 5.4.2.y	SMS Delivery - PEC

Figure 5.4.2.y.1 describes the scenario where a SMS is delivered from the SMSF for PEC mode

Figure 5.4.2.y.1 SMS delivery from SMSF - PEC

1.	"SMS to deliver" received by SMSF in order to deliver a MT SMS over NAS towards the UE: see applicable flows in table 5.4.2.3.1.

2.	UE reachability criteria met.

3.	The SMSF forwards the "SMS Deliver" message towards the UE.

4.	The SMSF receives "SMS Deliver Answer" message as the delivery success or failure of the SM transfer attempt.

4ch-a. The SMSF sends Charging Data Request [Event] to CHF for the MT SMS delivery.

4ch-b. The CHF creates a CDR for this MT SMS.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the SMSF.

Next change

##### 5.4.3.2.1	General

A SMS charging CHF CDR is used to collect charging information related to SMS chargeable events for PEC, IEC and ECUR.

End of changes