3GPP TSG-SA5 Meeting #127	S5-196238

# Sophia Antipolis, 14th Oct 2019 – 18th Oct 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0062 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add RESTful HTTP-based solution set of file-based performance assurance for integration with ONAP VES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-09-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ONAP DCAE VES Collector is the consumer of 3GPP perfoarmance data file report management service. It is required that RESTful HTTP-based solution set of file-based performance assurance for integration with ONAP VES is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new RESTful HTTP-based solution set for enabling ONAP DCAE VES Collector to consume 3GPP performance data file report management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st Change

12.3.x	RESTful HTTP-based solution set of file-based performance assurance for intergration with ONAP VES API

12.3.x.1	Mapping of operations

12.3.x.2	Mapping of notifications

12.3.x.2.1	Introduction

12.3.X.2.1.1	General

The 3GPP IS notifications are mapped to SS equivalents according to table 12.3.x.2.1.1.-1.

Table 12.3.x.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyFileReady | POST | /eventListener | M |

12.3.x.2.1.2 	Notification parameter mapping principles

3GPP performance data file notification parameters are mapped either to:

- ONAP VES API commonEventHeader fields– see Table 12.3.X.2.1.2-1, or to

- ONAP VES API notification fields – see mapping tables in following clause 12.3.X.2.X.

Table 12.3.X.2.1.2-1: Mapping of 3GPP IS notification parameters to ONAP VES common event header fields

| 3GPP IS notification parameter name | SQ | Corresponding ONAP VES common event header field name | SQ in ONAP VES common event header | Remark |
| --- | --- | --- | --- | --- |
| no corresponding IS parameter | N/A | domain | M | “notification” |
| notificationId | M | eventId | M |  |
| no corresponding IS parameter | N/A | eventName | M |  |
| eventTime | M | lastEpochMicrosec | M |  |
| no corresponding IS parameter | N/A | reportingEntityName | M |  |
| eventTime | M | startEpochMicrosec | M | Same value as lastEpochMicrosec. |
| no corresponding IS parameter | N/A | priority | M | Field not used by this solution set. |
| no corresponding IS parameter | N/A | sequence | M | Field not used by this solution set. |
| no corresponding IS parameter | N/A | sourceName | M | Field not used by this solution set. |
| no corresponding IS parameter | N/A | version | M | Version of the event header |
| no corresponding IS parameter | N/A | vesEventListenerVersion | M | Version of the ves event listener API. |

12.3.x.2.3 	Notificaiton notifyFileReady

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.3.x.2.1.3-1.

Table 12.3.x.2.3-1: Mapping of IS notification parameters to SS equivalents in ‘notificationFields’

| 3GPP IS notification parameter name | SQ | SS parameter name in ‘notificationFields’ – cf. A.x | SQ |
| --- | --- | --- | --- |
| objectClass objectInstance | M | changIdentifier | M |
| notificationType | M | changeType (value = ‘notifyFileReady’) | M |
| fileInfoList | M | arrayOfNamedHashMap | O |
| additionalText | O | arrayOfNamedHashMap | O |
| no corresponding IS parameter | N/A | newState | O |
| no corresponding IS parameter | N/A | notificationFieldsVersion | M |
| no corresponding IS parameter | N/A | oldState | O |
| no corresponding IS parameter | N/A | stateInterface | O |

12.3.x.3	Resource

12.3.x.3.1	Resource structure

Figure 12.3.x.3.1-1 shows the resource structure of file-based perforamance assurance MnS in the context of its integration with ONAP DCAE VES Collector.

Figure 12.3.x.3.1-1: Resource URI structure of file-based perforamance assurance MnS for integration with ONAP DCAE VES

Table 12.3.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.3.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| eventListener | /eventListener | POST | Send notifications |

12.3.x.3.2	Resource definations

See [21]

12.3.x.4	Data type definitions

See clause 12.3.1.1.4.

End of 1st Change