3GPP TSG-SA5 Meeting #127	S5-196239

# Sophia Antipolis, 14th Oct 2019 – 18th Oct 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0063 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Netconf-based solution set for provisioning MnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-09-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ONAP Controller is a potential consumer of 3GPP provisioning MnS.It is required that a new NETCONF-based solution set be specified for provisioning MnS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new NETCONF-based solution set for provisioning MnS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12.1.X (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

1st Change

2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration of networks and network slicing; Provisioning; Stage 1".

[6]	3GPP TS 28.554: "Management and orchestration of networks and network slicing; 5G End to end Key Performance Indicators (KPI); performance measurements and assurance data".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS) "

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[x]	RFC 6241 “Network Configuration Protocol (NETCONF)”.

2nd Change

12.1.X. NETCONF-based solution set of provisioning

12.1.X.1 Mapping of operations

12.1.X.1.1 Introduction

The IS operations are mapped to SS equivalents according to table 12.1.X.1.1-1

Table 12.1.X.1.1-1: Mapping of 3GPP IS operations to SS equivalents [x]

| IS operation | SS operation | Qualifier |
| --- | --- | --- |
| createMOI | <edit-config> | M |
| getMOIAttributes | <get-config> <get> (note 1) | M |
| modifyMOIAttibutes | <edit-config> | M |
| deleteMOI | <edit-config> | M |

Note 1: If the configuration data is required, Netconf <get-config> operation should be used. The configuration and state information is required, Netconf <get> operation should be used.

12.1.X.1.2 Operation “createMOI”

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.2-1 and table 12.1.1X.1.2-2.

Table 12.1.X.1.2-1: Mapping from IS createMOI input parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| managedObjectClass | config | M | XML element’s name inside the <config> element. |
| managedObjectInstance | config | M | A sequence of embedded XML elements inside the <config> element. XML elements for all containing MOs and their ids(keys) shall be included together wilt the XML elements representing the to be created MO and its key. |
| referenceObjectInstance | config | O | Not supported. |
| attributeListIn | config | M | The key leaf, the “attributes container” and leaf, leaf-list or list entries of YANG models representing the attributes. |

Table 12.1.X.1.2-2: Mapping from IS createMOI output parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| attributeListOut | no corresponding SS paramete | M | Not supported. (note 1) |
| status | - | M | OperationSucceeded if NETCONF rpc-reply contains <ok> element. OperationFailed if NETCONF-reply contains <rpc-error>. |

Note 1: Successful Netconf <edit-config> operations only return an <ok> element. Therefore the attributeListOut can be retrieved via a separate <get-config> operation.

Example:

Create ManagedFunction=myFunction, GNBDUFunction=1

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<edit-config>

<target>

<running/>

</target>

<default-operation>none</default-operation>

<error-option>rollback-on-error</error-option>

<config>

<ManagedFunction>

<id>myFucntion</id>

<GNBDUFunction>

<id>1</id>

<attributeList>

<attribute 1>value</attribute 1>

<attribute 2>value</attribute 2>

</attributeList>

</GNBDUFunction>

</ManagedFunction>

</config>

</edit-config>

</rpc>

<!-- createMO Response -->

<rpc-reply message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<ok/>

</rpc-reply>

12.1.X.1.3 Operation "getMOIAttributes"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.3-1 and table 12.1.X.1.3-2.

Table 12.1.X.1.3-1: Mapping of IS getMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | filter | M | XML element’s name inside the <filter> element. |
| scope | filter | M | A sequence of embedded XML elements inside the <filter> element. XML elements for the levels of the containment hierarchy and the base object shall be included together wilt the XML elements selecting the MOs to be queried. |
| filter | filter | M | XML element inside the <filter> element describing the filter test to be applied to the selected MOs. |
| attributeListIn | filter | M | The key leaf, the “attributes container” and leaf, leaf-list or list entries of YANG models representing the attributes. |

Table 12.1.X.1.3-2: Mapping of IS getMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| managedObjectClass | data | M | XML element’s name inside the <config> element. |
| managedObjectInstance | data | M | A sequence of embedded XML elements inside the <filter> element. XML elements for all containing MOs and their ids(keys) shall be included together wilt the XML elements representing the to be queried MO and its key. |
| attributeListOut | data | M | The key leaf, the “attributes container” and leaf, leaf-list or list entries of YANG models representing the attributes. |
| status | data | M | OperationSucceeded if NETCONF rpc-reply contains <ok> element. OperationFailed if NETCONF-reply contains <rpc-error>. |

Example #1:

getMOIAttributes for base object ManagedFunction=myFunction, scope = BASE_ONLY, filter=none,  attributesListIn=empty is realized as the following <get-config> operation.

<!-- getMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<filter type=”subtree”>

<ManagedFunction>

<id>myFucntion</id>

<attributeListIn/>

<ManagedFunction>

<scope>

<base> BASE_ONLY </base>

<level/>

</scope>

</filter>

</get-config>

</rpc>

Example #2:

getMOIAttributes for base object ManagedFunction=myFunction, scope = BASE_ALL, filter=, GNBDUFunction=1,  attributesListIn=empty

<!-- getMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<filter type="subtree">

<ManagedFunction>

<id>myFucntion</id>

<GNBDUFunction>

<id> 1 </id>

</GNBDUFunction>

</ManagedFunction>

<scope>

<base> BASE_ALL </base>

<level/>

</scope>

</filter>

</get-config>

</rpc>

Example #3:

getMOIAttributes for base object ManagedFunction=myFunction, scope = BASE_ SUBTREE, level=2,  filter=none, attributesListIn=empty

<!-- getMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<filter type=”subtree”>

<ManagedFunction>

<id>myFucntion</id>

<attributeListIn/>

<ManagedFunction>

<scope>

<base> BASE_ SUBTREE </base>

<level> 2 </level>

</scope>

</filter>

</get-config>

</rpc>

Example #4:

getMOIAttributes for base object ManagedFunction=myFunction, scope = BASE_NTH_LEVEL, level=2,  filter=none, attributesListIn=empty

<!-- getMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<filter type=”subtree”>

<ManagedFunction>

<id>myFucntion</id>

<attributeListIn/>

<ManagedFunction>

<scope>

<base> BASE_ NTH_LEVEL </base>

<level> 2 </level>

</scope>

</filter>

</get-config>

</rpc>

12.1.X.1.4 Operation "modifyMOIAttributes"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.4-1 and table 12.1.X.1.4-2.

Table 12.1.X.1.4-1: Mapping of IS modifyMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | config | M | A sequence of embedded XML elements inside the <config> element. XML elements for all containing MOs and their ids(keys) shall be included together wilt the XML elements representing the to be modified MO and its key. |
| scope | config | M | A sequence of embedded XML elements inside the <filter> element. XML elements for the levels of the containment hierarchy and the base object shall be included together wilt the XML elements selecting the MOs to be modified. |
| filter | config | M | XML element inside the <filter> element describing the filter test to be applied to the selected MOs. |
| modificationList | config | M | The key leaf, the “attributes container” and leaf, leaf-list or list entries of YANG models representing the attributes. |

Table 12.1.X.1.4-2: Mapping of IS modifyMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| modificationListOut | no corresponding SS parameter | M | Not supported. (note 1) |
| status | - | M | OperationSucceeded if NETCONF rpc-reply contains <ok> element. OperationFailed if NETCONF-reply contains <rpc-error>. |

Note 1: Successful Netconf <edit-config> operations only return an <ok> element. Therefore the attributeListOut can be retrieved via a separate <get-config> operation.

12.1.X.1.5 Operation "deleteMOI"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.5-1 and table 12.1.X.1.5-2.

Table 12.1.X.1.5-1: Mapping of IS deleteMOI input parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| baseObjectInstance | config | M | A sequence of embedded XML elements inside the <config> element. XML elements for all containing MOs and their ids(keys) shall be included together wilt the XML elements representing the to be deleted MO and its key. |
| scope | config | M | A sequence of embedded XML elements inside the <filter> element. XML elements for the levels of the containment hierarchy and the base object shall be included together wilt the XML elements selecting the MOs to be deleted. |
| filter | config | M | XML element inside the <filter> element describing the filter test to be applied to the selected MOs. |

Table 12.1.X.1.5-2: Mapping of IS deleteMOI output parameters to SS equivalents

| IS operation parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| deletionList | no corresponding SS parameter | M | Not supported. (note 1) |
| status | - | M | OperationSucceeded if NETCONF rpc-reply contains <ok/> element. OperationFailed if NETCONF-reply contains <rpc-error>. |

Note 1: Successful Netconf <edit-config> operations only return an <ok> element. Therefore the attributeListOut can be retrieved via a separate <get-config> operation.