3GPP TSG-SA5 Meeting #127	S5-196295

# 14-18 October 2019, Sophia Antipolis, France

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0141 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct measurements on RTT delay between PSA UPF and UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The end to end delay in 5G networks between UE and PSA UPF has been introduced in SA5#126 meeting. According to agreed SA2 CR, the timers during measurement between PSA UPF and UE include: - T1: local time T1 that PSA UPF sent the DL monitoring packets; - T2: T2 for NG RAN at the reception of the DL; - T5: local time T5 for NG RAN the UL monitoring packets sent out in the UL monitoring packets to the PSA UPF; - T6: the local time T6 for PSA UPF when receiving the UL monitoring packets identified by the sequency number. Method for PSA UPF to caculate the UL/DL packet delay between UE and PSA UPF is that based on the received UL/DL packet delay results of Uu interface and UL/DL packet delay between RAN and PSA UPF. The method to measure RTT delay between PSA UPF and UE will be corrected by this CR for reason as following: - The T6-T1 would include the time for application software and middle ware in UE, then, this caculation is not the measurement agreed in SA2. This contribution is to correct this mesurement in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct measurements on round-trip delay between PSA UPF and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The round-trip delay between PSA UPF and UE is not same method agreed in SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.6.1, 5.4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.4.6.1	Average round-trip packet delay between PSA UPF and UE

a)	This measurement provides the average round-trip GTP packet delay between PSA UPF and UE. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are not time synchronised.

b)	DER (n=1).

c)	The measurement composed of received UL/DL packet delay results of Uu interface from NG RAN and UL/DL packet delay between RAN and PSA UPF.

UL/DL packet delay between RAN and PSA UPF is obtained by the following method:

For each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4]):

- 	T1 that the DL GTP PDU was sent by the PSA UPF;

- 	T2 that is for the NG RAN at the reception of the DL;

- 	T5 that is for NG RAN the UL monitoring packets sent out in the UL monitoring packets to the PSA UPF;

-	T6 that the corresponding UL GTP PDU (with QMP indicator and the corresponding sequence number) was received by the PSA UPF.

-	The 5QI and S-NSSAI associated to the DL GTP PDU.

The PSA UPF counts the number (N) of DL GTP PDUs encapsulated with QFI, TEID, sequence number and QMP indicator for each 5QI and each S-NSSAI respectively, and takes the following calculation for each 5QI and each S-NSSAI:

i=1NT6i-T1iN

i=1NT6i-T1i-(T5i-T2i)N

d)	Each measurement is a real representing the average delay in microseconds.

e)	GTP.RttDelayPsaUpfUeMean.5QI, where 5QI identifies the 5QI; 
GTP.RttDelayPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

5.4.6.2	Distribution of round-trip packet delay between PSA UPF and UE

a)	This measurement provides the distribution of round-trip GTP packet delay between PSA UPF and UE. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are not time synchronised.

b)	DER (n=1).

c)	The measurement is composed of received UL/DL packet delay results of Uu interface from NG RAN and UL/DL packet delay between RAN and PSA UPF.

UL/DL packet delay between RAN and PSA UPF is obtained by the following method: :

For each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4]):

- 	T1 that the DL GTP PDU was sent by the PSA UPF;

- 	T2 that is for the NG RAN at the reception of the DL;

- 	T5 that is for NG RAN the UL monitoring packets sent out in the UL monitoring packets to the PSA UPF;

-	T6 that the corresponding UL GTP PDU (with QMP indicator and the corresponding sequence number) was received by the PSA UPF.

-	The 5QI and S-NSSAI associated to the DL GTP PDU.

The PSA UPF 1) takes the following calculation for each DL GTP PDU (packet i) encapsulated with QFI, TEID, sequence number and QMP indicator for each 5QI and each S-NSSAI respectively, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

T6i-T1i

(T6i-T1i)-(T5i-T2i)

d)	Each measurement is an integer representing the number of DL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.RttDelayPsaUpfUeDist.5QI.Bin.DSCP, Where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.RttDelayPsaUpfUeDist.Bin.SNSSAI, Where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

End of Modified Sections