3GPP TSG-SA5 Meeting #127	S5-196309

# 14-18 October 2019, Sophia Antipolis, France

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0031 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add clarification of the NSI and NSSI performance assurance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The procedure of the NSI and NSSI performance assurance based on the slice QoE analytics provided by NWDAF has been approved during the SA5#126 meeting. However, there is no definition of SLS. Reference to latest version of TS 23.288 should be added for better understanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definition of SLS and reference to TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of SLS and analytics provided by NWDAF are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Service level specification: specification of the minimum acceptable standard of service. It may consisits of required network performance and service quality parameters which are included in SLA.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SLA	Service Level Agreement

SLS	Service Level Specification

2nd modified section

Annex H (normative): 
NSI and NSSI performance assurance

H.1	General

After a network slice instance is created, 3GPP management system creates all resources to the NSI and configurations to satisfy the network slice requirements, including the performance of NSI requested according to network slice SLS (as part of SLA). In order to guarantee the performance of network slice, NSI modification can be triggered by NSI provisioning service. During the NSI performance supervision, the end to end KPIs defined in TS 28.554 [21], performance measurements defined in TS 28.552[20] and optionally slice QoE provided by NWDAF as defined in clause 6.4.5 of 3GPP TS 23.288 [22] should be used for SLA fulfilment evaluation. According to the evaluation result, NSI modification defined in TS 23.531[8] can be triggered for network slice performance assurance.

H.2	Procedure of NSI and NSSI performance assurance

This Figure H.2-1 illustrates the procedure of performance assurance for NSI(s) or NSSI(s).

Figure H.2-1: Procedure for NSI and NSSI performance assurance

1.	The authorized consumer requests NSMS_Producer to allocate a new NSI. NSMS_Producer consumes provisioning services provided by NSSMS_Producer to create the NSI. After NSI allocation, the NSMS_Producer and NSSMS_Producer perform NSI and NSSI performance supervision.

2.	The NSMS_Producer or NSSMS_Producer may get slice QoE analytics provided by NWDAF [20].

3.	The NSMS_Producer or NSSMS_Producer checks whether the performance requirements can be met by NSI or NSSI by utilizing the end to end KPIs defined in TS 28.554 [21], performance measurements defined in TS 28.552[20] and slice QoE analytics provided by NWDAF in 3GPP TS 23.288 [22].

4a. If the performance requirements of NSI cannot be met, the NSMS_Producer triggers the NSI modification procedure. NSMS_Producer modifies the capacity of the network slice (e.g. supported user number), or modifies the network slice configuration to guarantee the performance requirements.

4b. If the performance requirements of NSSI cannot be met, the NSSMS_Producer of CN modifies virtualized resources and the configuration of 5GC NFs to guarantee the performance requirements. NSSMS_Producer of AN reconfigures RRMPolicy as defined in TS 28.541 [3] to optimize performance.

End of modification