3GPP TSG-SA5 Meeting #128	S5-197047

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0135 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR Charging procedure with I-SMF insertion,change,removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing I-SMF insertion,change,removal in PDU Session charging procedure for ETSUN scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PDU Session charging procedure with I-SMF insertion, change , removal triggers respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can’t support ETSUN PDU Session charging with I-SMF insertion, change, removal procedures . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.x.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.x.5	PDU Session modification procedure with I-SMF involvement

The following figure 5.2.2.x.5-1 describes PDU session charging message flows, for the case where

-	the UE moves from SMF service area to new I-SMF service area, a new I-SMF is inserted (i.e. I-SMF insertion); or

-	the UE moves from old I-SMF service area to new I-SMF service area, the I-SMF is changed (i.e. I-SMF change); or

-	the UE moves from I-SMF service area back to SMF service area, the I-SMF is removed (i.e. I-SMF removal).

with simplified UE with 5GS message flows, based on TS 23.502 figure 4.23.4.3-1, specify the corresponding charging message flow between SMF and CHF for I-SMF insertion, change, removal respectively:

Figure 5.2.2.x.5-1: PDU Session Charging message flow for UE Triggered Service Request procedure with I-SMF insertion/change/removal

0.ch.	PDU Session served by both I-SMF and SMF has been established, with the ongoing Charging session.

1. I-SMF insertion message flows, upon UE moves from SMF service area to I-SMF service area, finally with Nsmf_PDUSession_Create Response message sent from SMF to the inserted I-SMF (Detail message flows could refer to TS23.502,4.23.4.3-1 step 3 to 8c), or finally with Nsmf_PDUSession_Update Response message sent from SMF to the inserted I-SMF (Detail message flows could refer to TS23.502,4.23.4.3-1 step 18 to 20c

2.ch-a. All counts are closed in SMF and a Charging Data Request [Update] message with SMF information and usage report is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF with the inserted I-SMF information.

2.ch-b. The CHF updates CDR for this PDU session.

2.ch-c. The CHF acknowledges by sending Charging Data Response[Update] message to the SMF.

3. I-SMF Change message flows, upon UE moves from old SMF service area to new I-SMF service area, finally with Nsmf_PDUSession_Update Response message sent from SMF to the inserted new I-SMF (Detail message flows could refer to TS23.502,4.23.4.3-1 step 3 to 8c, or step 18 to 20c.

4.ch-a. All counts are closed in SMF with the old I-SMF, and a Charging Data Request [Update] message including the old I-SMF information and usage report is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF with the new I-SMF information.

4.ch-b. The CHF updates CDR for this PDU session.

4.ch-c. The CHF acknowledges by sending Charging Data Response[Update] message to the SMF.

5. I-SMF Removal message flows, upon UE moves from I-SMF service area back to SMF service area, finally SMF send Nsmf_PDUSession_CreateSMContext Response message to AMF (Detail message flows could refer to TS23.502,4.23.4.3-1 step 10 to 16) ,  or SMF send Nsmf_PDUSession_UpdateSMContext Response message to AMF (Detail message flows could refer to TS23.502, 4.23.4.3-1 step 22 to 25)

6.ch-a. All counts are closed in SMF with the I-SMF, and a Charging Data Request [Update] message including I-SMF information is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF.

6.ch-b. The CHF updates CDR for this PDU session.

6.ch-c. The CHF acknowledges by sending Charging Data Response[Update] message to the SMF.

End of change