3GPP TSG-SA5 Meeting # 128	S5-197356

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0514 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce description of volume based charging for VoLTE in PS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | VBCLTE |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PS domain charging does not support volume based charging for VoLTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce volume based charging for VoLTE in PS offline and online charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definition for volume based charging for VoLTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.X (new), 5.3.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.1.X	Volume Based Charging for VoLTE Services

Volume based charging for VoLTE services allows the P-GW to collect charging information for VoLTE service, categorised by rating group or combination of rating group and service id. This functionality is supported by the P-GW by the integration of a PCEF. In the P-GW, FBC (as described in clause 5.2.1.3) applies to the VoLTE service data flows. Details of this functionality are specified in TS 23.203 [215].

NOTE: 	Some VoLTE service specific charging information include in the PGW-CDR in VoLTE bearer specific offline charging: caller information, callee information, and status of VoLTE service delivery.

The following chargeable events are specified for volume based charging for VoLTE services:

-	Start of VoLTE bearer context. Upon encountering this event, a new CDR for this VoLTE bearer context is created and the data volume is captured for the VoLTE bearer context.

-	End of VoLTE bearer context. The VoLTE bearer context CDR is closed upon encountering this trigger.

-	Expiry of an operator configured time limit per VoLTE bearer context. This event closes the VoLTE bearer context CDR, and a new one is opened if the VoLTE bearer context is still active.

-	Expiry of an operator configured data volume limit per VoLTE bearer context. This event closes the VoLTE bearer context CDR, and a new one is opened if the VoLTE bearer context is still active.

-	Change of charging condition: tariff time change. When this event is encountered, the current volume count is captured and a new volume count is started.

-	Expiry of an operator configured change of charging condition limit per VoLTE bearer context. This event closes the VoLTE bearer context CDR, and a new one is opened if the VoLTE bearer context is still active.

Management intervention may also force trigger a chargeable event.

Second change

#### 5.3.1.X	Volume Based Charging for VoLTE

Volume based charging for VoLTE services allows the P-GW to collect charging information for VoLTE service, categorised by rating group or combination of rating group and service id. This functionality is supported by the P-GW by the integration of a PCEF. In the P-GW, FBC (as described in clause 5.3.1.2) applies to the VoLTE service data flows. Details of this functionality are specified in TS 23.203 [215].

NOTE: 	Some VoLTE service specific charging information include in P-GW online charging: caller information, callee information, and status of VoLTE service delivery.

End of changes