3GPP TSG-SA5 Meeting #128	S5-197622

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0199 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update JSON definitions of ServiceProfile NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MA5SLA |  |  |  |  |  | Date: |  |  | 2019-11-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on S5-195055 (LS from GSMA to SA5 on NG116 Publication and GST Co-operation with 3GPP SA5), it is proposed to update network slicing NRM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the JSON solution set for ServiceProfile to align with stage 2 informaton model definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of 3GPP NRM support for some attributes of GSMA GST. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | J.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR package (CR 0177, CR0198 and CR 0199) for TS 28.541. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

J.4.3	JSON schema "sliceNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP Network Slice NRM",

"version": "15.3.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP Network Slice NRM"

},

"paths": {},

"components": {

"schemas": {

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY"

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"Category": {

"type": "string",

"enum": [

"CHARACTER",

"SCALABILITY"

]

},

"Tagging": {

"type": "string",

"enum": [

"PERFORMANCE",

"FUNCTION",

"OPERATION"

]

},

"Exposure": {

"type": "string",

"enum": [

"API",

"KPI",

"APIandKPI"

]

},

"ServAttrCom": {

"type": "object",

"properties": {

"category": {

"$ref": "#/components/schemas/Category"

},

"tagging": {

"$ref": "#/components/schemas/Tagging"

},

"exposure": {

"$ref": "#/components/schemas/Exposure"

}

}

},

"Support": {

"type": "string",

"enum": [

"NOT SUPPORTED",

"SUPPORTED"

]

},

"DelayTolerance": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"support": {"$ref": "#/components/schemas/Support"}

}

},

"DeterministicComm": {

"type": "object",

"properties":

{

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"availability": {"$ref": "#/components/schemas/Support"},

"periodicityList":{"type":"string"}

}

},

"dLThptPerSlice": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"guaThpt": {"type": "float"},

"maxThpt": {"type": "float"}

}

},

"dLThptPerUE": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"guaThpt": {

"type": "float"

},

"maxThpt": {

"type": "float"

}

}

},

"uLThptPerSlic": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"guaThpt": {

"type": "float"

},

"maxThpt": {

"type": "float"

}

}

},

"uLThptPerUE": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"guaThpt": {

"type": "float"

},

"maxThpt": {

"type": "float"

}

}

},

"maxPktSize": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"maxsize": {

"type": "integer"

}}

}

},

"maxNumberofConns": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"nOofConn": {

"type": "integer"

}}

}

},

"kPIMonitoring": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"kPIList": {

"type": "string"

}}

}

},

"supportedAccessTech": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"accTechList": {

"type": "integer"

}}

}

},

"userMgmtOpen": {

"type": "object",

"properties":

{"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"support": {"$ref": "#/components/schemas/Support"}

}

},

"v2XModels": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"v2XMode": {"$ref": "#/components/schemas/Support"}

}

},

"termDensity": {

"type": "object",

"properties": {

"servAttrCom": {"$ref": "#/components/schemas/ServAttrCom"},

"density": {

"type": "integer"

}}

},

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

}

}

},

"NetworkSlice": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

},

{

"type": "object",

"properties": {

"networkSliceSubnetRef": {

"$ref": "genericNrm.json#/components/schemas/Dn"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"serviceProfileList": {

"$ref": "#/components/schemas/ServiceProfileList"

}

}

}

]

}

}

}

]

},

"NetworkSliceSubnet": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

},

{

"type": "object",

"properties": {

"managedFunctionRefList": {

"$ref": "genericNrm.json#/components/schemas/DnList"

},

"networkSliceSubnetRefList": {

"$ref": "genericNrm.json#/components/schemas/DnList"

},

"operationalState": {

"$ref": "genericNrm.json#/components/schemas/OperationalState"

},

"administrativeState": {

"$ref": "genericNrm.json#/components/schemas/AdministrativeState"

},

"nsInfo": {

"$ref": "#/components/schemas/NsInfo"

},

"sliceProfileList": {

"$ref": "#/components/schemas/SliceProfileList"

}

}

}

]

}

}

}

]

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "nRNrm.json#/components/schemas/SnssaiList"

},

"plmnIdList": {

"$ref": "nRNrm.json#/components/schemas/PlmnIdList"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "ngcNrm.json#/components/schemas/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"$ref": "#/components/schemas/MobilityLevel"

},

"sst": {

"$ref": "nrNrm.json#/components/schemas/Sst"

},

"resourceSharingLevel": {

"$ref": "#/components/schemas/SharingLevel"

},

"availability": {

"type": "number"

},

"delayTolerance": {

"$ref": "#/components/schemas/DelayTolerance"

},

"deterministicComm": {

"$ref": "#/components/schemas/DeterministicComm"

},

"dLThptPerSlice": {

"$ref": "#/components/schemas/DLThptPerSlice"

},

"dLThptPerUE": {

"$ref": "#/components/schemas/DLThptPerUE"

},

"uLThptPerSlic": {

"$ref": "#/components/schemas/ULThptPerSlic"

},

"uLThptPerUE": {

"$ref": "#/components/schemas/ULThptPerUE"

},

"maxPktSize": {

"$ref": "#/components/schemas/MaxPktSize"

},

"maxNumberofConns": {

"$ref": "#/components/schemas/MaxNumberofConns"

},

"kPIMonitoring": {

"$ref": "#/components/schemas/KPIMonitoring"

},

"supportedAccessTech": {

"$ref": "#/components/schemas/SupportedAccessTech"

},

"userMgmtOpen": {

"$ref": "#/components/schemas/UserMgmtOpen"

},

"v2XModels": {

"$ref": "#/components/schemas/V2XModels"

},

"coverageArea": {

"type": "string"

},

"termDensity": {

"$ref": "#/components/schemas/TermDensity"

},

"activityFactor": {

"type": "float"

},

"uESpeed": {

"type": "integer"

},

"jitter": {

"type": "integer"

},

"survivalTime": {

"type": "string"

},

"reliability": {

"type": "string"

}

}

},

"ServiceProfileList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ServiceProfile"

}

},

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"snssaiList": {

"$ref": "nRNrm.json#/components/schemas/SnssaiList"

},

"plmnIdList": {

"$ref": "nRNrm.json#/components/schemas/PlmnIdList"

},

"perfReq": {

"$ref": "#/components/schemas/PerfReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "ngcNrm.json#/components/schemas/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"$ref": "#/components/schemas/MobilityLevel"

},

"resourceSharingLevel": {

"$ref": "#/components/schemas/SharingLevel"

}

}

},

"SliceProfileList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SliceProfile"

}

}

}

}

End of change