3GPP TSG-SA5 Meeting #129e 	S5-201246

# e-meeting, 24 February – 4 March 2020

# Source:	Ericsson

# Title:	Addition of CSIF and CHF flows

# Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

Adding the following changes to TS 28.201

# 2	References

[1]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI).

# 3	Rationale

Addition of flows between CSIF and CHF based on TS 32.290 [1], for the interaction with NWDAF this interaction is still FFS.

# 4	Detailed proposal

First change

#### 5.2.2.2	Analytics and performance charging from CSIF

##### 5.2.2.2.1	General

The subclause below describes analytics and performance charging from CSIF scenarios.

##### 5.2.2.2.2	Analytic and performance reporting

The following figure 5.2.2.2.2 describes analytic and performance report charging:

Figure 5.2.2.2.2: Analytic and performance reporting

1)	Collection of resource usage information: Collection of information related to the network slice analytics and performance is done by the CSIF.

2)	Trigger condition met: the CSIF determines based on the collected information that a report to the CHF is needed.

3)	Charging Data Request [Event]: The CSIF generates charging data related to the collected information and sends the request for the CHF to store related charging data for CDR generation purpose.

4)	Account, Rating: The CHF may rate the event and check account based on the request.

5)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

6)	Charging Data Response [Events]: The CHF informs the CSIF on the result of the request.

Second change

#### 5.2.2.3	Network data analytics subscription from NWDAF

##### 5.2.2.3.1	General

The subclause below describes network data analytics subscription scenarios.

Editor’s note: The need for specific flows or if the flows in TS 23.288 can be refenced is FFS.

End of changes