# 3GPP TSG SA WG5 Meeting #129e	S5-201272

# Feburary 24-28, 2020, e-meeting	revision of S5-19abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0048 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UL user plane packet delay measurement from UE to PSA UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020/2/28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to provide performance assurance of 5G services, it is important to measure end-to-end user plane packet delay between PSA UPF and UE. Performance measurement at gNB level may not satisfy this requirement, Packet loss, delay, or latency can happen between gBN and UPF and their measurement thus is also very important. Performance measurements from UE to gNB and gNB to UPF are well defined respectively but those of end-to-end between UE and UPF is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements: - Average UL one way user plane data packet delay from UE to PSA UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL packet delay between PSA UPF and UE cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.4.x.y UL packet delay between PSA UPF and UE

5.4.x.y.1 Average UL user plane packet delay between PSA UPF and UE

- This measurement provides the average UL one way user plane data packet delay from UE to PSA UPF. It uses actual user planc service packets as monitoring packets.  This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

- DER (n=1)

- This measurement is obtained as:

The monitoring packet is sampled from the user plane packets transmitted from UE to PSA UPF based on the measurement period specified in the createMeasurementJob operation. To distinguish the GTP-U packets for this measurement from the ones delivering the normal user plane packets, a new payload type QMP indicator for QoS monitoring in GTP-U header (between UPF and gNB) and in SDAP/PDCP header (between gNB and UE) is used.

The sampling rate is defined as a measurement policy.  The overhead of measurement depends on the granularity of the sampling rate.  To minimize the overhead and maximize accuracy, we recommend to apply different sampling rates based on the user plane data traffic charactristics.  More granular sampling rate is applied for the user plane session of shorter session duration.  The specific sampling rate is out of scope and implementation issue with operational experiences.

For each sampled GTP PDU monitoring response packet (packet i), the PSA UPF and gNB perform the following tasks:

- 	When getting the UL packet delay result of Uu interface, gNB encapsulates QMP indicator in the GTP-U header of the monitoring response packet and sends it to the PSA UPF. gNB also includes the sequence number, UL one way packet delay result of Uu interface (TULuu), local time T3 when the response packet sent out in the response packet.

-	The PSA UPF records T4 when the monitoring response packet was received;

-	The PSA UPF records the 5QI and S-NSSAI associated to the GTP PDU.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each 5QI and each S-NSSAI respectively, and takes the following calculation for each 5QI and each S-NSSAI:

i=1N(T4i-T3i+TULuui)N

d)	Each measurement is a real representing the average delay in microseconds.

e)	GTP.DelayUlPsaUpfUeMean.5QI, where 5QI identifies the 5QI; 
GTP.DelayUlPsaUpfUeMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

Next modified section