3GPP TSG-SA5 Meeting #129-e	S5-201329

# Online, , 24th Feb 2020 - 4th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0201 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change of PSA2 for UL CL or BP controlled by I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing I-SMF controlled change of PSA and UL CL or BP procedure related charging message flow for local traffic offload ETSUN scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add I-SMF controlled local UPF and local PSA (PSA2) UPF information and local traffic offload usage report charging message flow, based on TS23.502 clause 4.23.9.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support local traffic offload charging upon change of PSA and UL CL or BP controlled by I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.14.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.14.x	Change of PSA and UL CL or BP controlled by I-SMF

This clause based on TS23.502 clause 4.23.9.3 describe a simplified change of UL-CL/BP function, e.g. new additional PDU Session Anchor (i.e. PSA2) and releases the existing additional PDU Session Anchor (i.e. PSA0), via modifying IPv6 multi-homing or UL CL rule in the same Branching Point or UL CL under controlled by the same I-SMF, with charging message flow:

Figure 5.2.2.x.8-1: Change of PSA and UL CL or BP controlled by I-SMF

1.	UE has an established PDU Session with a UPF including the PDU Session Anchor 1(controlled by SMF), and the PDU Session Anchor 0 and an I-UPF acting as UL CL or BP (controlled by I-SMF).

1.ch.	Ongoing Charging session already created between SMF and CHF, with I-SMF and the controlled UPF information available in CHF.

2~4.	At some point the I-SMF decides to establish a new PDU Session Anchor 2 and releases the existing old PDU Session Anchor 2 e.g. due to UE mobility. The I-SMF may then select a new UPF acting as new I-UPF and replace the existing I-UPF which also act as UL-CL/BP before, informed SMF in PDU Session message flows.

4.ch-a. All counts are closed in SMF with I-SMF controlled old PSA2 information, and a Charging Data Request [Update] message including I-SMF and usage report with PSA2 information is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF, with I-SMF controlled new PSA2 information.

4.ch-b. The CHF updates CDR for this PDU session.

4.ch-c. The CHF acknowledges by sending Charging Data Response [Update] message to the SMF.

End of change