3GPP TSG-SA5 Meeting #129-e	S5-201522

# Online, 24th Feb 2020 - 4th Mar 2020                                                              Revision of S5-201119

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0038 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update KPI definitions to align with the new template |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA-KPI |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The KPI template has been modified, so the KPI definitions need to be updated to align with the new template. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of KPIs to align with the new template. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPI definitions will not align with the template. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3, 6.2.4, 6.2.5, 6.3.1, 6.3.2, 6.3.3, 6.3.4, 6.3.5, 6.3.6.1, 6.4.1, 6.4.2, 6.4.3, 6.4.4, 6.5.1.1, 6.5.2.1, 6.6.1, 6.6.2, 6.6.3, 6.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

6	End to end KPI definitions

6.1	KPI Overview

The KPI categories defined in [2] will be reused by the present document.

6.2	Accessibility KPI

6.2.1	Registered Subscribers of Network and Network Slice Instance through AMF

a)	AMFRegNbr.

b)	This KPI describe the total number of subscribers that are registered to a network slice instance. It

is obtained by counting the subscribers in AMF that are registered to a network slice instance. It is an Interger. The KPI type is CUM.

c)

d)	SubNetwork, NetworkSlice

6.2.2	Registered Subscribers of Network and Network Slice Instance through UDM

a)	UDMRegNbr.

b)	This KPI describe the total number of subscribers that are registered to a network slice instance. It

is corresponding to the measurement RM.RegisteredSubUDMNbrMean that counts subscribers registered in UDM. It is an Interger. The KPI type is CUM.

c)

d)	SubNetwork, NetworkSlice

6.2.3	Registration success rate of one single network slice instance

a)	RegSR.

b)	This KPI describes the ratio of the number of successfully performed registration procedures to the number of attempted registration procedures for the AMF set which related to one single network slice instance and is used to evaluate accessibility provided by the end-to-end network slice instance and network performance. It

is obtained by successful registration procedures divided by attempted registration procedures. It is a percentage. The KPI type is RATIO.

c)

Note:	Above measurements with subcounter .Type should be defined in 3GPP TS 24.501 [4].

d)	NetworkSlice

6.2.4	DRB Accessibility for UE services

a)	DRBAccessibility

b)	This KPI describes the DRBs setup success rate, including the success rate for setting up RRC connection and NG signalling connection. It

is obtained as the succeess rate for RRC connection setup multiplied by the success rate for NG signalling connection setup multiplied by the success rate for DRB setup. The success rate for RRC connection setup and for NG signalling connection setup shall exclude setups with establishment cause mo-Signalling [5]. It is a percentage. The KPI type is RATIO.

c)	DRBAccessibility 5QI = (∑RRC.ConnEstabSucc.Cause/∑RRC.ConnEstabAtt.Cause) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.5QI/DRB.EstabAtt.5QI) * 100

DRBAccessibility SNSSAI = (∑RRC.ConnEstabSucc.Cause/∑RRC.ConnEstabAtt.Cause) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.SNSSAI/DRB.EstabAtt.SNSSAI) * 100.

Note that the sum over causes shall exclude the establishment cause mo-Signalling [5].

d)	NetworkSlice.

6.2.5	PDU session Establishment Success Rate of one network slice (S-NSSAI)

a)	PDUSessionEstSR.

b)	This KPI describes the ratio of the number of successful PDU session establishment request to the number of PDU session establishment request attempts for 5G network for the SMF which related to one network slice (S-NSSAI) and is used to evaluate accessibility provided by the end-to-end network slice and network performance. It

is obtained by the number of successful PDU session requests divided by the number of attempted PDU session requests. It is a percentage. The KPI type is RATIO.

c)	 PDUESR= SM.PduSessionCreationSuccNSI.SNSSAI SM.PduSessionCreationReqNSI.SNSSAI

d)	NetworkSlice

6.3	Integrity KPI

6.3.1	Latency of 5G Network

6.3.1.0	KPI categories

a)	E2ELatency.

b)	This KPI describes the end to end packet transmission latency through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network. It

is the RTT end to end latency of UE IP packets transmitted from UE to the N6 interface in the 5G network. The N6 interface is the reference point between UPF and DN. It is a time interval (millisecond). The KPI type is MEAN.

c)	NetworkSlice, SubNetwork

6.3.1.1	Downlink latency in gNB-DU

a)	DLLat_gNB-DU.

b)	This KPI describes the gNB-DU part of the packet transmission latency experienced by an end-user. It is used to evaluate the gNB latency contribution to the total packet latency. It

is the average (arithmetic mean) of the time from reception of IP packet to gNB-DU until transmission of first part of that packet over the air interface, for a packet arriving when there is no previous data in queue for transmission to the UE. It is a time interval (microsecond). The KPI type is MEAN.

c)	DLLat_gNB-DU = DRB.RlcSduLatencyDl or optionally DownlinkLat.QoSx = DRB.RlcSduLatencyDl.QoSx where QOS identifies the target quality of service class.

d)	SubNetwork

6.3.1.2	Integrated downlink delay in RAN

a)	DLLat_NR.

b)	This KPI describes the average packet transmission latency through the RAN part to the UE. It is used to evaluate latency performance of NG-RAN in downlink. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3). It

is the average packets latency transmitted from RAN to UE. It is a Time interval (microsecond). The KPI type is MEAN.

c)	DLLat_NR= DRB.PdcpSduDelayDl + DRB.PdcpF1Delay + DRB.RlcSduDelayDl + DRB.AirIfDelayDl.

d)	NetworkSlice, SubNetwork

e)	In non-split gNB scenario, the value of DRB.PdcpF1Delay is set to zero for there are no F1-interfaces in this scenario.

6.3.2	Upstream throughput for network and Network Slice Instance

a)	UpstreanThr

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance. It

is obtained by upstream throughput provided by N3 interface from NG-RAN to all UPFs which are related to the single network slice instance. The KPI unit is kbit/s and the KPI type is CUM.

c)

d)	NetworkSlice, SubNetwork.

6.3.3	Downstream throughput for Single Network Slice Instance

a)	DownstreamThr.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance. It

is obtained by downstream throughput provided by N3 interface from all UPFs to NG-RAN which are related to the single network slice instance. The KPI unit is kbit/s and the KPI type is CUM.

c)	DTSNSI= UPFGTP.OutDataOctN3UPF

d)	NetworkSlice.

6.3.4	Upstream Throughput at N3 interface

a)	N3UpstreamThr.

b)	This KPI describes the total number of octets of all incoming GTP data packets on the N3 interface (measured at UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate upstream GTP throughput integrity performance at the N3 interface. It

is obtained by measuring the GTP data upstream throughput provided by N3 interface from NG-RAN to UPF, during the granularity period.  The KPI unit is kbit/s and the KPI type is MEAN.

c)	UGTPTS=SUM (GTP.InDataOctN3UPF)/timeperiod) at UPF

d)	NetworkSlice

6.3.5	Downstream Throughput at N3 interface

a)	N3DownstreamThr.

b)	This KPI describes the total number of octets of all downstream GTP data packets on the N3 interface (transmitted downstream from UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate integrity performance at N3 interface. It

is obtained by measuring the GTP data downstream throughput provided by N3 interface from UPF to NG-RAN, during the granularity period. The KPI unit is kbit/s and the KPI type is MEAN.

c)	DGTPTS=SUM (GTP.OutDataOctN3UPF)/timeperiod) at UPF

d)	NetworkSlice

6.3.6	RAN UE Throughput

6.3.6.1	Definition

a)	UEDLThrp, UEULThrp.

b)	This KPI describes how NG-RAN impacts the service quality provided to an end-user.

It is obtained by payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface. The KPI unit is kbit/s and the KPI type is MEAN.

c)	RAN UE Throughput DL = DRB.UEThpDl and

RAN UE Throughput UL = DRB.UEThpUl

or optionally RAN UE Throughput DL for single mapped 5QI or QCI = DRB.UEThpDl.QoS and

RAN UE Throughput UL for single mapped 5QI or QCI = DRB.UEThpUl.QoS

d)	 SubNetwork

6.3.6.2	Extended definition

To achieve a Throughput measurement (below examples are given for DL) that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in figure 1).

Time (slots)

Data arrives to

empty DL buffer

First data is

transmitted to the UE

The send buffer is

again empty

calulations since it

can be impacted

by packet size of

User Plane (UP) packets.

ThpTimeDl

Failed transmission (

”

Block

error

”

)

Successful transmission,

buffer not empty

Successful transmission,

buffer empty

ThpVolDl =

∑

Total DL transferred volume =

∑

(kbits)

+

(kbits)

UE Throughput in DL =

ThpVolDl / ThpTimeDl (kbits/s)

No transmission, buffer not

empty (e.g. due to contention)

The last slot shall always be removed from

Figure 1

To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

6.4	Utilization KPI

6.4.1	Mean number of PDU sessions of network and network Slice Instance

a)	PDUSesMeanNbr.

b)	This KPI describes the mean number of PDU sessions that are successfully established in a network slice instance.

is obtained by successful PDU session establishment procedures of SMFs which is related to the network slice instance. It is an integer. The KPI type is MEAN.

c)	PDUSesMeanNbr=Sum (SM.SessionNbrMean.SNSSAI) over SMFs.

d)	NetworkSlice

6.4.2	Virtualised Resource Utilization of Network Slice Instance

a)	VirtualResUtilizaiton.

b)	This KPI describes utilization of virtualised resource (e.g. processor, memory, disk) that are allocated to a network slice instance. It is obtained by the usage of virtualised resource (e.g. processor, memory, disk) divided by the system capacity that allocated to the network slice instance. It is a percentage, The KPI type is Ratio.

Note: 	In the present document, this KPI is for the scenario when NF is not shared between different network slice instances.

c)

d)	NetworkSlice

6.4.3	PDU session establishment time of network slice

a)	PDUEstTime.

b)	This KPI describes the time of successful PDU session establishment which related to one single network slice and is used to evaluate utilization provided by the end-to-end network slice and network performance. It

is obtained by measuring the time between the receipt by SMF from AMF of " Nsmf_PDUSession_UpdateSMContext Request ", which includes N2 SM information received from (R)AN to the SMF and the sending of a " Nsmf_PDUSession_CreateSMContext Request or Nsmf_PDUSession_UpdateSMContext Request PDU Session Establishment Request " message from AMF to the SMF. It is a time interval (millisecond). The KPI type is MEAN.

c)	 PDUEstTime = SM.PduSessionTimeMean.SNSSAI

d)	NetworkSlice

6.4.4	Mean number of successful periodic registration updates of Single Network Slice

a)	RegUpdMeanNbr.

b)	This KPI describes the mean number of successfully periodic registration updates in a network slice at the AMF. It

is obtained by summing successful of periodic registration updates at the AMFs which is related to the network slice after registration accept by the AMF to the UE that sent the periodic registration update request. It is an integer. The KPI type is MEAN.

c)	RegUpdMeanNbr=Sum (AM.RegNbrMean.SNSSAI) over AMFs

d)	NetworkSlice

6.5	Retainability KPI

6.5.1	QoS flow Retainability

6.5.1.1	Definition

a)	QoSRetain_R1, QoSRetain_R2.

b)	This KPI shows how often an end-user abnormally loses a QoS flow during the time the QoS flow is used. It is obtained by

number of QoS flows with data in a buffer that was abnormally released, normalized with number of data session time units. The unit of this KPI is “active release / second”. The KPI type is MEAN.

c)	To measure QoS flow Retainability for a single QoS level (R1) is fairly straight forward.

R1QoS_x=QF.RelActNbr.QoSQoS_xQF.SessionTimeQoS.QoSQoS_x
However to measure the QoS flow Retainability for UEs is not as straight forward.  The measurement R1 is defined to look at the activity level of just one QoS level at the time, so to use this formula and measurements in an aggregated way to get QoS flow Retainability on UE level will not be accurate (e.g. for an UE with multiple QoS flows there might be QoS flows that are active at the same time, hence aggregating the QoS level measurements for session time will give a larger session time than the total UE session time. See picture below).


Hence a measurement QoS flow Retainability on UE level is defined (R2) to provide a measurement for the overall QoS flow Retainability.

R2=QoSQF.RelActNbr.QoSQF.SessionTimeUE

d)	SubNetwork, NetworkSlice

e)	The definition of the service provided by 5GS is QoS flows.

6.5.1.2	Extended definition

The retainability rate is defined as:

Active QoS flow Time

Number of abnormally released QoS flow with data in any of the buffers

[Releases/Session time]

As for defining an abnormal QoS flow release with end-user impact, it shall only be considered an abnormal release of the QoS flows if the NG-RAN considers there to be data waiting for transfer in any of the buffers.

As for defining a QoS flow as active, a QoS flow shall be considered active if there recently has been any data transmission in any direction.

6.5.2	DRB Retainability

6.5.2.1	Definition

a)	DRBRetain.

b)	This KPI shows how often an end-user abnormally loses a DRB during the time the DRB is active. It is obtained by

number of DRBs that were abnormally released and that were active at the time of release, normalized with number of data session time units. The unit of this KPI is “active release / second”. The KPI type is MEAN.

c)	DRB Retainability for a single mapped 5QI level (R1) and for a single S-NSSAI (R1) are defined as:

R15QI_x=DRB.RelActNbr.5QI5QI_xDRB.SessionTime.5QI5QI_x

and

R1SNSSAI_x=DRB.RelActNbr.SNSSAISNSSAI_xDRB.SessionTime.SNSSAISNSSAI_x

d)	SubNetwork, NetworkSlice

e)	The definition of the service provided by 5GS is DRBs.

6.5.2.2	Extended definition

To define (from a DRB Retainability point of view) if a DRB is considered active or not, the DRB can be divided into two groups:

-	For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms.

-	For DRBs with continuous flow, the DRB (and the UE) is always seen as being active in the context of this measurement, and the session time is increased from the first data transmission on the DRB until 100 ms after the last data transmission on the DRB.

A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

6.6	Mobility KPI

6.6.1	NG-RAN handover success rate

a)	NRHoSR.

b)	A KPI that shows how often a handover within NR-RAN is successful, regardless if the handover was made due to bad coverage or any other reason. This KPI is obtained by successful handovers to the same or another gNB divided by attempted handovers to the same or another gNB.

c)NGRANHOSR=MM.HoExeInterSucc+MM.HoExeIntraSuccMM.HoExeInterReq+MM.HoExeIntraReq×MM.HoPrepInterSucc+MM.HoPrepIntraSuccMM.HoPrepInterReq+MM.HoPrepIntraReq×100%

MM.HoExeInterSucc, MM.HoExeIntraSucc, MM.HoExeInterReq, MM.HoExeIntraReq, MM.HoPrepInterSucc, MM.HoPrepIntraSucc, MM.HoPrepInterReq and MM.HoPrepIntraReq.

d)	SubNetwork.

6.6.2	Mean Time of Inter-gNB handover Execution of Network Slice

a)	-	InterGNBHOMeanTime.

b)		This KPI describes the time of successful Mean Time of Inter-gNB handover which related to one single network slice and is used to evaluate utilization provided by the end-to-end network slice and network performance. This KPI is obtained by measuring the time between the receipt by the Source NG-RAN from the Target NG-RAN of a “Release Resource" and the sending of a " N2 Path Switch Request " message from Source NG-RAN to the Target NG-RAN over a granularity period. The unit of this KPI is millisecond.

c)

d)	NetworkSlice

6.6.3	Successful rate of mobility registration updates of Single Network Slice

a)	MobilityRegUpdateSR.

b)	This KPI describes the successful rate of mobility registration updates in a network slice e at the AMF. This KPI is obtained by deviding the number of successful mobility registration updates at the AMFs by number of mobility registration update requests received by the AMFs of single network slice.

MRUSR=RM.RegMobSucc.SNSSAIRM.RegMobReq.SNSSAI

c)

d)	NetworkSlice

6.7	Energy Efficiency (EE) KPI

6.7.1	NG-RAN data Energy Efficiency (EE)

6.7.1.1	Definition

a)	EE_MNDV.

b)	A KPI that shows mobile network data energy efficiency in operational NG-RAN. Data Volume (DV) divided by Energy Consumption (EC) of the considered network elements. The unit of this KPI is bit/J.

c)	EEMN,DV
=SamplesDRB. PdcpSduVolumnUl+DRB.PdcpSduVolumnDlSamplesPEE.Energy - for non-split gNBs;

=SamplesF1uPdcpSduVolumeUl+XnuPdcpSduVolumeUl+X2uPdcpSduVolumeUl+(F1uPdcpSduVolumeDl+XnuPdcpSduVolumeDl+X2uPdcpSduVolumeDl)SamplesPEE.Energy - for split-gNBs;

d)	SubNetwork

e)	The Data Volume (in kbits) is obtained by measuring amount of DL/UL PDCP SDU bits of the considered network elements over the measurement period. For split-gNBs, the Data Volume is calculated per Interface (F1-U, Xn-U, X2-U). The Energy Consumption (in kWh) is obtained by measuring the PEE.Energy of the considered network elements over the same period of time. The samples are aggregated at the NG-RAN node level. The 3GPP management system responsible for the management of the gNB (single or multiple vendor gNB) shall be able to collect PEE measurements data from all PNFs in the gNB, in the same way as the other PM measurements.

End of Modified Sections