3GPP TSG-SA5 Meeting #130e	S5-202108

20-28 April 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0217 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to DL packet delay in NG-RAN for network slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The DL packet delay in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU and the delay over Uu interface) is one significant part of the e2e delay of a Network Slice that has impact to users’ experience for some types of services (e.g., URLLC). The measurements on DL packet delay in NG-RAN for Network Slice are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements on DL packet delay in NG-RAN for Network Slice |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DL packet delay in NG-RAN for Network Slice cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.x	Delay measurements for Network Slice

### 6.x.1	Delay in NG-RAN for Network Slice

#### 6.x.1.1	Average DL delay in NG-RAN for Network Slice

a)	This measurement provides the average DL packet delay between NG-RAN and UE for a Network Slice, including the delay at gNB-CU-UP, on F1-U and on gNB-DU and the delay over Uu interface.

b)	CC.

c)	The MnS producer collects the following measurements of all the NR Cell CUs supporting the measured Network Slice identified by the S-NSSAI:

-	DRB.DelayDlNgranUeAve.SNSSAI, which is the average DL delay between NG-RAN and UE;

-	QosFlow.PdcpSduVolumeDl_Filter (where the filter contains S-NSSAI), which is the DL Cell PDCP SDU Data Volume for split gNBs or non-split gNBs.

The MnS producer calculates the measurements above-mentioned collected in the granularity period:

NR Cell CU(DRB.DelayDlNgranUeAve.SNSSAI*QosFlow.PdcpSduVolumeDl_Filter (S-NSSAI)) NR Cell CUQosFlow.PdcpSduVolumeDl_Filter (S-NSSAI)

d)	Each measurement is a real representing the average delay in microseconds.

e)	DRB.DelayDlSliceNgranUeAve.

f)	NetworkSlice.

g)	Valid for packet switched traffic.

h)	5GS.

#### 6.x.1.2	Distribution of DL delay in NG-RAN for Network Slice

a)	This measurement provides the distribution of DL packet delay between NG-RAN and UE for a Network Slice, including the delay at gNB-CU-UP, on F1-U and on gNB-DU and the delay over Uu interface.

b)	CC.

c)	The MnS producer collects the following measurements of all the NR Cell CUs supporting the measured Network Slice identified by the S-NSSAI:

-	DRB.DelayDlNgranUeDist.SNSSAI.Bin, which is the distribution of DL delay between NG-RAN and UE.

The MnS producer calculates the measurements above-mentioned collected in the granularity period for each bin indicating delay range:

NR Cell CUDRB.DelayDlNgranUeDist.SNSSAI.BinNR Cell CUBin(DRB.DelayDlNgranUeDist.SNSSAI.Bin)*100%

d)	Each measurement is real representing the percentage GTP PDUs measured with delay within the range of the bin.

e)	DRB.DelayDlSliceNgranUeAve.Bin, Where Bin indicates a delay range which is vendor specific.

f)	NetworkSlice.

g)	Valid for packet switched traffic.

h)	5GS.

Next Modified Sections

# A.x	Monitoring of delay in NG-RAN for Network Slice

A Network Slice may be used to support the delay a sensitive service, such as URLLC. The performance with regards to the delay of the Network Slice is very significant to the satisfaction of the end users. The delay in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU and the delay over Uu interface) takes up the main portion of the e2e delay of the Network Slice. Therefore the delay in NG-RAN needs to be monitored for the Network Slice.

As the performance in UL and DL differs, it is important to monitor the UL and DL user plane delay separately.

End of Modified Sections