3GPP TSG-SA5 Meeting #130-e	S5-202050

# Online, , 20th Apr 2020 - 28th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0102 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Remove subscribe and unsubscribe operation from ProvMnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, REST_SS |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Configurable subscriptions to notifications have been added to 3GPP TS 28.622 and 3GPP TS 28.623. For this reason dedicated subscribe and unsubscribe operations are not needed any more in the Provisioning MnS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The dedicated subscribe and unsubscribe operations are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple possibilities for subscribing and unsubscribing to receive notifications exist leading to problems when conflicting client requests are received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.5 (Void), 11.1.1.6 (Void), 12.1.1.1.1, 12.1.1.1.6 (Void), 12.1.1.1.7, (Void), 12.1.1.3.1, 12.1.1.3.2.2 (Void), 12.1.1.4.2.12 (Void), 12.1.1.4.2.13 (Void), 12.1.1.4.2.14 (Void) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-201390 |  |  |  |  |  |  |  |  |

First modification

#### 11.1.1.5	Void

#### 11.1.1.6	Void

Next modification

# 12	Management services – Stage 3

## 12.1	Generic provisioning management service

### 12.1.1	RESTful HTTP-based solution set

#### 12.1.1.1	Mapping of operations

##### 12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| createMOI | PUT | http://{URI-DN-prefix}/{root}/ProvMnS/{MnSVersion}/ {LDN-first-part}/{className}={id} | M |
| getMOIAttributes | GET | http://{URI-DN-PREFIX}/{root}/ProvMnS/{MnSVersion}/{LDN-first-part}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | http://{URI-DN-prefix}/{root}/ProvMnS/{MnSVersion}/{LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | http://{URI-DN-prefix}/{root}/ProvMnS/{MnSVersion}/{LDN-first-part}/{className}={id} | M |

Next modification

##### 12.1.1.1.6	Void

##### 12.1.1.1.7	Void

Next modification

#### 12.1.1.3	Resources

##### 12.1.1.3.1	Resource structure

Figure 12.1.1.3.1-1 shows the resource structure of the Provisioning MnS.

Figure 12.1.1.3.1-1: Resource URI structure of the Provisioning MnS

Table 12.1.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| MOI | /{className}={id} | PUT | Creates a resource representing a managed object instance |
| MOI | /{className}={id} | GET | Retrieves one or multiple resources representing managed object instances |
| MOI | /{className}={id} | PATCH | Modifies one or multiple resources representing managed object instances |
| MOI | /{className}={id} | DELETE | Deletes one or multiple resources representing managed object instances |
| notificationSink | /notificationSink | POST | Sends notifications |

Next modification

###### 12.1.1.3.2.2	Void

###### 12.1.1.3.2.3	Void

Next modification

###### 12.1.1.4.2.12	Void

###### 12.1.1.4.2.13	Void

###### 12.1.1.4.2.14	Void

End of modifications