3GPP TSG-SA5 Meeting #130e 	S5-202216

# e-meeting 20-28 April 2020

# Source:	Telefónica S.A.

# Title:	pCR 28.807 Neutral host in the management of PNI-NPN

# Document for:	Approval

Agenda Item:	6.6.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TS 28.807 1.0.0: Study on management aspects of non-public networks

# 3	Rationale

It is proposed to include discussion of management of Public Network Integrated NPN (PNI-NPN), when this NPN is supported by a neutral host. This constitutes an alternative approach to the “NPN supported by network slice instances of a PLMN” scenario.

# 4	Detailed proposal

This document proposes the following changes in TS 28.807 [1].

1st Change

## 4.3	Management of public network integrated NPN

### 4.3.2	NPN supported by a neutral host

A PNI-NPN can be made available by a 3rd party infrastructure provider taking the role of neutral host.

In the context of 3GPP, neutral host represents the owner of a facility that can be used to provide wholesale, mobile localized coverage solutions to multiple MNOs, each taking the role of hosted client. Examples of facilities that can be used for this purpose include venues like stadiums, shopping malls, museums and transportation hubs (e.g. airports, underground stations, railways, etc.).

The neutral host’s owned facility resources can be used for the provision of different PNI-NPNs to different MNOs. Every PNI-NPN can be seen as an end-to-end network composed of two differentiated segments: one public, consisting of network functions deployed using PLMN network resources; and one private, consisting of network functions built upon neutral host’s owned facility resources. The MNO becomes the PNI-NPN service provider, using the PNI-NPN to deliver communication services for non-public use.

From a management viewpoint, the following issues apply to the neutral host:

- The set of neutral host’s managed resources include the neutral host’s owned facility resources and the functions built on top of them.

NOTE 1: Examples of these resources can include RAN nodes (e.g. gNB, eNB, etc.) and/or non-3GPP access nodes (e.g. WiFi access points).

- The set of neutral host’s managed resources collectively act a shared platform to the hosted PLMN operators.

NOTE 2: The neutrality aspect in this context does not imply strict equality between hosted PLMN operators, as the resources offered to each PLMN operator are subjected to commercial agreement between the neutral host and the PLMN operator, and policy-based management maybe applied.

- The neutral host can rely on network slicing mechanisms to make offered resources available to each hosted PLMN operator.

End of change