3GPP TSG-SA5 Meeting #130e 	S5-202322

# e-meeting 20-28 April 2020

# Source:	Huawei

# Title:	pCR TS 28.313 clarification on RAN-related discriptions of SON management service

# Document for:	Approval

Agenda Item:	6.4.9

1	Decision/action requested

The group is asked to discuss and approve.

2	References

[1]             3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage 2".

3	Rationale

TS 38.300[1] describes the Automatic Neighbour Cell Relation Function. SON cases including MRO and RACH optimization are being discussed in RAN3. In order to align with RAN cases, this contribution propose to add RAN-related reference descriptions.

4	Detailed proposal

1st modified section

7	Management services for SON

Editor’s note: this clause contains the management services for SON and definition of information, such as targets, control, parameters, and PM measurements that are needed to support SON functions.

7.1	Management services for D-SON management

7.1.1	RACH Optimization (Random Access Optimisation)

7.1.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

7.1.2.1	MnS Component Type B definition

7.1.2.1.1	Targets information

The targets of RACH optimization are shown in Table 7.1.2.1.1-1.

Table 7.1.2.1.1-1.  RACH optimization targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| UE access delay probability | The probability distribution of UE access delay that is used to minimize the access delays for the UEs under the coverage of popular SSBs. | CDF of access delay |

Editor's Note:	The definition of targets to support the following targets needs clarification from RAN3.

-  Minimize the delays for the UEs to request the other SIs

-  Minimize the imbalance of UEs access delays on uplink (UL) and supplementary uplink (SUL) channel

-  Minimize the beam failure recovery delays for the UEs in RRC_Connected.

-  Minimize the failed/unnecessary RACH attempts on RACH resource before success.

7.1.2.1.2	Control information

The parameter is used to control the RACH optimization function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| RACH optimization control | This attribute allows authorized consumer to enable/disable the RACH optimization functionality. See attribute rachOptimizationControl in TS 28.541 [13]. | Boolean On, off |

7.1.2.1.3	Parameters to be updated

7.1.1.3	MnS Component Type C definition

7.1.1.3.1	Performance measurements

Performance measurements related to the RACH optimization are captured in Table 7.1.1.3.1-1:

Table 7.1.1.3.1-1.  RACH optimization related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Distribution of RACH preambles sent | Distribution of the number of preambles UEs sent to achieve synchronization, where the number of preambles sent corresponds to PREAMBLE_TRANSMISSION_COUNTER (see clause 5.1.1 in TS 38.321 [4]) in UE. | UE access delay probability |
| Distribution of UEs access delay | Distribution of the time needed for UEs to successfully attach to the network. | UE access delay probability |

7.1.2	MRO (Mobility Robustness Optimisation)

7.1.2.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

7.1.2.2	MnS Component Type B definition

7.1.2.2.1	Targets information

The targets of MRO are shown in the Table 7.1.2.2.1-1.

Table 7.1.2.2.1-1.  MRO targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total intra-RAT handover failure rate | (the number of failure events related to intra-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total inter-RAT handover failure rate | (the number of failure events related to inter-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |

7.1.2.2.2	Control information

The parameter is used to control the MRO function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| MRO function control | This attribute allows the operator to enable/disable the MRO functionality. See attribute mroControl in TS 28.541 [13]. | Boolean On, off |

7.1.2.2.3	Parameters to be updated

7.1.2.3	MnS Component Type C definition

7.1.2.3.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.3.1-1.  MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover eventsl | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes. | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes. | Total inter-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell. | Imbalance of UE access delay on UL and SUL probability |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure. | Total intra-RAT handover failure rate |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time. | Total intra-RAT handover failure rate |
| Number of inter-RAT too late handover failures | Deteccted when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC. | Total inter-RAT handover failure rate |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE. | Total inter-RAT handover failure rate |

7.1.3	PCI configuration

7.1.3.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |

7.1.3.2	MnS Component Type B definition

7.1.3.2.1	Control information

The parameter is used to control the D-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the D-SON PCI configuration functionality. | enable, disable |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI configuration control.

7.1.3.2.2	Parameters to be updated

The table below lists the parameter related to the D-SON PCI configuration function.

| Parameters | Definition | Legal Values |
| --- | --- | --- |
| PCI list | The list of PCI values to be used by D-SON PCI configuration function to assign the PCI for NR cells. | List of integers |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI list.

7.1.3.3	MnS Component Type C definition

7.1.3.3.1	Alarm information

The table below lists the alarms related to D-SON PCI configuration,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add new alarms or modify existing alarms in TS 28.545 or TS 32.111-2) to support PCI collision and confusion.

7.1.4	ANR management

This management service is used for management of ANR, and ANR is specified in TS 38.300 [7], clauses 15.3.3.

Stage 2 for ANR management is located in TS 28.541 [13], clauses 4.3.2.2, 4.3.2.3, 4.3.32.2 and 4.3.32.3.

Stage 3 for ANR management is located in TS 28.541 [13], clauses C.4.3, D.4.3, and E.5.

7.2	Management services for C-SON

7.2.1	PCI configuration

7.2.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 11.1.1 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.2.1 of TS 28.532 [3]: - notifyNewAlarm - notifyClearedAlarm | It is supported by Fault Supervision MnS for NF, as defined in TS 28.545 [10]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

7.2.1.2	MnS Component Type B definition

7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. | disable, enable |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI configuration control.

7.2.1.3	MnS Component Type C definition

7.2.1.3.1	Alarm information

The table below lists the alarms related to PCI configuration are generated from the NR cells,

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI collision alarm | The collision alarm is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion alarm | The confusion alarm is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Editor’s Note: need to consider ways (e.g. add a new attribute or modify an existing attribute in 28.541) to support PCI list.

7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1.  PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| PCI of candidate cells | The measurement contains cumulative counter with subcounters that is identified by the PCI value(s) of the candidate cells, and is derived from MeasResultListNR (see clause 6.3.2 in TS 38.331 [9]) where it contains PCI in PhysCellId, and RSRP/RSRQ in MeasQuantityResults of candidate cells. It is generated when the RSRP received from the candidate cells exceeds certain thresholds. |  |

Editor’s Note: Performance measurement needs to be defined in the TS 28.552 [5].

End of modified section