3GPP TSG-SA5 Meeting #131e 	S5-203115

- e-meeting 25th May-3rd June 2020

- Source:	Huawei

- Title:	trigger addition for CSIF based architecture.

- Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This is a pCR to TS 28.202 to add applicable trigger, for CSIF based architecture.

# 2	References

[1]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to add applicable trigger, for the CSIF based architecture.

# 4	Detailed proposal

First change

#### 5.2.1.2	Applicable triggers in Network Slice Management charging

A set of trigger conditions are defined for the MnS producer to invoke a Charging Data Request [Event] towards the CHF.

Table 5.2.1.2.1 summarizes the set of default trigger conditions and their category which shall be supported by the MnS producer.

Table 5.2.1.2.1: Default Trigger conditions in MnS producer

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| NSI Allocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Modification Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Deallocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
|  |  |  | Not Applicable |  | Charging Data Request [Event] |

Table 5.2.1.2.2 summarizes the set of default trigger conditions and their category which shall be supported by the eCTF/CSIF.

Table 5.2.1.2.2: Default Trigger conditions in eCTF/CSIF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Receiving Notification event | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the MnS producer (CTF) or eCTF/CSIF for these chargeable events, is specified in clause 5.2.3.

Editor's Note: the relationship between the eCTF and CSIF as well as naming are ffs

End of change