3GPP TSG-SA5 Meeting #131e 	S5-203135

- e-meeting 25th May-3rd June 2020

- Source:	Nokia

- Title:	Enahncing the Handover optimization use case in subclause 6.5

- Document for:	Approval

Agenda Item:	6.6.4

# 1	Decision/action requested

Please approve the propose to update of the use case and the corresponding requirements.

# 2	References

[1]	3GPP TR 28.809 Study on enhancements of Management Data Analytics

# 3	Rationale

This contribution enhances the current Handover optimization use case regarding mobility management reated issues in [1] highlighting the resource provision and/or selection of a particular target gNB considering service requirements and network conditions.

# 4	Detailed proposal

1st Modified Section

## 6.5 Mobility management related issues

### 6.5.1	Handover optimization

#### 6.5.1.1	Use case

Current handover procedures are mainly based on radio conditions  for selecting the target gNB upon a handover. The target gNB accepts or rejects the handover (HO) request depending on various conditions. In virtualized environment, the HO may be rejected due to inadequate available resources within the target gNB. The notion of resources may include virtual resources (e.g., compute, memory) and/or radio resources (e.g., PRB, RRC connected users). If the HO request is rejected, a UE will try to connect to a different gNB until the request is successfully accepted. Several target gNBs can be tried until the request is successfully accepted. This process can result in wastage of UE and network resources, while it may also introduce service disruption due to increased latency and radio link failures (RLFs). It also introduces inefficiency in the HO or other network procedures.

To address this handover optimization issue, it is desirable to use MDAS (Management data analytic service) to provision and/or select a particular target gNB for handover in order to reduce or even avoid HO rejections.

The MDAS producer provides a HO optimization analytics report containing the current and future/predicted resource consumption, network capabilities and other KPIs’ status with respect to a specific service or slice as well as considering the expected QoE or QoE statistics for the target gNB. The analytics report also provides recommended actions to optimize the resource configuration or the selection of target gNB to accommodate the handover. Based on the report, the MDAS consumer adjusts (e.g., scale-out/up the virtual resource, re-schedule/optimize radio resource) the resources before continuing with the handover and/or adjusts the selection of the target gNB by also considering the overlapping coverages of inter-frequency and inter-RAT deployments.

#### 6.5.1.2	Potential requirements

REQ-HO_OPT_CON-1		The MDAS producer should have a capability to provide the analytics report describing the resource consumption to authorized consumers based on the current and future virtual resource consumption of gNB.

REQ-HO_OPT_CON-2		The MDAS producer should have a capability to provide the analytics report describing the resource consumption to authorized consumers based on the current and future radio resource consumption of gNB.

REQ-HO_OPT_CON-3		The analytics report describing the resource consumption should contain the following information describing the current and future resource consumption:

-	Assigned virtual, radio, and transport resources for target gNB.

-	Consumed virtual, radio, and transport resources for target gNB.

-	Projected virtual, radio and transport resource usage in near future for target gNB.

-	Indication on whether the target gNB is optimal for handover.

-	Indication of the QoE at the UE  or at the target gNB.

-	Recommended action to optimize the target gNB and/or the selection of the target gNB for handover, for example, with respect to a specific service or slice.

REQ-HO_OPT_CON-4		The MDAS producer should have a capability to provide an analytics report indicating a selection priority, i.e. mobility ranking, for the target cell, among a set of candidate inter-frequency cells.

REQ-HO_OPT_CON-5		The MDAS producer should have a capability to provide an analytics report indicating a list of target cells to spare, i.e. avoid, a handover for an indicated time period.

REQ-HO_OPT_CON-6		The analytics report describing inter-frequency target cell selection for handover should provide an analytics report for provisioning or selecting a target gNB with respect to a specific service or slice.

REQ-HO_OPT_CON-7		The analytics report describing inter-frequency target cell selection for handover should provide an analytics report for provisioning or selecting a target gNB with respect to QoE targets.

#### 6.5.1.3	Possible solutions

TBD

End of Modifications