3GPP TSG-SA5 Meeting #131e 	S5-203147

e-meeting 25th May-3rd June 2020 	(revision of xx-yyxxxx)

- Source:	Huawei

- Title:	Clarify fault management use case

- Document for:	Approval

Agenda Item:	6.6.4

# 1	Decision/action requested

The group is asked to approve the proposal.

# 2	References

[1]	3GPP TR 28.809: “Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

MDA can provide analytics report for both RAN and 5GC domain or both as described in clause 5.2. The current fault management use case is not clear whether it is a RAN domain use case or a cross domain use case. This contribution suggests to update the fault management use case to cover both domain specifc and cross domain secnarios for MDAS.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 6.4	Fault management related issues

### 6.4.1	Alarm incident analysis

#### 6.4.1.1	Use case

In 5G network, millions of alarms are generated due to the network complexity. Since the topological relations between different network elements and logical relations between different generated alarms, a series of alarms caused by a same root cause should be correlated with each other. In addition, the same root causes may give rise to the network performance deterioration. For example, an alarm in a lower layer of the communication protocol stack may cause multiple alarms in higher layers. Sequence of alarms may be generated in multiple domains along a communication link if one fault in the source domain occurs.

Large amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of same root cause should be correlated and analysed. Some ML models and algorithms may be used to group or filter the correlated alarms and indicate the root cause. Also, the historical alarms, performance measurements and network topology data can be utilized as the foreknowledge.

To improve the efficiency of network operation and maintenance, the MDAS producer is able to provide the analytics result including the root alarm or root cause by correlate and group the related alarms and performance measurements into an alarm incident.

#### 6.4.1.2	Potential requirements

REQ-ALARM_MDA-01:	The MDAS producer shall have a capability to provide the analytics report describing the alarm incident analysis.

REQ-ALARM_MDA-02:	The analytics report describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms, performance measurements

-	The start time and end time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level

-	Affected objects

-	Recommended actions

#### 6.4.1.3	Possible Solutions

##### 6.4.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to identify the alarm incident. The required data can be from RAN domain or CN domain or both. As the table in 6.4.1.3.34shows, the analytics report is able to be provided by the MDAS producer to describe the root causes and recommendations of identified alarm incident. It can be a domain specific or cross domain analytics report. This procedure may be triggered by the request or periodically.

##### 6.4.1.3.2	Data required for alarm incident analysis for RAN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for RAN domain used to construct the alarm incident.

Table 6.4.1.3.2-1: Potential data required for alarm incident analysis

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of NG-U, the alarm of radio frequency unit, the alarm of cell outage. The types of alarms are FFS. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; PDCP Data Volume: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; TB related measurements: The TB transmitted in a cell, see clause 5.1.1.7 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Inter-gNB handovers: Number of requested/successful handover/failed handover preparations/resource allocations/executions, see clause 5.1.1.6.1 of TS 28.552 [8]. Intra-gNB handovers: Number of requested/successful handover executions, see clause 5.1.1.6.2 of TS 28.552 [8]. RRC connection establishment related measurements: Attempted/ successful RRC connection establishments, see clause 5.1.1.15 of TS 28.552 [8]. RRC connection Re-establishment and RRC connection Resuming related measurement, see clause 5.1.1.17 and 5.1.1.18 of TS 28.552 [8]. Packet loss rate, Packet drop rate and Packet delay including the average delay and distribution of delay in CU-UP, F1-U, gNB-DU, CU-UP both of downlink and uplink. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |

Editor’s Notes: The data above may not be the complete list for alarm incident detection, other types of management data may also be utilized.

##### 6.4.1.3.3	Data required for alarm incident analysis for CN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for CN domain used to construct the alarm incident.

Table 6.4.1.3.3-1: Potential data required for alarm incident analysis

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of AMF, the alarm of UPF. The types of alarms are FFS. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. Mobility related measurements for AMF: Number of requrested/failed PDU sessions/QoS flows for inter-AMF handovers and number of attempted/successful/failed handover between 5GS and EPC, see clause 5.2.5.1, 5.2.5.3 and 5.2.5.4 of TS 28.552 [8]. Session related measurements for SMF: Number of successful/failed PDU session creation and number of requested/successful/failed PDU session modifications, see clause 5.3.1 of TS 28.552 [8]. QoS flow monitoring for SMF: Number of requested/successful/failed QoS flows to create/modify, see clause 5.3.2 of TS 28.552 [8]. Session related measurements for UPF: Number of requested/failed N4 session establishments, see clause 5.4.3.1 of TS 28.552 [8]. Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |

Editor’s Notes: The data above may not be the complete list for alarm incident detection, other types of management data may also be utilized.

##### 6.4.1.3.4	Analytics report for alarm incident analysis

Following table provides the potential information of the domain specific or cross domain analytics report for alarm incident analysis based on the required data received as described in 6.4.1.3.2 and 6.4.1.3.3.

| Analytics Report of alarm incident | Information | Description |
| --- | --- | --- |
|  | Alarm Incident Identifier | The alarm incident id or name for correlated alarms and performance measurements, e.g., NgU transmission alarm incident, Xn transmission alarm incident. |
|  | List of Correlated AlarmInfo | List of Alarm name or alarm ID, e.g., alarm of NgU setup, alarm of user plane link failure, alarm of user plane failure, alarm of cell outage. |
|  | List of Correlated performance measurements info | Performance measurements and the corresponding value, e.g., NgU handover failure rate, NgU setup failure rate. |
|  | Location | The geographical area or the cells where the alarm incident exists |
|  | Start Time | The start time of alarm incident |
|  | Stop Time | The end time of alarm incident |
|  | Affected objects | The MOIs, e.g., the MOIs of cells or subnetworks or network slices affected by the alarm incident |
|  | Root cause or Root alarm | Root alarm identified or predicted by root cause decision model, e.g. alarm of NgU setup |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident |
|  | Recommended actions | The recommend actions to clear the alarm incident. The recommended actions could be to replace the hardware unit, reconfigure the protocol, e.g., Xn application protocol |

End of changes